3GPP TSG-RAN WG2 Meeting #128	R2-2410594

# Orlando, USA, Nov. 18th – 22nd, 2024

Agenda item:		8.2.2 (FS_Ambient_IoT_solutions)

Source:	LG Electronics Inc.

Title: 	Discussion on functionality aspects of ambient IoT

Document for:	Discussion and Decision

1.	Introduction

This document discusses functionality of ambient IoT (A-IoT). In this contribution, we discuss mainly energy status indication and segmentation. The related agreements in R2-127bis meeting are as follows.

Agreements (AS ID) - RAN2 assumes that if “AS ID” is defined it is used at least for purpose of D2R scheduling and R2D reception. Up to RAN1 to decide whether a “AS ID” is defined. - RAN2 assumes this “AS ID” should be a short AS layer ID, rather than the full upper layer device ID. FFS on the length. FFS if AS ID can be based on partial upper layer device ID. - From RAN2 perspective, following options are possible for “AS ID”: Option 1: a random ID if used in Msg1 can be reused; Option 2: reader assigns this “AS ID”. FFS via which R2D message. - RAN2 will aim to define one common design for all RA procedures (if technically possible) Agreements (Energy status indication) - Capture in the TR the option: the device may include energy status indication in D2R messages (e.g. MSG1,MSG3 and Command Response message), if the device can. 1 bit indication can be captured in the TR. We will capture the use case in the TR. FFS whether it is reader controlled. Agreements (Need of MSG size) - From RAN2 perspective, it is beneficial for the reader to know an estimate of expected D2R message size. Two options can be captured: 1) from the CN and 2) from the device (simple message size indication). Capture the advantages/disadvantages. - The D2R message size would be beneficial but it is not essential. - Ask SA2 if it is possible to provide the expected (e.g. approximate, estimate, exact, max) future response D2R message size. Is it always available, sometimes, or never. Agreements (Segmentation) 1 RAN2 will study segmentation and RAN2 will need to give some guidance once there is more clarity on solution, on the minimum TB size. 2 Study the functionality of segmentation in the MAC. We focus on D2R and can consider the support for R2D later depending on SA2 and RAN1. The solution should aim to be applicable to both direction (i.e. unified solution), if we decide to support on both directions. 3 Do not support sequence number for A-IoT segmentation functionalities. 4 Do not support segment number and the number of segments for A-IoT segmentation functionalities. 5 Capture the following option in TR: An indication is used to indicate to reader whether the data is segmented and whether it is last segment. FFS the details on how this is designed and number of bits (one or two). 6 RAN2 assumes that for the device we will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all buffered segment(s) are stored in upper layer(s).

2.	Discussion

Energy status indication

An A-IoT device may indicate its energy status to the reader in case the device does not have energy for the follow up messages. The indication may be included in D2R messages (e.g., MSG1, MSG3 and command response message). Based on the energy status indication, the reader is aware of device’s need of additional energy harvesting and may allocate a time duration for device’s energy harvesting. The device performs energy harvesting during the duration. After that, the reader and the device are able to perform data communication successfully. Figure 1 shows an example scenario for use of energy status indication assuming a CFRA for a designated device.

Figure 1 Example for use of simple energy status indication in a CFRA case

In a CBRA case, when a device includes energy status indication in MSG1 transmission, the reader is not able to identify the device if the device ID is not included in the received MSG1. In this case, the reader may send an additional subsequent paging after the expected duration for device’s energy harvesting or may defer transmission of a subsequent paging until the device is charged enough.

When a device includes energy status indication in MSG3 or a command response message, the reader is generally able to identify the device because the device ID is included in MSG3 or AS ID is expected to be assigned. In this case, the reader may trigger the device by a paging for the designated device after the expected duration for device’s energy harvesting.

In the above-mentioned use cases, a device includes energy status indication in a D2R message based on its own decision. The reader is not in control of device’s energy status indication. The reader may just take the indication into account, for example, for adjusting time points of triggering devices to let devices harvest energy.

Proposal 1. An A-IoT device includes energy status indication in a D2R message based on its own decision. i.e., The reader is not in control of device’s energy status indication.

Segmentation

In the previous meeting (R2-127bis meeting), it is agreed that RAN2 will study segmentation. For A-IoT segmentation functionality, sequence number and segment number are not supported for a simple segmentation operation. To indicate to reader whether the data is segmented or not and whether it is the last segment or not, an indication is used. How this is designed and number of bits (one or two) need to be further discussed. In case of other quite complicated protocol, 2-bit segmentation indication is usually used. For example, segmentation info (SI) field of NR RLC is 2-bit long. For A-IoT segmentation functionality, transmissions of segments belonging to a message is expected to be performed in a stop and wait protocol considering that sequence number and segment number are not supported. If a stop and wait protocol is used, segments belonging to the same message are transmitted back-to-back and 1-bit indication would be enough.

Proposal 2. 1-bit segmentation indication is used for A-IoT segmentation functionality assuming that a stop and wait protocol is used for transmissions of segments.

To handle failure of segment transmission, an immediate retransmission by reader control may be considered. However, we think, considering that segment number is not supported, retransmission is effective only for limited failure cases. It does not handle various failure cases. For example, as shown in Figure 2, if R2D transmission to trigger the second segment (i.e., segment 2) transmission is lost after successful the first segment transmission and R2D transmission to trigger the retransmission is received on the device, the device will transmit the first segment (i.e., segment 1) again instead of transmitting the second segment (i.e., segment 2). Since the reader expects transmission of segment 2, the received segment (i.e., segment 1) will be placed next to segment 1 instead of being discarded because segment number is not supported and there is no way to detect duplication. It will cause that the message is not correctly re-assembled.

Figure 2 A failure of A-IoT segmentation operation

To avoid incorrectly handling failure of segment transmission, when a segment transmission failure happens, the message transmission procedure is stopped and re-access is triggered to recover the failure.

Proposal 3. When a segment transmission failure happens, the procedure is stopped and re-access is triggered instead of an immediate retransmission.

3.	Conclusion

This document discusses energy status indication and segmentation.

Proposal 1. An A-IoT device includes energy status indication in a D2R message based on its own decision. i.e., The reader is not in control of device’s energy status indication.

Proposal 2. 1-bit segmentation indication is used for A-IoT segmentation functionality assuming that a stop and wait protocol is used for transmissions of segments.

Proposal 3. When a segment transmission failure happens, the procedure is stopped and re-access is triggered instead of an immediate retransmission.