3GPP TSG-RAN WG2 Meeting #128	R2-2410601

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.1.5

Source:	T-Mobile USA, Nokia

Title:	Requirements for Model Transfer/Delivery

WID/SID:	NR_AIML_air-Core – Release 19

Document for:	Discussion and Decision

# 1	Introduction

This paper will discuss requirements for model transfer/delivery and some assumptions that could help guide future discussion on solutions and down-scoping of solutions for model transfer/delivery.

# 2	Discussion

### 2.1	Study Outcomes

Four RAN impacts of model transfer/delivery were discussed and compared between different model transfer/delivery solutions, which included control plane (CP) and user plane (UP) based RAN, core network (CN) and location management function (LMF) approaches and OTT and OAM based approaches, both assumed to be UP based. The discussion areas, A1 – A4 are below.

- A1 – Large, no upper limit model/model parameter size

- A2 – Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs

- A3 – Network controllability on model transfer/delivery and management at gNB

- A4 – Model transfer/delivery QoS (for DRB) including latency, etc.) and priority (for SRB)

The requirements and assumptions in the next section are directly related to these discussion areas.

### 2.2 Requirements

As a starting point, we think that the following requirements should guide the development of a model transfer/delivery solution:

Proposal 1: Adopt the following list of requirements for model transfer/delivery.

- Model transfer/delivery traffic should be differentiated from other user traffic.

- Model transfer/delivery traffic should be transferred at a different priority than user traffic.

- There should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models cannot be downloaded.

- Model transfer/delivery is initiated by the UE.

- Models need to be addressable such that the UE can request the transfer/delivery of a specific one.

- The NW is in control of if and when to transfer / deliver a model to the UE.

And that the following assumptions should be made:

Proposal 2: Adopt the following assumptions for discussion about model transfer/delivery.

- UE’s can only store a very limited set of trained models.

- Trained models could be per gNB, per UE hardware/software, per morphology.

- For beam management trained models are applicable to a small number of sites, e.g., cell, gNB, or area, etc.

- gNB can evaluate current conditions to determine whether to transfer a model, e.g., if a UE is at the cell edge, based on cell load, etc.

# 3	Conclusion

This document proposed the following:

Proposal 1: Adopt the following list of requirements for model transfer/delivery.

- Model transfer/delivery traffic should be differentiated from other user traffic.

- Model transfer/delivery traffic should be transferred at a different priority than user traffic.

- There should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models cannot be downloaded.

- Model transfer/delivery is initiated by the UE.

- Models need to be addressable such that the UE can request the transfer/delivery of a specific one.

- The NW is in control of if and when to transfer / deliver a model to the UE.

Proposal 2: Adopt the following assumptions for discussion about model transfer/delivery.

- UE’s can only store a very limited set of trained models.

- Trained models could be per gNB, per UE hardware/software, per morphology.

- For beam management trained models are applicable to a small number of sites, e.g., cell, gNB, or area, etc.

- gNB can evaluate current conditions to determine whether to transfer a model, e.g., if a UE is at the cell edge, based on cell load, etc.