3GPP TSG-RAN WG2 #128 	R2-2410605

Orlando, FL, USA, November 18th - 22nd, 2024

Agenda Item:	8.2.3

Source:	Ericsson

Title:	Discussion on DL messages for Ambient IoT UEs

Document for:	Discussion/Decision

# 1	Introduction

RAN2 has started work on the support of Ambient IoT devices WI (RP-234058). The SID [1] has the following objectives led by RAN2:

- RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential.

In RAN2#127bis, the following agreements were made:

- The A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. FFS how to indicate. Wait for further information and requirements from other WGs and make this decision in normative phase. - Based on this information the device determines whether to skip sending the response to paging. - From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs. RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs. - If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA

In this contribution we continue to discuss downlink messaging for A-IoT devices and address the open issues from the previous meetings.

# 2	Discussion

## 2.1 Reader awareness

In RAN2#127, it was agreed that the following information is considered useful if it is visible to the reader: the service type, e.g., inventory, command, whether the requested service is intended for one or multiple devices, and the approximate number of targeted devices. However, it was FFS whether such information should be mandatory or optional. We think that the information on service type, and even command type, and whether service request is intended for a single device, or multiple devices should be provided to the reader mandatorily. This is so that it would be possible for the reader to configure the downlink message accordingly, e.g., contention based vs. contention free random access. However, for approximate number of targeted devices we think that it can be optional as it may not be always possible for the network to have such information.

It is mandatory to inform the reader about the service type, command type and whether the DL message is addressed to single or multiple devices.

It is optional to inform the reader about the approximate number of targeted devices.

Send an LS to SA2 and RAN3 regarding the information visible to the reader.

## 2.2 Multiple/subsequent A-IoT paging

In RAN2#127, it was agreed to support a mechanism for the reader to trigger multiple/subsequent A-IoT paging messages that are associated with the same request from the CN. In RAN2#127bis, it was agreed that A-IoT paging message can include information to avoid duplicate response from the device to a reader. It is FFS how to indicate such information. In our view, this information can either be generated by the reader or the CN which can then be provided along with the paging request:

- Reader generated: when the reader receives a service request from the CN, it generates the information associated with that service request and provides it in the paging message. In the subsequent paging message(s) triggered by the reader, same information is provided in the paging message.

- CN generated: the information associated with the service request is generated by the CN and provided to the reader along with that service request. The reader provides the information in the paging message. In the subsequent paging message(s) triggered by the reader, same information is provided in the paging message. It is also possible that the information generated by the CN is provided encrypted in an encapsulated message provided by the CN, i.e., not visible to the reader.

If it is the CN that generates the information, CN can coordinate so that same information is provided to multiple readers, in the service request from the CN, e.g., inventory. This would make it possible to avoid receiving redundant responses from an A-IoT device in case it happens to be covered by multiple readers. Otherwise, i.e., if it is the reader that generates the information, there may be a need for coordination between readers or additional information may need to be provided to identify the readers. This can be beneficial since the device may determine whether to respond based on the information provided in the paging message with no need to decode the message from the CN, if the message is encrypted.

We suggest capturing both options with pros and cons in the TR and inform SA2 and RAN3 that it may be beneficial if the information associated with the service request is generated by the CN and provided in the paging message.

Capture the following options on how information associated with the service request is generated and pros and cons in the TR:

- reader generates the information associated with the service request from the CN.

- CN generates the information and provides it to the reader along with the service request from the CN, e.g., transparently or encrypted.

Device behaviour after receiving (subsequent) A-IoT paging

When an A-IoT device receives a paging message and observes that the information provided in the paging message is different from the one provided in the previous paging message, the A-IoT device performs the requested procedure regardless of whether it has completed the same procedure in the previous paging round. Otherwise, i.e., if the information provided in the paging message is the same as the one provided in the previous paging message, the device only needs to perform the requested procedure if it has not completed the procedure successfully in the previous paging round. Otherwise, it can ignore the paging message. Note that successful completion may be either due to receiving an acknowledgment explicitly or implicitly after transmitting Msg3 for 3-step contention-based RA. This is a separate discussion.

If the paging message is associated with a CN request different than the one associated with the paging message in the previous round, the device performs the requested procedure.

If paging message is associated with a CN request same as the one associated with the paging message in the previous round, the device performs the requested procedure only if it has not completed the procedure successfully in an earlier paging round associated with the same CN request.

Otherwise, the device can ignore the paging message.

## 2.3 Multiple reader scenario for A-IoT devices

Multiple readers may be deployed to serve an area, and their coverage can overlap. For example, in [1], a reader can transmit paging messages to A-IoT device(s) whose responses are received by another reader.

One may also need to consider the case where an inventory procedure started by one reader is interleaved by another inventory procedure started by another reader assuming that there is no coordination between readers in time and/or frequency domains. For example, an A-IoT device which is in the coverage of two readers as shown in Figure 1, may receive paging messages from both readers assuming that there is no coordination between the readers in time and/or frequency domains. In the absence of a (reader) identifier, it will be difficult for the device to realize which reader it has responded, or it should respond to.

Figure 1 A-IoT device in the overlapping coverage.

Based on the following agreement: “A-IoT paging message indicate information from which the device can determine resources to be used for response”, the information within a paging message indicates the resources to be used when transmitting the response. For example, for slotted-Aloha access, where devices determine the slot for the uplink message transmission, an A-IoT device under coverage of multiple readers may get confused if subsequent paging messages are received. It can be beneficial to introduce a reader identifier for an A-IoT device to determine when/how to transmit. Such identifier can be provided in the paging message.

Figure 2 A-IoT device receiving paging message from multiple readers in overlapping coverage area.

Besides, in some use cases (e.g., for proximity determination), the CN may need the device to responsd to all A-IoT paging messages sent from multiple readers even if the A-IoT paging messages are associated with the same CN request. The CN could instruct the reader to include the reader ID in the A-IoT paging messages in case it only needs the device to respond once to the A-IoT paging messages associated with the same CN request and sent from different readers, otherwise the CN could instruct the reader to not include the reader ID in the A-IoT paging messages if it needs the device to response to the A-IoT paging messages sent from different readers even if the A-IoT paging messages are associated with the same CN request, in this case the device will always respond.

The coverage from multiple readers may overlap, e.g., where a transmitting reader can be different than the receiving reader, or an inventory procedure started by one reader can be interleaved by another inventory procedure started by another reader.

In some use cases (e.g., for proximity determination), the CN may need the device to respond to the A-IoT paging messages sent from multiple readers even if the A-IoT paging messages are associated with the same CN request.

Study if it would be beneficial to introduce a reader ID that can be provided in the paging message.

## 2.4 Indication of the random-access type

In RAN2#127bis it was agreed that “From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs. If multiple device IDs in single paging is supported and A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA.”

A related discussion is if the reader should indicate explicitly whether a device responds to a paging message with a contention free or contention based random access. In our view there is no need to indicate such explicitly. For contention based random access, the reader only needs to indicate information that is required for a device to determine which slot to respond randomly, yet for contention free random access, the reader needs to indicate the dedicated resources for the targeted device(s). This means that the content of such paging messages will be different for contention free, and contention based random access.

From RAN2 perspective it is feasible to support paging multiple device IDs depending on TB size.

When A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA.

There is no need to indicate explicitly whether a device should respond to the paging message with contention free or contention-based RA.

# 3	Conclusion

In this contribution we discussed downlink/paging messages for A-IoT devices. Based on the discussion in the previous sections we made the following observations:

Observation 1	The coverage from multiple readers may overlap, e.g., where a transmitting reader can be different than the receiving reader, or an inventory procedure started by one reader can be interleaved by another inventory procedure started by another reader.

Observation 2	In some use cases (e.g., for proximity determination), the CN may need the device to respond to the A-IoT paging messages sent from multiple readers even if the A-IoT paging messages are associated with the same CN request.

Observation 3	From RAN2 perspective it is feasible to support paging multiple device IDs depending on TB size.

Observation 4	When A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA.

Based on the discussion in the previous section, we propose the following:

Proposal 1	It is mandatory to inform the reader about the service type, command type and whether the DL message is addressed to single or multiple devices.

Proposal 2	It is optional to inform the reader about the approximate number of targeted devices.

Proposal 3	Send an LS to SA2 and RAN3 regarding the information visible to the reader.

Proposal 4	Capture the following options on how information associated with the service request is generated and pros and cons in the TR:

a) 	reader generates the information associated with the service request from the CN.

b)	 CN generates the information and provides it to the reader along with the service request from the CN, e.g., transparently or encrypted.

Proposal 5	If the paging message is associated with a CN request different than the one associated with the paging message in the previous round, the device performs the requested procedure.

Proposal 6	If paging message is associated with a CN request same as the one associated with the paging message in the previous round, the device performs the requested procedure only if it has not completed the procedure successfully in an earlier paging round associated with the same CN request.  Otherwise, the device can ignore the paging message.

Proposal 7	Study if it would be beneficial to introduce a reader ID that can be provided in the paging message.

Proposal 8	There is no need to indicate explicitly whether a device should respond to the paging message with contention free or contention-based RA.

# References

TR 38.848, “Study on Ambient IoT (Internet of Things) in RAN (Release 18)” V18.0.0 (2023-09)

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA, Maastricht, Netherlands, March 18-21, 2024.

# Annex A.	Text Proposal

## 6.3	Protocol stack and signalling procedures

### 6.3.1	General aspects and overall procedure

The study aims that the design on the A-IoT radio interface between reader and A-IoT device is common for Topology 1 and Topology 2. The difference of topologies is transparent to the A-IoT devices and has no impact on A-IoT devices. Unless explicitly stated, the descriptions in clause 6.3 apply to all A-IoT device types and both Topology 1 and Topology 2.

Figure 6.3.1-1 Overall AS procedures between A-IoT device and reader

The overall AS procedures can be formulated as:

-	Step A: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message indicating device(s) that need to respond. See clause 6.3.3.

NOTE 1:	In the clause 6.3, the term of “A-IoT paging message” is equal to the “(initial) trigger message”. For simplification, only the former is used.

-	Step B: D2R data (device ID) transmission. Triggered A-IoT device(s) perform the device ID transmission via the A-IoT random access procedure or without using the A-IoT random access procedure. See clause 6.3.4 (and 6.3.5).

-	Step C1: Possible R2D data transmission (e.g., for sending the command). See clause 6.3.5.

-	Step C2: Possible D2R data transmission (e.g., the corresponding response to command). See clause 6.3.5.

Then, above AS procedure can support indoor inventory and indoor command use cases by the following manners:

-	For the detailed use case of “inventory-only”, it is supported by the procedure with Step A and Step B as baseline.

-	For the detailed use case of “inventory and command”, it is supported by the procedure with Step A, Step B, Step C1 and Step C2, as baseline.

NOTE 2:	For the use case of “inventory and command”, it does not imply that the A-IoT paging message includes both the inventory and command and it does not imply the inventory and command are received by the reader at the same time from upper layer.

-	For the detailed use case of “command-only”:

-	It can be also supported by the baseline procedure with Step A, Step B, Step C1 and Step C2.

-	In addition, another candidate to support this use case is following, whose feasibility still depends on the conclusion from [7] and [8]:

- -	Step A’: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message including the command, indicating device(s) to process/respond the command.

- -	Step C2: Possible D2R data transmission (e.g., the device ID or the corresponding response to command), via the A-IoT random access procedure or without using the A-IoT random access procedure.

The following information are considered useful to be visible to the reader from CN:

-	The A-IoT service type (e.g., inventory, command)

-	Whether the service is targeted for one or more than one A-IoT devices

-	The approximate number of target A-IoT devices of this service

### It is mandatory to inform the reader about the service type, command type and whether the DL message is addressed to single or multiple devices. It is optional to inform the reader about the approximate number of targeted devices.

### <skip>

### 6.3.3	A-IoT paging

In A-IoT AS layer, the A-IoT paging functionality is to use A-IoT paging message to indicate device(s) that need to respond.

As to the A-IoT paging message, the identifier may be required to identify the device/group of devices in this trigger message (e.g., for the case of reaching a single or a group of devices). Following cases are studied:

-	The A-IoT paging message containing an identifier of a single A-IoT device

-	The A-IoT paging message containing a group ID that maps to multiple A-IoT devices

-	The A-IoT paging message that does not contain any identifier, i.e., indicating all A-IoT devices that can receive the A-IoT paging message need to respond

-	The A-IoT paging message containing multiple identifiers of A-IoT devices. The need for this use case is still to be confirmed/dependent according to the conclusion in [7]. From RAN2 perspective, it is feasible to support paging multiple identifiers of A-IoT devices, pending on TB size and multiplexing design of A-IoT paging message.

NOTE 1:	The details of the above identifier and group ID and also the use case/scenario are studied in [7].

As to the A-IoT paging message, it can additionally indicate the information from which the device(s) can determine the resource(s) to be used for D2R response message(s). It can be further considered on more details for the discussion in sub-clause 6.1.

For A-IoT device paging functionality, it is understood that the legacy paging message, legacy paging occasion and legacy DRX from NR are not supported (See TS 38.300 [9] for references for any legacy NR functionality). From RAN2 perspective, it is assumed that the A-IoT device can receive as long as there is enough energy (it can be further considered on the discussion in sub-clause 6.1 and 6.2).

It is supported that the reader can send multiple (subsequent) A-IoT paging messages that are associated with the same service request from the CN. The duplicated response from devices for the same service request should be avoided. The A-IoT paging message can include information to avoid this duplicated response from the device to a reader. This information should be short and simple. This information can be generated as follows:

-	When reader receives a service request from the CN, it generates this information associated with that service request and provides it in the paging message. In subsequent paging message(s) triggered by the reader, same information is provided in the paging message(s).

-	The information associated with the service request is generated by the CN and provided to the reader with that service request. The reader provides the information in the paging message. In subsequent paging message(s) triggered by the reader, same information is provided in the paging message(s). It is possible that the information generated by the CN is provided encrypted in an encapsulated message provided by the CN, i.e., not visible to the reader.

If CN generates this information, CN can coordinate so that same information is provided to multiple readers associated with the same service request from the CN, e.g., inventory. This would make it possible to avoid triggering redundant responses from an A-IoT device in case it happens to be covered by multiple readers. If the reader generates this information, there may be a need for coordination between readers unless additional information is provided to identify the readers.

It can be further discussed on how to design this information in A-IoT paging message (e.g. as stage-3 details and also considering the aspects from other WGs for this). Then, based on this information, the device determines whether to skip sending the response to A-IoT paging message or not (if the device had successfully responded the same service before).

If the paging message is associated with a CN request different than the one associated with the paging message in the previous paging round, the device performs the requested procedure. If paging message is associated with a CN request same as the one associated with the paging message in the previous paging round, the device performs the requested procedure only if it has not completed the procedure successfully in an earlier paging round associated with the same CN request. Otherwise, the device can ignore the paging message.