3GPP TSG-RAN WG2 Meeting #128         											                      R2-2410641

Orlando, USA, Nov 18th – 22th, 2024                                                         Revision of R2-2408623

Agenda item:	8.9.3

Source:	MediaTek Inc.

Title:	Discussion on enhanced EDT

Document for:	Discussion and Decision

# Introduction

Rel-19 IoT NTN work item was revised in RAN#105 [1] and one of the objectives is to enhance EDT procedure:

- Support of Capacity enhancements for uplink

  - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]:

    - Msg3 transmission without msg1/ Random Access Response (RAR)

    - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In RAN2#127bis, RAN2 has reached the following agreement for CB-Msg3.

Agreements:

1.	RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19.

2.	RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed.

3.	At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details.

4.	CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

5.	RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA)

6.	At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission

7.	The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

8.	CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

In this contribution, we continue discussing the CB-Msg3 on details of DSA mechanism, RNTI, TA issue, and details of Msg4.

# Discussion

Details of DSA mechanism

Diversity Slotted Aloha (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) were discussed in the RAN2 meetings. RAN2 has reached consensus to continue working on DSA in the RAN2#127bis and RAN2 does not intend to work on CRDSA in Rel-19. We continue discussing the details of DSA mechanism.

The previous analysis of DSA indicates that the Packet Loss Ratio (PLR) of DSA exceeds that of SA as the system load increases.

Observation 1: DSA outperforms SA in terms of lower PLR only when the system load is relatively low.

Although in the last RAN2 meeting we agreed to focus on a working point where the PLR is lower than 10%, we still need to face the reality that the PLR can be higher than 10% due to a relatively high system load.

RAN2#127bis agreement: R2-2409231 Report of [AT127bis][301][R19 IoT NTN] Working point for CB-msg3 NEC discussion Rel-19 IoT_NTN_Ph3-Core Based on the divergent views, we might try to focus the discussion on a working point where the PLR is lower than 10%. In evaluation of possible methods to improve UL capacity for CB-msg3 ETD-like transmission we focus on a working point where the PLR is lower than 10% (this does not mean that we are mandating any network to work at any working point)

Therefore, the DSA scheme should be enabled only when the network ascertains the system load is relatively low (i.e., where the DSA outperforms SA). When the DSA is disabled, SA is assumed to be used.

Proposal 1: The DSA for CB-Msg3 is optionally configured by the network. If it is not configured, SA is used as the default.

For DSA, the UE needs to transmit multiple times (i.e., N replicas) for UL data. Although DSA can benefit from a lower PLR when the system load is relatively low, it consumes more power than SA due to transmitting N replicas. For UEs that are sensitive to power consumption (e.g., battery based IoT device), DSA may not be preferred. Therefore, DSA can be an optional feature for UEs. For cells with DSA enabled, UEs without DSA capability can use SA for CB-Msg3. Since the CB-Msg3 resource is provide via system information, the network does not need to know whether the UE supports DSA, so a UE capability parameter is not needed.

Proposal 2: DSA is an optional feature without UE capability parameter. If DSA is not supported by a UE, SA can be utilized even when the cell enables DSA.

The DSA with 3 replicas consumes more power than DSA with 2 replicas. Additionally, based on the pervious analysis, the PLR of DSA with 3 replicas increases faster when the system load increase compared to DSA with 2 replicas. The area in terms of PLR where DSA outperforms SA is further limited. Therefore, we believe the number of replicas for DSA in IoT NTN should be no more than 2.

Proposal 3: The number of replicas for DSA in IoT NTN is 2.

For NB-IoT, multiple non-anchor carriers can be used to expand the system capacity. On the top of time-level or subcarrier-level resources, the carrier-level UL/DL resources for CB-Msg3 can be utilized. When the carrier-level UL/DL resources are configured, to keep the UE complexity low, the replicas of DSA should be transmitted on a same carrier.

Proposal 4a: For NB-IoT, carrier-level UL/DL resources can be configured for CB-Msg3.

Proposal 4b: For NB-IoT, when the carrier-level UL/DL resources are configured, the replicas of DSA are transmitted on the same carrier.

Normally, the replicas of DSA are depicted as spreading in time domain. That is, a subsequent transmission is allocated after a certain time (e.g., in terms of subframe or radio frame) following the previous transmission. The frequency domain (i.e., selection of the subcarrier) of each DSA replicas should be randomly selected to reduce collision. If replicas are transmitted at the same times but on the different subcarriers, the transmit power will be averaged to each replica. This will reduce UL link budget. So, this approach is more suitable for those UEs in the edge of the coverage, which needs full transmission power to reach the satellite. Therefore, we make the following proposal:

Proposal 5: DSA replicas are transmitted at different times. The frequency domain is randomly selected.

How the UL resource is configured for DSA is another question. There will be two options. One option is that UE send each replica on consecutive periodical UL resources. UE monitors the possible Msg4s with two timers.

The other option is all DSA replicas can be sent in one periodical UL resource. Two or more PUSCH occasions are provide in one UL resource. UE monitors the possible Msg4 with one timer.

The first option is simpler, it has less spec impact. But it needs to start two or more timers to monitor Msg4. It also needs to keep the periodicity of UL resource smaller than the RTT. Otherwise, it becomes retransmission rather than DSA. The second option has more spec impact, but it needs one timer to monitor Msg4, and there is not limitations on the periodicity of the UL resource. We slightly prefer option 1.

Proposal 6: For the DSA transmission, RAN2 assumes that the UE sends each replica on consecutive periodical UL resources.

If the network cannot decode all the DSA replicas, and when the UE failed to decode the Msg4, can UE retransmit the UL data using DSA? A retransmission using DSA will increase the system load, and the PLR will increase accordingly. It will also increase UE power consumption compared to a retransmission using SA. We believe retransmission using DSA should only be allowed when the system load is very low. A retransmission number for the DSA can be optionally configured by the network. For UEs not supporting DSA, retransmission with SA is used instead.

Proposal 7: A retransmission number of DSA can be optionally configured by the network for UEs supporting DSA.

RNTI

A new RNTI is needed for monitoring PDCCH for Msg4. RAN2 has agreed that the RNTI is derived based on the resources associated with the PUSCH occasion used for the contention based Msg3 EDT transmission.

RAN2#127bis Agreements:

The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

For the network, the new RNTI is used to scramble the PDCCH for the Msg4 transmission. UE also needs it to monitor PDCCH. How the RNTI is calculated is determined by how the Msg4 is organized.

If there is no DL response to the UL EDT transmission, there will be only a contention resolution MAC CE in the Msg4. So that there will be enough room left in a MAC PDU for other information. The Msg4s responding to the Msg3s from the same time domains but the different frequency domains can be included in the same MAC PDU. The signalling overhead efficiency will be improved accordingly.

Therefore, the time domain information (e.g., subframe index, SFN) will be part of formulation of new RNTI. The frequency domain information (e.g., subcarrier index) will not.

Proposal 8a: The Msg4s responding to the Msg3s from the same time domains but the different frequency domains can be included in the same MAC PDU.

Proposal 8b: The time domain information of CB-Msg3 UL resource (e.g., subframe index, SFN) is taken into account for new RNTI calculation. The frequency domain information (e.g., subcarrier index) is not.

Additionally, since the carrier-level UL/DL resource can be provided in NB-IoT, UEs can send UL transmissions on different UL carriers and then monitor DL responses on the same DL carrier. To differentiate Msg4s responding to different UL carriers, the carrier index is needed in the formulation of new RNTI.

Proposal 9: For NB-IoT, the carrier index of CB-Msg3 UL resource is taken into account for new RNTI calculation.

When the DSA approach is applied, there will be multiple replicas to be transmitted. The UL resources in terms of time domain and/or frequency domain for replicas should be different. Hence the RNTIs for the replicas should be derived separately based on their own time domain information. UE should monitor the RNTIs for replicas simultaneously.

Proposal 10: The RNTIs for the replicas should be derived separately, and the UE should monitor the RNTIs for the replicas simultaneously.

TA issue

We would like to introduce a new optional cell-specific TA adjustment to be compatible for the network-implemented TA margin solution. In this solution, to counter possible UE overcompensation, the network may configure a positive value in the TAC in RAR and subtract the same value from common TA. When the network needs to address the UE overcompensation, it simply decreases the positive value in the TAC in RAR for a specific UE. Without this solution, the network would have to put a negative value in the TAC in RAR which is not allowed in the current LTE/NR specification.

For enhanced EDT without Msg1/RAR, if this network-implemented TA margin method is still adopted, it may cause some problem to the UEs transmitting Msg3 without Msg1/Msg2. The common TA has been reduced by a certain value for the existing legacy UEs, it would be wrong for the new UEs doing TA pre-compensation based on the modified common TA value. We propose to add a cell-specific TA adjustment for the UEs transmitting Msg3 without RAR, so that the TA calculation would be remain correct.

A similar discussion occurred in the NR NTN support of regenerative payload on whether to introduce a negative value for common TA, where the common TA is supposed to be 0 so that it couldn’t be reduced any further. See the LS [2] for reference.

Proposal 11: Introduce a new optional cell-specific TA adjustment for UEs transmitting Msg3 without Msg1/Msg2.

Details of Msg4

Because the Msg3 is contention based, contention must be resolved in Msg4. Hence, a contention resolution MAC CE should be included in the Msg4. The details of contention resolution MAC CE can be further discussed.

Proposal 12: A contention resolution MAC CE must be included in the Msg4 for enhanced EDT procedure.

Since the contention resolution is essential and cannot be done in DCI due to the size limitation, using the L1 ACK as the Msg4 is not suitable.

Proposal 13: Do not support using L1 ACK as the Msg4 for the enhanced EDT procedure.

# Conclusion

Details of DSA mechanism

Observation 1: DSA outperforms SA in terms of lower PLR only when the system load is relatively low.

Proposal 1: The DSA for CB-Msg3 is optionally configured by the network. If it is not configured, SA is used as the default.

Proposal 2: DSA is an optional feature without UE capability parameter. If DSA is not supported by a UE, SA can be utilized even when the cell enables DSA.

Proposal 3: The number of replicas for DSA in IoT NTN is 2.

Proposal 4a: For NB-IoT, carrier-level UL/DL resources can be configured for CB-Msg3.

Proposal 4b: For NB-IoT, when the carrier-level UL/DL resources are configured, the replicas of DSA are transmitted on the same carrier.

Proposal 5: DSA replicas are transmitted at different times. The frequency domain is randomly selected.

Proposal 6: For the DSA transmission, RAN2 assumes that the UE sends each replica on consecutive periodical UL resources.

Proposal 7: A retransmission number of DSA can be optionally configured by the network for UEs supporting DSA.

RNTI

Proposal 8a: The Msg4s responding to the Msg3s from the same time domains but the different frequency domains can be included in the same MAC PDU.

Proposal 8b: The time domain information of CB-Msg3 UL resource (e.g., subframe index, SFN) is taken into account for new RNTI calculation. The frequency domain information (e.g., subcarrier index) is not.

Proposal 9: For NB-IoT, the carrier index of CB-Msg3 UL resource is taken into account for new RNTI calculation.

Proposal 10: The RNTIs for the replicas should be derived separately, and the UE should monitor the RNTIs for the replicas simultaneously.

TA issue

Proposal 11: Introduce a new optional cell-specific TA adjustment for UEs transmitting Msg3 without Msg1/Msg2.

Details of Msg4

Proposal 12: A contention resolution MAC CE must be included in the Msg4 for enhanced EDT procedure.

Proposal 13: Do not support using L1 ACK as the Msg4 for the enhanced EDT procedure.

# References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)

[2] R2-2407623, “LS on common TA in a regenerative payload scenario”, RAN2