3GPP TSG-RAN WG2 Meeting #128	R2-2410656

Orlando, USA, 18 – 22 November, 2024

Title: 	Discussion on MRO enhancements for Rel-18 mobility features

Source: 	Huawei, HiSilicon

Agenda item:	8.10.2

Document for:	Discussion/Decision

- Introduction

In this paper, we mainly analyze possible RAN2 impacts for LTM, CHO with candidate SCGs.

- Discussion

Discussion on LTM

RAN2#127bis made the following agreements:

Agreements - The UE shall log cell IDs such as reestablishment cell ID, source, failed, reconnect cell ID, following the same principle as RLF, HOF and successful recovery, incl. the time between UE executing the LTM command and the failure. - If RA-based LTM failure happens the UE logs and reports RACH info in the RLF report. Additional information is TBD. - Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined). - Include an explicit indicator in SHR whether the successful LTM execution was RACH-less or RACH-based. Can sort out the details during stage-3 implementation. - Unless RAN3 defines a NW-based solution: Introduce an explicit indication in RLF-Report to indicate whether a neighbour cell is an LTM candidate cell. - UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR.

To facilitate the further discussion, we illustrate the steps during the LTM procedure to have a retro on the design of Rel-18 LTM feature, compared to those during legacy L3 handover.

Figure 1: Handover procedures for Legacy L3 HO and LTM

The division of failure cases for LTM is similar as those for handover, since the LTM is built and augmented on top of L3 handover. For handovers, the SON features including the radio link failure (RLF) report and successful handover report (SHR) provides the UE feedback information. The RLF and SHR could be augmented to also provide useful feedback information to address the LTM mobility issues. We will discuss the possible MRO enhancement for those SON reports.

RLF report for LTM

- Access type

- An FFS from RAN2#127 meeting is whether the specific access type, i.e. RACH-based or RACH-less, is explicitly or implicitly indicated in RLF report. The specific access type is determined by whether the UE has available TA after reception of LTM cell switch command, which could be gained either from network triggered PDCCH ordered TA acquisition procedure or UE-based TA measurement. Some companies think if the RA information is included in the RLF report, it can be used as an implicit indication. However, the RA information is logged only when the failure occurs due to random access problem or beam failure recovery failure. For other failure causes, the RA information is not known by the network. Therefore, based on the agreed logging information from previous RAN2 meeting, we could not derive whether it is RA-based or RA-less LTM cell switch execution implicitly. Therefore, it is suggested to indicate the specific access type explicitly.

Proposal 1: RAN2 include the specific access type in the RLF report explicitly.

The UE logs and reports whether and how the UE got the TA value

Per current specification, the UE could perform the UE-based TA measurement and perform PDCCH ordered TA acquisition simultaneously. How the UE performs cell switch upon reception of cell switch command is divided into the following cases:

Case1: UE receives the TA value from cell switch command, regardless whether UE has valid measured TA.

->RA-less access based on TA value from cell switch command

Case2: UE does not receive any TA in cell switch command, and only got the UE measured TA

->RA-less access based on UE measured TA value

Case3: UE does not have neither the TA value from cell switch command, nor UE measured TA value

->RA-based

With the indication of access type included in the RLF report, the UE only need to log whether the network provides the TA in the cell switch command. So that, the network could distinguish between the TA from the network and the TA from UE-based measurement to make proper adjustment of the LTM related parameters, e.g. whether to enable the UE-based TA measurement.

Proposal 2: RAN2 include in the RLF report whether the UE receives the TA from the cell switch command.

RACH information in RLF report

Currently UE includes RACH information (ra-InformationCommon) in RLF Report if the failure cause is random access problem or the beam failure recovery failure. The LTM cell switch execution could be RACH-less or RACH-based. If it is RACH-based, it is recommended to reuse the existing RACH information (ra-InformationCommon). If it is RACH-less, RAN2 discuss whether to include early sync info to existing RACH information to identify the issues of network-based TA procedure, e.g. the PDCCH order triggered TA acquisition is out-of-date. The detailed early sync info is FFS.

Proposal 3: RAN2 reuse the existing ra-InformationCommon for the RA-based LTM failure

Proposal 4: RAN2 discuss whether to include early sync info to existing RACH information. The detailed early sync info is FFS.

Beam Info

Furthermore, we discuss the beam info indicated in the LTM cell switch command. In case of RACH-less cell switch, the UL transmission may fail due to the bad choice of the target beam. It is suggested that UE logs the beam info in the cell switch command. Combining this beam info with the L1 measurement result at the failure, the network could perform analysis regarding the choice of target beam.

Proposal 5: RAN2 agree to enhance the RLF report with the beam information contained in the LTM cell switch command.

LTM candidate Cells

The last RAN2 meeting has agreed to introduce an explicit indication in RLF report to indicate the LTM candidate cells. There are two alternatives as follows:

Alt.1: indicate a LTM candidate in the neighbour cell measurement results

Alt.2: introduce a LTM candidate cell list

For alt.1, it supports to indicate the LTM candidate for both RLF report and SHR report, by a flag of the LTM candidate within the MeasResultNR explicitly for each neighbour cell, since the two reports contain the neighbour cell measurement results.  However, for alt.2, a separate LTM candidate list needs to be introduced to RLF and SHR report individually. Therefore, we propose to adopt alt.1., which is applicable to both RLF and SHR report.

Proposal 6: Introduce a flag of the LTM candidate within the MeasResultNR explicitly for each neighbour cell.

SHR report for LTM

stage-3 implementation

Some companies think the RA information included in SHR report could be used as an implicit indication of RACH-less or RACH-based LTM switch. Similar to RA information in RLF report, the RA information is not always included in the RLF report. Only when T304 trigger is fulfilled, the UE logs the RA information. Otherwise, the RA information is absent. That is why RAN2 thinks an explicit indicator of RACH-less or RACH based LTM switch is needed. Such an indicator is needed for both RLF report and SHR report. From our point of view, it is straightforward to use a flag of whether it is RACH-less for both SHR report and RLF report.

Proposal 7: Introduce a flag of whether it is RACH-less for both SHR report and RLF report.

Discussion on CHO with candidate SCGs

Agreements - UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS. - Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

RAN2#127bis made the following agreements:

About assumptions on configurations

According to RAN2#126 Chair Notes [1], for the assumptions on configurations, the following options were proposed for discussions:

Option 1: R18 configurations of CHO with candidate SCGs

Option 2: R17 configurations of CHO with target SCG

Option 3: R16 configurations of CHO

Some companies proposed to discuss Option 1 only, while some companies proposed to discuss mixed configurations, e.g. Option 1 + Option 2, or Option 1 + Option 3. Since the Option 1 is the target case for this Rel-18 feature objective, the usage of mixed configurations are just a supplement to Option 1 in case that these parameters of Option 1 could not be configured properly and thus raise the risk of mobility failure issues.

We think RAN2 can focus on Option 1 first, and other options (including mixed configuration) can be de-prioritized.

Proposal 8: RAN2 focus on only R18 configurations of CHO with candidate SCGs first.

Discussions on scenarios and issues

At RAN2#126, we submitted [2] and we showed 9 failure cases for CHO with candidate SCGs. As below, we provide more technical analysis  on these scenarios and possible issues.

Discussions on Case 2)

For Case 2), it is about too late handover, and then UE will suffer RLF on PCell.

Figure 4: Handover procedure for Case 2)

For RLF report, currently the IE lastHO-Type is used to indicate the type of the last executed handover before the last detected connection failure, and it has defined CHO and DAPS. For Case 2), if a new HO type CHO with candidate SCGs can be added, the network can do optimizations based on this kind of handover. For now, we see no strong need to enhance SCGFailureInformation for Case 2).

Proposal 9: For too late handover, it is proposed RAN2 to discuss the need of introducing a new HO type CHO with candidate SCGs in the IE lastHO-Type in RLF report.

For each candidate PCell, there could be multiple candidate PSCells, i.e. a list of candidate PSCells. When UE is configured with CHO with candidate SCGs, the UE could receive at most 8 candidate configurations, each of which is for one PCell-PSCell association or one PCell only. We also notice that each of the multiple PCell-PSCell associations could have its own elapsed time between the point in time of the first fulfilled condition and RLF, as agreed in last RAN2 meeting. We do not intend to record such a time for every PCell-PSCell association. In our understanding, the first fulfilled condition means the first fulfilled condition among all the PCell-PSCell association. Therefore, the RLF report only needs to log the time info once, i.e. earliest triggering point of PCell/PSCell till the RLF.

Proposal 10: RAN2 discuss to only include one time information, namely between the earliest triggering point of PCell/PSCell till the RLF.

Discussions on Case 7)

For Case 7), both PCell mobility and PScell mobility are triggered. And then, either PCell or PScell or both fail.

Figure 5: Handover procedure for Case 7)

For Case 7), if PCell fails and PScell succeeds, the UE will do RRC re-establishment and also generate RLF report. In this case, the UE can indicate the cause (e.g. PCell failure+PScell success) to the network, so that the network can optimize configuration parameters correspondingly.

If PCell succeeds and PScell fails, the UE will change its PCell successfully, but PScell change fails. The UE would report the SCG failure for the failed target PSCell to the successful target PCell. Hence, there is no impact on SCGFailureInformation.

If both PCell and PScell fail, similar to the first case for Case 7), the cause can be indicated to the network for optimizations.

Proposal 11: For the case of PCell failure and PScell success, and the case of PCell failure and PScell failure, it is proposed RAN2 to discuss the need of introducing the causes in the RLF report.

Proposal 12: For the case of PCell success and PScell failure, it is proposed RAN2 to confirm no impact on SCGFailureInformation message.

Discussions on Case 8)

For Case 8), both PCell and PScell change successfully, and then either PCell or PScell or both fail in a short time. This belongs to "failure shortly after a successful handover".

Figure 6: Handover procedure for Case 8)

We think Case 8) can be categorized into near failure case, and then successful handover report can be enhanced for PCell/PScell mobility.

Firstly, at RAN2#127bis meeting, UE logs the time GAP between the first met execution time and the second met execution time in RLF report. It can be recommended to be included in either SHR or SPR. In addition, the trigger based on the time period can be considered, e.g. if the time period is above a threshold, it may be near failure, and such successful handover reports should be triggered.

Secondly, for Case 8), the UE may generate SHR and SPR. Then whether/how to co-ordinate both reports may be discussed. For example, the UE may generate SHR for PCell mobility and generate SPR for PScell mobility in Case 8), and two reports are actually related to one single R18 mobility procedure (i.e. CHO with candidate SCGs).

Proposal 13: For a successful CHO with candidate SCGs, time gap between the first met execution time and the second met execution time could be included in SHR or SPR. Further discuss the time gap as a SHR/SPR trigger.

Proposal 14: For a successful mobility procedure of CHO with candidates SCGs, RAN2 to discuss whether SHR and SPR need to be correlated or not.

Other scenarios (see no issues and no enhancements needed)

Apart from the above scenarios, there are more scenarios but we observe no issues and thus no enhancements are needed.

For Case 1), it is about too late handover, and PCell/PScell mobility is not triggered. In this case, we think legacy SON reporting can work.

Figure 7: Handover procedure for Case 1)

For Case 4), we think CHO failure reporting can be followed. In this case, no candidate SCG mobility is triggered, so no extra information needs to be collected.

If R16 CHO fails, legacy SON reporting is followed.

Figure 8: Handover procedure for Case 4)

For Case 5), it is CHO only and SCG is not changed. This assumes that UE has both R16 CHO and R18 CHO with candidate SCGs. If R16 CHO shortly RLF, legacy SON reporting is followed.

Figure 9: Handover procedure for Case 5)

For Case 6), it is CHO only and SCG is not changed. This assumes that UE has both R17 (CHO+SCG)+R18 CHO with candidate SCGs. If R17 CHO+SCG succeeds, and SCG failure happens quickly, legacy SCGFailureInformation is followed.

Figure 10: Handover procedure for Case 6)

For 9d, there are two failures during the access procedure, priority of this case? According to internal discussion, when UE is in DC and stable state, Fast MCG recovery can happen. So 9b does not exist.

Figure 11: Handover procedure for Case 9)

- Conclusion

Based on the discussions above, we have the following observations and proposals:

LTM

Proposal 1: RAN2 include the specific access type in the RLF report explicitly.

Proposal 2: RAN2 include in the RLF report whether the UE receives the TA from the cell switch command.

Proposal 3: RAN2 reuse the existing ra-InformationCommon for the RA-based LTM failure

Proposal 4: RAN2 discuss whether to include early sync info to existing RACH information. The detailed early sync info is FFS.

Proposal 5: RAN2 agree to enhance the RLF report with the beam information contained in the LTM cell switch command.

Proposal 6: Introduce a flag of the LTM candidate within the MeasResultNR explicitly for each neighbour cell.

Proposal 7: Introduce a flag of whether it is RACH-less for both SHR report and RLF report.

CHO with candidate SCGs

Proposal 8: RAN2 focus on only R18 configurations of CHO with candidate SCGs first.

Proposal 9: For too late handover, it is proposed RAN2 to discuss the need of introducing a new HO type CHO with candidate SCGs in the IE lastHO-Type in RLF report.

Proposal 10: RAN2 discuss to only include one time information, namely between the earliest triggering point of PCell/PSCell till the RLF.

Proposal 11: For the case of PCell failure and PScell success, and the case of PCell failure and PScell failure, it is proposed RAN2 to discuss the need of introducing the causes in the RLF report.

Proposal 12: For the case of PCell success and PScell failure, it is proposed RAN2 to confirm no impact on SCGFailureInformation message.

Proposal 13: For a successful CHO with candidate SCGs, time gap between the first met execution time and the second met execution time could be included in SHR or SPR. Further discuss the time gap as a SHR/SPR trigger.

Proposal 14: For a successful mobility procedure of CHO with candidates SCGs, RAN2 to discuss whether SHR and SPR need to be correlated or not.

References

Draft_R2_126_Meeting_Report_v2

R2-2405334, MRO enhancements for Rel-18 mobility features, Huawei, HiSilicon

Draft_RAN2_127_Meeting_Report_v1