# 3GPP TSG-WG2 Meeting #128                                            R2-2410672

Orlando, USA, 18th – 22nd November 2024

Source: 			CMCC

Title:  		    Discussion on functionality for A-IoT

Agenda item:	8.2.2

Document for: 	Discussion

Introduction

With the past RAN2#127bis meeting, we have great progress on segmentation, energy status report, message size reporting and AS ID. However, some details are still FFS. In addition, some issues are not be discussed for the limit of time. In this contribution, we will further discuss the leftover issues on functionalities for A-IoT.

Discussion

Segmentation

In the last meeting, RAN2 agreed to focus D2R segmentation and some preliminary guidance on it.

- Agreements on segmentations

- RAN2 will study segmentation and RAN2 will need to give some guidance once there is more clarity on solution, on the minimum TB size.

- Study the functionality of segmentation in the MAC.   We focus on D2R and can consider the support for R2D later depending on SA2 and RAN1.   The solution should aim to be applicable to both direction (i.e. unified solution), if we decide to support on both directions.

- Do not support sequence number for A-IoT segmentation functionalities.

- Do not support segment number and the number of segments for A-IoT segmentation functionalities.

- Capture the following option in TR: An indication is used to indicate to reader whether the data is segmented and whether it is last segment.    FFS the details on how this is designed and number of bits (one or two).

- RAN2 assumes that for the device we will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all buffered segment(s) are stored in upper layer(s).

As we agreed in last meeting, AS layer buffer is not supported for segmentation. Solutions for device to get or maintain the information of segmentation are open. In our understanding, one direct way is A-IoT device maintains the “start point” of data to be transmitted, the subsequent transmission can be continued based on it. How to specify the “start point” can be discussed in work item. Besides that, the reader can assist the A-IoT devices to get the “start point” with indicator(s) in R2D. Two alternatives can be considered: Alternative 1. Reader indicates the “start point” and “end point” of segment to be transmitted; Alternative 2. Reader indicates the “end point” of segment received. For Alternative 1, reader can indicate the size of D2R data with the “start point” and “end point”, considering the service type or message size reporting (if supported). With Alternative 1, how to segment the D2R data is under reader scheduling. For Alternative 2, the reader always indicate the “end point” of segment, the A-IoT devices decide the data size of segment by its implementation, e.g., considering the resource allocated for D2R or energy status. For study item, we suggest considering the above solutions and down select in work item.

Proposal 1: Considering following potential solutions for D2R segmentation,

- Alternative 1: Reader indicates the “start point” and “end point” of segment to be transmitted

- Alternative 2: Reader indicates the of “end point” of segment received, the size depends on device’s decision

- Alternative 3: Device maintains the start point of data to be transmitted, the size depends on device’s decision

For segment transmission, buffer based re-transmission for D2R segmentation can not be supported, considering that neither RLC-like re-transmission nor AS buffer are supported for A-IoT. However, we indentify that feedback for segmentation is beneficial for A-IoT devices to decide following behaviours. For segmentation Alternative 1 and Alternative 2 buffer-less solutions, the re-transmission can be implemented by reader repeat the “start point” and “end point” in R2D message. For segmentation Alternative 3, if device receives transmission failure indicated by reader, it can continue the last “start position” without updated. So, we understand with the proposed 3 alternatives, buffer-less re-transmission can be handled without specification impacts. The details can be left to work item.

Proposal 2: Support optional explicit R2D failure/success feedback indication for D2R segments.

Proposal 3: Do not support buffer based re-transmission for D2R segmentation.

Proposal 4: Buffer-less re-transmission can be discussed in work item.

The motivation to introduce segmentation in A-IoT is to handle the use case that D2R data size is bigger than allocated resource in R2D. For Msg1 and Msg3, the message size is always stable. The reader can indicate proper D2R resource to device to transmit Msg1 and Msg3. So, the Msg1 and Msg3 segmentation are not supported, D2R segmentation is supported for D2R data only.

Proposal 5: Msg1 and Msg3 segmentation are not supported, D2R segmentation is supported for D2R data only.

Some solutions shall be considered for reader to aware when to stop D2R data monitoring. Of course, this issue may be impacted by the segmentation mechanism. For example, if the size of segment is indicated by reader as alternative 1, when the D2R data size is less than the indicated size, the reader can distinguish it is the lase segment. For alternative 2 and 3, an simple message size indicator as end maker can be considered to be transmitted along with the segment, or the device can set message size as “0” independently.

Proposal 6: Simple message size indicator can be considered for last segment indicator.

Message size reporting

As agreed in last meeting, the expected D2R message size is beneficial for reader to perform resource allocation and scheduling.

Agreements

- From RAN2 perspective, it is beneficial for the reader to know an estimate of expected D2R message size.  Two options can be captured: 1) from the CN and 2) from the device (simple message size indication).    Capture the advantages/disadvantages.

- The D2R message size would be beneficial but it is not essential.

- Ask SA2 if it is possible to provide the expected (e.g. approximate, estimate, exact, max) future response D2R message size.  Is it always available, sometimes, or never.

The motivation to further discuss the need of simple message size reporting is to inform the reader about the remaining data to be transmitted. With the progress in the past meetings, the gNB can know the message type, e.g., inventory or command for CN. The rough total data size can be estimated by the reader. Whether The D2R message can be informed by the CN is waiting for SA2 response. The leftover issue is about when to report message size for devices. As per the progress in RA procedure, the size of Msg1 and Msg 3 are fixed. In addition, we suggest no segmentation for Msg 1 and Msg 3 as Proposal 4. So, the message size reporting is not needed during the RA procedure to inform the reader about the D2R message size. Only when the device starts the data transmission, the message reporting is beneficial. As to whether the message reporting as be transmitted along with Msg 3, we prefer not. We understand that an independent Msg 3 without data and message size reporting may lower the complexity of device and reader implementation.

Proposal 7: Message size reporting along with Msg 1 and Msg 3 are not supported.

Energy status report

RAN2 has agreed to support 1 bit to indicate energy status report, if device capability allow.

Capture in the TR the option: the device may include energy status indication in D2R messages (e.g. MSG1, MSG3 and Command Response message), if the device can.  1 bit indication can be captured in the TR.    We will capture the use case in the TR.   FFS whether it is reader controlled.

For R19 ambient IoT, efforts are made to minimize the size, complexity, and power consumption of A-IoT devices, aiming to facilitate the deployment of billions of IoT devices for a wide array of applications and to enhance value across the entire value chain. For some device types, especially for device type 1, the energy status of A-IoT devices cannot be measured with precision by the devices themselves. They can provide limited information to the reader. Whether the UE reports the energy status correctly is untestable. If operator and network side rely on the untestable reporting, the significance is minimal, and it may lead to erroneous scheduling. In addition, different operators have different network deployment strategies. We don’t see any need to introduce energy status reporting for all scenarios, at least for now. So, energy status reporting should be under reader control. If the network doesn’t support energy status reporting feature or decides not to require energy status reporting from the devices, network can inform the devices avoid reporting.

Proposal 8: Energy status reporting is always under reader control.

AS layer ID

Agreements on AS ID - RAN2 assumes that if “AS ID” is defined it is used at least for purpose of D2R scheduling and R2D reception. Up to RAN1 to decide whether a “AS ID” is defined. - RAN2 assumes this “AS ID” should be a short AS layer ID, rather than the full upper layer device ID. FFS on the length. FFS if AS ID can be based on partial upper layer device ID. - From RAN2 perspective, following options are possible for “AS ID”: - Option 1: a random ID if used in Msg1 can be reused; - Option 2: reader assigns this “AS ID”. FFS via which R2D message. - RAN2 will aim to define one common design for all RA procedures (if technically possible)

RAN2 has agreed two optional solutions for AS ID. As we analysis in RA paper [3], for option 1, the random ID generated by device may be reported by other devices within the same paging around, i.e., multiple devices report the same random ID in Msg1. For such a case, reader can assign an “AS ID” via Msg 2 or request the device to generate and report a new ID.

Proposal 9: Reader can either request device to generate and report a new ID or assign an ID to device via Msg 2, if random ID in Msg 1 has been reported by other devices.

Information visibility to network/reader

RAN2 have concluded that the service type of A-IoT message, the approximate number of target devices and targeted for one or more devices are useful information for reader to obtained from the CN.

At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command). FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

2	RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

For A-IoT radio between device and reader, it is the network to allocate RA resource and data transmission resource to A-IoT radio. Random access and data transmission procedure are both fully controlled by network. For different kinds of procedure, the resource allocation can be different. For example, if inventory procedure is triggered, the expected data are RN16 and device ID, the D2R data can be almost 16 or 96bits from high layer perspective; if read command is triggered, the acknowledgement can be 1000bits at most; if write command is included in the AIOT paging message, the D2R feedback can be one or limit number of bits. For effective resource scheduling, the reader at least needs to understand the type of the command from CN, e.g. read/write/disable to assistant resource allocation.

Proposal 10: RAN2 assumes that the A-IoT command type is visible to gNB, i.e., gNB is aware of read/write/disable.

RAN2 have concluded that the (proximity) number of target devices from CN is necessary for gNB to assistant RA type selection and resource allocation. If no device number are provided by the CN, the gNB or reader can estimate the device number to assistant the access resource configuration to avoid waste of resource or collision, as we discussed in RA paper [3].

In [3], we also provide one candidate solution for device number estimation and suggest capturing it on the TR.

Proposal 11: If no device number can be obtained from CN, RAN2 are suggested to support estimate the (approximate) number of devices in gNB or reader.

Device population estimation

There is an agreement captured in TR that the approximate number of target A-IoT device per service is useful in RAN. The number can be provided by CN if possible.

The following information are considered useful to be visible to the reader from CN: - The A-IoT service type (e.g., inventory, command) - Whether the service is targeted for one or more than one A-IoT devices - The approximate number of target A-IoT devices of this service (if available)

However, if CN cannot provide such information, reader may need to estimate the number of target device by itself.

In addition, in some privacy sensitivity use cases, reader may also need to estimate device number by itself. For example, counting the number of people in a building, it’s impossible and maybe illegal to read their ID by inventory, estimating their population can be the only option in this case. Hence device population estimation by reader should be studied and specified.

Proposal 12: RAN2 should specify how reader estimates the number of target devices per service if it’s not provided by CN in stage-2.

As a start, in UHF RFID, EPC C1G2 uses a collision-based binomial algorithm, sometimes referred as Q-algorithm to estimate device population. Reader will adjust the Q value based on whether there is collision or not in a time slot to get a proper value of Q, typically, 2Q should be roughly equals to the number of target device.

There are many problems with Q-algorithm, for example, it’s not accurate, and in Ambient IoT, it will consume much device energy. Here is a brief introduction of the proposed device population estimation method, which provides a both accurate and energy-efficient approach to estimate device population. Details can be found in Annex A and B.

Reader can indicate the length of random number, target number and comparison operation to device. For every occasion, device generates a RN with length indicated by reader, then compares it to the target number. Only If the result matches the operation, device will send Msg1 without data. In this way, devices response in every access occasion independently with the target response probability.

By counting the number of empty occasions, successful occasions, collided occasions, the reader can calculate the estimated population based on the method in Annex A.

Proposal 13: RAN2 is suggested to capture Annex A in TR as a candidate solution for device population estimation, which introduces a dedicated R2D message containing length of RN, target number and operation.

Proximity determination

As described in the revised SID of A-IoT, the feasibility and functionality for proximity determination should be studied.

Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects).

RAN1 has start the discussion on this issue and has some preliminary progress:

Conclusion [RAN1#118]

Proximity determination is concluded to be feasible with either of the two solutions below. Potential specification impact or not will not be determined in the Rel-19 study item.

Solution 1

For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission

- then the device is determined as near to the reader based on measurements at the reader side

Solution 2

For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission then the device is determined as near to the reader

Considering the two solutions in RAN1, the reader judges the devices are nearby once the reader receive the response from the device or the quality of the response is higher than some thresholds. As per the discussion and conclusion in RAN P#105, RAN1 will not further discussion on proximity determination in SI. From RAN2’s perspective, both solution 1 and solution 2 can be covered by current inventory and command procedure. No additional impacts are identified.

Proposal 14: From RAN2’s perspective, both solution 1 and solution 2 for proximity determination can be covered by current inventory and command procedure. No additional impacts are identified.

Protocol stack

With the progress in RAN2, the functionalities supported for A-IoT include paging, Random access, data transmission, segmentation and energy status report. From our perspective, all the functionalities can be handled by A-IoT MAC layer. No new L2 AS layer is necessary to introduce to A-IoT protocol stack.

Proposal 15: No new L2 AS layer is needed for A-IoT protocol stack.

Proposal 16: RAN2 is suggested to capture the protocol stack in Figure 1 A-IoT topology 1.

Figure.1 Protocol stack for A-IoT topology 1

Capability

As per the SID, overall objective shall be to study a harmonized air interface design with minimized differences for Ambient IoT devices type. From RAN2’s perspective, the supporting functionalities are common for devices autonomously or under network control. For physical layer design, no conclusion from RAN1 shows the differentiation among different types of devices. We can wait the progress of RAN1 in work item stage before we decide whether to support capability report. In addition, we can further check whether the capability information can be provided by core network with SA2, if progress in RAN1 shows differentiation among different types of devices at work item. From the perspective of network deployment by operators, deploying the same type of devices within a coverage area is conducive to resource allocation and device management.

Observation 1: From operator’s perspective, deploying the same type of devices within a coverage area is conducive to resource allocation and device management.

Proposal 17: Pending discussion on capability reporting to work item.

Conclusion

According to the above discussion, the following observation and proposals are given:

segmentation

Proposal 1: Considering following potential solutions for D2R segmentation,

- Alternative 1: Reader indicates the “start point” and “end point” of segment to be transmitted

- Alternative 2: Reader indicates the of “end point” of segment received, the size depends on device’s decision

- Alternative 3: Device maintains the start point of data to be transmitted, the size depends on device’s decision

Proposal 2: Support optional explicit R2D failure/success feedback indication for D2R segments.

Proposal 3: Do not support buffer based re-transmission for D2R segmentation.

Proposal 4: Buffer-less re-transmission can be discussed in work item.

Proposal 5: Msg1 and Msg3 segmentation are not supported, D2R segmentation is supported for D2R data only.

Proposal 6: Simple message size indicator can be considered for last segment indicator.

Message size reporting

Proposal 7: Message size reporting along with Msg 1 and Msg 3 are not supported.

Energy status reporting

Proposal 8: Energy status reporting is always under reader control.

AS ID

Proposal 9: Reader can either request device to generate and report a new ID or assign an ID to device via Msg 2, if random ID in Msg 1 has been reported by other devices.

Information visibility to network/reader

Proposal 10: RAN2 assumes that the A-IoT command type is visible to gNB, i.e., gNB is aware of read/write/disable.

Proposal 11: If no device number can be obtained from CN, RAN2 are suggested to support estimate the (approximate) number of devices in gNB or reader.

Device population estimation

Proposal 12: RAN2 should specify how reader estimates the number of target devices per service if it’s not provided by CN in stage-2.

Proposal 13: RAN2 is suggested to capture Annex A in TR as a candidate solution for device population estimation, which introduces a dedicated R2D message containing length of RN, target number and operation.

Proximity determination

Proposal 14: From RAN2’s perspective, both solution 1 and solution 2 for proximity determination can be covered by current inventory and command procedure. No additional impacts are identified.

Protocol stack

Proposal 15: No new L2 AS layer is needed for A-IoT protocol stack.

Proposal 16: RAN2 is suggested to capture the protocol stack in Figure 1 A-IoT topology 1.

Figure.1 Protocol stack for A-IoT topology 1

Capability

Observation 1: From operator’s perspective, deploying the same type of devices within a coverage area is conducive to resource allocation and device management.

Proposal 17: Pending discussion on capability reporting to work item.

Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

[2] RAN#105 meeting report.

[3] R2-2410315, Further discussion on ambient IoT random access, CMCC

Annex A (informative): Device population estimation method

Suppose there are x devices and m access occasion to estimate device population. Device can response in every access occasion independently with probability p, which can be done with a random number generator (RNG) used to randomly select access occasion. RNG should be able to generate uniformly distributed random number (RN). Reader can indicate the length of RN, target number and operation to device.

For every occasion, device generates a RN with length indicated by reader, then compares it to the target number. Only If the result matches the operation, device will send Msg1 without data. The following table is an example.

| Response probability | Length of random number | Target number | Operation |
| --- | --- | --- | --- |
| 0.125 | 3 | ‘000’ | ‘==’ |
| 0.25 | 2 | ‘00’ | ‘==’ |
| 0.375 | 3 | ‘010’ | ‘<=’ |
| 0.5 | 1 | ‘0’ | ‘==’ |
| 0.625 | 3 | ‘101’ | ‘<=’ |
| 0.75 | 2 | ‘00’ | ‘!=’ |
| 0.875 | 3 | ‘000’ | ‘!=’ |
| 2-k | k | ‘000…0’k zeros | ‘==’ |

Reader can count the number of empty occasions where no device responses, donated as e; successful occasions where only one tag response, donated as s; and collided occasions where more than one device response, donated as c. Then we have,

1=p+q,  p>0, q>0

m=e+s+c,  e>0, c>0,s>0

And,

Pe=qx≈em

Ps=1x∙p∙qx-1=x∙p∙qx-1≈sm

Pc=i=2x1x∙pi∙qx-i≈cm

Hence,

x1=logemlogq

Where x1 is a rough estimation on device population. To get a more precise value, c can be used since it is negatively correlated with e.

fx=1-Pc=Pe+Ps=qx-1∙q+x∙p≈1-cm,  x∈N+

It can be proven that fx is constant decreasing in its domain (see Annex B), thus, a numerical result can be obtained using the c method. Let x2 be the numerical result obtained using the bisection method. The more accurate estimated value of x can be the mean of  x1 and  x2.

x=x1+x22

Reader typically uses small p and/or large m when there are many devices, to get a more precise estimation value, reader can use a larger m.

Annex B (informative): Proof of monotonicity

Here is the proof of the monotonic decrease of fx in Annex A.

fx=Pe+Ps=qx-1∙q+x∙p,  x∈N+

Its differentiation,

f'x=qx-1∙lnq∙q+x∙p+p≤qx-1∙lnq+p,  x∈N+

And the sign of f'x is the same as,

gt=1-t+lnt,  t∈(0, 1)

Observing that,

limt→1-g(t)=0

And its differentiation,

g't=1t-1>0,  t∈(0, 1)

This completes the proof.