3GPP RAN WG2 Meeting #128                                                     	R2-2410679

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	8.2.2

Source:	HONOR

Title:	Discussion on functionality aspects for A-IoT

Document for:	Discussion and Decision

1. Introduction

This contribution discusses the functionality aspects for Ambient IoT, including AS ID handling, Energy indication and segmentation.

2 Discussion

2.1 AS ID handling

The AS ID will be used for D2R scheduling and R2D reception and the uniqueness should be guaranteed to avoid collision and incorrect response. Both the using random ID directly and partial upper layer device ID have the collision risk. For random ID, there may be collision between the ID generated in the current slot the and  the random ID generated before. Therefore, the AS ID should be assigned by the reader for the uniqueness. And the reader-assigned AS ID could be random ID received in Msg1 or ID generated by the reader.

Proposal 1: It is proposed that the Reader assigns the AS ID to guarantee the uniqueness, the AS ID could be random ID or new AS ID.

There are multiple R2D transmissions which could be used for AS ID assignment, for example A-IoT paging message, Msg2, Msg4 (if exists) or the following R2D message, which could be found in the figure below.

The validity period of AS ID is under discussion which depends on whether AS ID should be kept after power off and has not reached the consensus.

For one-shot AS ID, reader will recycle the AS ID after the procedure is completed or device is depleted. The AS ID should be assigned as early as possible to enjoy more benefits bought by AS ID. The possible R2D message is A-IoT Paging message and the AS ID could be assigned when paging message is used to trigger specific devices, e.g device ID is visible.

For long-term AS ID, the lifetime is quite long and there may not matter much to assign the ID earlier or later. Assigned via paging message is also one option that could be adopted for triggering specific devices, e.g, device ID is visible.

Observation 1: The AS ID should be assigned as early as possible to enjoy more scheduling benefits bringing by AS ID.

Observation 2: A-IoT paging message is generally the first message to start the procedure.

In some cases, the A-IoT paging message could trigger “full” or “group” of devices and it is complex for the reader to assign the AS ID for all of them in this phase. MSG2 is a good option for the ID assignment. This is because the MSG2 is always constructed when there is follow-up data and the reader could identify  one device after receiving RN16 and this makes assignment possible.

Observation 3: If A-IoT paging message aims to multiple devices without device ID, MSG2 could be used for AS ID assignment used for follow-up date transmission and reception.

Proposal 2: RAN2 to discuss and decide to assign AS ID via A-IoT paging message and MSG2 in RA procedure.

2.2 Energy bit indication

In last RAN2#127bis meeting [1], the below agreements on energy bit indication have been made.

Agreements on energy bit indication

Capture in the TR the option: the device may include energy status indication in D2R messages (e.g. MSG1,MSG3 and Command Response message), if the device can.  1 bit indication can be captured in the TR.    We will capture the use case in the TR.   FFS whether it is reader controlled.

1bit indication is one possible energy report option which could only indicate the energy is enough or not. It is useful to report when the energy is depleted. Then, the reader could stop scheduling the device on receiving the indication which could avoid resource wastage.

Observation 4: The reader could stop scheduling the device if it is unavailable due to power off, which could avoid resource wastage.

Considering complexity and capability, it is hard to do precise energy measurement and evaluate whether the energy is enough for following R2D reception. The 1bit indication should indicate the device will stop receiving the follow R2D message.

Proposal 3:  Energy bit indication indicates the energy is depleted and the device could not support following R2D reception.

Some devices could harvest energy in time or in some pattern and it is possible to predict when the energy will be enough for communication. The reader could suspend or terminate the procedure depending on whether the device could come back in time. For example, the reader will resume or re-start the procedure if the device could come back before the required latency for response.

Observation 5: The reader may continue or re-start the uncompleted procedure via re-transmitting the last message or re-paging the device if the device could complete energy harvesting in certain period.

If there is harvesting with regularity, the availability information could be sent to the reader by CN or the device if possible.

Proposal 4:  RAN2 to discuss whether and how the reader could know the availability of device again, e.g via energy harvesting duration or harvesting completed indicator from the CN or device side.

The energy measurement will also consume device’s energy and frequent measurement may lead to A-IoT service failure. As it is the reader to use the information indicated by the energy indicator, it should be controlled by the reader on when to report the information. For example, the reader could ask the device to report the energy bit to decide whether segmentation could be performed. Or if the network provides information to a device to determine available/unavailable duration, the energy measurement and report should not be needed.

Observation 6: Energy measurement will consume some energy and it should not be performed frequently.

Proposal 5:  The energy bit report should be network controlled.

2.3 Segmentation

Segmentation is one possible functionality on MAC which may depends on device capability, energy status and coverage etc. Generally, more segments imply multiple exchanges between device and reader which will consume more energy of A-IoT device. The energy depleted could happen during segments transmission phase which may lead to A-IoT service failure. Less segments could reduce the failure probability.

Proposal 6: It is proposed to divide the message into as less segments as possible, e.g., two segments to reduce energy consumption and A-IoT service failure.

From RAN1 perspective, a maximum TB size of around 1000 bits in PHY for R2D and D2R directions can be supported [2]. The reader could allocate resource for the complete D2R message if it knows the D2R message size. Otherwise, the reader will try to allocate resource blindly and may receive the first segments from the device. Then, the device could evaluate the left message size in the buffer.

To support less segments, it is beneficial for the device to report the left message size waiting for scheduling to reduce the number of segments.

Proposal 7: RAN2 assumes to report the left message size if segmentation is performed in the device.

The use case for R2D segmentation is not clear. Segmentation should be optional functionality and the reader could perform segmentation only after acquiring the capability which may increase device complexity caused by the capability report. In addition, some messages aim to multiple devices and it should not be segmented if any of them does not support segmentation.

Observation 7: The R2D message aims to multiple devices could not be segmented if any device does not support segmentation.

Since no sequence number and ARQ, the device could not know whether it has received the whole R2D message and this may lead to the reception of the A-IoT command failure.

Observation 8: Without ARQ and SN, the device will reassemble the R2D message incorrectly if there are segments lost which will lead to service failure.

Proposal 8: R2D segmentation should not be supported.

3 Conclusions

In this contribution, we analyze the possible A-IoT device functionalities and made the following observations and proposals:

Observation 1: The AS ID should be assigned as early as possible to enjoy more scheduling benefits bringing by AS ID.

Observation 2: A-IoT paging message is generally the first message to start the procedure.

Observation 3: If A-IoT paging message aims to multiple devices without device ID, MSG2 could be used for AS ID assignment used for follow-up date transmission and reception.

Proposal 2: RAN2 to discuss and decide to assign AS ID via A-IoT paging message and MSG2 in RA procedure.

Observation 4: The reader could stop scheduling the device if it is unavailable due to power off, which could avoid resource wastage.

Observation 5: The reader may continue or re-start the uncompleted procedure via re-transmitting the last message or re-paging the device if the device could complete energy harvesting in certain period.

Observation 6: Energy measurement will consume some energy and it should not be performed frequently.

Observation 7: The R2D message aims to multiple devices could not be segmented if any device does not support segmentation.

Observation 8: Without ARQ and SN, the device will reassemble the R2D message incorrectly if there are segments lost which will lead to service failure.

Proposal 1: It is proposed that the Reader assigns the AS ID to guarantee the uniqueness, the AS ID could be random ID or new AS ID.

Proposal 3:  Energy bit indication indicates the energy is depleted and the device could not support following R2D reception.

Proposal 4:  RAN2 to discuss whether and how the reader could know the availability of device again, e.g via energy harvesting duration or harvesting completed indicator from the CN or device side.

Proposal 5:  The energy bit report should be network controlled.

Proposal 6: It is proposed to divide the message into as less segments as possible, e.g., two segments to reduce energy consumption and A-IoT service failure.

Proposal 7: RAN2 assumes to report the left message size if segmentation is performed in the device.

Proposal 8: R2D segmentation should not be supported.

4. References

[1]	R2-24xxxx Report of 3GPP TSG RAN WG2 meeting #127bis

[2]	R2-2409405	Reply LS to RAN2 on data block sizes for Ambient IoT