3GPP RAN WG2 Meeting #128	R2-2410686

# Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	8.7.6

Source:	HONOR

Title:	Discussion on XR rate control

Document for:	Discussion and Decision

1. Introduction

In RAN#105 meeting, the following objective of Rel-19 XR work item was agreed [1]:

Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

In RAN2#127bis meeting, the following agreements on XR rate control have been achieved [2].

Agreements on XR rate control - FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc. - RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications: - Extend the Bit Rate field - Define a new bit rate table to provide sufficient granularity for XR traffic - Introduce new values for the bitRateMultiplier - Send LS to SA4 asking about range/granularity which is required

In this contribution, firstly we discuss the shortcomings of L4S and its relationship with recommended bit rate. Then we provide an analysis of the benefits of recommended bit rate based on QoS flow.

2. Discussion

2.2. XR rate control

In last meeting, RAN2 has agreed to analyse the relationship between XR rate control indication and L4S. In this contribution, we first discuss the basic procedure of L4S and its shortcomings. Then we give the proposal on how XR rate control indication interworks with L4S.

L4S is a rate adaption mechanism applied at the TCP/IP protocol. It exposes congestion information by marking ECN bits in the IP header of the user IP packets between the UE and the application server to trigger application layer rate adaption. ECN marking for L4S is enabled on a per QoS Flow. SMF decides whether NG-RAN based ECN marking for L4S is used. The following diagram depicts the rate adaption process based on L4S mechanism.

When gNB detects uplink congestion, it exposes congestion information to AF by marking ECN bits in the IP header. Then AF notifies UE the congestion information by marking ECN-Echo bit in the TCP header.

As UE lacks congestion degree information, rate adaptation can only be performed gradually until AF no longer indicates congestion information.

Observation 1: Rate adaption based on the L4S mechanism has the following drawbacks:

- reaction to rate adaption is slow.

- no accurate congestion information.

- only applicable to rate decrease adaption, not rate increase adaption.

Rate adaption based on rate control indication can solve these problems well. When gNB detects uplink congestion or congestion relief, it can indicate the recommended target bit rate directly to the UE.

In our view, both XR rate control indication and L4S are feasible solutions to handle congestion. They can be used in different scenarios. XR rate control indication can be used in cases which requires rapidly and more accurate congestion handling while L4S can be used in general congestion handling with less overhead. The gNB can choose the congestion control mechanism based on the SMF's indication or implementation for uplink congestion.

Proposal 1: The gNB can choose the rate adaption method based on the SMF's indication or implementation for uplink congestion.

XR services may comprise multiple different traffic flows, including video, audio, and haptic control, etc. It is possible that some of these traffic flows with similar QoS requirements are mapped to the same DRB. Right now, RAN2 is discussing whether the rate control indication should be per DRB or per QoS flow. In our view, if XR rate control indication is per DRB, UE cannot know which traffic flows need rate adaption and what is the recommended target bit rate for each traffic flow. In addition, rate adaption based on L4S is enabled on a per QoS Flow. Therefore, the rate control indication based on QoS Flow is preferred.

Proposal 2: Rate control indication based on QoS Flow is preferred.

3. Conclusions

This contribution provides the following proposals.

Observation 1: Rate adaption based on the L4S mechanism has the following drawbacks :

- reaction to rate adaption is slow.

- no accurate congestion information.

- only applicable to rate decrease adaption, not rate increase adaption.

Proposal 1: The gNB can choose the congestion control method based on the SMF's indication or implementation.

Proposal 2: Rate control indication that based on QoS Flow is preferred.

4. References

[1] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[2] Draft_RAN2_127b_Meeting_Report_v1