3GPP TSG-RAN WG2 Meeting #128	R2-2410689

Orlando, USA, Nov. 18th – 22nd , 2024

Agenda Item:	8.6.4

Source:	HONOR

Title: 	Discussion conditional LTM

Document for:	Discussion and decision

1	Introduction

For the conditional LTM, UE evaluates conditions for triggering LTM where the design of the evaluation condition could refer to that of event triggered L1 measurement reporting. The discussion about measurements related enhancements for purpose of supporting LTM was already started several meetings ago and the following agreements were made during last RAN2 meeting.

Agreements on C-LTM

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

In this contribution, we discuss the general concept of the conditional LTM like execution trigger events and the early UL synchronization. Then we propose some solutions/way-forwards accordingly.

- Discussion

During the previous discussion, RAN2 already confirmed that the event triggered L1 measurement is designed for LTM early synchronization and LTM cell switch procedure. Considering cell switch procedure of L3 cell switch, L3 measurement is the only pre-operation before the cell switch. L3 evaluation could be performed upon the measurement configuration. For L1/L2 triggered measurement, L1 measurement is one of the important parts in the preparation phase before the LTM cell switch execution. Unlike L3 measurement, L1 measurements could result in more signalling overhead and power consumption. Especially for the CSI-RS based measurement, dynamic activation/de-activation solution is introduced to reduce the overhead from perspectives of both NW and UE. It is not necessary to perform the L1 measurements for all candidates when the serving cell becomes good enough or for all RS types or when the candidate cell is not good enough.

Proposal 1: Event triggered L1 measurement should be designed for the one additional LTM purposes: trigger L1 measurement (e.g., CSI-RS based L1 measurement) for conditional LTM.

During L3 mobility like CHO, UE synchronizes to that candidate cell after detaching the source gNB. For the LTM with less latency, the UE needs to perform the early synchronization (DL and/or UL) before the LTM cell switch execution. During previous discussion, RAN2 already confirmed that the event triggered L1 measurement is designed for LTM early synchronization. This enables that the early synchronization could be obtained earlier than the cell switch by performing synchronization upon the satisfaction of an easily satisfied condition and executing the cell switch upon a stricter condition. What is more, we still need to discuss the early synchronization of C-LTM is triggered by the UE or the NW based on L1 measurement result. On one hand, we could reuse the PDCCH order triggered early TA requisition defined in Release 18 LTM with less specification impacts. But it is possible that UE has not received PDCCH order before it triggers the execution of the LTM based on the L1 measurement results. On the other hand, the early synchronization is also trigger by the UE based on the preconfigured L1 measurement event. For example, UE could send the preamble or perform the UE-based TA measurement with a lower threshold and triggered the execution with a higher threshold.

Observation: The UE could send the preamble for early TA acquisition or perform the UE-based TA measurement with a lower threshold and triggered the execution with a higher threshold to achieve RACH-less.

Proposal 2: In addition to the PDCCH order triggered, the early synchronization of C-LTM could be triggered by the UE based on L1 measurement results and the pre-configured condition.

For Rel-18 LTM, the TA is carried on the LTM switch command to trigger the execution. Since the execution for the conditional LTM is triggered by UE when the L1 measurement results meet the pre-configured condition, the TA needs to be sent to the UE ahead to enable the RACH-less switch. There are possible solutions to send the TA in advance mentioned during the initial phase during Release 18, for example 1) reuse the LTM command, 2) RAR, 3) MAC CE (e.g., new defined MAC CE or reuse the Timing advanced command).

Proposal 3: RAN2 to discuss how UE obtains the TA before the execution of the C-LTM, for example: 1) reuse the LTM command, 2) RAR, 3) MAC CE (e.g., new defined MAC CE or reuse the Timing advanced command).

With the early UL synchronization designed in Rel-18, UE could perform the RACH-less LTM on the triggered candidate. To avoid signalling and power overhead resulted from frequent TA acquisition, TA could be maintained in the UE after pre-acquisition before the execution and UE could use the corresponding TA when executing the switch to the candidate if the TA of this candidate is still valid.

Proposal 4: UE could maintain the TA for the candidates after receiving the TA.

3	Conclusions

In this contribution, we discussed the conditional LTM for Rel-19 mobility WI. Based on the discussion, the following observation and proposals are concluded:

Proposal 1: Event triggered L1 measurement should be designed for the one additional LTM purposes: trigger L1 measurement (e.g., CSI-RS based L1 measurement) for conditional LTM.

Observation: The UE could send the preamble for early TA acquisition or perform the UE-based TA measurement with a lower threshold and triggered the execution with a higher threshold to achieve RACH-less.

Proposal 2: In addition to the PDCCH order triggered, the early synchronization of C-LTM could be triggered by the UE based on L1 measurement results and the pre-configured condition.

Proposal 3: RAN2 to discuss how UE obtains the TA before the execution of the C-LTM, for example: 1) reuse the LTM command, 2) RAR, 3) MAC CE (e.g., new defined MAC CE or reuse the Timing advanced command).

Proposal 4: UE could maintain the TA for the candidates after receiving the TA.

4	References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Rel-19, Sep 9-12, 2024