3GPP TSG-RAN-WG2 Meeting #128	R2-2410704

Orlando, USA, 18th– 22nd Nov, 2024

Agenda item	:		8.13.2

Source	: 	Sharp

Title	: 	discussion on Relay discovery and (re)selection for multi-hop relay

Document for	:	Discussion and Decision

Introduction

In RAN2#127 meeting, RAN2 reached following agreements [1];

Agreements: - From RAN2 perspective, models A and B can both be supported. - Reuse existing AS discovery protocol to transmit discovery message for multi-hop U2N relaying. - Reuse SL-SRB4 to transmit discovery message for multi-hop U2N relaying. - Reuse existing resource pools defined for discovery message transmission and reception - Both of resource allocation mode 1 or mode 2 can be supported as in Rel-17/18 at least by relay UEs ; FFS mode 1 for remote UE. - Configuration can be provided by SIB/dedicated message or pre-configured as in Rel-17/18. - If the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell, the Remote UE can perform discovery transmission, as in Rel-17/18. - If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds). - FFS discovery conditions for the intermediate relay UEs.

In this paper, we discuss how to introduce multi-hop relay.

Discussion

According to the WID, RAN2 needs to discuss (A) framework of discovery and (re)selection, (C) SRAP and QoS handling, and (D) C-plane procedure. And RAN2 needs to consider forward compatibility while RAN2 discusses one additional hop relay. Considering future compatibility, the same term (e.g. intermediate relay UE) should be used for the new relays (both for first/second relay UEs), but for convenience, we will use the same term as the WID in this document.

In this document, we consider multi-hop systems as following the requirements specified in TS 22.261[2] (the list of KPIs is provided in the Appendix as an example). And we will consider the TR23.700-03[3] as baseline of the discussion.

Proposal 1. RAN2 will take into account the requirement specified in TS22.261 and the solutions and conclusion in TR23.700-03.

  - Discovery transmission

According to the SA’s potential solution, the discovery procedure has the following characteristics;

- Model A: discovery message is firstly transmitted by “last relay UE.” And, similar to U2U relay discovery, intermediate UEs forward the discovery to the remote UE.

- Model B: discovery message is firstly transmitted by “remote UE.” If first relay UE is last relay UE, first relay UE responses to discovery message directly, else, similar to U2U relay discovery, first relay UE forwards the discovery message to the other relay UE and then responses to discovery message when receives response message from the other relay UE.

Based on this, we need to consider following aspects;

- The condition under which the last relay UE initiates discovery Model A

- The condition under which the intermediate relay UEs forward discovery message for Model A

- The condition under which the remote UE initiates discovery Model B and/or relay (re)selection

- The condition under which the intermediate relay UEs forward discovery message for Model B

- The condition under which the last relay UE responds to discovery message for Model B

For discovery transmission conditions, RAN2 reached following agreements;

If the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell, the Remote UE can perform discovery transmission, as in Rel-17/18.

If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds).

FFS discovery conditions for the intermediate relay UEs.

For the condition under which the last relay UE initiates discovery Model A, it was already agreed as above, i.e. the condition is the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold.

For the condition under which the intermediate relay UEs forward discovery message for Model A, it is still FFS. The situation of intermediate relay UE is similar with U2U relay UE during model B discovery. Therefore, we can reuse the condition, i.e. intermediate relay UE forwards discovery message of model A if PC5 RSRP between UE transmitted the discovery message to the intermediate relay UE and itself is above a threshold.

Proposal 2. Intermediate relay UE forwards discovery message of model A if PC5 RSRP between UE transmitted the discovery message to the intermediate relay UE and itself is above a threshold.

For the condition under which the remote UE initiates discovery Model B, it was already agreed as above, i.e. the condition is the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell. Furthermore, we want to clarify that there is no PC5 RSRP condition of remote UE for discovery transmission since remote UE should be able to reselect other first relay UE to reduce a number of hops.

Proposal 3. Remote UE is not configured with PC5 threshold to limit model B discovery transmission.

For the condition under which the intermediate relay UEs forward discovery message for Model B, it is still FFS. As mentioned above, for the PC5 condition, RAN2 should reuse U2U relay mechanism, i.e. Intermediate relay UE forwards solicitation message of model A if PC5 RSRP between UE transmitted the solicitation message to the intermediate relay UE and itself is above a threshold.

Proposal 4. Intermediate relay UE forwards solicitation message of model A if PC5 RSRP between UE transmitted the solicitation message to the intermediate relay UE and itself is above a threshold.

For the Uu condition under which the intermediate relay UEs forward discovery message for Model A and B, it should be discussed. In the email discussion [4], RAN2 discussed whether the discovery condition for the intermediate relay UEs is needed. In LTE U2N relay, upper bound for relay UE was introduced to avoid wasteful relay UE operation since remote UE’s position should be cell edge or OOC. So, in accordance with this principle, upper-bound is needed. Some companies said that the threshold condition should not be configured to support relay behaviour while candidate intermediate relay UE is OOC or in non-relay-support cell. For OOC/non-relay-support cell case, the upper bound is not configured via SIB/dedicated RRC signalling. In this case, UE can act as intermediate relay UE (it might be new condition).

Observation 1. In LTE, Relay UE is configured with upper bound Uu threshold to avoid wasteful discovery transmission.

Proposal 5. Intermediate Relay UE is configured with upper bound Uu threshold to avoid wasteful discovery transmission.

For the condition under which the last relay UE responds to discovery message for Model B, it was discussed in the email discussion. In Rel-17/18, PC5 AS condition for the last relay UE (i.e. PC5 link quality between the remote UE and the last relay UE) can be evaluated by the remote UE since it was single-hop. However, in multi-hop case, the remote UE cannot evaluate PC5 AS condition for the last relay UE. For discovery model A, PC5 AS condition is NOT needed since intermediate relay UE which directly receives discovery announce message from the last relay UE can evaluate PC5 AS condition for the last relay UE when the intermediate relay UE determines whether to forward it. However, for discovery model B, PC5 AS condition is needed since there is no UE which evaluates PC5 link quality between the intermediate relay UE and the last relay UE if the relay UE doesn’t evaluate PC5 AS condition. This is similar idea with the agreement of R18 U2U relaying; “For Model-B discovery, after receiving a discovery message from a relay UE, a target remote UE transmits the discovery response message only if the PC5 RSRP between the target remote UE and the relay UE is above a configured threshold.” Therefore, we think PC5 AS condition is needed for Model B discovery at the last relay UE.

Proposal 6. Last relay UE responds to solicitation message if PC5 RSRP between UE transmitted the solicitation message to the last relay UE and itself is above a threshold.

Conclusion

Proposal 1. RAN2 will take into account the requirement specified in TS22.261 and the solutions and conclusion in TR23.700-03.

Proposal 2. Intermediate relay UE forwards discovery message of model A if PC5 RSRP between UE transmitted the discovery message to the intermediate relay UE and itself is above a threshold.

Proposal 3. Remote UE is not configured with PC5 threshold to limit model B discovery transmission.

Proposal 4. Intermediate relay UE forwards solicitation message of model A if PC5 RSRP between UE transmitted the solicitation message to the intermediate relay UE and itself is above a threshold.

Observation 1. In LTE, Relay UE is configured with upper bound Uu threshold to avoid wasteful discovery transmission.

Proposal 5. Intermediate Relay UE is configured with upper bound Uu threshold to avoid wasteful discovery transmission.

Proposal 6. Last relay UE responds to solicitation message if PC5 RSRP between UE transmitted the solicitation message to the last relay UE and itself is above a threshold.

Reference

- R2-24xxxxx, Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands

- TS22.261, Service requirements for the 5G system

- TR23.700-03, Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 3

- R2-24xxxxx, Report of [Post127][401][Relay] Multi-hop relay discovery and (re)selection (LG)

Appendix

- 7.7	KPIs for UE to network relaying in 5G system

In several scenarios, it can be beneficial to relay communication between one UE and the network via one or more other UEs. The functional requirements related to relaying can be found in clause 6.9.2. Performance requirements for relaying in different scenarios can be found in table 7.7-1.

Table 7.7-1: Key Performance for UE to network relaying

| Scenario | Max. data rate (DL) | Max. data rate (UL) | End-to-end latency (note 7) | Area traffic capacity (DL) | Area traffic capacity (UL) | Area user density | Area | Range of a single hop (note 8) | Estimated number of hops |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| InHome Scenario (note 1) | 1 Gbit/s | 500 Mbit/s | 10 ms | 5 Gbit/s/ home | 2 Gbit/s /home | 50 devices /house | 10 m x 10m – 3 floors | 10 m indoor | 2 to 3 |
| Factory Sensors (note 2) | 100 kbit/s | 5 Mbit/s | 50 ms to 1 s | 1 Gbit/s /factory | 50 Gbit/s /factory | 10000 devices /factory | 100 m x 100 m | 30 m indoor / metallic | 2 to 3 |
| Smart Metering (note 3) | 100 bytes / 15 mins | 100 bytes / 15 mins | 10 s | 200 x 100 bytes / 15 mins /hectare | 200 x 100 bytes / 15 mins /hectare | 200 devices /hectare | 100 m x 100 m | > 100 m indoor / deep indoor | 2 to 5 |
| Containers (note 4) | 100 bytes / 15 mins | 100 bytes / 15 mins | 10 s | 15000 x 100 bytes / 15 mins /ship | 15000 x 100 bytes / 15 mins /ship | 15000 containers /ship | 400 m x 60 m x 40 m | > 100 m indoor / outdoor / metallic | 3 to 9 |
| Freight Wagons | 100 bytes / 15 mins | 100 bytes / 15 mins | 10 s | 200 x 100 bytes / 15 mins /train | 200 x 100 bytes / 15 mins /train | 120 wagons /train | 1 km | > 100 m outdoor / tunnel | 10 to 15 |
| Public Safety (note 5) | 12 Mbit/s | 12 Mbit/s | 30 ms | 20 Mbit/s /building | 40 Mbit/s /building | 30 devices /building | 100 m x 100 m – 3 floors | > 50 m indoor (floor or stairwell) | 2 to 4 |
| Wearables (note 6) | 10 Mbit/s | 10 Mbit/s | 10 ms | 20 Mbit/s per 100 m2 | 20 Mbit/s per 100 m2 | 10 wearables per 100 m2 | 10 m x 10 m | 10 m indoor / outdoor | 1 to 2 |
| NOTE 1: Area traffic capacity is determined by high bandwidth consuming devices (e.g. ultra HD TVs, VR headsets), the number of devices has been calculated assuming a family of 4 members. NOTE 2: Highest data rate assumes audio sensors with sampling rate of 192 kHz and 24 bits sample size. NOTE 3: Three meters (gas, water, electricity) per house, medium density of 50 to 70 houses per hectare. NOTE 4: A large containership with a mix of 20 foot and 40 foot containers is assumed. NOTE 5: A mix of MCPTT, MCVideo, and MCData is assumed. Average 3 devices per firefighter / police officer, of which one video device. Area traffic based on 1080 p, 60 fps is 12 Mbit/s video, with an activity factor of 30% in uplink (30% of devices transmit simultaneously at high bitrate) and 15% in downlink. NOTE 6: Communication for wearables is relayed via a UE. This relay UE can use a further relay UE. NOTE 7: End-to-end latency implies that all hops are included. NOTE 8: 'Metallic' implies an environment with a lot of metal obstructions (e.g. machinery, containers). 'Deep indoor' implies that there can be concrete walls / floors between the devices. NOTE 9: All the values in this table are example values and not strict requirements. |  |  |  |  |  |  |  |  |  |