3GPP TSG-RAN WG2 Meeting #R2-128		R2-2410738 Orlando, USA, Nov 12-22, 2024

Agenda Item:	8.5.3

Source: 	Rakuten

Title:  	Discussion on on-demand SIB1 for NES

Document for:	Discussion and decision

# Introduction

RAN#105 approved the Rel-19 WI on enhancements of network energy savings for NR (RP-242354) and it includes the following objective highlighted below:

2. Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

In this contribution we discuss some important aspects associated with OD-SIB1, focussed on the above objective.

# Discussion

In this document, we refer to the following nomenclature.

- The gNB associated with the NES cell as NES cell gNB

- The gNB-CU associated with the NES cell as NES cell gNB-CU

- The gNB-DU associated with the NES cell as NES cell gNB-DU

- The gNB associated with the Cell A as Cell A gNB

- The gNB-CU associated with the Cell A as Cell A gNB-CU

- The gNB-DU associated with the Cell A as Cell A gNB-DU

## Introduction

RAN1#116 (February 2024) made the following assumptions for the purpose of discussion:

For discussion purpose, the following assumption will be used in RAN1

- Cell A: A cell that is periodically transmitting at least its own SIB1.

- NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

The cell A in the below example is a macro cell and the NES cell is a small cell which has at least partial overlapping coverage with the macro cell.

Figure 1: NES Cell and Anchor cell topography for OD-SIB1

## UL WUS configuration of NES cell in RRC Connected mode

UL WUS configuration of the NES cell is a must for a UE in order to request OD-SIB1 transmission from the NES cell. Provisioning of UL WUS configuration in IDLE/INACTIVE is already being specified.

Another use-case is where a UE obtains the UL-WUS configuration from the Cell A or NES cell directly where the UL-WUS configuration of the NES cell is provided to the UE when it was in RRC Connected before being moved to IDLE/INACTIVE.

This use-case could prove very useful as it ensures that the UE receives the UL WUS configuration already in RRC Connected and hence doesn’t have to visit Cell A for acquiring the UL WUS configuration of NES cell.

RAN2 discusses and agrees a solution to the use-case of providing UL WUS configuration to the UE while in RRC Connected mode.

## Detection and access of NES cell in Case 2(Option 1+B+X)

Case 2 was agreed to be considered for normative work and is defined below,

  - Option 1: UE transmits UL WUS to NES Cell

  - Option B: UE obtains the UL WUS configuration from Cell A

  - Option X: UE receives on-demand SIB1 from NES Cell

From a UE pov, before coming to NES cell, it has already performed a cell reselection to the cell A, as it could not find the SIB1 to camp/enter the NES cell. Moreover, the OD-SIB1 is a feature designed to enhance network energy savings. Hence, we think that a UE should be permitted to go to NES cell, only when really needed. This ensures that the energy savings gains are maximised. Some additional RRM criteria may be imposed before a UE is redirected to the NES cell in case 2. Some example criteria are,

- Cell A is overloaded with RRC IDLE/INACTIVE UEs.

- Radio conditions in Cell A is poor compared to NES cell.

- Radio condition in NES cell is better than a predefined threshold.

- UE is intending to move to RRC Connected for data transmission and NES cell can offer capacity.

These criteria could be predefined in RAN2, and the UE performs the RRM criteria check based on indication in Cell A SIB.

Once the UE satisfies the minimum criteria, it performs RACH, based on the UL-WUS configuration and requests OD-SIB1. The NES cell starts broadcasting OD-SIB1 and the UE can perform cell reselection.

When in IDLE mode, UE is camped on cell A before it can retrieve UL-WUS configuration of an NES cell.

RAN2 discusses and agrees the minimum criteria that a UE should satisfy in cell A, before requesting OD-SIB1 transmission from NES cell to camp there.

## OD-SIB1 transmission due to IDLE/INACTIVE UE overload in Cell A

When an overload condition is detected in Cell A with respect to IDLE/INACTIVE mode UEs, a percentage of IDLE/INACTIVE UEs have to be offloaded to the NES cells. Further, all new UEs coming to Cell A have to be re-directed to the NES cells until the overload condition eases.

When the IDLE/INACTIVE UEs overload condition is detected, the anchor cell gNB shall offload UEs to one or more NES cells having overlapping coverage with it, an exemplary topography of anchor cell with overlapping NES cell(s) is shown in Figure 1.

When an overload condition of IDLE/INACTIVE UEs is detected in Cell A, the Cell A gNB shall initiate offloading of IDLE/INACTIVE UEs to NES cells.

To execute this offloading, the Cell A modifies its SIB that broadcasts UL WUS to include the list of NES cell IDs to which offloading is intended. In addition to this, the cause “Overload_Based_Offloading” may also be indicated, so that the UEs can transmit a UL WUS to the NES cell and perform cell-reselection to NES cell after acquiring the information in the modified SIB. Cell A gNB sends a System Information Modification notification to all UEs camped on it to read the modified SIB. This enables the UEs to read the modified Cell A SIB, acquire UL-WUS configuration, evaluate the radio condition of NES cell, transmit UL WUS request and perform cell re-selection to NES cell.

Alternatively, if the Cell A gNB wants to take proactive action to just prevent new incoming UEs, it could send a backhaul signaling to the NES cell gNB to initiate OD-SIB1 transmission, so that all the incoming UEs could directly camp on the NES cell without coming to Cell A.

System Information Modification notification could be used in this case also, to trigger cell A UEs to move to NES cell.

During overload, Cell A gNB initiates System Info Modification to trigger camped UEs to read modified SIB, acquire UL-WUS configuration, evaluate the radio condition of NES cell, transmit UL WUS request and perform cell re-selection to NES cell.

The Cell A SIB shall indicate the list of NES Cell IDs selected for offloading UEs and the cause.

RAN2 asks RAN3 to discuss and agree a solution for Cell A gNB to request NES cell gNB to trigger OD-SIB1 transmission without a UL WUS.

## Synergizing OD-SIB1 transmission in an NES cell

RAN1#118 made the below agreements.

Agreement At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options: - Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission - Option 2: starting time and duration are indicated in the UL WUS configuration Agreement At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on reference time point to determine the window starting time, RAN1 to down select from the following two options: - Option 1: The reference time point is defined based on the RAR reception time of the UL-WUS transmission - FFS: Definition of RAR reception time - Option 2: The reference time point is defined based on the UL-WUS transmission time - Option 3: The reference time point is defined based on the RAR window of the UL-WUS transmission

OD-SIB1 transmission is based on UL wake-up signal directly sent to NES cell from individual UEs and there could be multiple UEs in the coverage area of an NES cell who are eligible to camp on the NES cell, waiting for OD-SIB1 transmission.

NES cell triggering OD-SIB1 transmission per UE is sub-optimal from energy savings perspective. Moreover, the other eligible UEs in IDLE/INACTIVE mode are not aware of OD-SIB1 transmission, as they may not try to perform cell re-selection at the same time.

NES cell triggering OD-SIB1 transmission per UE is sub-optimal from energy savings perspective.

Eligible UEs in IDLE/INACTIVE mode which are camped on Cell A are not aware of NES cell’s OD-SIB1 transmission.

To mitigate this, the NES cell may decide when to start the OD-SIB1 transmission. For example, the gNB could wait for a predefined time interval before initiating OD-SIB1 transmission. Internally, the gNB may initiate the OD-SIB1 transmission only after more UEs have sent UL wake-up signal to the NES cell.

This is supported by the RAN1 agreements highlighted above where the start time of the OD-SIB could be indicated to the UE requesting OD-SIB1 transmission. It doesn’t necessarily have to start immediately. Subsequently, after the predefined time interval has elapsed, the gNB may initiate the OD-SIB1 transmission at the NES cell.

Since the objective is to maximise energy saving gains, the NES cell gNB may indicate to the cell A gNB over backhaul to send SystemInfoModification notification to the rest of the IDLE/INACTIVE UEs camped on Cell A to notify them that OD-SIB1 transmission has been initiated, so that they can access the OD-SIB1 transmission without sending a UL WUS request.

When the OD-SIB1 transmission is planned to be initiated, the NES cell gNB shall notify the Cell A gNB about the NES Cell ID and its OD-SIB1 transmission. RAN2 asks RAN3 to discuss and agree a way forward for this issue.

The Cell A gNB shall send a SystemInfoModification notification to all the IDLE/INACTIVE UEs to inform them about the NES Cell ID and its OD-SIB1 transmission.

The IDLE/INACTIVE UEs on Cell A, who already have the WUS configuration of the given NES cell, may access OD-SIB1 of NES cell without sending a UL WUS request and perform cell re-selection to NES cell.

# Conclusion

We have the following observations:

Observation 1	When in IDLE mode, UE is camped on cell A before it can retrieve UL-WUS configuration of an NES cell.

Observation 2	NES cell triggering OD-SIB1 transmission per UE is sub-optimal from energy savings perspective.

Observation 3	Eligible UEs in IDLE/INACTIVE mode which are camped on Cell A are not aware of NES cell’s OD-SIB1 transmission.

We have the following proposals:

Proposal 1	RAN2 discusses and agrees a solution to the use-case of providing UL WUS configuration to the UE while in RRC Connected mode.

Proposal 2	RAN2 discusses and agrees the minimum criteria that a UE should satisfy in cell A, before requesting OD-SIB1 transmission from NES cell to camp there.

Proposal 3	When an overload condition of IDLE/INACTIVE UEs is detected in Cell A, the Cell A gNB shall initiate offloading of IDLE/INACTIVE UEs to NES cells.

Proposal 4	During overload, Cell A gNB initiates System Info Modification to trigger camped UEs to read modified SIB, acquire UL-WUS configuration, evaluate the radio condition of NES cell, transmit UL WUS request and perform cell re-selection to NES cell.

Proposal 5	The Cell A SIB shall indicate the list of NES Cell IDs selected for offloading UEs and the cause.

Proposal 6	RAN2 asks RAN3 to discuss and agree a solution for Cell A gNB to request NES cell gNB to trigger OD-SIB1 transmission without a UL WUS.

Proposal 7	When the OD-SIB1 transmission is planned to be initiated, the NES cell gNB shall notify the Cell A gNB about the NES Cell ID and its OD-SIB1 transmission. RAN2 asks RAN3 to discuss and agree a way forward for this issue.

Proposal 8	The Cell A gNB shall send a SystemInfoModification notification to all the IDLE/INACTIVE UEs to inform them about the NES Cell ID and its OD-SIB1 transmission.

Proposal 9	The IDLE/INACTIVE UEs on Cell A, who already have the WUS configuration of the given NES cell, may access OD-SIB1 of NES cell without sending a UL WUS request and perform cell re-selection to NES cell.

# Reference

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.

RAN1 Chairman Notes : RAN1 #116bis