3GPP TSG-RAN WG2 Meeting #128	R2-2410752

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda item:	8.6.2

Source:	Samsung

Title:	Further Considerations to Support Inter-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

Introduction

In RAN2#127 meeting, RAN2 reached the following agreements for inter-CU LTM procedure and some details:

Agreements on inter-CU LTM

- The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

- For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

- The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

- Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline:

- The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part.

- FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

- The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message.

- Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed.

- The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration.

- The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part.

- The MN can request an SCG reference configuration from any of the involved SNs.

5.	For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN.

6.	The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells.

7.	The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN.

8.	In order to support subsequent inter-CU SCG LTM, the MN needs to transfer, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

9.	Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message.

10.	Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM.

11.	RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

In this contribution, we will further address inter-CU LTM in details.

Discussion (non-DC part)

Issue 1: Security aspects

The Reply LS from SA3 (S3-244316) is given as below:

1 Overall description SA WG3 thanks RAN WG2 for the LS on the security handling for inter-CU LTM in non-DC cases. SA WG3 has been considering the security mechanisms proposed for the inter-CU LTM feature. For this feature, SA WG3 want to preserve the principle that when there is a PDCP anchor change, then it is necessary to perform a key change (i.e. for solutions or handovers that do not require a PDCP anchor change, there is no need to provide rekeying). The rekeying method preferred by most companies in SA WG3 is that after every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signalling to be used by the UE for key derivation at the next inter-CU LTM cell switch. This solution was preferred as it provides the NCC value using the protected RRC signalling and hence is considered to be secure. SA WG3 understands that, RAN WG2 is aiming to avoid RRC configuration/signalling between cell switches for subsequent LTM procedures. If it is necessary for the RAN WGs to avoid RRC signalling between cell switches, then SA WG3 can analyse further methods like, - Security threat of the unprotected MAC CE to deliver the NCC value. - Sending NCC value per candidate cell via RRC message during LTM preparation phase. SA WG3 will keep RAN WG2 and RAN WG3 informed as we make further progress on this topic.

From the above reply LS, we can have the following observations:

- NCC over MAC CE is a concern from SA3 point of view

- NCC over RRC signalling for each inter-CU LTM cell switch is the preference of most companies in SA3. However, it is not aligned with the intention of subsequent LTM

- Depending on RAN2’s response, SA3 can further evaluate the solutions on the table.

According to the above observations, RAN2 may need further address followings:

- The security issue on NCC over MAC CE

NCC is just a number from 0 to 7, and the security key cannot be known solely from NCC. Moreover, from security point of view, LTM cell switch MAC CE already includes several information related to the LTM execution.  It is better to further evaluate the security threat of the unprotected MAC CE to deliver the NCC value.

Proposal 1-1: RAN2 is kindly asked SA3 to further evaluate the security threat of delivering NCC over MAC CE.

- Willingness of supporting subsequent LTM without additional signalling for inter-CU LTM

RAN2 already agreed to support subsequent LTM for inter-CU LTM. As a result, both RAN2 and RAN3 made several agreements to realize this feature.

RAN2 agreement

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

RAN3 agreements

−	Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM.

-	Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

−	Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM

Moreover, even if NCC over MAC CE has security issue, there are still other ways to resolve this concern without using RRC signalling, e.g.,

    - using NCC index, e.g., using specific index for each NCC value, which is configurable via RRC, and MAC CE only includes such NCC index

    - using delta value of NCC reference value, e.g.,  reference NCC is configured to the UE via RRC, and the MAC CE only includes a delta value towards such reference

Thus, it is better to keep the benefit of subsequent LTM.

Proposal 1-2: RAN2 is kindly asked to confirm that the support of subsequent LTM should avoid the RRC signalling between cell switch.

- Solution analysis

According to  reply LS, the solutions on the table can be further categorized, i.e.,

| Category | Mapped RAN2’s options | Pros. and Cons. |
| --- | --- | --- |
| Category A: NCC over MAC CE | Option 1A: the NCC value to be used at inter-CU LTM execution is included in the LTM cell switch command MAC CE. | Standard impact: - NCC over MAC CE - NCC value from gNB-CU to gNB-DU whenever a new NCC is from the CN - UE behavior of applying new NCC Pros.: simple, and well-support the subsequent LTM without RRC signaling Cons.: potential security concern from SA3 point of view |
| Category B: NCC over RRC signalling for each cell switch | Option 4: After every inter-CU LTM cell switch execution, the UE is provided via RRC signalling with the NCC value to be used by the UE for key derivation at the next inter-CU LTM cell switch. | Standard impact: - RRC signaling for NCC notification - UE behavior of applying new NCC Pros.: no security concern for NCC delivery from SA3 point of view Cons.: RRC signaling during subsequent LTM |
| Category C: pre-configure a list of NCC | Option 1B: the UE is preconfigured with a list of NCC values in a ciphered and integrity protected RRC message and the index of an NCC value in the list is included in the LTM cell switch command MAC CE. | Standard impact: - CN signaling enhancement, e.g., path switch procedure needs provide a list of NCC - NCC list distribution among different candidate gNBs - UE behavior of applying new NCC Pros.: no RRC signaling for NCC update during subsequent LTM Cons.: size limitation of NCC. Specifically, NCC only has a number from 0 to 7. The preconfigured NCC list may not be sufficient for candidate cell configurations and any L3 HO after LTM configuration. |
|  | Option 2: Similar to Rel-18 S-CPAC key update mechanism, the UE is preconfigured from the source gNB with a list of NCC values per CU using RRC signalling (that is both integrity protected and ciphered). It is expected that the participating gNBs (CUs) would need to be aware of the list and how the UE applies the list during LTM cell switches: Option 2A: UE chooses the first unused NCC for the target CU upon inter-CU LTM execution. Option 2B: As an alternative to choosing the next unused NCC (as in option 2A), horizontal key derivation is used in this option if the LTM cell switch is between the same two CUs. |  |
|  | Option 3: After the execution of inter-CU LTM cell switch, the participating gNBs are expected to be updated with new K-gNB* to be used for the next inter-CU LTM cell switch. The UE and CN are aware of how the UE would use the next NCC value. Option 3B: UE is pre-configured by the CN (via source gNB RRC signalling) with a list of NCC values and the UE chooses the first unused NCC value as the next NCC value. |  |

From RAN2 point of view, the above solutions can be further analysed considering the standard impact. RAN2 may provide some suggestions on those options for SA3 further analysis.

Proposal 1-3: RAN2 is kindly asked to further analyse the solutions on the table, and give suggestions to SA3 among the following three categories (if possible):

- Category A: NCC over MAC CE (Option 1A)

- Category B: NCC over RRC signalling between LTM cell switch (Option 4)

- Category A: pre-configure a list of NCC (Option 1B/2A/2B/3B)

Issue 2: fast failure recovery

The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

For failure case, the following four cases can be considered (as shown in Fig. 1):

- Case 1 (addressed in last meeting): failure after intra-CU cell switch (e.g., T304 expiry or the RLF shortly after successful cell switch), and the selected cell is an intra-CU cell w.r.t. the source cell

- Case 2: failure after inter-CU cell switch (e.g., T304 expiry or the RLF shortly after successful cell switch), and the selected cell is an intra-CU cell w.r.t the target cell

- Case 3: failure after intra-CU cell switch (e.g., T304 expiry or the RLF shortly after successful cell switch), and the selected cell is an inter-CU cell w.r.t. the source cell

- Case 4: failure after inter-CU cell switch (e.g., T304 expiry or the RLF shortly after successful cell switch), and the selected cell is an intra-CU cell w.r.t the source cell

Fig. 1 failure cases

Case 1 has been addressed by the above agreements. To apply the Rel-18 handling on failure, the UE has to determine whether the selected cell is an intra-CU cell or not. The new Rel-19 ID can be used for this purpose. Specifically, if the new Rel-19 ID of the selected cell is same as the one of the source cell (for intra-CU LTM cell switch failure) or the target cell (for inter-CU LTM cell switch failure), Rel-18 handling on failure can be applied.

Proposal 1-4: during failure recovery procedure, the UE uses the new Rel-19 ID to determine whether the selected cell is an intra-CU cell or not.

According to the current Rel18 procedure, after failure inter-CU LTM cell switch, the UE will revert back to the configuration of source PSCell, including the security key. Thus, for case 2~4, the UE will revert back to use source cell security key. Among those cases, if “revert back” operation is applied, the fast recovery can be used for case 4. On the other hand, if the “revert back” operation is not applied (i.e., after LTM cell switch, the UE keeps the configuration at the candidate cell side), the fast recovery can be used for case 2. RAN2 can discuss the fast recovery support for case 2 and case 4.

Proposal 1-5: RAN2 is kindly asked to discuss whether and how to support the fast recovery for the following two cases:

- Case 2: select an intra-CU cell w.r.t the target cell failure after inter-CU cell switch failure (to support fast recovery, the UE needn’t revert back to the source cell configuration after failure)

- Case 4: select an intra-CU cell w.r.t the source cell after inter-CU cell switch failure (to support fast recovery, the UE needs revert back to the source cell configuration after failure, as Rel-18 handling)

For case 3, the security key update is necessary during the failure recovery procedure, and two options can be considered:

- Option 1: wait for SA3 inputs

- Option 2: do not perform fast recovery, and the legacy re-establishment procedure is applied

Since the security key update results in the interruption, the benefit of fast recovery becomes small. Moreover, SA3 is still working on security key update solutions. Thus, it is better to use the legacy re-establishment procedure.

Proposal 1-6: For Case 3 (i.e., selecting inter-CU cell after intra-CU LTM failure), RAN2 is kindly asked to agree that the legacy re-establishment procedure is used for failure recovery.

Issue 3: Early Sync RACH resource allocation

During Rel-18 discussion, RAN2 made the following agreement

- RAN2#121bis-e From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order). - RAN2#123-bis RAN2 understands that the source DU needs to know the early RACH configuration for each candidate cell, so that the source cell can know how to set the PDCCH order information for early RACH; The candidate DU provides the TA value and its associated information to the source DU via the CU, e.g. preamble index, RO information (i.e. RA-RNTI) and candidate cell identity, so that the source DU can identify the UE. RAN3 can design the necessary network signalling.

The above agreements indicated that the early sync RACH resource allocated by one candidate cell is shared by multiple UEs. This sharing method can save the RACH resource at each LTM candidate cell. Following this RAN2 agreement, RAN3 designed that the early sync resource is shared among UEs in the same gNB-DU. Thus, the early sync. RACH resource at each candidate cell is allocated per gNB-DU, as shown by an example in Fig. 2. Specifically, the Cell1 allocates three early sync. RACH resource sets, i.e.,

- Set0: used for the LTM cell switch from Cell0 to Cell1

- Set1: used for the LTM cell switch from Cell2 to Cell1

- Set2: used for the LTM cell switch from Cell3 to Cell1

Fig. 2 Early sync. RACH resource sharing per gNB-DU

In Rel-19, the inter-CU LTM has the similar issue, i.e., whether the early sync. RACH resource is shared among multiple UEs or not. The motivation in Rel18 on shared resource is also applicable for inter-CU LTM case.

Proposal 1-6: RAN2 is kindly asked to agree that the early sync. RACH resource of each LTM candidate cell is shared among multiple UEs for inter-CU LTM.

To realize the resource sharing, two options can be considered, i.e., per gNB-DU sharing or per gNB sharing. However, this should be discussed in RAN3 since either options have RAN3 impact. RAN2 can make decision on the sharing early sync. RACH resource, and send the LS to RAN3, then RAN3 handles the detailed solutions.

Proposal 1-7: if RACH resource sharing is agreeable, an LS to RAN3 can be prepared.

Discussion (DC part)

Inter-CU SCG LTM

Issue 1: candidate configuration and reference configuration for inter-CU SCG LTM

In last meeting, the following agreements were reached:

- The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part. - The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration. - The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part.

According to the highlighted part, it can be understood that the reference configuration for inter-CU SCG LTM can include the MCG part, which is up to NW implementation. Consequently, the candidate configuration can also include the MCG part, which is up to NW implementation. The UE generates the configuration based on the received reference configuration and the candidate configuration, which contains both MCG part and SCG part.

Proposal 2-1: for inter-CU SCG LTM, RAN2 is kindly asked to agree the followings:

- Up to NW implementation, the reference configuration can include the MCG part

- Up to NW implementation, the candidate configuration can include the MCG part

- For each candidate PSCell, the generated configuration at UE side contains both MCG part and SCG part

Inter-CU MCG LTM

Issue 1: supported scenario

According to the WID, the inter-CU MCG LTM needs consider the following scenarios

- As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

“SN unchanged” indicates that the PSCell can be changed as long as SN is kept the same.

Proposal 2-2: RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

In last meeting, some companies raised to support inter-CU MCG LTM with SN addition. This scenario is not in the scope of WID. If this scenario can be supported for free, it is OK to include this. Otherwise, RAN2 needn’t address this scenario in Rel19.

Proposal 2-3: RAN2 is kindly asked to support inter-CU MCG with SN addition if it does not introduce additional specification impact.

Issue 2: reference configuration for inter-CU MCG LTM

Similar to no-DC case, the reference configuration including MCG part can be configured to the UE. The question is whether the reference configuration can include SCG part or not. Since the SN is unchanged, when preparing the SCG part, the SN can evaluate whether the reference configuration for SCG part can save signalling or not. Thus, SN can determine whether the reference configuration for SCG is needed or not. If it is needed, SN can provide the reference configuration to MN directly (without request from MN), and the MN can configure the reference configuration including both MCG and SCG part.

Proposal 2-4: For inter-CU MCG LTM with unchanged SN, RAN2 is kindly asked to agree the followings:

- The reference configuration can include the SCG part

- During inter-CU MCG LTM preparation, the SN can determine whether to provide the reference configuration of SCG part to the MN or not.

Issue 3: candidate configuration for inter-CU MCG LTM

For inter-CU MCG LTM, the SCG configuration may be changed for different candidate PCells. Thus, NW can determine to include the SCG part in the candidate configuration. For each candidate PCell, the UE can generate the configurations including both MCG and SCG parts.

Proposal 2-5: For inter-CU MCG LTM with unchanged SN, RAN2 is kindly asked to agree the followings:

- Up to NW implementation, the candidate configuration can include the SCG part

- For each candidate PCell, the generated configuration at UE side contains both MCG part and SCG part

Conclusion

- Non-DC

Security-related:

Proposal 1-1: RAN2 is kindly asked SA3 to further evaluate the security threat of delivering NCC over MAC CE.

Proposal 1-2: RAN2 is kindly asked to confirm that the support of subsequent LTM should avoid the RRC signalling between cell switch.

Proposal 1-3: RAN2 is kindly asked to further analyse the solutions on the table, and give suggestions to SA3 among the following three categories:

- Category A: NCC over MAC CE (Option 1A)

- Category B: NCC over RRC signalling between LTM cell switch (Option 4)

- Category A: pre-configure a list of NCC (Option 1B/2A/2B/3B)

Failure handling:

Proposal 1-4: during failure recovery procedure, the UE uses the new Rel-19 ID to determine whether the selected cell is an intra-CU cell or not.

Proposal 1-5: RAN2 is kindly asked to discuss whether and how to support the fast recovery for the following two cases:

- Case 2: select an intra-CU cell w.r.t the target cell failure after inter-CU cell switch failure (to support fast recovery, the UE needn’t revert back to the source cell configuration after failure)

- Case 4: select an intra-CU cell w.r.t the source cell after inter-CU cell switch failure (to support fast recovery, the UE needs revert back to the source cell configuration after failure, as Rel-18 handling)

Proposal 1-6: For Case 3 (i.e., selecting inter-CU cell after intra-CU LTM failure), RAN2 is kindly asked to agree that the legacy re-establishment procedure is used for failure recovery.

RACH resource sharing

Proposal 1-6: RAN2 is kindly asked to agree that the early sync. RACH resource of each LTM candidate cell is shared among multiple UEs for inter-CU LTM.

Proposal 1-7: if RACH resource sharing is agreeable, an LS to RAN3 can be prepared.

- DC

Inter-CU SCG LTM

Proposal 2-1: for inter-CU SCG LTM, RAN2 is kindly asked to agree the followings:

- Up to NW implementation, the reference configuration can include the MCG part

- Up to NW implementation, the candidate configuration can include the MCG part

- For each candidate PSCell, the generated configuration at UE side contains both MCG part and SCG part

Inter-CU MCG LTM

Proposal 2-2: RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

Proposal 2-3: RAN2 is kindly asked to support inter-CU MCG with SN addition if it does not introduce additional specification impact.

Proposal 2-4: For inter-CU MCG LTM with unchanged SN, RAN2 is kindly asked to agree the followings:

- The reference configuration can include the SCG part

- During inter-CU MCG LTM preparation, the SN can determine whether to provide the reference configuration of SCG part to the MN or not.

Proposal 2-5: For inter-CU MCG LTM with unchanged SN, RAN2 is kindly asked to agree the followings:

- Up to NW implementation, the candidate configuration can include the SCG part

- For each candidate PCell, the generated configuration at UE side contains both MCG part and SCG part

References