3GPP TSG-RAN WG2 Meeting #R2-128		R2-2410763 Orlando, USA, Nov 12-22, 2024

Agenda Item:	8.6.2

Source: 	Rakuten

Title:  	LTM in DC scenarios

Document for:	Discussion, Decision

# Introduction

In the WID [1], the following objective is defined for inter-CU LTM:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - As secondary priority, support the case when NR-DC is configured and CU is acting as SN and MCG is unchanged - As secondary priority, support the case when NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released - Note: The case that LTM is configured in both MCG and SCG is excluded - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

The WID objective includes configuring LTM for UEs in Dual Connectivity.

In this contribution, we describe LTM failure in a DC setup and discuss a reliable and fast mechanism to recover from LTM failure.

# Discussion

## Simultaneous LTM in MN and SN

Agreements on inter-CU LTM: (R2#126)

Clarification of DC

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

Figure 1: Simultaneous LTM in MN and SN

It was agreed in Rel18 that intra-CU LTM can be configured at both MN and SN simultaneously without needing any further co-ordination. Subsequently, in R2#126, it was agreed that there is no restriction on intra-CU LTM configuration at MN and SN simultaneously. In intra-CU LTM, change of security keys was not permitted. From [4], it entails that intra gNB inter gNB-CU-UP relocation is another use-case where security key change has to be supported inside the same gNB.

Consider Figure 1, where there is a MN with 2 gNB-DUs and SN with 5 gNB-DUs. All intra SN PSCell changes except the one in RED (SN1-DU3) are possible to be executed autonomously. In the case where PCell undergoes an inter-DU LTM inside the same MN, it is possible that the source and target MN-DUs are served by different CU-UPs and hence needing a security key change.

Moreover, there is a possibility of frequencies, band combinations etc being different at the source and target MN-DUs as well. We think that there are multiple ways of addressing this issue.

- Option 1: To adapt the same solution that RAN2 agrees for inter-CU LTM with security key change.

- Option 2: To exchange the negotiated MN and SN configurations corresponding to their respective DUs/cells during F1 setup.

- Option 3: To negotiate the MN and SN configurations corresponding to their respective DUs/cells during UE’s candidate cell preparation.

RAN2 to discuss the impacts of security key change on SN due to Intra-MN LTM and agree a way forward to address the issue.

## LTM Failure in Dual Connectivity

The dual connectivity can be configured to a UE in two variants as shown below.

Figure 2: NR-DC over Xn (Variant 1)

Figure 3: NR-DC over F1(Variant 2)

NR-DC can be configured over Xn where MN and SN roles are assigned to two different gNBs and the Pcell configured in the MN gNB belongs to Master Cell Group (MCG) and the PSCell configured in the SN belongs to the Secondary Cell Group (SCG). NR-DC can also be configured over F1 where MN and SN roles are assigned to the same gNB. The Pcell belongs to one gNB-DU and that belongs to Master Cell Group (MCG) and the PSCell may belong to the same or a different gNB-DU and belongs to the Secondary Cell Group (SCG).

The MCG is configured with SRB 0/1/2 and SCG is configured with SRB3.

In case of LTM cell switch failure, following is the behaviour (from TS 38.300)

-	When initial LTM execution attempt fails or HO fails, the UE performs cell selection and if the selected cell is an LTM candidate cell and if network configured the UE to try LTM after LTM execution failure, then the UE attempts LTM execution once, otherwise re-establishment is performed.

This implies the following in our understanding

- Whenever there is an LTM cell switch failure, the UE performs a cell selection based on the prevalent radio conditions. If the selected cell happens to be an LTM candidate cell, the UE performs another LTM execution, post failure (only if the network has configured).

- This is attempted only once and if failure occurs again, rrc re-establishment is initiated.

- RRC re-establishment will mean both MCG and SCG bearers will have to be released and set-up again.

The LTM failure recovery is autonomous in nature and doesn’t require an LTM switch command (DL MAC CE) from the network.

In the context of NR-DC the LTM failure recovery is possible to be addressed differently than what is specified in R18. In a DC setup, unless there is a simultaneous MCG and SCG failure, MCG SRB is available whenever there is an SCG LTM failure and SCG SRB is available whenever there is an MCG LTM failure.

Barring a simultaneous MCG and SCG failure, MCG SRB is available whenever there is an SCG LTM failure and SCG SRB is available (if SRB3 is configured) whenever there is an MCG LTM failure.

Since the whole objective of an LTM failure recovery is to avoid RRC re-establishment, the active and available radio link can be used to recover the failed link i.e after the LTM recovery attempt as specified in R18, instead of triggering RRC re-establishment, the active SRB (MCG SRB in case of SCG LTM failure and SCG SRB in case of MCG LTM failure) could be configured to be used by the UE to send the latest measurement reports to the gNB-CU. This could be forwarded to the MN/SN which underwent an LTM failure. The MN/SN that underwent failure could prepare a new candidate cell based on the latest measurement reports and send the new LTM candidate cell configuration to the UE using the active radio link. The LTM cell switch command could also be sent together with the new LTM candidate cell configuration as well.

Post LTM failure and recovery, the latest measurement results are sent from UE to the gNB-CU which are forwarded to the MN/SN that underwent failure.

Latest measurement results are used to prepare a fresh LTM candidate/target cell.

The newly prepared LTM candidate/target cell configuration and the LTM cell switch command for the new candidate/target cell are delivered to the UE using an RRC message over the active radio link.

The UE performs an LTM cell switch to the newly prepared candidate/target cell and applies the candidate/target cell configuration delivered using the RRC message. Thus, RRC re-establishment is avoided, and the UE recovers from LTM failure.

# Conclusion

We have the following observations:

Observation 1	Barring a simultaneous MCG and SCG failure, MCG SRB is available whenever there is an SCG LTM failure and SCG SRB is available (if SRB3 is configured) whenever there is an MCG LTM failure.

We have the following proposals:

Proposal 1	RAN2 to discuss the impacts of security key change on SN due to Intra-MN LTM and agree a way forward to address the issue.

Proposal 2	Post LTM failure and recovery, the latest measurement results are sent from UE to the gNB-CU which are forwarded to the MN/SN that underwent failure.

Proposal 3	Latest measurement results are used to prepare a fresh LTM candidate/target cell.

Proposal 4	The newly prepared LTM candidate/target cell configuration and the LTM cell switch command for the new candidate/target cell are delivered to the UE using an RRC message over the active radio link.

# References

RP-240299, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #103, Maastricht, Netherlands, March 18-21, 2024

RAN2 Chairman Notes R2_125bis_ChairNotes

3GPP TS 38.300. NR; NR and NG-RAN Overall Description; Stage 2 (Release 18)

R2-2408867: Initial considerations for inter-CU LTM