3GPP TSG-RAN WG2 Meeting #128	R2-2410796

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.3.2.1

Source:	Nokia

Title:	On the RRM measurement prediction aspects

WID/SID:	FS_NR_AIML_Mob- Release 19

Document for:	Discussion and Decision

# 1	Introduction

In the previous RAN2 meetings, companies presented and exchanged their views on the highly prioritized scenarios for the RRM measurement prediction use-case. In this contribution, we present our views on the intra-frequency spatial domain prediction, L3 filtering for RRM prediction and corresponding configuration aspects.

# 2	Discussion

## 2.1	FR2 to FR2 Intra-Frequency Spatial Domain Evaluation

One of the sub-use-cases identified in 3GPP TR 38.774 study item on AI/ML for inter-cell mobility in NR is intra-cell measurement prediction, which is an extension of the TR 38.843 beam prediction use case to intra-cell mobility.

Figure 2.1-1: 3GPP TS 38.300 Measurement Model

## 2.1.1 Simulation setup

In RAN2#126, the following was agreed.

10 Methodology of Intra_F_C_S: Intra-frequency intra-cell spatial domain prediction is done by measuring sub set of configured SSB as input to the model to predict L3 cell level measurements for every instance of the same cell. It is only evaluted for FR2 intra-frequency scenario and RRM sub case 1 and 3. Several measurement reduction rates should be aligned among company without defining detail pattern. The detail rate values are FFS.

In the agreed Case 1, an ML model is trained to predict the skipped beam-level measurements (A or A1) and then the cell-level filtered measurement is derived with the TS 38.300 baseline method. In Case 3, either B or C is directly predicted from the subset of beam-level measurements (A or A1). In case the unfiltered cell level measurement B is predicted with ML, the baseline layer 3 filter is applied in a separate step to get the layer 3 cell measurement C. Table 2.1.1-1 shows the simulation assumptions used in the evaluation of the different cases.

Table 2.1.1-1: Simulation parameters for the 7-site scenario based on TR38.843

| Reported simulation assumptions | UE trajectory option | Option 3 |
| --- | --- | --- |
|  | UE trajectory boundary processing option | Option 3 |
|  | UE speed | 30/60/120 km/h |
|  | SoftLOS | Yes |
|  | Spatial consistency option | Option B |
|  | Number of TX beams | 32 |
|  | Number of RX beams | 4 |
|  | Measurement reduction rate | 50% |
| Data Size (Number of Samples) | Training | 360K |
|  | Testing | 180K |
| AI/ML model input/output | Model input | Reduced set of filtered L1 measurements |
|  | Model output | Filtered L3 measurements |
| AI/ML model description | Model type | Random Forests |
|  | Model complexity in number of parameters | 0.3M |

## 2.1.2 Simulation results

In the simulation results of our RAN127bis contribution [1], we have shown that non-ML approach (take max) outperforms ML-based random forests for spatial domain measurement reduction. In addition to having a lower computational complexity, such models are also simpler to deploy and manage when compared to deep learning models with hundreds of thousands of parameters.

Observation 1: For spatial domain measurement reduction, low complexity ML models yield a competitive performance relative to more complex models (e.g., random forest).

In the simulation results mentioned above, we performed the beam measurement reduction by measuring only a fixed subset of beams (e.g. measuring only the even-indexed beams at each timestep). This subset of measured beams is "fixed" in the sense that the same beams are measured at every time step. We may change how to define the subset (e.g. measuring only the even-indexed or odd-indexed beams, every third or every fourth beam, etc.), but once the subset is defined, it does not change from one timestep to the next.

However, our evaluation results have shown that less prediction error can be achieved by allowing this subset of measured beams to change over time in a way that increases the probability that this subset contains the best beam. Since the UE’s best beam index can change over time, it is more reasonable to allow the measured subset of beams to change over time as well, so as to make sure the best beam is contained in the subset in as many timesteps as possible, since the L3 measurement is derived from the measurement of the best L1 beam.

To evaluate this observation, we run the simulation scenario in Table 2.1.1-1, where the measured subset of beams changes in an iterative manner over time, based on the previous timestep's best beam and its most correlated beams. The predicted layer 3 measurements are derived from the RSRP of the best beam among those that are measured. Table 2.1.2-1 shows the results of this evaluation.

Table 2.1.2-1: Evaluation results (RSRP mean absolute error in dB) for the iterative selection of the subset of beams to measure for each speed class and measurement saving rate (10%-90%)

|  | 10% | 30% | 50% | 70% | 90% |
| --- | --- | --- | --- | --- | --- |
| 30 km/h | 0.000264 | 0.002404 | 0.012578 | 0.06234 | 1.30469 |
| 60 km/h | 0.000439 | 0.003477 | 0.014861 | 0.07300 | 1.80343 |
| 120 km/h | 0.000587 | 0.005066 | 0.019094 | 0.08862 | 2.22447 |

Observation 2: Iteratively selecting the subset of beams to measure gives negligible mean absolute error for up to 70% measurement saving rate.

We also note that if the measured subset of beams at each beam sweep includes the best beam at that time, we would get zero error between ground truth L3 RSRP and the predicted L3 RSRP, because in this case, both of them are equal to the RSRP of the best beam. This can be demonstrated by the “hit rate”, which is the proportion of beam sweeps in which the measured subset of beams contained the best beam, as shown in Table 2.1.2-2.

Table 2.1.2-2: The proportion of beam sweeps in which the measured subset of beams contained the best beam each speed class and measurement saving rate (10%-90%)

|  | 10% | 30% | 50% | 70% | 90% |
| --- | --- | --- | --- | --- | --- |
| 30 km/h | 0.99971 | 0.99777 | 0.98956 | 0.95740 | 0.74238 |
| 60 km/h | 0.99958 | 0.99690 | 0.98781 | 0.95362 | 0.70324 |
| 120 km/h | 0.99953 | 0.99598 | 0.98583 | 0.94735 | 0.66974 |

Proposal 1: RAN2 to study two sub-use-cases of spatial measurement reduction:

- Fixed selection of the measured beams, i.e., the UE always measures the same set of beams in every beam sweep.

- Iterative selection of the measured beams, i.e., the UE updated the set of beams that it measures in each beam sweep.

## 2.2	Layer-3 Filtering in Time Domain Prediction Case B

Layer 3 filter applied to the beam consolidated measurements is defined in 3GPP TS 38.331 5.5.3.2 as:

Fn = (1 – a)*Fn-1 + a*Mn where Mn is the latest received measurement result from the physical layer; Fn is the updated filtered measurement result, that is used for evaluation of reporting criteria, for measurement reporting or for U2N Relay (re)selection evaluation; Fn-1 is the old filtered measurement result, where F0 is set to M1 when the first measurement result from the physical layer is received; and for MeasObjectNR, a = 1/2(ki/4), where ki is the filterCoefficient for the corresponding measurement quantity of the i:th QuantityConfigNR in quantityConfigNR-List, and i is indicated by quantityConfigIndex in MeasObjectNR; for other measurements, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by the quantityConfig; for UTRA-FDD, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by quantityConfigUTRA-FDD in the QuantityConfig;

In RAN2#127, the following was agreed.

3 In the definition of 3 RRM sub-cases, all cell level measurement result(s) refers to L3 filtered cell level measurement

In intra-frequency temporal domain case B evaluation, the L3 filtering should consider the impact of the skipped measurement steps in the layer 3 filter implementation. This includes both the predicted or the measured time-steps and the input to the ML model in inference. To facilitate the simulation work, the following example and three options are given in [2].

Figure 2.2-1: Temporal domain case B example 2

- Filtering option 1: L3 cell result at T3 is filtered based on the L1 filtered result at T3 and L3 filtered cell result at T2.

- Filtering option 2: L3 cell result at T3 is filtered based on the L1 filtered result at T3; in other words, not consider old L3 filtered result.

- Filtering option 3: L3 cell result at T3 is filtered based on the L1 filtered result at T3 and L3 filtered cell result at T1 (i.e. the old L3 filtered cell result from last actual measurement).

Considering the Rel. 18 and 19 studies on AIML, it has become clear that the outcomes of AIML models may not always be stable, and there can be significant prediction errors in predicted measurements. If we attempt to use the L3 filtering described in TS 38.331, these prediction errors can leak into future measurements due to the recursive nature of the filter (i.e. current filtered samples depend on previously filtered samples, so if the previously filtered samples contain errors, they will affect the current samples). To mitigate such issues, additional filtering would be needed. An example of such filtering is shown in Figure 2.2-2.

Figure 2.2-2: Proposed L3 filtering

A different filter forgetting factor may be used according to whether the previous or current time instance is measured (i.e. coming from the physical device) or predicted (i.e. coming from an ML model). We evaluate this by introducing prediction errors as random Gaussian noise into every other measurement sample from UE RSRP measurements coming from the agreed on FR1 scenario, and then filtering the unfiltered RSRP measurements such that the forgetting factor of predicted samples (aP) is smaller than the forgetting factor for measured samples (which is typically equal to 0.5). Our results show that selecting the right forgetting factor for measured and predicted steps can improve the accuracy, as show in Figure 2.2-3.

Figure 2.2-3:  Mean absolute error between the ground truth filtered measurements and the predicted filtered measurements after applying the proposed L3 filter using several values of aP

Observation 3: When measurement prediction is used, configuring specific filter forgetting factors for measured and predicted timesteps improves the prediction error of L3-filtered measurements.

Following the formulation from TS 38.331 clause 5.5.3.2, the filter can be applied as follows: when Fn-1 is predicted or computed from a predicted value and Mn is measured, Fn is calculated as:

Fn = (1 – ap)*Fn-1 + ap*Mn

and when Fn-1 is measured and Mn is predicted, Fn is calculated as:

Fn = ap*Fn-1 + (1 – ap) *Mn

where aP is smaller than 0.5. This gives the net effect of assigning a lower weight to the predicted samples compared to the measured ones.

Proposal 2: RAN2 to study the use of introducing additional filtering for measured and predicted time steps.

## 2.3	Configuration of RRM temporal prediction

In temporal domain RRM measurement prediction (both case A and case B), the choice of observation window (OW) and prediction window (PW) lengths significantly impacts the performance of the ML model. While larger observation and prediction windows provide more data for training, they can also increase prediction error. This poses a challenge in dynamic mobility and radio environments, where fixed OW and PW lengths cannot effectively balance AI/ML performance and measurement reduction gain.

In an earlier contribution [1], our evaluations show that a larger observation window and prediction window may result in a higher MAE in temporal domain RRM measurement prediction. Lower UE speeds generally result in better intra-frequency RRM prediction performance. This indicates that a constant window configuration may be inadequate in more dynamic radio environments. Observations from other contributions [3] further support this notion. For temporal domain case A, using a shorter OW at 60 Km/h with a fixed OW/PW ratio leads to improved prediction performance. Conversely, longer OW at 60 Km/h can worsen prediction for a given PW. Increasing the PW length results in a minor increase in prediction error (~0.4 dB) but reduces measurement overhead.

Observation 4: Fixed OW and PW lengths in temporal RRM prediction yield inconsistent MAE across UE speeds, performing better at lower speeds.

Proposal 3: RAN2 to discuss the configuration aspects for OW and PW for temporal RRM measurement prediction.

Considering legacy mobility procedures, the UE reports measurements based on a configuration provided by the network. This configuration includes detailed measurement and reporting configs that instruct the UE on when and how to report its measurements. These reports are critical to enabling the network to make timely and accurate handover decisions. Moving toward Release 19 and AI/ML-enabled use cases like RRM measurement prediction, similar to the legacy behaviour, it is expected that prediction configurations would also be provided by the network. The network would give the UE specific prediction configurations that guide how predictions are generated and reported. By extending the role of the network to include prediction-based configurations, the UE could leverage AI/ML techniques to anticipate network conditions, supporting a proactive mobility management approach that adapts more effectively to dynamic environments. With the addition of prediction capabilities in Release 19, a question arises about the appropriate reporting approach for these predictions. Just as the UE periodically reports measurements in Release 15 upon meeting specific events, a similar periodic reporting structure might be considered for predictions. This would enable the network to stay updated with predictive insights at regular intervals, facilitating better decision-making in fast-changing scenarios.

Observation 5: In legacy mobility procedures, the network provides configuration for UE to perform measurements and reporting. Similar approach can also be followed for reporting RRM measurement predictions.

Similar to the legacy framework, the UE might be configured to report predictions only under specific, non-periodic conditions or events, allowing for reporting of predictions when they are truly needed by the NW. This non-periodic approach could lower reporting overhead while still providing critical predictive data to the network when necessary.

Proposal 4: RAN2 to discuss the prediction reporting mechanisms.

# 3	Conclusion

This document has made the following observations:

Observation 1: For spatial domain measurement reduction, low complexity ML models yield a competitive performance relative to more complex models (e.g., random forest).

Observation 2: Iteratively selecting the subset of beams to measure gives negligible mean absolute error for up to 70% measurement saving rate.

Observation 3: When measurement prediction is used, configuring specific filter forgetting factors for measured and predicted timesteps improves the prediction error of L3-filtered measurements.

Observation 4: Fixed OW and PW lengths in temporal RRM prediction yield inconsistent MAE across UE speeds, performing better at lower speeds.

Observation 5: In legacy mobility procedures, the network provides configuration for UE to perform measurements and reporting. Similar approach can also be followed for reporting RRM measurement predictions.

And proposed the following:

Proposal 1: RAN2 to study two sub-use-cases of spatial measurement reduction:

- Fixed selection of the measured beams, i.e., the UE always measures the same set of beams in every beam sweep.

- Iterative selection of the measured beams, i.e., the UE updated the set of beams that it measures in each beam sweep.

Proposal 2: RAN2 to study the use of introducing additional filtering for measured and predicted time steps.

Proposal 3: RAN2 to discuss the configuration aspects for OW and PW for temporal RRM measurement prediction.

Proposal 4: RAN2 to discuss the prediction reporting mechanisms.

# 4	References

[1] R2-2408929, RRM Measurement Prediction Evaluation Results, Nokia, RAN2#127bis, Hefei, China, October, 2024.

[2] R2-2410190, Summary of [POST127bis][022][AI mobility] Simulation assumptions (OPPO), RAN2#128, Orlando, Florida, November, 2024.

[3] R2-2408925, Simulation results for RRM measurement predictions, InterDigital Inc., RAN2#127bis, Hefei, China, October, 2024.