3GPP TSG RAN WG2 #128	R2-2410805

Orlando, USA, November 18th – November 22nd, 2024

Agenda Item:	8.2.5

Source:	Fraunhofer HHI, Fraunhofer IIS

Title:	Discussion on Topology 2 related aspects

Document for:	Discussion & decision

# Introduction

In the RAN#102 meeting, a new SID on solutions for Ambient IoT (Internet of Things) in NR was approved [1]. Ambient IoT relies on ultra-low complexity devices with ultra-low power consumption for very-low-end IoT applications. The RAN2-led objectives are listed as follows:

Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signaling procedure to enable DO-DTT and DT data transmission and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential.

In previous RAN2 meetings [2][3], the following agreements related to paging/initial trigger message were made:

RAN2#127 Meeting Agreements: - RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2. - A-IoT air interface in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device. - RAN2 assumes that intermediate UE authorization is performed by upper layers. RAN2 impact can be studied after further SA/RAN3 progress. - RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied) - NW is in control of resources to be used by AIoT air interface. - Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO). May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible). RAN2#127bis Meeting Agreements: - Dedicated resource configuration is only given to the UE reader via dedicated signaling. Mechanisms for shared resource pool amongst readers are not considered in this release. - The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control. FFS how valid resource is determined – we will consider temporary out of connection scenarios. FFS on resource validity across multiple cells. - The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

In this contribution, we further discuss the considerations on the A-IoT topology 2, focusing on the resource validity aspects.

# Discussion

## Resource validity and reader behavior

In the last meeting, the topic of RRC state management for the intermediate UE received considerable attention, particularly concerning the UE’s behavior when transitioning into RRC_INACTIVE or RRC_IDLE states. These states allow the UE to enter a lower-power mode by suspending continuous communication with the gNB, which in turn reduces both signaling overhead and power consumption. This reduction is critical in managing energy efficiency, especially when the UE serves as an intermediary for AIoT operations. However, an important point discussed was the potential for the intermediate UE to retain its AIoT functionality, as long as the resources allocated by the gNB remain valid. Such an approach could allow the UE to operate independently for a specified duration, preserving AIoT operations during brief coverage losses or periods of intermittent connectivity.

Observation 1: Allowing a defined resource validity period could enable the intermediate UE to go temporarily out-of-coverage without the need to interrupt ongoing AIoT operations.

Typically, the gNB configures resources for the UE through a dedicated configuration message, such as RRC signaling. This configuration could serve as a baseline not only allocates the necessary resources but could also specify a validity period. This period would define how long the UE can retain access to those resources, ensuring that brief connectivity interruptions do not immediately disrupt AIoT operations. In effect, the validity period can act as a mechanism to keep operational stability even in variable coverage scenarios.

Figure 1. A validity timer can be triggered to keep resources during out-of-coverage

Proposal 1: Resource validity parameters should be defined and included in the initial configuration process, alongside resource allocation, allowing the UE to understand the duration for which it can maintain AIoT operations.

To maintain AIoT operations even when temporarily out of coverage, the UE could be configured with a timer-based validity approach, as shown in Figure 1. In this setup, once the gNB configures the UE's resources, an internal timer can be initiated to mark the start of the validity period when moving out of the service area of the gNB. This timer enables the UE to continue AIoT operations within the defined timeframe while acting as a data buffer, even if it temporarily moves out of coverage. During this period, the UE can retain access to the allocated resources without immediately releasing them, and so, effectively allowing it to maintain communication with nearby AIoT devices. As a result, the UE can continue serving as an intermediary node in AIoT scenarios by collecting and holding data until coverage is reestablished. When the UE returns within the coverage area, it can transmit any buffered data back to the gNB.

Proposal 2: A time-based validity period should be considered as a baseline for resource validity. Using an internal timer to determine the resource validity duration can allow continued operation in brief out-of-coverage scenarios.

In addition to time-based validity, an event-driven approach can provide an additional layer for managing resource validity. This approach enables the UE to actively monitor specific operational conditions, such as channel quality degradation or repeated access attempt failures. When these conditions persist, the UE can interpret them as indicators that the resource validity may be compromised. For instance, if the UE detects sustained degradation in the link quality or fails to access the gNB after multiple attempts, it may invalidate the resources.

Proposal 3: An event-driven approach can be adopted, either as a complement to or primary alternative for timer-based validity to determine the need for resource reconfiguration.

In scenarios where the intermediate UE operates across multiple cells, managing resource validity becomes essential. To support the UE's function as a relay node in AIoT, a broader resource validity management should be applied. In this regard, the gNB can configure a shared resource pool that remains valid across all cells within a specified area, or within those that maintain similar operational parameters such as carrier frequency. By allowing the UE to retain access to these resources even as it transitions between cells, this area-based configuration reduces the frequency of reconfiguration requests. For instance, as the UE moves within the tracking area, an area-based validity timer or other location-based parameters could ensure that configured resources remain available across multiple cells, facilitating seamless AIoT operation without additional signaling requirements.

Proposal 4: Extending resource validity to incorporate parameters like area-based settings or gNB configurations enables the configured resources to remain valid across a defined group of cells.

# Conclusion

In this contribution we discussed the A-IoT topology 2 consideration for A-IoT devices. Based on the discussion in the previous sections we made the following observations:

Observation 1: Allowing a defined resource validity period could enable the intermediate UE to go temporarily out-of-coverage without the need to interrupt ongoing AIoT operations.

Based on the discussion in the previous sections, we propose the following:

Proposal 1: Resource validity parameters should be defined and included in the initial configuration process, alongside resource allocation, allowing the UE to understand the duration for which it can maintain AIoT operations.

Proposal 2: A time-based validity period should be considered as a baseline for resource validity. Using an internal timer to determine the resource validity duration can allow continued operation in brief out-of-coverage scenarios.

Proposal 3: An event-driven approach can be adopted, either as a complement to or primary alternative for timer-based validity to determine the need for resource reconfiguration.

Proposal 4: Extending resource validity to incorporate parameters like area-based settings or gNB configurations enables the configured resources to remain valid across a defined group of cells.

# References

- 3GPP RP-234058, “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, 3GPP TSG-RAN #102, Edinburg, UK, Dec 11th-15th, 2023.

- 3GPP “Draft Report of 3GPP TSG RAN WG2 meeting #127”, 3GPP TSG RAN WG2 #127, Maastricht, Netherlands, Aug. 19th-23rd, 2024.

- 3GPP “Draft Report of 3GPP TSG RAN WG2 meeting #127bis”, 3GPP TSG RAN WG2 #127bis, Hefei, China, Oct. 14th-18th, 2024.