3GPP TSG-RAN WG2 Meeting #128	R2-2410870

Orlando, USA, November 18-22, 2024

Agenda Item:	8.8.2

Source:	THALES

Title:	Discussion on RAN2 Aspects for system-level Downlink Coverage enhancements

Document for:	Discussion and Decision

# 1	Introduction

The approved work item justifications and objectives for NR NTN Evolution in Rel-19 [1] emphasizes the need to accommodate satellite payload constraints which may limit the ability to have all its beams to be active at the nominal EIRP density and provide objectives for this Release :

- Offer optimized performance especially when addressing handset terminals (including smartphones with -5.5 dBi antenna gain) w.r.t. downlink coverage considering the NTN deployment constraints such as payload power limitation, large satellite foot print and limited feeder link bandwidth. DL coverage enhancements are needed to accommodate satellite payload constraints which may be unable to have all its beams active with the « nominal » EIRP density per beam (see Section 6.1.1 in TR 38.821) at a given time due to limited power and limited feeder link bandwidth, while maximizing the number of beams that can be activated simultaneously, and ensuring that all user terminals can be served across the satellite foot print while maximizing the overall satellite throughput and ensuring that all satellite’s radio cells are kept alive even without traffic but allowing new users to join or preventing impact on end-user QoS. DL coverage enhancements can be considered at both - Link level to improve the link margin of selected physical channels in order to accommodate the EIRP reduction in FR1-NTN. A link margin improvement for physical channels (e.g. PDSCH and PDCCH) may be considered without impact on SSB design. - System level to support an efficient dynamic and flexible power sharing between beams or different beam pattern/size (i.e., wide or narrow) across the satellite foot print for FR1-NTN and FR2-NTN.

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Notes for this objective: - SSB channel enhancement is not considered - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

As highlighted above, the RAN2 needs to provide solutions for DL coverage enhancements challenge:

    - Study and specify solutions including link level and or system level enhancements allowing dynamic and flexible power sharing between satellite beams or different satellite beam pattern/size across the satellite footprint. The LEO is considered in priority and the Rel-18 network energy saving techniques should be considered as baseline for the study.

In this contribution, we discuss the potential enhancements at the system level aspects for the Downlink coverage enhancements for NTN:

# 2	Discussion

## SSB periodicity

RAN1 already agreed to support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access up to 160 ms:

Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

In same time, the RAN1 agreed on the following configuration for downlink coverage enhancement evaluation (both system and link level):

| Scenario |  | Set1-1 FR1, Set1-2 FR1, Set1-3 FR1 and Set1-1 FR2 |
| --- | --- | --- |
| Total number of beam footprints |  | 1058 for Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1 800 for Set1-1 FR2 |
| Total number of simultaneously active beams |  | 106 for Set1-1 FR1 and Set1-3 FR1 16 for Set1-2 FR1 12 for Set1-1 FR2 |
| Hopping Ratio (Hr) | Hr | 0,100 (or 10%) for Set1-1 FR1 and Set1-3 FR1 0,015 (or 1,5%) for Set1-2 FR1 and Set1-1 FR2 |
| Size of the cluster of cells |  | 10 (1 beam is used to illuminate a cluster of 10 cells) for Set1-1 and Set1-3 FR1 66 (1 beam is used to illuminate a cluster of 66 cells) for Set1-2 FR1 67 (1 beam is used to illuminate a cluster of 67 cells) for Set1-1 FR2 |

It was agreed in RAN1#118bis to support a maximum of the additional default value of SSB periodicity (apart from the existing 20ms value) of at least 160ms. It is still FFS whether an SSB value of 320ms could be supported.

Based on RAN1#118bis agreement the SSB periodicity assumed by the UE for initial cell selection is could be set equal to 160ms. Thereby, cell supporting initial access could have PBCH periods of 5, 10, 20, 40, 80 or 160ms.

Observation 1	Based on RAN1#118bis agreement, the maximum default value of SSB periodicity of the additional default value (apart from the existing 20ms value) is at least 160ms. Therefore, cell supporting initial access could have PBCH periods of 5, 10, 20, 40, 80 or 160ms

It is worth noting that based on RAN2 agreement, SSB periodicity within 160ms won’t impact specification from RAN2 perspective.

Regarding the remaining FFS from previous RAN1 meeting, in our view, extending the periodicity of the Synchronization Signal Block (SSB) beyond the established 160ms (for example, to 320ms) in would have several implications:

- Increased Latency: A longer periodicity means that devices (UEs) attempting to access the network or perform cell search might experience increased latency since they would have to wait longer for the SSB to be broadcasted

- Handovers and Reselections: The time it takes to detect and measure neighboring cells, which is crucial for handovers and cell reselections, could be negatively impacted. This could result in slower or less reliable handovers, affecting user experience, especially at high speeds.

- Implementation Complexity at UE side

- Backward Compatibility: Ensuring compatibility with existing devices and network elements that are designed based on a 160ms periodicity would be necessary, posing an additional challenge.

- Specifications impacts: according to the above RAN2 agreement extending the SSB periodicity beyond the established 160ms will require several changes/adaptations: for example, ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17. More importantly, the requirements stemming from RAN4 RRM requirements (related to various aspects such as timing, radio measurements etc) collectively ensure that the SSB periodicity of maximum 160ms supports efficient network operation while maintaining a balance between access latency, mobility performance, and power consumption. Changes to this periodicity would necessitate a thorough review of these requirements to maintain network performance and user experience.

Proposal 1		Do not extend the SSB periodicity beyond the established value of 160ms

Proposal 2		The UE can deduces the cell support Downlink Coverage enhancement when the detected SSB periodicity is larger than 20 ms

Since a cell with beam hoping is like another cell, but with a transmission pattern, and the SSB is not extended above 160ms, there is no reason to bar specific NTN UE to access to the network.

Proposal 3		Do not introduce a cell barring bit for cell working with extended SSB periodicity

The beam hoping is a necessary enhancement for NTN network to allow constrained NTN payload to provide a better coverage by turning off some cell and turning on the other. It should be the most transparent as possible for the NTN UE (pre-Rel-19 and Rel-19 UEs). In RAN2#126, an agreement was captured to go on this direction :

Based on the solution being investigated in RAN1, RAN2 will further discuss whether/how legacy UEs might operate in a cell supporting DL coverage enhancements

Observation 2	Legacy UE can operate in a cell working with maximum SSB periodicity up to 160ms

## Cell DTX/DRX

The cell DTX/DRX mechanism can be re-used to handle the beam hoping. The cell-level DTX/DRX for Rel-18 NES already define patterns and behavior for UE and network. In top of that, the SSB is pattern should match to the DTX/DRX pattern that could be made by a proper configuration at the implementation.

Proposal 3		The cell-level DTX/DRX for Rel-18 NES could be re-used to handle system-level beam hoping mechanism for Downlink Coverage enhancement NTN Rel-19

Observation 2	The SSB pattern should match to the DTX/DRX pattern to avoid SSB transmission during DTX/DRX

We cannot assume a fixed dwell pattern in the cell lifetime. According to the satellite capacity, the number of users and the geography, the SSB periodicity and the DTX/DRX pattern can change. Also, for the mobility event in Earth-fixed cell, the target cells can be configured with a lower SSB periodicity and higher revisit time to handle the mobility of a group of UE.

Observation 3	The dwell pattern could not be fixed in time for a given cell, e.g. during a mobility event with Earth-Fixed Cells

The DTX/DRX is configured via UE-specific RRC signaling and can be activated/de-activated via dynamic L1/L2 signaling. Therefore, the DTX/DRX can be done by a quasi-dynamically with a question on the overhead and can be dynamically activated/de-activated.

Proposal 4		The UE is informed of the DRX/DTX pattern quasi-dynamically using existing procedure. FFS if enhancements are necessary to reduce overhead, especially when the NTN beam is turned on/off periodically

When a UE is trying to connect to a cell in coverage enhancement, according to RAN1, UE does not assume that SSB periodicity of 20 ms for initial attach. The value can be 20 to 160 ms and can be pre-configured by the operator in the UE. After decoding the SSB, the UE will try a PRACH. To be successful, the Msg1 and Msg3 should be received by the gNB when the cell is turned on. The Msg2 and Msg4 are transmitted when the cell is not in DTX mode. At the Msg1 transmission, the UE is not aware of the DTX/DRX pattern. The UE transmit the preamble on the next Random Access Opportunity.

Observation 4	UE is not aware of the DTX/DRX pattern at the initial attach

Proposal 5		RAN2 discusses the possible impacts and necessary enhancements on Random Access procedures when DTX/DRX is configured

Once the UE is connected, it is configured with the DTX/DRX pattern. During the DTX period, the UE is not required to monitor control channel or perform measurements on the serving cell. The C-DRX can be configured for the UE for power saving purpose. It should be aligned with the DTX cycle length.

Proposal 6		The connected UE is not required to monitor PDCCH or perform the serving cell during the configured DTX period when activated

Proposal 7		The connected UE should be configured with a C-DRX where the cycle length should be aligned with one or multiple times of the DTX cycle length

It is likely that neighbour cells are also configured with downlink coverage enhancement mode in a NTN deployment. The UE should measures neighbour cell when it is on. So, the UEs need to be informed for cell (re-)selection on the DTX/DRX pattern and the SSB periodicity.

Proposal 8		The UEs are informed of the DTX/DRX pattern of the neighbour cell. FFS if this indicated in SIB19 NTN-NeighCellConfig or by other means

If the DTX/DRX pattern of the neighbour cells is indicated to the UE there could be an issue with the timing difference between the serving and the neighbour cell of the DTX/DRX pattern. For a cells handled by the same satellite, the UE could assume the pattern timing are aligned but it does not correspond to the general case.

Observation 5	The DTX/DRX pattern of the neighbour cell indicated by the serving cell should have a difference timing

Proposal 9		If the DTX/DRX pattern of the neighbour cell is indicated in the service cell, RAN2 discusses how the DTX/DRX pattern timing is also indicated to allow the user to know when the neighbour cell is expected to be turn on

The handover should occur when the target cell is on, especially when configured with RACH. The HO can be configured with a time and the UE will trigger measurements for a cell (re-)selection. The same is applied for Conditional handover, location or time-based. For the RACH-less intra and inter-gNB handover, the re-synchronization to the target cell could occur when the DTX/DRX is active, leading to a RLF. The RACH-less could be ineffective if the UE is not configured with the DTX/DRX pattern of the target cell.

Proposal 10		RAN2 discusses the impacts of the DTX/DRX on mobility mechanism defined in previous release for NTN, e.g. Time and location-based CHO and RACH-less

# 3	Conclusion

In the previous sections we made the following observations and proposals:

<SSB periodicity>

Observation 1	Based on RAN1#118bis agreement, the maximum default value of SSB periodicity of the additional default value (apart from the existing 20ms value) is at least 160ms. Therefore, cell supporting initial access could have PBCH periods of 5, 10, 20, 40, 80 or 160ms

Proposal 1		Do not extend the SSB periodicity beyond the established value of 160ms

Proposal 2		The UE can deduces the cell support Downlink Coverage enhancement when the detected SSB periodicity is larger than 20 ms

Proposal 3		Do not introduce a cell barring bit for cell working with extended SSB periodicity

Observation 2	Legacy UE can operate in a cell working with maximum SSB periodicity up to 160ms

<Cell DRX/DTX>

Proposal 3		The cell-level DTX/DRX for Rel-18 NES could be re-used to handle system-level beam hoping mechanism for Downlink Coverage enhancement NTN Rel-19

Observation 2	The SSB pattern should match to the DTX/DRX pattern to avoid SSB transmission during DTX/DRX

Observation 3	The dwell pattern could not be fixed in time for a given cell, e.g. during a mobility event with Earth-Fixed Cells

Proposal 4		The UE is informed of the DRX/DTX pattern quasi-dynamically using existing procedure. FFS if enhancements are necessary to reduce overhead, especially when the NTN beam is turned on/off periodically

Observation 4	UE is not aware of the DTX/DRX pattern at the initial attach

Proposal 5		RAN2 discusses the possible impacts and necessary enhancements on Random Access procedures when DTX/DRX is configured

Proposal 6		The connected UE is not required to monitor PDCCH or perform the serving cell during the configured DTX period when activated

Proposal 7		The connected UE should be configured with a C-DRX where the cycle length should be aligned with one or multiple times of the DTX cycle length

Proposal 8		The UEs are informed of the DTX/DRX pattern of the neighbour cell. FFS if this indicated in SIB19 NTN-NeighCellConfig or by other means

Observation 5	The DTX/DRX pattern of the neighbour cell indicated by the serving cell should have a difference timing

Proposal 9		If the DTX/DRX pattern of the neighbour cell is indicated in the service cell, RAN2 discusses how the DTX/DRX pattern timing is also indicated to allow the user to know when the neighbour cell is expected to be turn on

Proposal 10		RAN2 discusses the impacts of the DTX/DRX on mobility mechanism defined in previous release for NTN, e.g. Time and location-based CHO and RACH-less

# References

RP-241789, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#105, 2024-09, Melbourne, AU.