3GPP TSG-RAN WG3 #126	R3-247211

Orlando, USA, 18th – 22nd November 2024

Agenda item:		13.2 Support for inter-CU LTM

Title:		discussion of Rel-19 inter-CU LTM issues

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

For the Rel-19 Inter-CU LTM, we continue to discuss and proposal to progress the project.

13. NR Mobility Enhancements WI WID [NR_Mob_Ph4-Core]: RP-242356 (target: RAN #109) [TU: 1 ( 1, 1, 1, 1)] QUOTA: 3

13.2. Support for inter-CU LTM Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: As secondary priority, support the case where CU is acting as SN and MN is unchanged As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first. Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation. Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time. Early data forwarding can be supported for inter-CU LTM. Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution. Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM. Introduce a new procedure on Xn to transfer the TA information. A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU. Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell. The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation. Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM. Change the name of LTM Cell Switch Notification message to Cell Switch Notification message. Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM. Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. RAN3#125bis: Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase. WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell. The error handling of multiple UE associations need to be considered. WA: The source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs via Handover Request message, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message. The LTM Configuration IDs are allocated by the source CU. WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB. Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message. How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress. Confirm the name of the new procedure as “LTM Configuration Update”. Introduce a new procedure for candidate gNB-initiated LTM cancellation. Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS. Continue to work on the solutions comparison and details…

2. Discussion

2.1	LTM Preparation

2.1.1	Initial LTM configuration request

WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

The error handling of multiple UE associations need to be considered.

Question: Handover request with single LTM candidate cell or multiple LTM candidate cells?

We have reached a Working Assumption that a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell. A question was raised that how does the candidate CU know the multiple HANDOVER REQUEST messages (each message per cell) is for the same UE, this was more or less confirmed that there is no problem for the candidate CU to associate the different HANDOVER REQUEST messages to the same UE because there is a mandatory NG-C UE associated signalling reference IE which is the AMF UE NGAP ID allocated in in AMF, will unique for the UE.

The next question is error handling e.g.

The error handling is complex, since it needs to consider that the same IE can be received, potentially with different values, across the multiple procedures. For examples, the non-LTM related IEs included in the multiple HO Requests may differ (e.g., different GUAMI). This requires more complex handling from implementation point of view.

The candidate gNB when receive two Handover Request message with different AP ID respectively, it will be able to identify only from the NG-C UE associated signalling reference IE whether these multiple Handover Request message is for the same UE. If multiple Handover Request messages each has different GUAMI, the candidate gNB can only identify these Handover Request messages are for different UEs. If the intention is for the same UE but the GUAMI is different, this will be a kind of implementation error which can be detected during the testing phase of the products.

Other information related with the UE Context Information, e.g. UE Security Capabilities, AS Security Information, UE Aggregate Maximum Bit Rate, and PDU Session related information, if they are different between the Handover Request message intended for the same UE, once the candidate CU knows the Handover Request messages are for the same UE, it would be easily to have error handling.

Proposal 1: To have the agreement on the “a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.”.

2.1.2	LTM configuration update

Last RAN3#125bis meeting we agreed a new procedure LTM Configuration Update is introduced with LTM CONFIGURATION UPDATE, LTM CONFIGURATION UPDATE ACKNOWLEDGE and LTM CONFIGURATION UPDATE FAILURE messages, these have been reflected in BL CR to 38.300 and BL CR to 38.423.

The BL CR to 38.423 for Inter-CU LTM in the LTM CONFIGURATION UPDATE message now contains:

- Target Cell Global ID (FFS)

- Early Sync Information  (detail is FFS)

- Early Sync Information for SUL (detail is FFS).

- LTM Information Request

The Target Cell Global ID IE can be a single cell provided that there are no multiple cells in the handover preparation phase.

Early Sync Information can simply reuse what has been in Rel-18 intra-CU LTM, provided by the candidate DU as in 9.3.1.328 of F1AP:

38.473v18.3.0 9.3.1.328 Early UL Sync Configuration This IE indicates the early UL sync configurations for the UE. IE/Group Name; Presence; Range; IE type and reference; Semantics description. RACH Configuration; M; OCTET STRING; Includes the EarlyUL-SyncConfig IE, as defined in TS 38.331 [8].. LTM gNB-DUs List; 0..1; This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s).. >LTM gNB-DUs Item IEs; 1..< maxnoofLTMgNBDUs>. >>LTM gNB-DU ID; M; gNB-DU ID 9.3.1.9. >>Preamble Index List; O; 9.3.1.329. Range bound; Explanation. maxnoofLTMgNBDUs; Maximum no. of gNB-DUs allowed to be configured with LTM towards one UE, the maximum value is 8..

Proposal 2: for the Early Sync Information from the candidate CU, can reuse what has been in Rel-18 intra-CU LTM provided by candidate DU.

We still assume the allocated Early UL Sync Configuration is valid for a list of DU in the source gNB-CU, and the Preamble Index List is similar to intra-CU LTM case that it is valid for a DU. The information in the Handover related messages and LTM configuration Update related message each is per UE oriented therefore no issue for the candidate CU to forward to the information to the source CU. Only when transfer the TA Information from the candidate DU to source CU, we should consider such case provided the TA Information will be using non-UE associated signaling message, then source CU-ID will be needed to be informed to gNB-DU.

Proposal 3: in order to correctly forward the TA information from candidate DU to source CU, consider to introduce an ID to identify the source CU.

2.2	LTM Execution

2.2.1	TA Information Transfer message

In the RAN3#124 meeting, it was already agreed to introduce a new procedure on Xn interface to transfer the TA information. But whether it is a UE associated message or non-UE associated message is still FFS, and in RAN3#125 meeting there is still no consensus. A simple way is to align with Rel-18 LTM in F1, no change to Rel-18 LTM, to introduce TA information transfer in XN with non-UE associated message.

Proposal 4: in order to align with Rel-18 LTM in F1, and with no change to Rel-18 LTM, to introduce TA information transfer in XN with non-UE associated message.

2.3	LTM Release

Last RAN3#125bis meeting we reached a “WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.”

RAN3#124 meeting has the following FFS: FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB. Last RAN3#125 this issue was not treated.

The condition of this issue is after the LTM Completion Phase i.e. after the UE has been cell switched to target CU, the ex-serving cells (of the ex-serving CU) can be kept for subsequent LTM. However at the time the UE successfully access to the new serving cell, if the new serving CU decides that there is no need to keep the ex-serving CU, whether to use UE Context Release message?

For the legacy handover procedure, after the target gNB has completed Path Switch towards CN, it initiates UE Context Release towards the source gNB. If we will follow this procedure sequence, then it would be natural to use UE Context Release procedure.

One other alternative way, can use Handover Cancel procedure, like cancel the prepared candidate gNB in the middle. This can work also.

We simply just need to decide one principle, i.e.  Handover Cancel procedure is used for the source gNB (or source CU) to cancel some of prepared Cells that was requested by the source gNB, the UE Context Release procedure is used to release the UE context of a gNB (i.e. release all prepared cells).

Proposal 5: To reuse the existing XnAP UE Context Release procedure at the source gNB if no LTM candidate cell(s) exist.

2.4	Support of subsequent LTM

Sync up of LTM information between candidate gNBs:

To support subsequent inter-CU LTM, the LTM information (e.g., early sync configuration, LTM configuration ID, and data forwarding addresses, etc.) may need to sync up among candidate gNBs.

In Rel-18 intra-CU inter-DU LTM in F1, the following note is captured in stage 2 (38.401):

NOTE 2: Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.

(the step 7 is CU->DU: UE CONTEXT MODICATION REQUEST; the step 19 is DU->CU: DU-CU CELL SWITCH NOTIFICATION; the step 22 is DU->CU: ACCESS SUCCESS)

For the Rel-19 inter-CU LTM, there are possible three options to support LTM information transfer to support subsequent LTM:

- Option 1: Send the collected LTM information to the candidate gNB(s) after the initial handover preparation.

- Option 2: Send the collected LTM information to the target gNB when the source gNB send the LTM Cell Switch Command to the UE (e.g., via LTM Cell Switch Notification message).

- Option 3: Send the collected LTM information to the target gNB when the source gNB receives the Handover Success message from the target gNB.

Considering to align with Rel-18 Intra-CU case that “Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.”, it would be then feasible to do the same, i.e. all three Option 1, 2 and 3 may be done by implementation.

Proposal 6: To align with Rel-18 intra-CU LTM, to take all Option 1, 2 and 3, which then can be done by implementation.

UE association between new serving gNB and other candidate gNB after cell switch

Last RAN3#125bis reached that “Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.”.

There are some options to establish the UE association between new serving gNB and other candidate gNBs: (Moderator summary from R3-245757)

- Option 1: During the LTM preparation phase, the candidate gNB allocates UE XnAP ID and send its XnAP ID to other candidate gNBs. [HW, LGE] - Option 2: After each cell switch, the new serving gNB signals its XnAP ID to other candidate gNBs, and then the candidate gNB reply with the Target node UE XnAP ID. [Nokia, HW, Ericsson] - Option 3: After each cell switch, the old serving gNB transfers the UE XnAP ID(s) used by the candidate gNBs to the new serving gNB. [ZTE]

Option 1 will need all the candidate gNBs to hold all the relation with other candidate gNBs. Option 2 and 3 will need only after the serving gNB change. It would be more efficient to have option 2 or option 3, if there will be no use case for the candidate gNBs to hold APID of other candidate gNBs before the serving gNB switching.

Then regarding whether option 2 or option 3, we can put the principle to say that the AP ID is certainly under control of the node who allocated the AP ID. It would then naturally to have option 2 (the new serving gNB signals its XnAP ID to other candidate gNBs).

Proposal 7: For the UE association between new serving gNB and other candidate gNB, from efficiency perspective, to exchange the AP ID between gNBs after the serving gNB change.

Proposal 8: For the UE association between new serving gNB and other candidate gNB, from principle perspective that APID is under control of the node who allocated the APID, it is proposed that the new serving gNB signals its XnAP ID to other candidate gNBs i.e. option 2.

3. Conclusion and proposals

Proposal 1: To have the agreement on the “a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.”.

Proposal 2: for the Early Sync Information from the candidate CU, can reuse what has been in Rel-18 intra-CU LTM provided by candidate DU.

Proposal 3: in order to correctly forward the TA information from candidate DU to source CU, consider to introduce an ID to identify the source CU.

Proposal 4: in order to align with Rel-18 LTM in F1, and with no change to Rel-18 LTM, to introduce TA information transfer in XN with non-UE associated message.

Proposal 5: To reuse the existing XnAP UE Context Release procedure at the source gNB if no LTM candidate cell(s) exist.

Proposal 6: To align with Rel-18 intra-CU LTM, to take all Option 1, 2 and 3, which then can be done by implementation.

Proposal 7: For the UE association between new serving gNB and other candidate gNB, from efficiency perspective, to exchange the AP ID between gNBs after the serving gNB change.

Proposal 8: For the UE association between new serving gNB and other candidate gNB, from principle perspective that APID is under control of the node who allocated the APID, it is proposed that the new serving gNB signals its XnAP ID to other candidate gNBs i.e. option 2.

# References

[1] RP-242356 Revised WID: NR mobility enhancements Phase 4 (WID [NR_Mob_Ph4-Core])

[2] R3-245747, Summary of offline on LTM procedures

[3] R3-245748, Summary of offline for two solutions for Inter-CU LTM

[4] R3-245859 == R3-247031 (BL CR to 38.300) Inter-CU LTM

[5] R3-245860 == R3-247032 (BL CR to 38.423) Xn Support for Inter-CU LTM

Annex: ONLY for Reference.

From BL CR 38.300 )R3-245859)

9.2.3.5.X C-Plane Handling of inter-gNB L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB to another gNB during NR operation for LTM. Figure 9.2.3.5.X-1 shows the inter-gNB LTM procedure for intra-NR.

Figure 9.2.3.5.X-1: Signalling procedure for inter-gNB LTM

The procedure for inter-gNB LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure inter-gNB LTM.

3.	The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). The source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM.

Editor’s Note: Working Assumption: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell

4.	Admission Control may be performed by the candidate gNB(s).

5.	The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response (HO REQUEST ACKNOWLEDGE) to the source gNB including the generated RRC configurations for the accepted candidate cell.

6.	The source gNB sends an LTM CONFIGURATION UPDATE to the candidate gNB(s) to update the LTM configurations of candidate cell(s).

7.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB.

Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional.

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

10/11. Early synchronization to some LTM candidate cell(s) belonging to the candidate gNB(s) may be performed. The candidate gNB(s) sends the TA INFORMATION TRANSFER message to the source gNB if early TA acquisition is performed to the candidate gNB(s).

Editor’s Note: Details are FFS on step 10/11.

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

13.	The source gNB determines to initiate inter-gNB LTM.

14.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE.

16.	The target gNB detects the UE access. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

19.	The UE sends an RRCReconfigurationComplete message to the target gNB.

20. The target gNB may send the UE CONTEXT RELEASE to inform the source gNB to release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

Editor’s Note: Working Assumption: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

TS38.473 The following LTM related IEs for Rel-18 intra-CU LTM in F1AP UE CONTEXT SETUP REQUEST message: LTM InformationSetup; 0..1; YES; reject. >LTM Indicator; M; ENUMERATED (true, …); -. >Reference Configuration; O; 9.3.1.292; -. >CSI Resource Configuration; O; 9.3.1.330; -. LTM Configuration ID Mapping List; O; 9.3.1.294; YES; reject. Early Sync Information Request; 0..1; YES; ignore. >Request for RACH Configuration; M; ENUMERATED (true, …); -. >LTM gNB-DUs List; 1; This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s).; YES; reject. >>LTM gNB-DUs Item IEs; 1..< maxnoofLTMgNBDUs>; -. >>>LTM gNB-DU ID; M; gNB-DU ID 9.3.1.9; -. The following LTM related IEs for Rel-18 intra-CU LTM in F1AP UE CONTEXT SETUP RESPONSE message: Early Sync Information; 0..1; YES; ignore. >TCI States Configurations List; M; OCTET STRING; Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8].; -. >Early UL Sync Configuration; O; 9.3.1.328; -. >Early UL Sync Configuration for SUL; O; Early UL Sync Configuration 9.3.1.328; This IE applies for SUL carrier.; -. LTM Configuration; 0..1; YES; ignore. >SSB Information; M; 9.3.1.202; Includes the SSB Information for the requested target cell.; -. >Reference Configuration Information; O; OCTET STRING; Includes the CellGroupConfig IE, as defined in TS 38.331 [8].; -. >Complete Configuration Indicator; O; ENUMERATED (complete, ...); -. >LTM CFRA Resource Configuration; O; OCTET STRING; Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8].; -. >LTM CFRA Resource Configuration for SUL; O; OCTET STRING; Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier.; -. The following LTM related IEs for Rel-18 intra-CU LTM in F1AP UE CONTEXT MODIFICATION REQUEST message: LTM Information Modify; 0..1; YES; reject. >LTM Indicator; M; ENUMERATED (true, …); -. >Reference Configuration; O; 9.3.1.292; -. >CSI Resource Configuration; O; 9.3.1.330; -. LTM CFRA Resource Config List; 0..1; YES; ignore. >LTM CFRA Resource Config Item IEs; 1 .. <maxnoofLTMCells>; EACH; ignore. >>Cell ID; M; NR CGI 9.3.1.12; -. >>LTM CFRA Resource Configuration; O; OCTET STRING; Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8].; -. >>LTM CFRA Resource Configuration for SUL; O; OCTET STRING; Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier.; -. LTM Configuration ID Mapping List; O; 9.3.1.294; YES; reject. Early Sync Information Request; 0..1; YES; ignore. >Request for RACH Configuration; M; ENUMERATED (true, …); -. >LTM gNB-DUs ID List; 1; This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s).; YES; reject. >>LTM gNB-DUs Item IEs; 1..< maxnoofLTMgNBDUs>; -. >>>LTM gNB-DU ID; M; gNB-DU ID 9.3.1.9; -. Early Sync Candidate Cell Information List; 0..1; YES; ignore. >Early Sync Candidate Cell Information Item IEs; 1 .. <maxnoofLTMCells>; EACH; ignore. >>Cell ID; M; NR CGI 9.3.1.12; -. >>TCI States Configurations List; O; OCTET STRING; Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8].; -. >>Early UL Sync Configuration; O; 9.3.1.328; -. >>Early UL Sync Configuration for SUL; O; Early UL Sync Configuration 9.3.1.328; This IE applies for SUL carrier.; -. >>TA Assistance Information; O; ENUMERATED (zero, …); The value "zero" corresponds to TA value of the cell being equal to zero.; -. >>UE Based TA Measurement Configuration; O; OCTET STRING; Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE.; -. Early Sync Serving Cell Information; 0..1; YES; ignore. >UE Based TA Measurement Configuration; O; OCTET STRING; Includes the ltm-ServingCellUE-MeasuredTA-ID contained in the LTM-Config IE, as defined in TS 38.331 [8], for the current serving cell.; -. LTM Cells To Be Released List; O; 9.3.1.291; YES; reject. The following LTM related IEs for Rel-18 intra-CU LTM in F1AP UE CONTEXT MODIFICATION RESPONSE message: Early Sync Information; 0..1; YES; ignore. >TCI States Configurations List; M; OCTET STRING; Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8].; -. >Early UL Sync Configuration; O; 9.3.1.328; -. >Early UL Sync Configuration for SUL; O; Early UL Sync Configuration 9.3.1.328; This IE applies for SUL carrier.; -. LTM Configuration; 0..1; YES; ignore. >SSB Information; M; 9.3.1.202; Includes the SSB Information for the requested target cell; -. >Reference Configuration Information; O; OCTET STRING; Includes the CellGroupConfig IE, as defined in TS 38.331 [8].; -. >Complete Candidate Configuration Indicator; O; ENUMERATED (complete, ...); -. >LTM CFRA Resource Configuration; O; OCTET STRING; Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8].; -. >LTM CFRA Resource Configuration for SUL; O; OCTET STRING; Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier.; -.