3GPP TSG-RAN WG3 Meeting #126                               R3-247293

Orlando, US, 18th – 22nd, Nov, 2024

Source:	CATT

Title:	Discussion on Direct AI/ML positioning

Agenda Item:	20.2

Document for:	Discussion and Decision

Introduction

In the last RAN3 meeting, we initially discussed the potential RAN3 impact to support Direct AI/ML Positioning, i.e. the Case 3b, NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning. According to the discussion [1], some agreements and open issues as below:

Case 3b: Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1. For case 3b: NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. The details of the measurements are still pending and subject to further discussion in RAN1.

In this contribution, we will further discuss the potential RAN3 impact to support Direct AI/ML positioning, taking the RAN1 progress/agreements into account. Base on the discussion, we will provide corresponding observations and proposals.

Discussion

Data collection

For the Data collection, corresponding RAN1 agreements or working assumptions could be found below:

RAN1#116bis Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement

RAN1#117 Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed

RAN1#118 Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications

In the last RAN3 meeting, it’s agreed that:

- NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF.

- The details of the measurements are still pending and subject to further discussion in RAN1.

Which means, the channel measurement, quality indicator of channel measurement, time stamp of channel measurement should be provide to LMF for training data collection for Case 3b. And RAN1 agreed that For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective. The other information, if needed, is pending to RAN1.

Proposal 1: In Case 3b, the information for training data collection, e.g. channel measurement, quality indicator of channel measurement, time stamp of channel measurement could be provided from gNB to LMF. The other information, if any, is pending to RAN1.

According to RAN1 agreements, the label and related information could be generated by PRU, Non-PRU UE with estimated location, or LMF. How to generate the label and related data is pending to RAN1 and RAN2, no impact to RAN3.

Proposal 2: In Case 3b, how to generate the label and related data is pending to RAN1 and RAN2, no impact to RAN3.

Inference

For Model Input/Report for inference, the corresponding agreements in RAN1 could be found below:

RAN1#116: Agreement - For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation. Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time. - FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time Agreement For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc.

RAN1#116 bis: Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT Conclusion - It is out of RAN1 scope to decide whether/how synthetic data (i.e., not direct physical data) and related entities are used in AI/ML based positioning. In RAN1 discussion, data (e.g., measurement data, label data) refer to physical data, not synthetic data.

RAN1#117: Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

RAN1#118: Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

RAN1#118bis: Agreement From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement.

In the existing NRPPa, above information could be provided to LMF in the TRP measurement Result, as shown below:

9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| … |  |  |  |  |  |  |

We assume NRPPa Measurement procedure could be reused to support the purpose of training data collection, inference and monitoring for Case 3b. Any other information need to be reported from gNB to LMF is pending to RAN1.

Proposal 3: In Case 3b, NRPPa measurement procedure could be reused for training data collection, inference and monitoring for Case 3b. Any other information need to be reported from gNB to LMF is pending to RAN1.

Performance Monitoring

For performance monitoring, corresponding RAN1 agreements are shown below:

RAN1#116 Agreement For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model.

For Case 3b, RAN1 discussed but not decided whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. In our understanding, RAN3 is not able to decide what should be provided from gNB to LMF for the performance monitoring in LMF. However, we assume at least the Part A (Part A: channel measurement, quality indicator of channel measurement, time stamp of channel measurement) parameters could be provided to LMF.

Proposal 4: In Case 3b, it’s not RAN3 to decide what should be provided from gNB to LMF for the performance monitoring in LMF. We assume at least the Part A (channel measurement, quality indicator of channel measurement, time stamp of channel measurement) could be provided for this purpose.

Conclusion

In this contribution, we discussed the RAN3 impact to support Direct AI/ML positioning for AI assisted Positioning Accuracy enhancements. Based on the discussion above, the following proposals are provided:

Proposal 1: In Case 3b, the information for training data collection, e.g. channel measurement, quality indicator of channel measurement, time stamp of channel measurement could be provided from gNB to LMF. The other information, if any, is pending to RAN1.

Proposal 2: In Case 3b, how to generate the label and related data is pending to RAN1 and RAN2, no impact to RAN3.

Proposal 3: In Case 3b, NRPPa measurement procedure could be reused for training data collection, inference and monitoring for Case 3b. Any other information need to be reported from gNB to LMF is pending to RAN1.

Proposal 4: In Case 3b, it’s not RAN3 to decide what should be provided from gNB to LMF for the performance monitoring in LMF. We assume at least the Part A (channel measurement, quality indicator of channel measurement, time stamp of channel measurement) could be provided for this purpose.

Reference

- RP-240774 Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RAN1#116/#116bis/#117/#118/#118bis Chair’s Notes