3GPP TSG-RAN WG3 Meeting #126															R3-247344

Orlando, US, 18th – 22nd November 2024

Agenda item:	14.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to BL CR for TS 38.300 and TS 38.413) Discussion on the support of MBS Broadcast Service

Document for:	Discussion and Decision

# 1	Introduction

Last RAN3 agreed the intended service area may be in a form of cell ID list, TAI list, geographical area or any combination. RAN3 also agreed the BL CR for TS 38.300 ([1]). But the TP may need some modification.

# 2	Discussion

Stage-2 TP

The agreed Stage-2 ([1]) is

To support the intended service area of a broadcast service (e.g. MBS broadcast) in case the satellite footprint covers a larger area, the intended service area may be in a form of cell ID list, TAI list, geographical area or any combination as defined in TS 38.413 [26].

Editor’s Note: The above paragraphs may be further reformulated.

Current TP need to be updated for following reason

- Current TP is unclear that the intended service area (in a form of cell ID list, TAI list, geographical area) is the one received by gNB from AMF via NGAP. It is not the one provided to the UE via Uu.

- It does not define the behavior of the gNB, i.e. use the received intended service area information to provide the related information to UE, and the Uu format is to be determined by RAN2.

So we propose to update the TP as below:

To support the intended service area of a broadcast service (e.g. MBS broadcast) in case the satellite footprint covers a larger area, the AMF provides the gNB the intended service area information that may be in a form of cell ID list, TAI list, geographical area or any combination as defined in TS 38.413 [26]. Based on the received intended service area information, the gNB provides the intended service area information to the UE as defined in TS 38.331 [12].

TP for TS 38.300 can be found in Annex A – TP for TS 38.300 BL CR

Proposal 1: Update the Stage-2 TP based on Annex A – TP for TS 38.300 BL CR

Stage-3 TP

In current NGAP spec, the gNB behavior for the received MBS Service Area IE is as below:

If the MBS Service Area IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account as specified in TS 23.247 [44].

If the MBS Service Area IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall update the MBS service area and send the BROADCAST SESSION MODIFICATION RESPONSE message.

Current Stage-3 TP need to be updated to capture the gNB behavior for the received intended service area information. For example,

If the MBS Service Area IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account as specified in TS 23.247 [44], and TS 38.300 [8] in case of NTN.

If the MBS Service Area IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall update the MBS service area and send the BROADCAST SESSION MODIFICATION RESPONSE message. In case of NTN, the NG-RAN node may also update the intended service area as specified in TS 38.300 [8].

TP for TS 38.413 can be found in Annex B – TP for TS 38.413 BL CR

Proposal 2: Update the Stage-3 TP based on Annex B – TP for TS 38.413 BL CR.

How to join/leave a MBS service

In current MBS system, the AMF transfers the BROADCAST SESSION SETUP REQUEST message, which contains the N2 SM information in the received Namf_MBSBroadcast_ContextCreate Request to all NG-RANs which support MBS in the MBS service area (Step 2). The AMF determines the target NG-RAN node(s) based on the MBS service area information received from MB-SMF and the information (gNB ID, supported TAI) received from the NG-RAN nodes during the NG Setup procedure or RAN Configuration Update procedure.

In TN, operator can deploy a new cell/gNB for an existing Tracking Area. Once the AMF know the gNB starts to serve a Tracking Area of the MBS Service Area, the AMF can initiate the Broadcast Session Setup procedure in the new gNB. Similar for removing a cell/gNB from serving a Tracking Area. The AMF can initiate the Broadcast Session Release procedure to a gNB when the gNB stops to serve a Tracking Area.

Similar procedure can be reused for NTN. When a gNB/satellites moves in (or move out) a geo-area, the AMF know the serving TAIs of the gNB, so the AMF can initiate Broadcast Session Setup procedure (or Broadcast Session Release procedure) to start (or stop) the broadcast session in the satellite’s coverage.

Observation: current MBS Broadcast Session Setup/Release procedure can be reused for NTN when a gNB/satellite move in (or move out).

Proposal 3: No enhancement to current MBS Broadcast Session Setup/Release procedure when a gNB/satellite move in (or move out).

# 5	Conclusion

In this contribution, we discussed how to support MBS broadcast service in NR NTN. Our proposals are:

TP for TS 38.300 can be found in Annex A – TP for TS 38.300 BL CR

Proposal 1: Update the Stage-2 TP based on Annex A – TP for TS 38.300 BL CR

TP for TS 38.413 can be found in Annex B – TP for TS 38.413 BL CR

Proposal 2: Update the Stage-3 TP based on Annex B – TP for TS 38.413 BL CR.

Observation: current MBS Broadcast Session Setup/Release procedure can be reused for NTN when a gNB/satellite move in (or move out).

Proposal 3: No enhancement to current MBS Broadcast Session Setup/Release procedure when a gNB/satellite move in (or move out).

# References

- R3-247033, Support for Regenerative Payload and MBS broadcast in NR NTN

# Annex A – TP for TS 38.300 BL CR

### 16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS;

-	The Cell Identity used for MBS broadcast service.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.

To support the intended service area of a broadcast service (e.g. MBS broadcast) in case the satellite footprint covers a larger area, the AMF provides the gNB the intended service area information that may be in a form of cell ID list, TAI list, geographical area or any combination as defined in TS 38.413 [26]. Based on the received intended service area information, the gNB provides the intended service area information to the UE as defined in TS 38.331 [12].

NEXT CHANGE

# Annex B – TP for TS 38.413 BL CR

## 8.17	Broadcast Session Management Procedures

### 8.17.1	Broadcast Session Setup

#### 8.17.1.1	General

The purpose of the Broadcast Session Setup procedure is to request the NG-RAN node to setup MBS session resources for a broadcast MBS session. The procedure uses non-UE associated signalling.

#### 8.17.1.2	Successful Operation

Figure 8.17.1.2-1: Broadcast Session Setup, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION SETUP REQUEST message to the NG-RAN node. If the NG-RAN node accepts all the MBS QoS flows in the MBS session at least in one of its cells, the NG-RAN node responds with the BROADCAST SESSION SETUP RESPONSE message.

If the MBS Service Area IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account as specified in TS 23.247 [44], and TS 38.300 [8] in case of NTN.

If the MBS Session FSA ID List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account to determine cells/frequencies within the MBS service area to broadcast MBS session data as specified in TS 23.247 [44].

If the Supported UE Type List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall, if supported, take it into account for configuring MBS session resources.

If the Associated Session ID IE is included in the BROADCAST SESSION SETUP REQUEST message the NG-RAN node shall, if supported, take this information into account to determine whether MBS session resource sharing is possible, as specified in TS 38.300 [8].

NEXT CHANGE

### 8.17.2	Broadcast Session Modification

#### 8.17.2.1	General

The purpose of the Broadcast Session Modification procedure is to request the NG-RAN node to update the MBS session resources or the area related to a previously established broadcast MBS session. The procedure uses non-UE associated signalling.

#### 8.17.2.2	Successful Operation

Figure 8.17.2.2-1: Broadcast Session Modification, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION MODIFICATION REQUEST message to the NG-RAN node.

If the MBS Service Area IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall update the MBS service area and send the BROADCAST SESSION MODIFICATION RESPONSE message. In case of NTN, the NG-RAN node may also update the intended service area as specified in TS 38.300 [8].

If the MBS Session Modification Request Transfer IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall replace the previously provided information by the newly received one and update the MBS session resources and area as requested and send the BROADCAST SESSION MODIFICATION RESPONSE message.

If the Supported UE Type List IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall, if supported, take it into account for configuring MBS session resources.

If the MBS NG-U Failure Indication IE is included in the BROADCAST SESSION MODIFICATION REQUEST message within the MBS Session Setup or Modification Request Transfer IE and set to "N3mb path failure", the NG-RAN node may provide new NG-U transport layer information to replace the failed one(s) or switch the data delivery to another 5GC as per the Broadcast MBS session restoration procedure for N3mb Path Failure specified in TS 23.527 [57].