3GPP TSG-RAN WG3 Meeting #126	R3-247414

Orlando, US, 18th – 22nd November, 2024

Title: 	(TP for AI/ML BLCR to TS 38.423) Discussion on the AI/ML-based Network Slicing

Source: 	Huawei

Agenda item:	11.2

Document Type:	Discussion and Decision

# 1. Introduction

In the last meeting RAN3 started discussing the objectives of the new Rel-19 WI “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing.

This contribution analyzes the technical enablers for the AI/ML-based Network Slicing use case to be introduced over the Xn interface, and provides proposals for discussion based on the outcome from the last RAN3 meeting.

# 2. Discussion

In the last RAN3 meeting the following agreements and open issues were achieved [4]:

Stage3 signalling design: Enhance the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure to enable requesting and reporting of the predicted radio resource status per slice and predicted slice available capacity over the Xn interface. Introduce Slice To Report List for Data Collection IE including PLMN ID and S-NSSAI in DATA COLLECTION REQUEST message. Slice UE performance: Observation based on current specs: Average packet delay UL/DL, average UE throughput (UL/DL), and average packet loss (DL) can be calculated per PLMN ID and per QoS level and per supported S-NSSAI. WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node. FFS if other finer granularity of UE performance is feasible.

We will first discuss further the Stage 3 signalling details for the Xn interface and then we provide our views on the granularity of the UE performance feedback for the AI/ML-based Network Slicing use case.

## 2.1 XnAP Stage 3 signalling details

RAN3 agreed that the Data Collection Reporting Initiation and Data Collection Reporting procedures over Xn need to be enhanced to request and report the predicted radio resource status per slice and the predicted slice available capacity. Moreover, the Slice To Report List for Data Collection IE was agreed to be introduced in the DATA COLLECTION REQUEST message pertaining the Data Collection Reporting Initiation procedure – this has already been reflected in the XnAP BLCR for AI/ML-based Network Slicing in [5].

Concerning the predicted radio resource status per slice, several contributions from the last meeting proposed to re-use the existing Predicted Radio Resource Status IE in the DATA COLLECTION UPDATE message which currently refers to legacy Radio Resource Status IE where only the SSB Area Radio Resource Status List IE is included. In such contributions, it is proposed to modify the semantics description of the Predicted Radio Resource Status IE in order to also include the Slice Radio Resource Status List IE already specified in the Radio Resource Status IE. We think that such approach does not work because, differently from the Resource Status Reporting (Initiation) procedures, the Data Collection Reporting (Initiation) procedures have been designed to support partial reporting. As a consequence, we think that we should introduce a new Predicted Slice Radio Resource Status IE distinct from the legacy Predicted Radio Resource Status IE in order to cover the case where the requesting node requests reporting of both the new Predicted Slice Radio Resource Status and the legacy Predicted Radio Resource Status IEs, but the reporting node can only provide one of these two predictions (and indicates the failure in providing the other one via the DATA COLLECTION RESPONSE message).

Moreover, in our understanding, the predicted radio resource status per slice is a per cell per slice information, hence the new Predicted Slice Radio Resource Status IE should be included in the Cell Measurement Result for Data Collection List IE of the DATA COLLECTION UPDATE message.

Proposal 1: RAN3 to agree to introduce a new Predicted Slice Radio Resource Status IE within the Cell Measurement Result for Data Collection List IE of the DATA COLLECTION UPDATE message which provides the predicted radio resource status per slice.

The new Predicted Slice Radio Resource Status IE refers to the existing Radio Resource Status IE where, in the semantics descriptions, it is specified that the new IE only includes the Slice Radio Resource Status List IE.

Proposal 2: RAN3 to agree that the new Predicted Slice Radio Resource Status refers to the existing Radio Resource Status IE and, in the semantics description, it is specified that the new IE only includes the Slice Radio Resource Status List IE.

When considering the predicted slice available capacity, instead, the straightforward way to enable the reporting of such information is to introduce a new Predicted Slice Available Capacity IE in the DATA COLLECTION UPDATE message. In our view the predicted slice available capacity is a per cell per slice information, hence the new Predicted Slice Available Capacity IE should be included in the Cell Measurement Result for Data Collection List IE of the DATA COLLECTION UPDATE message, same as for the proposed new Predicted Slice Radio Resource Status IE.

Proposal 3: RAN3 to agree to introduce a new Predicted Slice Available Capacity IE within the Cell Measurement Result for Data Collection List IE of the DATA COLLECTION UPDATE message which provides the predicted slice available capacity.

In our view the new Predicted Slice Available Capacity IE should refer to the existing Slice Available Capacity IE.

Proposal 4: RAN3 to agree that the new Predicted Slice Available Capacity refers to the existing Slice Available Capacity IE.

In order for the requesting node to request the reporting of the predicted radio resource status per slice and predicted slice available capacity, new bits for Predicted Slice Radio Resource Status and Predicted Slice Available Capacity IEs in the Report Characteristics for Data Collection IE of the DATA COLLECTION REQUEST message need to be introduced.

Proposal 5: RAN3 to agree to introduce new bits for Predicted Slice Radio Resource Status and Predicted Slice Available Capacity IEs in the Report Characteristics for Data Collection IE of the DATA COLLECTION REQUEST message.

When it comes to the partial reporting aspects, that is, how to indicate the failure in reporting some of the requested information by the requesting node, the following was captured in the meting minutes from the last meeting [4]:

The failure of the predicted radio resource status per slice and predicted slice available capacity should be indicated to the sending node? Per cell per slice?

Based on what we stated before that the new IEs proposed to be introduced in the Data Collection Reporting (Initiation) procedures – Predicted Slice Radio Resource Status and Predicted Slice Available Capacity IEs – should be per cell information, we think that two new bits should be introduced in the Cell Measurement Failed Report Characteristics IE of the DATA COLLECTION RESPONSE message so that the reporting node can indicate that either Predicted Slice Radio Resource Status or Predicted Slice Available Capacity cannot be reported to the requesting node (with an associated failure cause).

Proposal 6: RAN3 to agree to introduce new bits for Predicted Slice Radio Resource Status and Predicted Slice Available Capacity IEs in the Cell Measurement Failed Report Characteristics IE of the DATA COLLECTION RESPONSE message.

The TP for the XnAP BLCR in the Annex 1 reflects the Proposals above.

In the last meeting the contribution in [7] proposed to introduce the predicted Composite Available Capacity (CAC) to be reported per cell together with the predicted slice available capacity. The legacy CAC information indicates the amount of resources per cell and per SSB area that are available relative to the total NG-RAN resources per cell, while the slice available capacity indicates the amount of resources per network slice that are available per cell relative to the total NG-RAN resources per cell. And in legacy Resource Status Reporting over Xn, the slice available capacity is reported together with the CAC because CAC allows to correctly interpret the slice available capacity for cases where some slices share resources from the same resource pool. We think that, for the AI/ML-based Network Slicing, we should follow the legacy principle and hence discuss whether the predicted CAC should be introduced to be reported by the reporting node together with the predicted slice available capacity.

Proposal 7: RAN3 to discuss whether predicted composite available capacity should be introduced and configured to be reported together with the predicted slice available capacity.

## 2.2 Finer granularity of UE performance feedback

With regard to the feedback information, RAN3 agreed during the SI phase to collect the following information from (neighbour) gNBs, as reflected in TR 38.743 [3]:

#### 4.1.2.4 Feedback of AI/ML based Network Slicing To optimize the performance of AI/ML-based network slicing model, the following feedback can be considered to be collected from gNBs: - Measured Radio resource status per slice - Measured Slice available capacity - Legacy UE performance feedback for those UEs handed over from the source gNB - Finer granularity UE performance feedback for those UEs handed over from the source gNB to determine UE performance for a certain slice in use by a certain UE. NOTE: Exact level of finer granularity to be defined in normative phase.

The transfer over Xn of all the pieces of information listed above is already supported by the standard: measured radio resource status per slice and measured slice available capacity can be transferred over Xn via the Radio Resource Status Reporting (Initiation) procedures, while legacy Rel-18 UE performance feedback can be exchanged over Xn via the Data Collection Reporting (Initiation) procedures. In the current specification, the granularity of UE performance is per UE, and the current UE Performance IE includes Average UE Throughput DL/UL, Average Packet Delay and Average Packet Loss DL.

In the last meeting the majority of companies preferred to have UE performance to be reported on a per PDU session per slice granularity, meaning that UE performance should be collected and reported for each PDU session belonging to a certain slice in use by the UE. This led to the following Working Assumption and it is for further discussion whether other finer granularities are feasible [4]:

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node. FFS if other finer granularity of UE performance is feasible.

Still in the last meeting three possible options for the UE performance finer granularity have been discussed:

- Option 1: per PDU session per slice (S-NSSAI)

- Option 2: per slice (S-NSSAI)

- Option 3: per-QoS-flow-group

Among the three options above, Option 3 is the finest granularity, while Option 2 is the coarsest one. Therefore, it seems that Option 1 is a compromise which should provide enough granular information for the requesting node to evaluate its AI/ML model. However, we have some concerns on the actual feasibility of such granularity, and we suggest checking with SA5 before turning the WA from the last meeting into an agreement. Our understanding is that UE performance metrics can be measured on a per DRB granularity as per TS 28.558, and multiple DRBs can be mapped into the same PDU session. Therefore, we should ask SA5 how per PDU session measurement could be derived from the per DRB measurements (e.g., weighted averaging of per DRB measurements performed over all configured DRBs for the concerned UE) so to ensure that source and target gNBs apply the same methodology for deriving per PDU session per slice UE performance feedback before and after an AI/ML-based mobility action while the UE is using a certain slice, hence making the comparison between the two per PDU session measurements meaningful.

For Option 3 (per-QoS-flow-group UE performance), it is still unclear whether this could actually be implemented. In our understanding the “QoS-flow-group” represents the set of QoS flows that are mapped to a certain DRB. Therefore, in the source gNB side, the gNB can measure per DRB UE performance metrics (as per TS 28.558) and then implicitly derive the per-QoS-flow-group measurement (which corresponds exactly to the per DRB measurement). However, after the AI/ML-based mobility action for the UE using a certain slice, the target gNB may decide to map the set QoS flows (mapped into a single DRB in the source gNB) into multiple different DRBs. In other words, we think that only in case the target node decides to map the same set of QoS flows into a single DRB (as done by the source gNB) the comparison of UE performance before and after the AI/ML-based mobility action could be possible. However, it is kind of a corner case that this could occur, and for sure the target node cannot be forced to apply the same QoS-flows-to-DRB mapping as used by the source gNB.

When it comes to Option 2 (per slice UE performance), instead, it is worth to consider that, for the AI/ML-based Network Slicing, we focused during the SI on slice-level resource allocation and slice-based load balancing/mobility actions, therefore it would make sense to provide UE performance feedback on a per slice level as well. Moreover, in our understanding, per slice granularity of UE performance metrics is already supported by the standard as per SA5’s TS 28.558 [6].

Observation 1: Per slice UE performance is useful to evaluate AI/ML-based slice-level resource allocation and slice-based load balancing/mobility actions, and per slice granularity of UE performance metrics is already supported by the standard as per SA5’s TS 28.558.

Proposal 8: RAN3 to agree to liaise SA5 to check the feasibility of per PDU session per slice UE performance before turning the WA from RAN3#125bis into an agreement. A confirmation from SA5 on the standard support of per slice UE performance could also be requested.

A draft LS to SA5 can be found in Annex 2 of this paper.

# 3. Conclusion

Based on the above discussion, we have the following observation and proposals:

XnAP Stage 3 signalling details

Proposal 1: RAN3 to agree to introduce a new Predicted Slice Radio Resource Status IE within the Cell Measurement Result for Data Collection List IE of the DATA COLLECTION UPDATE message which provides the predicted radio resource status per slice.

Proposal 2: RAN3 to agree that the new Predicted Slice Radio Resource Status refers to the existing Radio Resource Status IE and, in the semantics description, it is specified that the new IE only includes the Slice Radio Resource Status List IE.

Proposal 3: RAN3 to agree to introduce a new Predicted Slice Available Capacity IE within the Cell Measurement Result for Data Collection List IE of the DATA COLLECTION UPDATE message which provides the predicted slice available capacity.

Proposal 4: RAN3 to agree that the new Predicted Slice Available Capacity refers to the existing Slice Available Capacity IE.

Proposal 5: RAN3 to agree to introduce new bits for Predicted Slice Radio Resource Status and Predicted Slice Available Capacity IEs in the Report Characteristics for Data Collection IE of the DATA COLLECTION REQUEST message.

Proposal 6: RAN3 to agree to introduce new bits for Predicted Slice Radio Resource Status and Predicted Slice Available Capacity IEs in the Cell Measurement Failed Report Characteristics IE of the DATA COLLECTION RESPONSE message.

The TP for the XnAP BLCR in the Annex 1 reflects the Proposals above.

Proposal 7: RAN3 to discuss whether predicted composite available capacity should be introduced and configured to be reported together with the predicted slice available capacity.

Finer granularity of UE performance feedback

Observation 1: Per slice UE performance is useful to evaluate AI/ML-based slice-level resource allocation and slice-based load balancing/mobility actions, and per slice granularity of UE performance metrics is already supported by the standard as per SA5’s TS 28.558.

Proposal 8: RAN3 to agree to liaise SA5 to check the feasibility of per PDU session per slice UE performance before turning the WA from RAN3#125bis into an agreement. A confirmation from SA5 on the standard support of per slice UE performance could also be requested.

A draft LS to SA5 can be found in Annex 2 of this paper.

# 4. Reference

- RP-242385, New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, 3GPP RAN#105

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- R3-245850, (BL CR to 38.423) Support of AI/ML assisted Network Slicing, ZTE Corporation, Qualcomm, China Unicom, Ericsson, Samsung, Nokia, Lenovo, CATT, Huawei, NEC, CMCC, China Telecom, Telecom Italia, Deutsche Telekom, Verizon Wireless, LGE, Jio, Rakuten

- 3GPP TS 28.558, Management and orchestration; UE level measurements for 5G system, Rel-18

- R3-245603, Extension of Data Collection Procedures for AI/ML Slicing, Nokia, Jio

# 5. Annex 1 – TP for AI/ML BLCR to TS 38.423 (based on R3-245850)

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 8.4.13	Data Collection Reporting Initiation

#### 8.4.13.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.4.13.2	Successful Operation

Figure 8.4.13.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start information reporting or to stop information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the Report Characteristics for Data Collection IE indicates cell-specific information reporting, the Cell To Report List for Data Collection IE shall be included.

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the Report Characteristics for Data Collection IE indicates slice-specific information reporting, the Slice To Report List for Data Collection IE shall be included.

If NG-RAN node2 is capable of providing all of the requested information, it shall initiate the information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

If NG-RAN node2 is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE or the Cell Measurement Initiation Result List IE or both in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the Requested Prediction Time IE in the DATA COLLECTION REQUEST message is present, it indicates the specific point in time to which the prediction of the requested information applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the UE Trajectory Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE trajectory collection and reporting. NG-RAN node2 shall report the UE trajectory only once. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Trajectory IE;

-	the number of visited cells within NG-RAN node2 is equal to the value of the Number of Visited Cells IE, if included;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to a cell belonging to an NG-RAN node different from NG-RAN node2.

The result of the UE trajectory collection is reported at the next available DATA COLLECTION UPDATE message.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects NG-RAN node2 performs measurements or predictions on. NG-RAN node2 shall include in the DATA COLLECTION UPDATE message:

-	the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted Number of Active UEs IE, if the second bit, "Predicted Number of Active UEs" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted RRC Connections IE, if the third bit, "Predicted RRC Connections" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput DL IE, if the fourth bit, "Average UE Throughput DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput UL IE, if the fifth bit, "Average UE Throughput UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Delay IE, if the sixth bit, "Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Loss DL IE, if the seventh bit, "Average Packet Loss DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Energy Cost IE, if the eighth bit, "Energy Cost" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Measured UE Trajectory IE, if the ninth bit, "Measured UE Trajectory" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

- 	the Predicted Slice Radio Resource Status IE, if the tenth bit, " Predicted Slice Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

- 	the Predicted Slice Available Capacity IE, if the eleventh bit, " Predicted Slice Available Capacity" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

#### 8.4.13.3	Unsuccessful Operation

Figure 8.4.13.3-1: Data Collection Reporting Initiation, unsuccessful operation

If none of the requested information can be initiated, NG-RAN node2 shall send the DATA COLLECTION FAILURE message with an appropriate cause value.

#### 8.4.13.4	Abnormal Conditions

For the same Measurement ID, if the initiating NG-RAN node1 does not receive either the DATA COLLECTION RESPONSE message or the DATA COLLECTION FAILURE message, the NG-RAN node1 may reinitiate the Data Collection Reporting Initiation procedure towards the same NG-RAN node, provided that the content of the new DATA COLLECTION REQUEST message is identical to the content of the previously unacknowledged DATA COLLECTION REQUEST message.

If the NG-RAN node2 receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "stop" and if the NG-RAN node2 Measurement ID value received in the DATA COLLECTION REQUEST message is not used, the NG-RAN node2 shall initiate DATA COLLECTION FAILURE message with an appropriate cause value.

If in the Report Characteristics for Data Collection IE bitmap all bits are set to "0" in the DATA COLLECTION REQUEST message, then NG-RAN node2 shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

If the NG-RAN node2 receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "start" and the NG-RAN node1 Measurement ID IE corresponding to an existing on-going Data Collection reporting, then NG-RAN node2 shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.26	DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory, Tenth Bit = Predicted Slice Radio Resource Status, Eleventh Bit = Predicted Slice Available Capacity. Other bits are ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Slice to Report List for Data Collection |  | 0..1 |  | S-NSSAI list to which the request applies. | YES | ignore |
| >>>Slice to Report Item for Data Collection |  | 1 .. < maxnoofBPLMNs > |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | - |  |
| >>>>S-NSSA List |  |  | 1 |  | - |  |
| >>>>>S-NSSA Item |  | 1 .. < maxnoofSliceItems> |  |  | - |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | - |  |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) | YES | ignore |
| UE Trajectory Collection Configuration | O |  | 9.2.3.185 |  | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.2.3.186 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.27	DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Delay, Fifth Bit = Average Packet Loss DL, Sixth Bit = Measured UE Trajectory. Other bits are ignored by the NG-RAN node1. | – |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Cell Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | reject |
| >Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Cell Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Cell Measurement Failure Cause Item |  | 1 .. <maxFailedCellMeasObjects> |  |  | – |  |
| >>>>Cell Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections, Fourth Bit = Predicted Slice Radio Resource Status, Fifth Bit = Predicted Slice Available Capacity. Other bits are ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.29	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result for Data Collection List |  | 0..1 |  |  | YES | ignore |
| >Cell Info Result for Data Collection Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Predicted Radio Resource Status | O |  | Radio Resource Status 9.2.2.50 | The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE. | – |  |
| >>Predicted Number of Active UEs | O |  | Number of Active UEs 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | RRC Connections 9.2.2.56 |  | – |  |
| >>Predicted Slice Radio Resource Status | O |  | Radio Resource Status 9.2.2.50 | The IE only includes the Slice Radio Resource Status List IE. |  |  |
| >>Predicted Slice Available Capacity | O |  | Slice Available Capacity 9.2.2.55 |  |  |  |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1. | – |  |
| >>UE Performance | O |  | 9.2.3.179 |  | – |  |
| >>Measured UE Trajectory | O |  | 9.2.3.182 | It contains information about cells that a UE has connected to. | – |  |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

# 6. Annex 2 – [Draft] LS to SA5 on granularity of UE performance metrics for AI/ML-based Network Slicing

3GPP TSG-RAN3 Meeting #126	R3-24xxxx

Orlando, US, 18th – 22nd November, 2024

Title:	[Draft] LS on granularity of UE performance metrics for AI/ML Network Slicing

Response to:

Release:	Release 19

Work Item:	NR_AIML_NGRAN_enh

Source:	Huawei [to be RAN3]

To:	SA5

Cc:	RAN2

Contact person:	Damiano Rapone

damiano.rapone@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

As part of the Rel-19 Work Item on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, RAN3 is currently discussing the granularity of UE performance metrics for a UE served by a certain slice to be reported from a target gNB to a source gNB over Xn after an AI/ML-based mobility action (i.e., handover) took place.

The UE performance metrics are Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay and Average Packet Loss DL, already specified in TS 38.423 in Rel-18 on a per UE granularity.

In RAN3#125bis RAN3 converged on a Working Assumption based as the following:

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node.

RAN3 understanding is that UE performance metrics can be measured on a per DRB granularity as per TS 28.558, and multiple DRBs can be mapped into the same PDU session. Therefore, RAN3 requires SA5 to indicate how per PDU session measurement can be derived from the per DRB measurements.

Moreover, RAN3 discussed the feasibility of UE performance on a per slice granularity and RAN3 understanding is that per slice (i.e., per S-NSSAI) UE performance metrics can already be provided according to what is specified in TS 28.558 clauses 6.3.1.1, 6.3.1.2.1, 6.3.1.4.1 and 6.3.1.4.2.

# 2	Actions

To SA5

ACTION: 	RAN3 respectfully asks SA5 to requires SA5 to indicate how per PDU session measurement can be derived from the per DRB measurements.

RAN3 respectfully asks confirmation from SA5 that UE performance on a per slice (i.e., per S-NSSAI) granularity UE performance metrics can already be provided according to what is specified in TS 28.558 clauses 6.3.1.1, 6.3.1.2.1, 6.3.1.4.1 and 6.3.1.4.2.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#127			2025-02-17 – 2025-02-21				Athens, GR

RAN3#127bis			2025-04-07 – 2025-04-11				China, CN