3GPP TSG RAN3 meeting #126    	         	                         R3-247650

18th – 22th Nov 2024  Orlando, USA

Agenda Item:	10.3.2

Source:	China Unicom

Title:	Discussion on SONMDT enhancements for network slicing

Document for:	Discussion

Introduction

The Work Item on SON/MDT enhancement has been agreed in RAN#102 (RP-234038) with the following objectives:

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] - Support of SON/MDT enhancements for [RAN3, RAN2]: - Intra-NTN mobility - Network Slicing - Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure

This paper discusses the SON/MDT enhancements for network slicing.

Discussion

MDT Enhancement for Network Slicing

The network can monitor the PDU session which is associated with the configured slice, if the PDU session is failed, no matter the setup fails or handover to target cell fails, and the failure reason is related to the slice, such as resources not available for the slice(s), slice(s) not supported by NG-RAN, etc, the network should report the MDT measurement report to the TCE, and the network can optimize the slice coverage or resource allocation according to the MDT measurement report.

For the MDT measurement reporting, it will has a large scale of reporting, since the MDT measurement reporting is aim to optimize the slice related fail, it can be event trigger. If the PDU session related to the configured slice(s) is(are) failed, the UE should record the slice fail reason and the PDU session information, then send the report to the network.

Proposal 1: The network can configure the event trigger immediate MDT measurement to avoid large scale of MDT measurement reporting.

Proposal 2: The UE shall report the MDT measurement reporting to the TCE when the PDU session related to the configured slice(s) is(are) failed.

MRO enhancement for Network Slicing

Slicing is introduced in 5G network, and the slice information can be configured together with the PDU session setup procedure. The network can provide differentiated services for different slices, but the optimization for slicing coverage is not together with the optimization of the radio signaling coverage. There may be slice coverage hole even if the radio coverage is well, when the UE already configured with a slice service may HO to a target cell, which cannot support this slice service and thus causing service discontinuity. The slice coverage may need to be enhanced.

In the UE Information report, the history of UE visited cells will be reported to the network, and the slice information associated with the PDU session of the UE can be reported to the network. When the handover happen, the History Information from the UE will be transferred from the source cell to the target cell in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the collected information and use it for future handover preparations. The target cell may analyse which future candidate cells support the services of certain slices, and which future candidate cells will have high success rate of handover, then it will make the final handover decision considering about this.

Proposal 3: The slices information associated with the PDU sessions of the UE can be reported to the network for slice optimization.

Conclusion

Proposal 1: The network can configure the event trigger immediate MDT measurement to avoid large scale of MDT measurement reporting.

Proposal 2: The UE shall report the MDT measurement reporting to the TCE when the PDU session related to the configured slice(s) is(are) failed.

Proposal 3: The slices information associated with the PDU sessions of the UE can be reported to the network for slice optimization.