3GPP TSG-RAN WG4 #113	R4-2417513

Orlando, US, November 18-22, 2024

Agenda item:	7.22.2.1		On-demand SSB SCell Operation

Source: 	Qualcomm Incorporated

Title: 	RRM requirements for on-demand SSB SCell operation

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 work on Rel-19 NES was started and the work plan has been agreed in [1]. According to the work plan, the first focus for on-demand SSB is on MAC CE based SCell activation. In a second step, RAN4 may discuss other SCell activation scenarios if needed.

Several agreements have been made for on-demand SSB according to [3] and additional open issues have been identified in [2]. In this paper we provide our views for the issues identified in the way forward as defined in [3] in RAN4 #112bis.

# Discussion

In our opinion, the following topics need to be addressed first

- L3 OD-SSB measurements for deactivated SCell. In order to keep the design simple, RAN4 should aim to treat cases #1 and #2 the same

- The condition of known SCell needs to be defined when OD-SSB are sent. It should follow the existing definition of known SCell as much as possible.

- The timeline for reception of on-demand SSB needs to be agreed, i.e., when the UE is ready to process the on-demand SSB

In the following we provide our view of the topics identified above.

## Deactivated SCell L3 Measurements

In RAN4 #112bis, L3 measurements based on OD-SSB have been discussed for the deactivated SCell for both cases #1 (al-ways-on SSB not present) and #2 (always-on SSB present). For case #1 it has been agreed that a fast L3 measurement within a window should be support and the definition of the terms ‘fast’ and ‘window’ is FFS. The same agreement has not yet mean achieved for case #2, but in our opinion, there is no reason why we should not do a fast measurement for case #2. Both cases #1 and #2 should be treated as similar as possible.

Observation 1: In the same way as agreed for case #1, a fast L3 SCell measurement for deactivated SCell should be supported for case #2

Observation 2: To keep the design for L3 measurements for deactivated SCell simple, both cases #1 and #2 should be treated as similar as possible.

Fast L3 measurement for deactivated SCell in our understanding means that the measurement period is shorter than measCycleScell, i.e., the measurement should be based on OD-SSB periodicity. Here it is implicitly assumed that OD-SBB periodicity is smaller than measCycleScell. Hence, we have the following proposals:

Proposal 1: RAN4 to introduce requirements for L3 measurement for deactivated SCell based on OD-SSB that follows OD-SSB periodicity.

Proposal 2: RAN4 to introduce requirements for L3 measurement for deactivated SCell that are identical for cases #1 and #2.

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1)

Agreements:

Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window.

- After the time window, UE can perform deactivated SCell measurement with a slow mode.

  - Option 2-1: fallback legacy requirement

  - Option 2-2: stop measurement

  - Option 2-3: No requirement

- Time window means the time duration starting from receiving OD-SSB activation indication

  - FFS the time window configuration or pre-defined

- FFS the fast L3 measurement requirement

- FFS the relation between time window and OD-SSB deactivation command in the time domain

When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

Issue 1-3-4: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2)

Agreements:

OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2)

  - Option 1: RAN4 requirement is defined based on OD-SSB periodicity.(CATT, Samsung, CMCC, ZTE)

  - Option 2: RAN4 requirement is defined based on legacy measCycleSCell (Apple, Xiaomi, vivo, OPPO)

  - Option 3:  RAN4 requirement is defined based on combining the always-on SSB and OD-SSB. (Ericsson, LG, ZTE)

  - Option 4: RAN4 for further discussion (Nokia, LG, CMCC, ZTE, Qualcomm)

  - Option 5: RAN4 requirement is defined based on the indicated measurement periodicity (either OD-SSB periodicity or measCycleSCell for always-on SSB) (vivo)

In the following we outline our view of L3 measurement for deactivated SCell. Figure 2-1 shows legacy measurement for the example of FR1. The UE is supposed to measure with a periodicity of measCycleSCell independent what the periodicity of the SSB is. The accuracy requirements for L3 measurement allow to combine measurement of 5 SSBs. For a SCell to be known, a valid measurement report needs to be sent to the network in the period 5*max(measCycleSCell, DRX cycle).

Figure 2-1: Deactivated SCell Measurements with always-on SSB

That means the UE is not supposed to measure each always-on SSB for the deactivated SCell but just up to five times within the period 5*max(measCycleSCell, DRX cycle). A NES gain can be achieved if the time span of those SSBs to be measured is reduced when OD-SSB are sent with a smaller periodicity. But on the other side, the UE should not be supposed to measure more often since then the power consumption of the UE would increase significantly. Hence, measurement requirements should be defined in a way that do not mandate the UE to measure more often. Also, no new accuracy requirements for measurements for deactivated SCell should be defined.

Observation 3: If the UE is supposed to measure the OD-SSB more often than in the existing requirements for legacy measurements, the power consumption of the UE would increase.

Proposal 3: RAN4 to define requirements that do not increase the frequency of L3 measurements for deactivated SCell.

Observation 4: No new accuracy requirements need to be defined for L3 measurements for a deactivated SCell to be known in case #2, scenario #2A, if the same number of SSB occurrences as for always-on SSB is allowed for on-demand SSB.

Proposal 4: No new measurement accuracy requirements should be defined for on-demand SSB.

Figure 2-2 illustrates how this could be achieved. A time window should be defined that comprises five SSBs and the UE is supposed to measure only in this time window. Due to the five SSBs in the time window the same accuracy requirement can be achieved.

Figure 2-2: Deactivated SCell Measurements with always-on SSB and OD-SSB

Proposal 5: RAN4 to define a time window for OD-SSB measurement of deactivated SCell that comprises five SSBs.

Proposal 6: The UE is only supposed to measure in this measurement time window.

To avoid that the frequency of OD-SSB based measurements increases, the measurement time window should have a periodicity of 5*max(measCycleSCell, DRX cycle). All other always-on SSB or OD-SSB in between two measurement time windows can be ignored by the UE for L3 measurements in deactivated SCell. Figure 2-2 illustrates the measurement and how often a L3 measurement report is sent.

Proposal 7: To avoid that the frequency of OD-SSB based measurements increases, the measurement time window should have a periodicity of 5*max(measCycleSCell, DRX cycle).

Proposal 8: All other always-on SSB or OD-SSB in between two measurement time windows can be ignored by the UE for L3 measurements in deactivated SCell.

In case that the measurement time window only consists of OD-SSB as it would the case for case #1, the OD-SSB are periodic across a time span. That simplifies potential combination of measurements. In case that the measurement time window includes also always-on SSB, both always-on SSB and OD-SSB should fit into the same periodic pattern as in illustrated Figure 2-3.

Figure 2-3: SSB Structure of Measurement Time Window

If this is the case, the processing of always-on SSB and OD-SSB is easier, and implementation gets simplified. If the measurement time window includes both OD-SSB and always-on SSB, requirements should only be defined for the case when both types of SSB fit into the same periodic structure.

Observation 5: SSB processing and implementation in the UE gets simplified if both always-on SSB and OD-SSB fit into the same periodic pattern.

Proposal 9: If the measurement time window includes both OD-SSB and always-on SSB, requirements should only be defined for the case when both types of SSB fit into the same periodic structure.

## Condition for Known/Unknown SCell

In RAN4 112# it was agreed that the same principle as in legacy is applied for the known cell condition in case of OD-SSB, i.e., the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command.

Issue 1-4-3: known cell condition design – principle

Agreement:

- The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command.

- When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met.

  - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB

Other principles are not precluded.

We first repeat two proposals from our paper in RAN4 #112bis since there has not yet been a formal agreement, though we have the impression that they are common understanding in the group.

Proposal 10: The condition for SCell to be known is based L3 measurements as in existing SCell activation scenarios.

Proposal 11: L1 measurements are not supported for deactivated SCell.

The condition of the SCell to be known is fulfilled if, among others, the UE has sent back a valid L3 measurement report within a certain period before the SCell activation. For OD-SSB this report should be based on OD-SSB based measurements. In TS 38.133, different time periods are defined for a SCell to be known depending of the mode of SCell activation. For example, for in chapter 8.3.2 the period is defined as 5*max(measCycleSCell, DRX cycle) for FR1 and as 4 seconds for FR2. In chapter 8.3.4 for direct SCell activation at SCell addition, the time is defined as 5 seconds for FR1 and again as 4 seconds for FR2. Since in some cases a fixed time is assumed for the period when a valid measurement report has to be sent, although in all legacy cases for always-on SSB the measurement period is measCycleSCell, it is seen that the period for the known SCell condition and the measurement period are not necessarily related. That also means that a new measurement period for OD-SSB does not imply that the period for the known SCell condition needs to be changed and potentially the legacy period in the known SCell condition can be reused.

Proposal 12: The measurement report to be sent for the known SCell condition should be based on OD-SSB in case OD-SSB are configured.

Observation 6: Different periods for a cell to be known are defined in RAN4 for different types of SCell activation.

Proposal 13: RAN4 to investigate whether the period for the known SCell condition can be reused from legacy or whether new period durations should be introduced.

## MAC CE Timeline

In [4], an LS was received from RAN1 with the following question:

RAN1 would like to ask RAN4 if the working assumption of T_min=. can be confirmed to agreement.

Figure 2-4 illustrates the RAN1 LS. A MAC CE triggered OD-SSB is received in slot #n and the corresponding PDSCH is acknowledged in slot #(n+m). In this slot the MAC CE is processed and the information contained in the MAC CE is available in the UE. This delay is unrelated to the OD-SSB and applies to each MAC CE. In case MAC CE triggers OD-SSB, the delay of m slots till HARQ feedback is sent to the network can remain unchanged.

Then additional 3 ms are left to the UE to get ready for the reception of the OD-SSB. Hence, T_min is the minimal time when the UE needs to be ready to process the OD-SSB. These additional 3 ms apply so far for A-TRS and aperiodic CSI-RS. However, the RAN1 LS extends the time of 3 ms when the UE is ready to process A-TRS or aperiodic CS-RS to SSB. Since A-TRS and aperiodic CSI-RS on the one side and SSB on the other side have usually different purposes, it cannot easily be assumed that the same timeline can be applied for these signals.

Figure 2-4: Illustration of T_min

Observation 7: Since A-TRS and aperiodic CSI-RS on the one side and SSB on the other side have usually different purposes, it cannot easily be assumed that the same timeline can be applied for these signals.

Hence, we propose that an additional margin  is applied to the timeline. It should be discussed further in RAN4 what margin is required and whether it is needed to define different margins for different scenarios.

Proposal 14: An additional margin  in slots should be added to T_min=m+3Nslotsubframe,μ+1, i.e., T_min=m+3Nslotsubframe,μ+∆. ∆ >1 is FFS in RAN4.

RAN4 should draft the LS reply to RAN1 accordingly that the RAN1 working assumption cannot be confirmed.

Proposal 15: RAN4 should agree that the RAN1 working assumption of Tmin=m+3Nslotsunframe,μ+1 cannot be confirmed.

## Miscellaneous

In RAN4 #112bis two LSs have been sent to RAN2 and RAN1 in [5] and [6] to get further clarification whether there is impact on SMTC configuration and what the relation between OD-SSB and always-on SSB in time, frequency, spatial relation is. Hence, discussions related to SMTC and the relation between OD-SSB and always-on SSB in time, frequency, spatial relation in RAN4 should be delayed till further clarification from RAN2 and RAN1 is received.

Proposal 16: Discussions related to SMTC and the relation between OD-SSB and always-on SSB in time, frequency, spatial relation should be delayed in RAN4 till further clarification from RAN2 and RAN1 is received.

However, there seems to be a common interest for the case that always-on SSB and OD-SSB are in the same frequency, have the same offset but different frequencies. Since this scenario is supported by RAN1 configuration, RAN4 should continue the discussions and specification effort for this reference scenario.

Proposal 17: RAN4 to focus first on the case that always-on SSB and OD-SSB are in the same frequency, have the same offset but different frequencies.

We also want to point out that the current discussions only consider measurements for a single deactivated NES SCell. Whether and/or how to extend this is multiple NES SCell should be subject to a separate discussion / decision.

Observation 8: It is observed that the current discussions only consider measurements for a single deactivated NES SCell.

Proposal 18: Whether and/or how to extend this is multiple NES SCell should be subject to a separate discussion / decision.

The existing specification in 38.133 allows interruptions during measurements on deactivated SCell when the measCycleSCell is larger than or equal to 640 ms. RAN4 should investigate whether there is impact if the measurement period for OD-SSB is changed.

Observation 9: The existing specification in 38.133 allows interruptions during measurements on deactivated SCell when the measCycleSCell is larger than or equal to 640 ms.

Proposal 19: RAN4 should investigate whether there is impact due to OD-SSB on the allowed interruptions during measurements on deactivated SCell.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #112bis. Our observations and proposals are summarized as follows:

Observation 1: In the same way as agreed for case #1, a fast L3 SCell measurement for deactivated SCell should be supported for case #2

Observation 2: To keep the design for L3 measurements for deactivated SCell simple, both cases #1 and #2 should be treated as similar as possible.

Proposal 1: RAN4 to introduce requirements for L3 measurement for deactivated SCell based on OD-SSB that follows OD-SSB periodicity.

Proposal 2: RAN4 to introduce requirements for L3 measurement for deactivated SCell that are identical for cases #1 and #2.

Observation 3: If the UE is supposed to measure the OD-SSB more often than in the existing requirements for legacy measurements, the power consumption of the UE would increase.

Proposal 3: RAN4 to define requirements that do not increase the frequency of L3 measurements for deactivated SCell.

Observation 4: No new accuracy requirements need to be defined for L3 measurements for a deactivated SCell to be known in case #2, scenario #2A, if the same number of SSB occurrences as for always-on SSB is allowed for on-demand SSB.

Proposal 4: No new measurement accuracy requirements should be defined for on-demand SSB.

Proposal 5: RAN4 to define a time window for OD-SSB measurement of deactivated SCell that comprises five SSBs.

Proposal 6: The UE is only supposed to measure in this measurement time window.

Proposal 7: To avoid that the frequency of OD-SSB based measurements increases, the measurement time window should have a periodicity of 5*max(measCycleSCell, DRX cycle).

Proposal 8: All other always-on SSB or OD-SSB in between two measurement time windows can be ignored by the UE for L3 measurements in deactivated SCell.

Observation 5: SSB processing and implementation in the UE gets simplified if both always-on SSB and OD-SSB fit into the same periodic pattern.

Proposal 9: If the measurement time window includes both OD-SSB and always-on SSB, requirements should only be defined for the case when both types of SSB fit into the same periodic structure.

Proposal 10: The condition for SCell to be known is based L3 measurements as in existing SCell activation scenarios.

Proposal 11: L1 measurements are not supported for deactivated SCell.

Proposal 12: The measurement report to be sent for the known SCell condition should be based on OD-SSB in case OD-SSB are configured.

Observation 6: Different periods for a cell to be known are defined in RAN4 for different types of SCell activation.

Proposal 13: RAN4 to investigate whether the period for the known SCell condition can be reused from legacy or whether new period durations should be introduced.

Observation 7: Since A-TRS and aperiodic CSI-RS on the one side and SSB on the other side have usually different purposes, it cannot easily be assumed that the same timeline can be applied for these signals.

Proposal 14: An additional margin  in slots should be added to T_min=m+3Nslotsubframe,μ+1, i.e., T_min=m+3Nslotsubframe,μ+∆. ∆ >1 is FFS in RAN4.

Proposal 15: RAN4 should agree that the RAN1 working assumption of Tmin=m+3Nslotsunframe,μ+1 cannot be confirmed.

Proposal 16: Discussions related to SMTC and the relation between OD-SSB and always-on SSB in time, frequency, spatial relation should be delayed in RAN4 till further clarification from RAN2 and RAN1 is received.

Proposal 17: RAN4 to focus first on the case that always-on SSB and OD-SSB are in the same frequency, have the same offset but different frequencies.

Observation 8: It is observed that the current discussions only consider measurements for a single deactivated NES SCell.

Proposal 18: Whether and/or how to extend this is multiple NES SCell should be subject to a separate discussion / decision.

Observation 9: The existing specification in 38.133 allows interruptions during measurements on deactivated SCell when the measCycleSCell is larger than or equal to 640 ms.

Proposal 19: RAN4 should investigate whether there is impact due to OD-SSB on the allowed interruptions during measurements on deactivated SCell.

# References

- R4-2412508, Work plan for R19 NES, RAN4 #112, August 2024

- R4-2415562, Topic summary for [112bis][215] Netw_Energy_NR_enh_Part1, RAN4 #112bis, October 2024

- R4-2416916, WF on RRM requirements for Netw_Energy_NR_enh_Part1, RAN4 #112bis, October 2024

- R4-2414911, LS on timeline for on-demand SSB operation in SCell, RAN4 #112bis, October 2024

- R4-2416911, LS on SSB adaptation, RAN4 #112bis, October 2024

- R4-2416913, LS on SSB relation in on-demand SSB and SSB adaptation on SCell, RAN4 #112bis, October 2024