3GPP TSG-RAN WG4 #113	R4-2417515

Orlando, US, November 18-22, 2024

Agenda item:	7.19.3.2		Other RRM Requirements

Source: 	Qualcomm Incorporated

Title: 						Additional RRM requirements for NR MIMO Phase 5

Document for:	Discussion

# Introduction

In RAN4 #112 the work on Rel-19 was started. Several agreements were made to identify RRM requirements and additional open issues have been identified [2]. The following agreements were made:

In this paper we provide our views for the open issues identified in the way forward as defined in [2] in RAN4 #112bis.

# Discussion

## CSI Enhancement to support up to 128 Ports

In RAN #105 in September modifications were made to the MIMO Phase 5 work item [3] and SRS port group has been added as new topic. Issue 1-1 addresses the question whether SSRS port grouping has RRM impact.

Issue 1-1: Whether RRM impacts exist by the added SRS port grouping for CSI enhancement?

In last RAN plenary meeting RAN#105, another sub-bullet was approved for CSI enhancement (Objective#2):

- SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

Way forward:

- Option 1: Previous RAN4 agreement still apply “No RRM impact by CSI enhancement to support up to 128 ports”.

- Option 2: Wait for RAN1.

In RAN1 #118bis a fixed mapping has been agreed that SRS group 0 comprises the even SRS ports and SRS group 1 comprises the uneven SRS ports. So far, no impact on RRM core requirements is seen by SRS port grouping but this may depend on further RAN1 progress

Observation 1: So far, no impact by SRS port grouping for RRM core requirements is seen.

Proposal 1: RAN4 to wait for further RAN1 progress whether RRM core requirements are impacted by SRS port grouping.

## UE Reporting Enhancements for CJT Deployments

Issue 2-1 addresses the accuracy requirements of reporting of phase, time delay and frequency. The goal was to identify whether one-shot measurements are sufficient or whether measurements based on multiple observations and averaging of measurements are required. Since multiple observations also involve the definition of a measurement reporting delay, RRM core requirements would be impacted. In case of one-shot measurement, only RRM performance requirements are impacted. Unfortunately, the time between RAN4 #112bis and RAN4 #113 was too short to provide meaningful simulation results.

Observation 2: The time between RAN4 #112bis and RAN4 #113 was too short to provide meaningful simulation results. Discussions on core requirements may need to be delayed till RAN4 #114.

Issue 2-1: Whether RRM core requirements impacts exist by UE reporting enhancement for CJT calibration?

Agreement:

- RAN4 to simulate the performance of offsets for phase, time delay and frequency, based on one-shot measurement and multiple-sample measurement, and discuss the impact on RRM core requirement later.

- Further discuss the simulation assumptions offline during this meeting.

Way forward:

Companies are encouraged to bring simulation assumption/results in the next meeting. Simulation assumption in Appendix A can be used as a starting point for information only.

Since also Issue 2-2 and Issue 2-3 are impacted by the availability of simulations results of Issue 2-1, RAN4 work on UE reporting enhancement for CJT calibration should be delayed till enough companies provides simulation results.

Issue 2-2: Whether RRM performance requirements impacts exist by UE reporting enhancement for CJT calibration?

Way forward:

- Option 1: Define accuracy requirements.

- Option 2: FFS.

Issue 2-3: Whether report mapping tables should be specified by UE reporting enhancement for CJT calibration?

Way forward:

- Option 1: Define mapping tables for frequency/delay/phase offset reporting in performance part.

Proposal 2: RAN4 should delay work on UE reporting enhancement for CJT calibration till enough companies have provided simulation results.

## 3-Antenna-port Codebook-based UL Transmission

For 3Tx support, it has been concluded that no RRM core requirements are impacted. We also do not see that RRM performance requirements are impacted by 3Tx support.

Issue 3-1: RRM core/performance impacts by introducing 3TX support?

Agreement:

- No new RRM core requirements by introducing 3TX support.

Hence, we propose also to agree in RAN4 that no RRM performance requirements are impacted.

Proposal 3: No RRM core or performance requirements need to be defined for non-coherent uplink codebook.

## Asymmetric DL sTRP / UL mTRP

In RAN #105 in September modifications were made to the MIMO Phase 5 work item [3] and support of 2 TA has been added as new feature for DL sTRP / UL mTRP.

Issue 4-1: Whether RRM impacts exist by 2TA enhancement?

Agreement:

- RAN4 to update the RRM requirements for 2TA enhancement [for asymmetric DL sTRP/ UL mTRP]

- As baseline, the Rel-18 framework can be considered as starting point, and the update is needed to accommodate the new scenario in this WID.

- Further discuss the details in the next meeting.

It has been agreed in RAN1 #118bis for the DL sTRP / UL mTRP scenario that only one downlink reference timing is supported and applied to both TAGs. This is different to Rel-18 support of two TA for multi-DCI where two downlink reference timings are associated with the two TAGs

Observation 3: In DL sTRP / UL mTRP scenario, both TAGs have the same downlink reference time.

The downlink reference time is provided by the DL sTRP and the TA is updated for each TAG separately. A definition similar the one for single TA can be reused.

Proposal 4: For DL sTRP/UL mTRP operation with two TA, one for each TAG, the uplink transmission(s) timing take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of the corresponding downlink frame of the reference signal associated with UL/joint TCI state.

Issue 4-2: RRM core impacts of Active uplink TCI state switching delay for unified TCI?

Way forward:

- Option 1: No RRM impacts on UL TCI state switching delay requirement for unified TCI states.

- Option 2: RAN4 to specify UL TCI state switching requirements.

- Option 3: FFS.

In RAN1 #117, the following conclusion was drawn that Rel-17 unified TCI/ICBM and Rel-18 unified TCI framework is reused. Since the UL mTRP do not send any DL signals, the TCI states of the UL mTRP do not have any source RS that is identical to the PL-RS or do not have any source RS that is QCLed Type D to the PL-RS. However, this is the prerequisite for the uplink TCI state switching delay requirements to apply. Hence, we do not think that the existing requirements for active uplink TCI state switching in sections 8.16 and 8.23 of TS 38.133 can be directly applied to DL sTRP / UL mTRP without any modifications.

Observation 4: The existing requirements for active uplink state switching in TS 38.133 cannot be directly applied to DL sTRP / UL mTRP without any modifications if the uplink TCI switches involve the UL mTRPs.

The uplink TCI state switching requirements in sections 8.16 and 8.23 of TS 38.133 are only applicable if the source RS of the UL TCI state is the PL-RS or is QCLed Type D to the PL-RS. RAN4 should discuss whether it is sufficient to enhance these applicability rules to the case when the SRS is the source RS.

Proposal 5: RAN4 should discuss whether it is sufficient to enhance the existing applicability rules to the case when the SRS is the source RS for the UL TCI state.

The UL TCI state switch delay requirements are defined based on the PL-RS maintenance. RAN4 should investigate whether the existing conditions for PL-RS maintenance and the definitions of e.g., Tfirst_target-PL-RS and Ttarget_PL-RS can be reused.

Proposal 6: RAN4 should investigate whether for the existing conditions for PL-RS maintenance the definitions of e.g., Tfirst_target-PL-RS and Ttarget_PL-RS can be reused for UL TCI state switch.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #112bis. Our observations and proposals are summarized as follows:

CSI enhancement to support up to 128 ports

Observation 1: So far, no impact by SRS port grouping for RRM core requirements is seen.

Proposal 1: RAN4 to wait for further RAN1 progress whether RRM core requirements are impacted by SRS port grouping.

UE reporting enhancement for CJT deployments

Observation 2: The time between RAN4 #112bis and RAN4 #113 was too short to provide meaningful simulation results. Discussions on core requirements may need to be delayed till RAN4 #114.

Proposal 2: RAN4 should delay work on UE reporting enhancement for CJT calibration till enough companies have provided simulation results.

3-Antenna-port Codebook-based UL Transmission

Proposal 3: No RRM core or performance requirements need to be defined for non-coherent uplink codebook.

Asymmetric DL sTRP/UL mTRP

Observation 3: In DL sTRP / UL mTRP scenario, both TAGs have the same downlink reference time.

Proposal 4: For DL sTRP/UL mTRP operation with two TA, one for each TAG, the uplink transmission(s) timing take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of the corresponding downlink frame of the reference signal associated with UL/joint TCI state.

Observation 4: The existing requirements for active uplink state switching in TS 38.133 cannot be directly applied to DL sTRP / UL mTRP without any modifications if the uplink TCI switches involve the UL mTRPs.

Proposal 5: RAN4 should discuss whether it is sufficient to enhance the existing applicability rules to the case when the SRS is the source RS for the UL TCI state.

Proposal 6: RAN4 should investigate whether for the existing conditions for PL-RS maintenance the definitions of e.g., Tfirst_target-PL-RS and Ttarget_PL-RS can be reused for UL TCI state switch.

# References

- R4-2415660, Topic summary for [112bis][213] NR_MIMO_Ph5_Part2, RAN4 #112bis, October 2024

- R4-2416921, WF on RRM requirements for NR_MIMO_Ph5_Part2, RAN4 #112bis, October 2024

- RP-242394, WID revision: NR MIMO Phase 5, RAN #105, September 2024