3GPP TSG-RAN WG4 #113	R4-2417516

Orlando, US, November 18-22, 2024

Agenda item:	7.19.3.1		Enhancement for UE-initiated/Event-driven Beam Management

Source: 	Qualcomm Incorporated

Title: 	RRM requirements for enhanced UE-initiated/event-driven beam management

Document for:	Discussion

# Introduction

In RAN4 #112 the work on Rel-19 MIMO Phase 5 was started.  Several agreements were made for UE-initiated/Event-driven beam management and additional open issues have been identified in [2]. The following agreements were made:

Issue 1-3: Applicability of Event triggered measurement reporting delay requirements?

< Agreement>

- Applicable for all events agreed in RAN1/2

- Applicable to both Mode A and B

- Note: applicable to the event/mode supported by UE

Issue 1-1: How to define Event triggered measurement reporting delay?

< Agreement>

- Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air.

- Ending point is the point when the UE transmits a [first PUCCH] triggered by the event.

- “[first PUCCH]” can be updated based on further RAN1/2 agreement.

Issue 1-7: Whether to support CSI-RS based L1-RSRP for aperiodic?

<Agreement >

- RAN4 to define CSI-RS based UEIBM L1-RSRP requirements for periodic CSI-RS. CSI-RS based UEIBM L1-RSRP requirements for semi-persistent CSI-RS are FFS pending on RAN1 further agreement to support semi-persistent CSI-RS.

In this paper we provide our views for the open issues identified in the way forward as defined in [2] in RAN4 #112bis.

# Discussion

In the following we discuss several of the open issues listed in [2]. One of the main open issues is the exact definition of the measurement reporting delay. We use Figure 2-1 to illustrate the individual contributions to the measurement reporting delay. In RAN4 #112bis the agreement has been reached that the measurement reporting delay is defined from a start point to an end point with the following definitions of the start and end points: The start point is the time when the event is met over the air and the end point is the time when the UE transmits a [first PUCCH] that is triggered by the event. It can be seen from Figure 2-1 that there are several contributions to the overall measurement reporting delay, i.e., Tgap, Tmeas and TPUCCH. In the following the individual contributions are addressed separately.

Figure 2-1: Illustration of Delay Contributions till [first PUCCH] is sent

The first open issue listed in the way forward is Issue 1-2: How to define measurement period in event triggered measurement reporting delay. In Figure 2-1 this contribution to the overall delay is described by Tmeas.

Issue 1-2: How to define measurement period in Event triggered measurement reporting delay?

<Way forward >

- Option 1:

- If multiple beams are measured with different measurement period, RAN4 to discuss how to consider the total measurement time.

- Option 2:

- It is proposed to take measurement period defend for Rel-18 LTM as baseline

- Option 3:

- RAN4 should investigate whether those measurement periods can be reused for defining measurement reporting delay requirements for UE-initiated/event-driven beam management

- Option 4:

- Use legacy measurement period for TL1-RSRP_Measurement_Period_SSB for SSB based intra-cell , inter-cell, TL1-RSRP_Measurement_Period_CSI-RS for CSI RS based.

- Option 4a:

    - Remove Treport or set to 0 in legacy requirements.

- Option 4b:

    - Remove considerations for HST enhancements/ scenarios

    - From legacy measurement period, for new beam report further discuss if M is needed or can be removed/assumed 1.

    - From legacy measurement period, for new beam report further discuss if P can be the same or modified for simplification.

- Option 4c:

    - For FR2, the T_L1_measurement_period is scaled by N.

- Other options are not precluded.

Tmeas is the required time by the UE to identify the event that triggers a beam management report and is analyzed further in the next step. So far, the agreement in RAN1 is that L1-RSRP is supported as a measurement quantity. Defining L1-SINR as a second measurement is under discussion in RAN1 but has not yet been agreed. Requirements for L1-RSRP reporting based on SSB and CSI-RS measurement are already defined in TS 38.133, section 9.5 fulfilling the absolute and relative measurement accuracy requirements defined in TS 38.133, section 10.1. The WID targets FR2 [4], hence FR1 does not need to be considered in the following analysis.

Observation 1: The WID targets FR2, hence FR1 does not need to be considered for defining the measurement report delay.

The measurement periods TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS are defined for FR2 as well. If these measurement periods are reused, no additional accuracy requirements are required for L1-RSRP.

Proposal 1: RAN4 to reuse the measurement periods as already defined for L1-RSRP measurements for FR2, i.e., TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS

This is in line with option 4 and we believe also with option 3 under Issue 1-2. Option 4b in Issue 1-2 proposes to remove HST enhancements. We do not see why this should be done, instead we expect that FR2 HST, once deployed, could benefit from UE-initiated / event-driven beam management. Also, the impact to the specification should be low once the measurement report delay is defined for the baseline scenario.

Observation 2: RAN4 to analyze further whether UE-initiated / event-driven beam management can also be applied for FR2 HST in case the highSpeedMeasFlagFR2-r17 is set.

As mentioned above, since existing L1-RSRP measurement periods are reused for UE-initiated / event-driven beam management, no additional measurement accuracy requirements need to be defined.

Proposal 2: No additional accuracy requirements are needed for L1-RSRP measurements (Option 1 in Issue 1-12).

Issue 1-12: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management?

<Way forward >

- Option 1:

- The existing accuracy requirements for L1 measurements can be applied. No new accuracy requirements.

- Option 2:

- For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline.

- For CSI-RS based beam reprt, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements.

- Option 3:

- Discuss the accuracy for the two trigger-event solutions respectively.

- For the basic trigger-event detection, tight the existing L1-RSRP accuracy requirement;

- For the optional trigger-event detection, reusethe existing L1-RSRP accuracy requirement.

- Other options are not precluded.

The next contribution to address is TPUCCH. This delay contribution is addressed by Issue 1-5, but we do not think that all options under Issue 1-5 are still applicable anymore after the agreement in RAN4 #112 that the end point for the reporting delay is the transmission of the [first PUCCH]. For example, option 3 does not apply anymore since it still takes delays after [first PUCCH] into account.

Observation 3: Not all options in Issue 1-5 are still applicable after it has been agreed in RAN4 #112 that the end point for the reporting delay is the transmission of the [first PUCCH].

Issue 1-5: How to consider UL resource in Event triggered measurement reporting?

<Way forward >

- Option 1:

- RAN4 needs to consider the impact of UL resource waiting time for L1 event triggered reporting.

- Option 2:

- The beam reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

- No need to differentiate mode A and mode B

- Option 3:

- Report delay is defined as Tfirst-PUCCH or TPUCCH + TDCI + T1. Where TDCI = 0 for Mode B, Time to receive DCI with UL grant in Mode A; T1 = K2 for Mode A; Tfirst-PUSCH or TPUSCH for Mode-B

- Option 4:

- To determine how to count for the uncertainty in acquiring UL resource for UEIBR, RAN4 waits further conclusion from RAN1 on whether the first PUCCH is SR or not (for Mode-A).

- Other options are not precluded.

Once the triggering event has been identified by the UE, the [first PUCCH] can be sent by the UE when the UL resources are available. This is illustrated in Figure 2-1. After the event has been detected by the UE, there is waiting time, indicated as TPUCCH in the figure, till the UE can sent the [first PUCCH].

The situation is like the case of event-triggered periodic reporting of L3 measurements as described in sections 9.2.4.2 and 9.2.4.3 of TS 38.133. Here the measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. […] This measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on.

Observation 4: For event-triggered periodic reporting of L3 measurements delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH and the case of no UL resources are excluded from the reporting delay.

The same principle should be reused for UE-initiated/event-triggered beam reporting. Requirements should be defined in such a way that the case of no uplink resources is excluded and the waiting time till UL resources are available is also excluded.

Proposal 3: Delay uncertainty till the [first PUCCH] can be sent after the event has been identified by the UE and the case of no UL resources should not be included in the measurement reporting delay for UE-initiated/event-triggered beam reporting.

Figure 2-1 shows an additional contribution to the overall measurement reporting delay, i.e., Tgap. Tgap is the time between the occurrence of the event over the air and the availability of the first periodic CSI-RS resource for measuring. Tgap represents a delay uncertainty and is bounded by the periodicity of the CSI-RS resources. It is a similar uncertainty as TEvent_DU in the conditional handover. In the definition of the test case itself when RRM performance requirements are defined, the test can be defined in such a way that the delay uncertainty Tgap is not counted, see for example the test case definition for conditional handover in section A.6.3.3 in TS 38.133.

Observation 5: Tgap is the time between the occurrence of the event over the air and the availability of the first periodic CSI-RS resource for measuring.

Observation 6: In the definition of the test case itself when RRM performance requirements are defined, the test can be defined in such a way that the delay uncertainty Tgap is not counted.

Issue 1-8 is related to the question whether CSI-RS based L1-RSRP should be defined for inter-cell beam management.

Issue 1-8: CSI-RS based L1-RSRP for inter-cell BM?

<Way forward >

- Option 1:

- RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case

- Option 2:

- For CSI-RS based inter-cell L1-RSRP measurement, since no specific discussion before, RAN4 needs to identify whether additional clarifications on the following points are necessary:

  - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC;

  - The prerequisite on L3 measurement on NSC.

- Option 3:

- RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario in FR2.

- RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

Other options are not precluded.

In Rel-17, measurement requirements for inter-cell beam management have only been introduced for SSB based L1-RSRP. We propose not to introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case.

Observation 7: In Rel-17, measurement requirements for inter-cell beam management have only been introduced for SSB based L1-RSRP but not for CSI-RS based L1-RSRP.

Proposal 4: RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case.

Depending on whether the target TCI state is known, or unknown, different TCI state switch delays are defined in [3]. The condition of a known TCI state includes that the TCI switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and that the UE has sent a valid L1-RSRP report for the target TCI state. The existing delay requirement already covers the case that the target TCI state is known but not yet in the active TCI state list. This adds additional switch delay. If there is no L1-RSRP report before the TCI state switch command, the TCI state is unknown which adds additional delay in the TCI state switch. The existing conditions for known and unknown TCI state can be reused.

Issue 1-10: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM?

<Way forward >

- Option 1:

- Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM.

- Option 2:

- No impact of UE triggered beam report to known/unknown conditions for TCI state.

- The known/unknown condition would be determined by the same criteria as legacy.

- Other options are not precluded.

Proposal 5: The existing conditions for known and unknown TCI state can be reused for UE-initiated/event-triggered beam management.

Issue 1-9: Whether to specify new unified TCI switching delay requirements?

<Way forward >

- Option 1:

- RAN4 should investigate whether any new required TCI state switch delays need to be defined or existing requirements are sufficient

- Option 2:

- Wait further RAN1/RAN2 progress.

- Option 3:

- No need to define new TCI state switching delay requirements. Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching.This can be revisited if the RS in the RS resource set can be updated by MAC-CE in further RAN1 conclusion.

- Other options are not precluded.

After the gNB receives the beam report and executes a TCI state switch to a target TCI state, this target TCI state can be known or unknown since there is no guarantee how quickly the gNB would trigger the switch after the report has been received. Also, the target TCI state does not need to be in the active TCI state list for PDSCH. In section 8.10.3, 8.10.4 and 8.10.5 of TS 38.133, the TCI state switch delays are defined for MAC CE based, DCI based and RRC based switching are defined, taking all the conditions mentioned above into account. Based on current RAN1 agreements, we believe that these TCI state dela switches can also be reused for UE-initiated / event-triggered beam management.

Proposal 6: The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management.

Issue 1-4: How to consider the time window of triggering event determination in Event triggered measurement reporting?

<Way forward >

- Option 1:

- RAN4 needs to consider the impact of triggering window defined in RAN1.

- Option 2:

- It is proposed to define requirements for both the case without filtering and the case with filtering (i.e. timer and counter M).

- It is proposed that same reporting requirements are applied to the case without filtering and the case with filtering (i.e. timer and counter M)

- Option 3:

- No additional time-window-based filtering or other filtering should be applied for the L1-RSRP measurements

- Option 4:

- Discuss the necessity of filtering based on the two trigger-event detection solutions approved in RAN1.

  - For the basic solution, one-shot measurement is defined by RAN1. No room to consider filtering.

  - For the optional solution, multi-shot measurement with at least M instance is defined by RAN1. Filtering is feasible.

- Option 5:

- RAN4 to discuss how to define one Event-2 instance for following Event-2 case:

  - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

- Option 6:

- Wait further RAN1 progress.

- Other options are not precluded.

Finally, Issue 1-4 addresses whether and how to consider the time window of triggering the event determination in the definition of the requirements. This issue is related to an agreement in RAN1 #117 for event-2 ‘quality of at least one new beam becomes a threshold value better than the current beam. It says that the UE initiated beam report occurs if within a configurable time window, the number of event-2 instances for at least one same new beam is greater than a configurable number M. That means that the report occurs if the event has been observed M times in a certain time window. It has been agreed in RAN1 that this window approach is an optional feature.

Even if a time window is applied, the report is still an L1-RSRP report. Hence, we do not think that the UE should apply any additional filtering. It should be left to gNB whether any additional filtering is applied before the gNB decides whether a TCI state switch should be triggered.

Proposal 7: No additional time window based filtering or other filtering should be applied by the UE for the L1-RSRP measurements, independent whether a time window is configured or not.

If no filtering is applied, it is not obvious whether additional reporting delay requirements for the configurable time window needs to be defined. In case that no filtering is applied in the UE, the measurement period would simply extend by a factor of M based on its configured value. Hence, we do not see that defining additional requirements adds much new insight in the UE behavior.

Observation 8: In case that no filtering is applied in the UE, the measurement period would simply extend by a factor of M based on its configured value.

Proposal 8: RAN4 should not define additional requirements for the window-based observation whether the event has occurred M times.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #112bis. Our observations and proposals are summarized as follows:

Observation 1: The WID targets FR2, hence FR1 does not need to be considered for defining the measurement report delay.

Proposal 1: RAN4 to reuse the measurement periods as already defined for L1-RSRP measurements for FR2, i.e., TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS

Observation 2: RAN4 to analyze further whether UE-initiated / event-driven beam management can also be applied for FR2 HST in case the highSpeedMeasFlagFR2-r17 is set.

Proposal 2: No additional accuracy requirements are needed for L1-RSRP measurements (Option 1 in Issue 1-12).

Observation 3: Not all options in Issue 1-5 are still applicable after it has been agreed in RAN4 #112 that the end point for the reporting delay is the transmission of the [first PUCCH].

Observation 4: For event-triggered periodic reporting of L3 measurements delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH and the case of no UL resources are excluded from the reporting delay.

Proposal 3: Delay uncertainty till the [first PUCCH] can be sent after the event has been identified by the UE and the case of no UL resources should not be included in the measurement reporting delay for UE-initiated/event-triggered beam reporting.

Observation 5: Tgap is the time between the occurrence of the event over the air and the availability of the first periodic CSI-RS resource for measuring.

Observation 6: In the definition of the test case itself when RRM performance requirements are defined, the test can be defined in such a way that the delay uncertainty Tgap is not counted.

Observation 7: In Rel-17, measurement requirements for inter-cell beam management have only been introduced for SSB based L1-RSRP but not for CSI-RS based L1-RSRP.

Proposal 4: RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case.

Proposal 5: The existing conditions for known and unknown TCI state can be reused for UE-initiated/event-triggered beam management.

Proposal 6: The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management.

Proposal 7: No additional time window based filtering or other filtering should be applied by the UE for the L1-RSRP measurements, independent whether a time window is configured or not.

Observation 8: In case that no filtering is applied in the UE, the measurement period would simply extend by a factor of M based on its configured value.

Proposal 8: RAN4 should not define additional requirements for the window-based observation whether the event has occurred M times.

# References

- R4-2415659, Topic summary for [112bis][212] NR_MIMO_Ph5_Part1, RAN4 #112bis, October 2024

- R4-2416856, WF on RRM requirements for NR_MIMO_Ph5_Part1, RAN4 #112bis, October 2024

- R4-2416873, Ad-hoc minutes for NR_MIMO_Ph5, RAN4 #112bis, October 2024

- RP-242394, WID revision: NR MIMO Phase 5, RAN #105, September 2024