# 3GPP TSG-RAN WG4 Meeting # 113	R4-2417517

Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 	More on Delta Ppowerclass and Delta TRxSRS for 4Tx

Agenda Item:	5.7.1

Document for:	Approval

# Introduction

This contribution is an update of [1] to discuss issues raised in [2] for ΔPPowerClass and ΔTRxSRS for 4Tx.

# Discussion

## Overview

To come right to the point, our final proposal is applying the same requirements for PC1.5 UE with txDiversity4Tx-r18 to also the PC1.5 without txDiversity4Tx-r18, i.e., it is very similar to what CR [3] addressed.

Our last contribution of [1] concluded that in order to move forward, RAN4 should follow a way that 2Tx has taken. Nonetheless, we couldn’t reach a consensus. Firstly, in a case that a UE indicates PC1.5 without txDiversity4Tx-r18, CR from Huawei [4] and CR [3] from Ericsson are very different. The former follows PC2 with 2Tx, while the latter doesn’t address it at all. In the case of UE indicates PC1.5 with txDiversity4Tx-r18, those two CRs are almost the same.

Observation 1: In a case that a UE indicates PC1.5 without txDiversity4Tx-r18, CR from Huawei [R4-2411236] and CR [R4-2413358] from Ericsson are very different. The former follows PC2 with 2Tx, while the latter doesn’t address it at all. In the other case that UE indicates PC1.5 with txDiversity4Tx-r18, those two CRs are almost the same.

In the subsequent clauses, we address those two cases separately.

## PC1.5 4Tx without txDiversity4Tx-r18

A point for this case is how to handle switching cases that a UE doesn’t use PA(s) with the highest power ability for SRS AS. Originally, we suggested that we apply the same approach for PC2 2Tx without txDiversity-r16 to that for PC1.5 4Tx without txDiversity4Tx-r18 for the sake of consistency in order to move forward, even though we don’t think that mixing ΔPPowerClass and ΔTRxSRS is the right way. Now, one camp wants to keep the existing PC2 requirements as they are, while the other camp doesn’t like to introduce the same approach for 2Tx into 4Tx requirements.

Observation 2: The deadlock comes from the situation that a first camp wants to keep the existing PC2 requirements as they are, a second camp doesn’t like to introduce the same approach for 2Tx into 4Tx requirements and a third camp wants to keep consistency across 2Tx and 4Tx.

Technically, the requirement for PC2 2Tx without txDiversity-r16 would not be useful, given that UE shall transmit the same power across ports as much as possible during SRS AS. Otherwise, this just invites more channel estimation errors.

Observation 3: Technically, the requirement for PC2 2Tx without txDiversity-r16 would not be useful, given that UE shall transmit the same power across ports as much as possible during SRS AS. Otherwise, this just invites more channel estimation errors while unfortunately it has been in the specification for several years.

Therefore, the remaining approach is we allow inconsistency between PC2 without txDiversity-r16 and PC1.5 without txDiversity4Tx-r18. More specifically, we keep the current PC2 UE without txDiversity-r16, while we don’t introduce the similar mechanism into the requirements for PC1.5 with txDiversity4Tx-r18, i.e., the same ΔTRxSRS applies regardless of txDiversity4Tx-r18 indication. This is actually very similar to what the CR [3] addressed.

Observation 4: the remaining approach is we allow inconsistency between PC2 without txDiversity-r16 and PC1.5 without txDiversity4Tx-r18. More specifically, we keep the current PC2 UE without txDiversity-r16, while we don’t introduce the similar mechanism into the requirements for PC1.5 with txDiversity4Tx-r18.

Proposal 1: Allow inconsistency between PC2 without txDiversity-r16 and PC1.5 without txDiversity4Tx-r18, i.e., keep the current PC2 UE without txDiversity-r16, while don’t introduce the similar mechanism into the requirements for PC1.5 without txDiversity4Tx-r18.

## PC1.5 with 4Tx with txDiversity4Tx-r18

As mentioned earlier, the content for this part in the CR [3] and [4] is very similar. The below is an excerpt of a part of the CR [3], where parts coloured in yellow, green and grey are the main differences compared to the CR [4].

For the yellow part, it’s expected that the author of [3] used the yellow text to have a common term applicable to UE regardless of supporting txDiversity4Tx-r18.

Firstly, we think that it is better not to create a new way of expression like “equipped with X (2 or 4) Tx connectors”, but rather to use what we have in the specification. Hence, we suggest notto use equipped with 4 Tx connectors, but rather use “with four transmit antenna connectors”, and do not touch PC2 with TxD capability. We also understand the motivation to change the 3 dB for PC2 TxD case. But if we changed this part, we would need to remove a case of mixing ΔPPowerClass and ΔTRxSRS mixing part, i.e., PC2 UE without TxD capability.

For green part, if we go with this, we also need to add t1r2, t1r4 before as ‘t1r8’ for 6 dB case as well.

For grey part, we don’t think this is needed. Because it doesn’t matter, given that two SRS ports are used for each SRS resource set in this context.

Proposal 2: Introduce the following texts only for PC1.5 4Tx ΔPPowerClass, i.e., do not touch PC2 TxD ΔPPowerClass part.

Proposal 3: Do not introduce new changes into the existing ΔTRxSRS part.

# Summary

We obtained following observations and proposals. A companion CR based on this contribution is also submitted [5].

Observation 1: In a case that a UE indicates PC1.5 without txDiversity4Tx-r18, CR from Huawei [R4-2411236] and CR [R4-2413358] from Ericsson are very different. The former follows PC2 with 2Tx, while the latter doesn’t address it at all. In the other case that UE indicates PC1.5 with txDiversity4Tx-r18, those two CRs are almost the same.

Observation 2: The deadlock comes from the situation that a first camp wants to keep the existing PC2 requirements as they are, a second camp doesn’t like to introduce the same approach for 2Tx into 4Tx requirements and a third camp wants to keep consistency across 2Tx and 4Tx.

Observation 3: Technically, the requirement for PC2 2Tx without txDiversity-r16 would not be useful, given that UE shall transmit the same power across ports as much as possible during SRS AS. Otherwise, this just invites more channel estimation errors while unfortunately it has been in the specification for several years.

Observation 4: the remaining approach is we allow inconsistency between PC2 without txDiversity-r16 and PC1.5 without txDiversity4Tx-r18. More specifically, we keep the current PC2 UE without txDiversity-r16, while we don’t introduce the similar mechanism into the requirements for PC1.5 with txDiversity4Tx-r18.

Proposal 1: Allow inconsistency between PC2 without txDiversity-r16 and PC1.5 without txDiversity4Tx-r18, i.e., keep the current PC2 UE without txDiversity-r16, while don’t introduce the similar mechanism into the requirements for PC1.5 without txDiversity4Tx-r18.

Proposal 2: Introduce the following texts only for PC1.5 4Tx ΔPPowerClass, i.e., do not touch PC2 TxD ΔPPowerClass part.

Proposal 3: Do not introduce new changes into the existing ΔTRxSRS part.

# References

[1] R4-2411235, Delta Ppowerclass and Delta TRxSRS for 4Tx, Huawei, HiSilicon, RAN4#112

[2] R4-2400341, Delta PpowerClsss for 4Tx for SRS antenna switching, Nokia, RAN4#110

[3] R4-2413358, (NR_ENDC_RF_FR1_enh2-Core) CR to 38.101 Rel-18: DeltaP_PowerClass correction for SRS AS for 4Tx, Ericsson, RAN4#112

[4] R4-2411236, Introduction of Delta TRxSRS for 4Tx, Huawei, HiSilicon, RAN4#112

[5] R4-2417518, DeltaP_PowerClass correction for SRS AS for 4Tx, Huawei, HiSilicon, RAN4#113