3GPP TSG-RAN WG4 Meeting #113	R4-2417560

Orlando, US, 18th – 22nd November 2024

Agenda item:	7.16.2

Source:	MediaTek Inc.

Title:	Discussion on UE requirements with 8Rx MMSE-IRC receiver

Document for:	Discussion

# Introduction

A WF for UE 8Rx MMSE-IRC receiver requirements was agreed in RAN4#112-bis meeting [1]. In this contribution, we provide initial simulation analysis and our views on outstanding issues.

- Discussion

## 2.1 PDSCH requirements with inter cell interference

In the last meeting, RAN4 agreed to use the settings/parameters in [1] for initial simulation analysis first. Based on the simulation results, RAN4 can further determine the settings/parameters used in the final requirements. We provide our initial simulation results for 8Rx MMSE-IRC receiver with inter cell interference in Table 1.

Table 1. Simulation results for 8Rx MMSE-IRC receiver with inter cell interference

|  | MIMO | Rank (target UE) | MCS (target UE) | INR1 | INR2 | Channel Model | MMSE-IRC | MMSE | Gain over MMSE | Corresponding SINR (IRC) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline | 2T8R | 1 | 17 | 7.77 | 2.29 | TDLC300-100 | 6.4 | 10.4 | 4.0 | -3.0 |
|  | 2T8R | 2 | 17 |  |  | TDLA30-10 | 8.2 | 12.9 | 4.7 | -1.2 |
|  | 4T8R | 4 | 13 |  |  |  | 8.8 | 14.4 | 5.6 | -0.6 |
|  | 2T8R | 1 | 17 | 7.58 | N/A | TDLA30-10 | 3.8 | 7.9 | 4.1 | -4.5 |
|  | 2T8R | 2 | 17 |  |  |  | 7.7 | 11.9 | 4.2 | -0.6 |
|  | 4T8R | 4 | 13 |  |  |  | 8.3 | 13.4 | 5.1 | 0.0 |
| Simplified | 2T8R | 1 | 17 | 7.77 | 2.29 | TDLC300-100 | 8.2 | 11.2 | 3.0 | -1.2 |
|  | 2T8R | 2 | 17 |  |  | TDLA30-10 | 10.8 | 14.2 | 3.4 | 1.4 |
|  | 4T8R | 4 | 13 |  |  |  | 15.0 | 17.1 | 2.1 | 5.6 |
|  | 2T8R | 1 | 17 | 7.58 | N/A | TDLA30-10 | 5.5 | 9.1 | 3.6 | -2.8 |
|  | 2T8R | 2 | 17 |  |  |  | 10.3 | 13.4 | 3.1 | -5.2 |
|  | 4T8R | 4 | 13 |  |  |  | 14.4 | 16.1 | 1.7 | 6.1 |

The rationale for evaluating the performance MMSE receiver is to find enough SNR gap between MMSE-IRC and MMSE. By doing this, we can differentiate two different UE implementation, i.e., MMSE-IRC and MMSE. If the SNR gap between MMSE-IRC and MMSE receiver is large enough under some settings (e.g., rank, MCS), we can use that settings to test UE and UE with MMSE implementation cannot pass the requirements derived from MMSE-IRC implementation. From simulation results, the SNR gap between MMSE-IRC and MMSE are all larger than 1.7dB. Therefore, we think all settings on the table now can be used to differentiate the UE implementation of MMSE-IRC and MMSE.

Observation 1: The rationale for evaluating the performance MMSE receiver is to find enough performance gap between MMSE-IRC and MMSE with some suitable settings (e.g., rank, MCS).

Observation 2: From our simulation results, the SNR gap between MMSE-IRC and MMSE are all larger than 1.7dB. For PDSCH requirements with inter cell interference, all different settings (e.g., different rank, MCS, propagation conditional combinations) in the previous meeting’s WF can be used to differentiate the UE implementation of MMSE-IRC and MMSE.

Besides SNR gap between MMSE-IRC and MMSE receiver, the SINR values are also considered in the discussions of Rel-17 2Rx/4Rx MMSE-IRC receiver WI. We provide “corresponding SINR for SNR” in the last column of Table 1 based on Figure 1 and 2.

| Figure 1. SINR versus SNR for two interfering cells | Figure 2. SINR versus SNR for one interfering cell |
| --- | --- |

From the last column of Table 1, the SINRs for Rank 1 are all above -4.5 dB, which is larger than -6dB. Therefore, for both baseline and simplified receiver, we propose to use the following settings for PDSCH requirements with inter cell interference.

- Two interference cell: TDLC300-100, 2T8R Rank 1, and MCS 17

- One interference cell: TDLA30-10, 2T8R Rank 1, and MCS 17

Observation 3: The SINRs for Rank 1 are all above -4.5 dB, which is larger than -6dB.

Proposal 1: Use the following settings to define PDSCH requirements with inter cell interference

  - Two interference cells: TDLC300-100, 2T8R Rank 1, and MCS 17

  - One interference cell: TDLA30-10, 2T8R Rank 1, and MCS 17

## 2.2 Wideband CQI reporting requirements with inter-cell interference

Based on [1], there are lots of outstanding issues for 8Rx MMSE-IRC CQI requirements. For the test metric, we support to reuse the same test metric from Rel-17 2/4Rx and determine the values of throughput ratio and BLER.

a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ g . b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

Proposal 4: For 8Rx MMSE-IRC receiver CQI testing, reuse the same test metric from Rel-17 2/4Rx and determine the values of throughput ratio and BLER through simulations.

For the configuration of test setup, we prefer to start with Rel-17 2/4Rx setting as

- Propagation condition:

  - TDLA30-5 for the serving cell

  - AWGN for the interference cell

- Rank for the serving cell:

  - Rank 1 for the serving cell

- Antenna configuration and MIMO correlation

  - 2T8R ULA low for the serving cell

  - 1T8R for the interference cell

- CodebookSubsetRestriction

  - 000001

- RI Restriction

  - No RI restriction

- Interference power level

  - INR = 10.04 dBs

For the CSI-IM and NZP CSI-RS configurations, we suggest following the agreed principles in Rel-17 2/4Rx inter-cell interference requirements:

  - CSI-IM on serving cell overlapping with PDSCH from interference cell

  - NZP CSI-RS on serving cell overlapping with NZP CSI-RS from interference cell

However, the current Rel-17 2/4Rx inter-cell interference requirements did not follow above agreements. For FDD requirements in Clauses 6.2.2.1.2.3 and 6.2.3.1.2.3, the CSI-IM on target cell, (kCSI-IM,lCSI-IM) = (4, 9), overlaps with ZP CSI-RS (k0=Row5,(4) and I0 = 9) from interference cell, not the PDSCH from interference cell.

Figure 3. CSI-RS configurations in Rel-17 FDD 2/4Rx inter-cell interference requirements

Observation 4: The current CSI-RS configuration for Rel-17 2/4Rx inter-cell interference requirements did not follow agreements in Rel-17. The CSI-IM on target cell, (kCSI-IM,lCSI-IM) = (4, 9), overlaps with ZP CSI-RS (k0=Row5,(4) and I0 = 9) from interference cell, not the PDSCH from interference cell.

Therefore, we suggest using the following setting for ZP CSI-RS configuration and reuse NZP CSI-RS and CSI-IM resource settings.

- ZP CSI-RS configuration (First subcarrier index in the PRB used for CSI-RS (k0))

  - Row 5,(4) for the serving cell

  - Row 5,(6) for the interference cell

Proposal 5: Based on the above discussions, we propose the following settings for initial simulation.

- Propagation condition:

  - TDLA30-5 for the serving cell

  - AWGN for the interference cell

- Rank for the serving cell:

  - Rank 1 for the serving cell

- Antenna configuration and MIMO correlation

  - 2T8R ULA low for the serving cell

  - 1T8R for the interference cell

- CodebookSubsetRestriction

  - 000001

- RI Restriction

  - No RI restriction

- Interference power level

  - INR = 10.04 dBs

- ZP CSI-RS configuration (First subcarrier index in the PRB used for CSI-RS (k0))

  - Row 5,(4) for the serving cell

  - Row 5,(6) for the interference cell

## 2.3 PDSCH requirements with intra-cell inter-user interference

In the last meeting, RAN4 agreed to use the settings/parameters in [1] for initial simulation analysis first. Based on the simulation results, RAN4 can further determine the settings/parameters used in the final requirements. We provide our initial simulation results for 8Rx MMSE-IRC receiver with intra-cell inter-user interference in Table 2.

Table 2. Simulation results for 8Rx MMSE-IRC receiver with intra-cell inter-user interference

|  | MIMO | Rank for target + Co-UE | MCS (target UE) | Precoder selection | Channel Model | Antenna correlation | Modulation order for the co-scheduled UE | MMSE-IRC | MMSE | Gain over MMSE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline | 2T8R | 1+1 | MCS 13 | Random | TDLA30-10 | ULA Low | 16QAM | 4.5 | 8.7 | 4.2 |
|  |  |  |  | Orthogonal |  |  |  | 2.9 | 3.0 | 0.1 |
|  | 4T8R | 2+2 | MCS 13 | Orthogonal |  |  |  | 7.6 | 11.5 | 3.9 |
| Simplified | 2T8R | 1+1 | MCS 13 | Random | TDLA30-10 | ULA Low | 16QAM | 5.3 | 8.8 | 3.5 |
|  |  |  |  | Orthogonal |  |  |  | 2.9 | 3.0 | 0.1 |
|  | 4T8R | 2+2 | MCS 13 | Orthogonal |  |  |  | 9.7 | 15.9 | 6.2 |

Based on our results, it shows that the SNR gap between MMSE-IRC and MMSE is larger than 3.5 dB for the case of 1+1, random precoder and 2+2 orthogonal precoder.

Observation 5: SNR gap between MMSE-IRC and MMSE is larger than 3.5 dB for the case of 1+1, random precoder and 2+2 orthogonal precoder

Based on observation 1 and 5, we propose to use the following setting to define PDSCH requirements with intra-cell inter-user interference.

- 2T8R, 1+1, MCS13, random precoder

- 4T8R, 1+1, MCS13, orthogonal precoder

Proposal 6: Use the following settings to define PDSCH requirements with intra-cell inter-user interference

- 2T8R, 1+1, MCS13, random precoder

- 4T8R, 1+1, MCS13, orthogonal precoder

## 2.4 Applicability rule and release independent

Test applicability - Candidate options: - Option 1: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule - Option 2: Applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, if same test scenarios/configurations as 2RX/4RX are defined

We support new requirements for 8Rx MMSE-IRC receiver to be mandatory requirements for all 8Rx UEs. However, if RAN4 agreed to define two set of requirements for baseline and simplified, we need to introduce new applicability rules as what we have in Table 5.1.1.3-1 of TS38.101-4. Otherwise, we think there is no necessary to introduce new applicability rules. Also, we are willing to introduce applicability rules to skip 2/4Rx requirements if 8Rx tests are passed.

Table 5.1.1.3-1: Requirements applicability for optional UE features UE feature/capability [14]; Test type; Test list; Applicability notes. Baseline SU-MIMO 8Rx receiver; FR1 FDD; PDSCH; Clause 5.2.4.1.1(Tests 1-1,3-1). FR1 TDD; PDSCH; Clause 5.2.4.2.1(Tests 1-1,3-1). Simplified SU-MIMO 8Rx receiver; FR1 FDD; PDSCH; Clause 5.2.4.1.1(Tests 2-1,4-1). FR1 TDD; PDSCH; Clause 5.2.4.2.1(Tests 2-1,4-1). PBCH; Clause 5.4.2.1 (Test 3-1) Clause 5.4.3.1 (Test 3-1).

Proposal 7: We support new requirements for 8Rx MMSE-IRC receiver to be mandatory requirements for all 8Rx UEs. However, if RAN4 agreed to define two set of requirements for baseline and simplified, we need to introduce new applicability rules as what we have in Table 5.1.1.3-1 of TS38.101-4.

Proposal 8: Support defining applicability rules to skip 2/4Rx requirements if 8Rx tests are passed.

Release independent - Candidate options: - Option 1: The new requirements for 8Rx should be release independent from Rel-18 Other options are not precluded

As there are some many options/settings for requirements are under discussion. Based on the workplan [2], we have 5 meeting including this October meeting. We propose RAN4 start discussing the issue of release independent when RAN4 agree on baseline assumptions, test parameters and test metrics, i.e., after RAN4#113.

Proposal 9: RAN4 start discussing the issue of release independent when RAN4 agree on baseline assumptions, test parameters and test metrics, i.e., after RAN4#113.

# Conclusion

In this contribution, we provide our initial simulation results and views on UE demodulation requirements with 8Rx MMSE-IRC receiver. Our observations and proposals are captured below:

Observation 1: The rationale for evaluating the performance MMSE receiver is to find enough performance gap between MMSE-IRC and MMSE with some suitable settings (e.g., rank, MCS).

Observation 2: From our simulation results, the SNR gap between MMSE-IRC and MMSE are all larger than 1.7dB. For PDSCH requirements with inter cell interference, all different settings (e.g., different rank, MCS, propagation conditional combinations) in the previous meeting’s WF can be used to differentiate the UE implementation of MMSE-IRC and MMSE.

Observation 3: The SINRs for Rank 1 are all above -4.5 dB, which is larger than -6dB.

Proposal 1: Use the following settings to define PDSCH requirements with inter cell interference

  - Two interference cells: TDLC300-100, 2T8R Rank 1, and MCS 17

  - One interference cell: TDLA30-10, 2T8R Rank 1, and MCS 17

Proposal 4: For 8Rx MMSE-IRC receiver CQI testing, reuse the same test metric from Rel-17 2/4Rx and determine the values of throughput ratio and BLER through simulations.

Observation 4: The current CSI-RS configuration for Rel-17 2/4Rx inter-cell interference requirements did not follow agreements in Rel-17. The CSI-IM on target cell, (kCSI-IM,lCSI-IM) = (4, 9), overlaps with ZP CSI-RS (k0=Row5,(4) and I0 = 9) from interference cell, not the PDSCH from interference cell.

Proposal 5: Based on the above discussions, we propose the following settings for initial simulation.

- Propagation condition:

  - TDLA30-5 for the serving cell

  - AWGN for the interference cell

- Rank for the serving cell:

  - Rank 1 for the serving cell

- Antenna configuration and MIMO correlation

  - 2T8R ULA low for the serving cell

  - 1T8R for the interference cell

- CodebookSubsetRestriction

  - 000001

- RI Restriction

  - No RI restriction

- Interference power level

  - INR = 10.04 dBs

- ZP CSI-RS configuration (First subcarrier index in the PRB used for CSI-RS (k0))

  - Row 5,(4) for the serving cell

  - Row 5,(6) for the interference cell

Proposal 6: Use the following settings to define PDSCH requirements with intra-cell inter-user interference

- 2T8R, 1+1, MCS13, random precoder

- 4T8R, 1+1, MCS13, orthogonal precoder

Proposal 7: We support new requirements for 8Rx MMSE-IRC receiver to be mandatory requirements for all 8Rx UEs. However, if RAN4 agreed to define two set of requirements for baseline and simplified, we need to introduce new applicability rules as what we have in Table 5.1.1.3-1 of TS38.101-4.

Proposal 8: Support defining applicability rules to skip 2/4Rx requirements if 8Rx tests are passed.

Proposal 9: RAN4 start discussing the issue of release independent when RAN4 agree on baseline assumptions, test parameters and test metrics, i.e., after RAN4#113.

# Reference

- R4-2416555, WF for [112bis][320] NR_demod_Ph5_Part2_UE, Nokia