3GPP TSG-RAN WG4 Meeting #113												R4-2417591

Orlando, USA, 18 – 22 November, 2024

Agenda item:			7.12.2.2

Source:	Huawei, HiSilicon

Title:	on NTN OTA tests

Document for:

# Introduction

The WF in the last meeting [1] highlights the following issue on the use of circular polarization for TRP and TRS tests. This contribution discusses the issue further and proposes a TRS measurement time reduction method.

Issue 3-3-4: How to resolve UE antenna circular polarization issue

Agreements:

  - RAN4 to further study the need for circular polarization for NR NTN conformance testing

  - Feedback from OEMs and Satellite Vendors on the need for CP testing and test system impacts to support CP OTA conformance testing from TE/System vendors is requested.

# Discussion

All the satellites in operation today use circularly polarized signals to reduce propagation impact. However, current TRP and TRS measurement systems employ linear polarization, so do most, if not all, smartphones. If linear polarization is used to measure TRP and TRS for NTN devices, it would drive device development and optimisation in the wrong direction. Namely device manufacturers would optimise the device performance for linear polarization while circular polarization is used in reality.

An alternative to the use of circular polarization is to continue to use linear polarization with an offset to account for the performance difference or loss between linear and circular polarization. Such an approach is impractical because (a) it is difficult to derive or agree on an offset (b) this would still drive device performance optimisation in a wrong direction.

The use of circular polarization would require some modifications to existing lab configurations as shown in [2], but such modifications are implementable without a huge amount of disruption.

Proposal 1: circular polarization shall be used for TRP and TRS measurement for NTN device performances.

TS 38.521-5 [3] references tables in section 5.6.2.1 of TS 38.508-1 [4] for delay parameters. For example, Table 5.6.2.1-1 for GSO shows one-way delay of 127.33ms while Table 5.6.2.1-2 for LEO-600 and Table 5.6.2.1-3 for LEO-1200 show one-way delays of 2.03ms and 4.04ms, respectively.

Table IV in [2] shows 16% measurement time reduction when RTT (Round Trip Time) is set zero compared to RTT of 240ms emulating GSO scenario while the repeatedly measured TRS values with different RTT in Table III are almost same. Namely RTT can be set to zero or a very low value to reduce measurement time. An MU term could also be considered if the impact of setting RTT to zero or a very low value for GSO cases is deemed significant.

However, measurement time reduction would be very limited or none at all for NGSO such as LE600 and LE1200 scenarios because the inherent delay is very low already.

Proposal 2: set NTN RTT (Round Trip Time) to zero or a very low value to reduce TRS measurement time.

# Conclusion

This contribution has the following proposals.

Proposal 1: circular polarization shall be used for TRP and TRS measurement for NTN device performances.

Proposal 2: set NTN RTT (Round Trip Time) to zero or a very low value to reduce TRS measurement time.

# Reference

- R4-2416590 Way Forward for [112bis][323] TRP_TRS_Ph3

- Over-the-Air Performance Evaluation of NB-IoT Over Non-Terrestrial Networks, J. Luo et al, Session 15 on Advanced Antenna Measurements, the 46th​ ​Annual​ ​Meeting​ ​and​ ​Symposium​ ​of​ ​the​ ​Antenna​ ​Measurement​ ​Techniques Association, 27 Oct – 1 November 2024, Cincinnati, OH,​ ​USA

- TS 38.521-5, v18.3, User Equipment (UE) conformance specification; Radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance

- TS 38.508-1, v18.4, User Equipment (UE) conformance specification; Part 1: Common test environment