# 3GPP TSG-RAN WG4 Meeting #113	R4-2417622

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.18.2

Source: 	OPPO

Title: 	Study of impacts on RAN4 requirements for AI mobility

Document for:	Approval

- Introduction

In RAN#102 meeting, the SID on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR is approved. In RAN#105 meeting, the related work plan in RAN4 has been further checked and updated as follows:

Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

Some background information derived from RAN2 SI is as follows:

To evaluate potential benefits and gains of AI/ML aided mobility, following aspects are agreed in SID[1] for study:

- Measurement prediction (including UE sided and NW sided model)

- HO failure/RLF prediction (only UE sided model)

- Measurement events prediction (only UE sided model)

And as clarified in RAN2 on TR 38.744, the study of the use cases is driven mainly by two study goals.

- The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements.

- The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption).

Regarding the RRM measurement prediction

- 3 sub-use cases are considered for cell-level RRM measurement prediction

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

- Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

- Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

- Temporal/frequency/spatial domain measurement prediction

- For temporal domain measurement prediction,

In RAN2#125bis meeting, it has been agreed as:

- For temporal domain measurement prediction, we will consider the AI-PHY beam management Case A and Case B from the RAN1 AI/ML PHY TR and it applies to both beam level and cell level. As baseline we will focus on pure temporal predication.

In addition, based on the post email discussion after RAN2#127 meeting, temporal domain case A and case B have been further clarified as:

Intra-frequency temporal domain case A: continuous measurement results in prediction window are predicted by continuous historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either one or more sampling periods (with sliding L1/L3 filtering option) or one or more measurement periods (with non-sliding L1/L3 filtering option), where measurement result(s) are actually measured before sliding.

Figure 1

Intra-frequency temporal domain case B: measurement results in prediction window are predicted by historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option) and measurement result(s) in previous prediction window is/are skipped during window sliding

Figure 2-1 temporal domain case example 1

Figure 2-2 temporal domain case example 2

- For frequency domain measurement prediction, typical feature is to use the measurement result from one frequency to predict the measurement result at another frequency, which can be applicable to FR1 to FR1 cases in R19 evaluation.

- For spatial domain measurement prediction, typical feature is to predict cell-level measurements through measurements of partial beams, which can be applicable to sub-use case 1 and 3 in R19.

- Prioritization for different RRM measurement prediction scenarios

Different sub-use cases and different scenario assumptions mentioned above may involve a lot of combinations such as FR1 to FR1, FR2 to FR2, intra/inter-frequency, intra/inter-cell, case A/B, test goal 1/2, etc. In R19, to simplify the scope, RAN2 has decided to focus on the following scenarios and conditions, and provided an explanation of the priorities in RAN2#126.

The prioritization among evaluation scenarios in RAN2 is captured as follows:

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

- Discussion

In this contribution, we will further evaluate RAN4 impacts based on RAN2 progress and current RAN4 discussions.

Performance Metrics/KPIs for Measurement Prediction use case

In RAN4#112bis meeting, RAN4 has agreed to start the discussion with RRM measurement prediction use-case.

3 sub-use cases are agreed in RAN2 and considered in RAN4 for cell-level RRM measurement prediction:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

- Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

- Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

Here, from the point view of AI/ML model implementation：

- Sub-use case 1: The AI/ML model employs actual L1 beam-level measurements as the input to predict L1 beam-level measurement outputs. Subsequently, the inferred L1 beam-level measurement results could be leveraged to generate L3 cell-level measurement outcomes. The entire process could be denoted by L1-L1-L3, with two segments: the L1-L1 AI/ML prediction forms the first segment, while the second involves L1-L3, potentially through legacy methodologies.

Figure 3-1

- Sub-use case 2: The AI/ML model uses actual L3 cell-level measurements as input to predict their L3 output. This entire process, could be represented by L1-L3-L3, also splits into two parts: initially obtaining actual L3 cell-level measurement results from L1 measurements (L1-L3, e.g., in legacy way with filter design or up to company’s implementation), followed by the L3-L3 RRM measurement prediction by AI/ML.

Figure 3-2

- Sub-use case 3: a L1-L3 end-to-end way, involves the AI/ML model using L1 beam-level measurements as input to foretell the L3 cell-level measurement output directly, using L1 as input and L3 as eventual output.

Figure 3-3

Each of these sub-use cases utilizes L1-level measurements as the origin and aims for L3-level measurement predictions as their destinations, the entire process or a portion thereof being replaced by AI/ML modules or solutions. The differences primarily pertain to various implementation methodologies for transitioning from L1 measurement to L3 prediction.

For RAN4 testing, RAN4 can treat this without excessive fragmentation into different AI/ML implementation strategies and the synergy between AI and traditional algorithms. All three sub-use cases should use L3-level measurements as test objects for RAN4 testing (Point C in the figure 4), rather than check L1/L3 beam-level results(e.g. point A or E in figure 4). Concerning sub-use case 1 and sub-use case 2, some additional parameters(if needed) may be included as part of the test conditions in the form of side conditions (e.g., L1/L3 filter).

Figure 4 L1/L3 measurement framework

Observation 1: For Sub-use case 1/2/3, differences primarily pertain to various implementation methodologies for transitioning from L1 measurement to L3 prediction, the entire process or a portion thereof being replaced by AI/ML modules or solutions. For RAN4 testing, RAN4 can treat this without excessive fragmentation into different AI/ML implementation strategies and the synergy between AI and traditional algorithms.

Proposal 1: All three sub-use cases should use L3-level measurements as test objects for RAN4 testing (Point C ), rather than L1/L3 beam-level results(e.g. point A or E).

- Concerning sub-use case 1 and sub-use case 2, some additional parameters(if needed) may be included as part of the test conditions in the form of side conditions (e.g., L1/L3 filter).

Regarding the performance metrics, measurement prediction accuracy could be utilized for all 3 sub-use cases and the definition from RAN2 could be reused in RAN4.

- Metrics for measurement prediction accuracy

Measurement prediction accuracy for cell-level RRM measurement prediction is defined as Average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases, and the RSRP difference values should be an absolute value before they are averaged.

- Assumptions on measurement reduction rate

MRRT and MRRS that defined in RAN2 can be reused:

MRRT = skipped measurement time instances / total measurement time instances

MRRS = skipped beams to be measured/ total beams to be measured

Proposal 2: In RAN4, reuse the measurement prediction accuracy that defined in RAN2 as candidate performance metrics for all RRM sub-use cases,

- Metrics for measurement prediction accuracy: Measurement prediction accuracy for cell-level RRM measurement prediction is defined as Average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases, and the RSRP difference values should be an absolute value before they are averaged.

Proposal 3: In RAN4, reuse the assumptions on measurement reduction rate that defined in RAN2:

MRRT = skipped measurement time instances / total measurement time instances

MRRS = skipped beams to be measured/ total beams to be measured

Impacts of measurement errors on the prediction accuracy performance

In RAN4# 112bis meeting, 4 options have been proposed for discussion on the impacts of measurement errors

- Option 1: RAN4 to study the prediction accuracy performance with consideration of the impact of measurement error.

- Option 2: suggest RAN2 to take into account the measurement errors in RAN2 evaluation.

- Option 3: Wait for RAN2 evaluation results and if RAN2 doesn’t consider measurement errors then RAN4 should do it after RAN2 evaluations.

- Option 4: Wait for more progress in AI/ML BM to study the impact of measurement error on the prediction accuracy requirements for AI mobility

Our understanding is:

- Whether and how to evaluate the impacts of measurement error on performance requirement should be considered within RAN4 discussions, do not need to let RAN2 do extra works.

- The current discussion in RAN4 BM already draws attention to the impact of measurement error, including how this evaluation could be conducted and its potential influence on performance requirements. To avoid redundant work of these assessments, we’d better not to introduce similar evaluations/works in the research of RAN4 AI mobility study at this stage.

- Also, RAN4 needs to consider the TU budget. It is challenging to complete within 0.5 TU for the measurement error evaluations/simulations. Aside from the discussions centered on measurement error, there are a lot of unresolved issues we need to address in R19. We do not think that initiating evaluations on measurement error is the most critical task within the current 0.5 TU. framework.

Proposal 4: Whether and how to evaluate the impacts of measurement error on performance requirement should be considered within RAN4 discussions, do not need to let RAN2 do extra works.

Proposal 5: To avoid redundant work of these assessments, not to introduce similar evaluations/works in the research of RAN4 AI mobility study at this stage.

Proposal 6: Initiating evaluations on measurement error is not the most critical task within the current 0.5 TU framework.

- Impacts on Core requirements

In the last meeting, RAN4 agreed to start the discussion for scenario 2, 3, 4 as listed below:

| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| --- | --- | --- | --- | --- |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

For each scenario, further consideration is needed on how to define it in RAN4 tests. This includes the configuration of the observation window (e.g., window length, measurement sample configuration within the window), the configuration of the prediction window (e.g., window length, samples to be predicted within the window), and other test settings/conditions. The above aspects can be referenced based on the progress of RAN2's research, particularly the evaluation assumptions.

- Based on the post email discussion after RAN2#127 meeting[2], for temporal domain case A, following limitations have been made in RAN2 and could be considered in RAN4 discussion:

  - The ratio between observation window and prediction window is at least limited to value range {5,4,3,2,1,1/2,1/3,1/4,1/5}

  - The window length should be multiple times of sampling period or measurement period of corresponding frequency range.

- Based on the post email discussion after RAN2#127 meeting[3], for FR1 intra-frequency temporal domain case B, following recommended value range have been suggested in RAN2 and could be also considered in RAN4 discussion:

  - MRRT: {50%~80%}

  - OW: {40ms ~ 2000ms}

  - PW: {40ms~800ms}

Proposal 7: For each scenario, further consideration is needed on how to define it in RAN4 tests, e.g., configurations of the observation window and prediction window.

Proposal 8: For temporal domain case A, following limitations have been made in RAN2 and could be considered in RAN4:

- the ratio between observation window and prediction window is at least limited to value range {5,4,3,2,1,1/2,1/3,1/4,1/5}

- the window length should be multiple times of sampling period or measurement period of corresponding frequency range.

Proposal 9: For FR1 intra-frequency temporal domain case B, following recommended value range have been suggested in RAN2 and could be also considered in RAN4 discussion:

- MRRT: {50%~80%}

- OW: {40ms ~ 2000ms}

- PW: {40ms~800ms}

- Conclusions

In this contribution, we discussed the Rel-19 AI/ML impacts to RAN4 and got the following proposals

Observation 1: For Sub-use case 1/2/3, differences primarily pertain to various implementation methodologies for transitioning from L1 measurement to L3 prediction. For RAN4 testing, RAN4 can treat this without excessive fragmentation into different AI/ML implementation strategies and the synergy between AI and traditional algorithms.

Sub-use case 1

Sub-use case 2

Sub-use case 3

Proposal 1: All three sub-use cases should use L3-level measurements as test objects for RAN4 testing (Point C ), rather than L1/L3 beam-level results(e.g. point A or E).

- Concerning sub-use case 1 and sub-use case 2, some additional parameters(if needed) may be included as part of the test conditions in the form of side conditions (e.g., L1/L3 filter).

Proposal 2: In RAN4, reuse the measurement prediction accuracy that defined in RAN2 as candidate performance metrics for all RRM sub-use cases,

- Metrics for measurement prediction accuracy: Measurement prediction accuracy for cell-level RRM measurement prediction is defined as Average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases, and the RSRP difference values should be an absolute value before they are averaged.

Proposal 3: In RAN4, reuse the assumptions on measurement reduction rate that defined in RAN2:

MRRT = skipped measurement time instances / total measurement time instances

MRRS = skipped beams to be measured/ total beams to be measured

Proposal 4: Whether and how to evaluate the impacts of measurement error on performance requirement should be considered within RAN4 discussions, do not need to let RAN2 do extra works.

Proposal 5: To avoid redundant work of these assessments, not to introduce similar evaluations/works in the research of RAN4 AI mobility study at this stage.

Proposal 6: Initiating evaluations on measurement error is not the most critical task within the current 0.5 TU framework.

Proposal 7: For each scenario, further consideration is needed on how to define it in RAN4 tests, e.g., configurations of the observation window and prediction window.

Proposal 8: For temporal domain case A, following limitations have been made in RAN2 and could be considered in RAN4:

- the ratio between observation window and prediction window is at least limited to value range {5,4,3,2,1,1/2,1/3,1/4,1/5}

- the window length should be multiple times of sampling period or measurement period of corresponding frequency range.

Proposal 9: For FR1 intra-frequency temporal domain case B, following recommended value range have been suggested in RAN2 and could be also considered in RAN4 discussion:

- MRRT: {50%~80%}

- OW: {40ms ~ 2000ms}

- PW: {40ms~800ms}

- References

RP-234055 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

R2-2407849 summary of [POST127][030][AI mobility] RRM simulation assumptions - Phase 1

R2-2407885 summary of [POST127][030][AI mobility] RRM simulation assumptions - phase 2