3GPP TSG-RAN4 Meeting #113                                              R4-2417644

# Orlando, USA, 18th – 22nd November 2024

Agenda Item:	5.9.4

Source: 	CAICT

Title: 	(LTE_NBIoT_eMTC_NTN_req-Perf) Discussion on the understanding of the Note in Table 8.2.1.2-1 of TS 36.102

Document for:	Discussion and Endorsement

Introduction

There is a Note in Table 8.2.1.2-1 of TS 36.102 [1] saying that “For UE supports NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also applies to UE according to the UE category and capability”. This paper is for the discussion of the understanding of this Note.

Discussion

The following is from TS 36.102 [1]：

Table 8.2.1.2-1: Requirements applicability for optional UE features

| UE feature/capability | Test list | Applicability notes |
| --- | --- | --- |
| NTN access (ntn-Connectivity-EPC-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1 |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2 |
| NTN scenario support (ntn-ScenarioSupport-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
| Operation in coverage enhancement mode A (ce-ModeA-r13) | Clause 8.2.1.1 (Test 1, Test 2) | The requirements apply only for UE Category M1 |
| Operation in coverage enhancement mode B (ce-ModeB-r13) | Clause 8.2.1.1 (Test 3) | The requirements apply only for UE Category M1 |
| Note: For UE supports NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also applies to UE according to the UE category and capability |  |  |

There are two understandings for the above green highlighted Note:

Understanding 1: The Note in Table 8.2.1.2-1 of TS 36.102 only applies to GSO IoT NTN UE and does not apply to NGSO IoT NTN UE. For NGSO IoT NTN only UE that does not support TN and GSO, the requirements in TS36.101 Clause 8 and Clause 9 do not apply.

Understanding 2: The Note in Table 8.2.1.2-1 of TS 36.102 applies to both GSO IoT NTN UE and NGSO IoT NTN UE. For NGSO IoT NTN only UE that does not support TN and GSO, the requirements in TS36.101 Clause 8 and Clause 9 also apply to UE according to the UE category and capability.

Proposal 1: Understanding 1 is correct and the Note in Table 8.2.1.2-1 of TS 36.102 needs to be revised to imply that it only apples to GSO IoT NTN UE and does not apply to NGSO IoT NTN UE.

Proposal 2: Understanding 2 is correct and the Note in Table 8.2.1.2-1 of TS 36.102 remains as it is.

Conclusion

This paper discussed the understanding of the Note in Table 8.2.1.2-1 of TS 36.102 and provided corresponding proposals based on each understanding:

Understanding 1: The Note in Table 8.2.1.2-1 of TS 36.102 only applies to GSO IoT NTN UE and does not apply to NGSO IoT NTN UE. For NGSO IoT NTN only UE that does not support TN and GSO, the requirements in TS36.101 Clause 8 and Clause 9 do not apply.

Understanding 2: The Note in Table 8.2.1.2-1 of TS 36.102 applies to both GSO IoT NTN UE and NGSO IoT NTN UE. For NGSO IoT NTN only UE that does not support TN and GSO, the requirements in TS36.101 Clause 8 and Clause 9 also apply to UE according to the UE category and capability.

Proposal 1: Understanding 1 is correct and the Note in Table 8.2.1.2-1 of TS 36.102 needs to be revised to imply that it only apples to GSO IoT NTN UE and does not apply to NGSO IoT NTN UE.

Proposal 2: Understanding 2 is correct and the Note in Table 8.2.1.2-1 of TS 36.102 remains as it is.

# References

- TS 36.102 V18.6.0, Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access