3GPP TSG-RAN4 #113	R4-241xxxx

Orlando, USA, 18th Nov. – 22nd Nov. 2024

Title:	FR2 test setup for AI/ML beam management evaluation

Source:			Anritsu Corporation

Agenda Item:			7.17.4.2

Document for:			Approval

# 1.	Introduction

At the RAN4 #112-bis meeting in Hefei, we had an extensive discussion on a test setup for the AI/ML beam management. And the associated way forward [1] was created, extracted for reference.

- Issue 2-9: Test setup requirements Agreement: - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A

In this contribution we share our proposals on a practical test setup for the AI/ML beam management evaluation.

# 2.	Discussion

  - General view on the FR2 test setup for AI/ML beam management evaluation

As of now, we have roughly three types of FR2 OTA (Over The Air) test setups, 1AoA (Angle of Arrival), 2AoA and MPAC (Multi Probe Anechoic Chamber). If we create a new test setup for AI/ML FR2 Beam Management evaluation following the current RAN4 discussion, and if we choose a solution that we extend an existing MPAC test system equipped with multiple probes in a chamber, it means that we will have another different OTA chamber only for this test, with which we do not have any other usage for OTA tests at this moment.

Observation 1: If we simply extend the existing MPAC system and create a test system for the AI/ML beam management evaluation, we will have another different OTA chamber.

Observation 2: We should note that we don’t have other usages with the AI/ML beam management test system.

Proposal 1: Agree to avoid creating independent FR2 OTA chamber for the conformance test of AI/ML beam management.

2.2 Issues with AI/ML beam management evaluation test system

Preconditions of AI/ML beam management (Set A/ Set B Tx beams)

As agreed at the previous meeting [2], the new test system needs to be equipped with an ability to emulate a lot of Tx beams, [64 to 128] beams for set A to obtain the ground truth.

Observation 3: The new test system needs to equip an ability to emulate a lot of Tx (BS) beams, [64 to 128] beams for Set A to obtain the ground truth.

Physical restrictions with CDL channel model

When we consider a characteristic of the FR2 beam management evaluation, it is straightforward to provide multiple downlink signals simultaneously and physically from multiple directions. However, if we try to apply the CDL channel model for this test, we need to emulate multiple clusters (different AoD and AoA) and many corresponding test probes along with the test equipment become inevitable. Also, since we have multiple CDL channel models now from CDL-A to CDL-E, we have another variation of clusters.

Observation 4: If we try to apply the CDL channel model with a straightforward way for the FR2 Beam management test, where multiple DL signals are provided simultaneously and physically from multiple directions to emulate multiple clusters, many corresponding test probes along with the test equipment become inevitable.

Observation 5: Multiple CDL channel models will increase a variation of cluster patterns (AoDs and AoAs) to be emulated.

Limitation with an extension of MPAC

As provided the simulation results at the previous meeting [3], if we try to achieve emulating multiple Tx (BS) beams (Set A/B), clusters and CDL channel models by extending the existing MPAC for MIMO OTA, we need to consider hundreds of test probe placements in an OTA chamber, which is not practical anymore.

Observation 6: We need to consider hundreds of patterns of probe placement in the chamber if we try to simulate CDL models following the MIMO OTA methodology, which is not practical.

On the other hand, if we try to conduct tests with simplified CDL model, there might be an issue with a test coverage based on an appropriate test environment.

Observation 7: A simplified CDL model may cause an issue with insufficient test conditions.

2.3 Consideration on an alternative test setup

Conditions to solve the above issues are as follows.

Observation 8: Following conditions should be fulfilled to achieve the FR2 beam management test setup.

- Reuse the existing OTA chamber.

- Achievable number of Tx (BS) beams are same with set A.

- Clusters can be emulated based on the number of agreed set A beams.

In addition to the conditions above, we have following views on the performance of AI features.

- In the real fields, a direction of UE and Rx beam steering in a UE can be arbitrary when an RSRP measurement is made by the UE for the inputs of the AI/ML inference. Thus, UE shall be able to conduct the AI/ML inference regardless of variety in the RSRP measurement results obtained by arbitrary directions of UE and Rx beam steering.

- In a case where we incorporate UE Rx beam steering patterns in the channel models of a fading simulator, there is a limitation with a test that a UE Rx beam steering patterns, which are inherently controlled autonomously by the UE, cannot be tested while the UE itself controls them. However, given the need to be able to perform AI/ML inferences with arbitrary UE orientation and/or receive beam patterns, it can be considered that testing under the condition of incorporating all UE orientations and receive beam patterns into the channel model can be effective.

In the test system proposed later, the AI/ML estimation results will be tested in such a way that the UE Rx beam is locked and yet the UE Rx beam characteristics are incorporated into a fading simulator. Test results in such a test environment are considered justified based on above.

Observation 9: The AI needs to be able to predict the optimum BS beam and RSRP from a limited number of measurements, independent of the UE orientations and UE Rx beam patterns.

Observation 10: Given the above, even if the AI/ML estimation results were tested in such a way that the UE Rx beam is locked and the UE Rx beam characteristics are incorporated into a fading simulator, the test results in such a test environment are justified, as far as tested with various enough UE orientations and UE Rx beam patterns in the channel-model simulations.

In addition to the ideas above, considering the proposals in the previous contribution from MediaTek [4], which proposed the use of 1AoA test system, we would like to propose a following test setup, BTS simulator + Fading simulator + 1AoA OTA chamber.

Figure 2.3-1: Test setup of AI/ML beam management performance evaluation

Here are the supplemental explanations of each equipment in the figure above.

BTS simulator

Legacy features as the system simulator are necessary.

Fading simulator

CDL channel model + multiple UE antenna beam steering patterns + UE orientations are all contained in its simulation.

A difference between the legacy MPAC and this test setup is that the CDL models in the MPAC tests do not include the antenna beam steering patterns and UE orientations in their tests.

It is preferred that the antenna beam steering patterns are also standardized. TBD if we need to measure antenna beam patterns of each DUT independently.

OTA chamber

The legacy 1AoA DFF/IFF antenna can be reused.

A probe number can be 1 if we consider the rank 2 test.

FFS on a need of 2AoA 4-layer tests.

DUT

Same as demodulation tests, a UE beam is locked at bore sight direction and the UE is connected to the test system by the wireless (virtual) cable connection.

An interface and/or signalling are necessary to report inferred results to the test system based on a measurement of DL signals from the fading simulator.

2.4 Verdict of the measurement results

FFS on a verdict of the measurement results and corresponding inferences. For example, it can be considered that the report from UE based on the inference and the ground truth are compared.

Observation 11: FFS on a verdict of measurement results and corresponding inferences.

2.5 Summary

In summary, we have the following proposals and observation.

Proposal 2: As for the AI/ML beam management test, we evaluate the performance of the feature with the following test setup: BTS simulator + Fading simulator + 1AoA OTA chamber.

Proposal 3: Following aspects need further studies.

Specification of the report and the interface between the UE and the test system.

CDL models to be supported (LOS and/or NLOS)

Definitions of UE antenna beam steering patterns to be contained in the simulated channel model

UE orientations to be contained in the simulated channel model

2-layer test is the starting point. FFS for a need of 4-layer support.

Verdict of the measurement results and/or inferences.

Total test time.

Observation 12: Feedback to the issue 2-9 in the previous way forward [1] in a case we use the test system described at clause 2.3 is as follows.

CDL model: Same as existing CDL models. No need to simplify the legacy CDL models.

AoD range: Same as Set A.

Number of beams: Same as Set A.

Number of probes: One with cross polarized antenna (2 layers), FFS for 4-layer case.

- Conclusion

In this contribution we discussed a practical test setup for the AI/ML beam management evaluation.

Observation 1: If we simply extend the existing MPAC system and create a test system for the AI/ML beam management evaluation, we will have another different OTA chamber.

Observation 2: We should note that we don’t have other usages with the AI/ML beam management test system.

Proposal 1: Agree to avoid creating independent FR2 OTA chamber for the conformance test of AI/ML beam management.

Observation 3: The new test system needs to equip an ability to emulate a lot of Tx (BS) beams, [64 to 128] beams for Set A to obtain the ground truth.

Observation 4: If we try to apply the CDL channel model with a straightforward way for the FR2 Beam management test, where multiple DL signals are provided simultaneously and physically from multiple directions to emulate multiple clusters, many corresponding test probes along with the test equipment become inevitable.

Observation 5: Multiple CDL channel models will increase a variation of cluster patterns (AoDs and AoAs) to be emulated.

Observation 6: We need to consider hundreds of patterns of probe placement in the chamber if we try to simulate CDL models following the MIMO OTA methodology, which is not practical.

Observation 7: A simplified CDL model may cause an issue with insufficient test conditions.

Observation 8: Following conditions should be fulfilled to achieve the FR2 beam management test setup.

- Reuse the existing OTA chamber.

- Achievable number of Tx (BS) beams are same with set A.

- Clusters can be emulated based on the number of agreed set A beams.

Observation 9: The AI needs to be able to predict the optimum BS beam and RSRP from a limited number of measurements, independent of the UE orientations and UE Rx beam patterns.

Observation 10: Given the above, even if the AI/ML estimation results were tested in such a way that the UE Rx beam is locked and the UE Rx beam characteristics are incorporated into a fading simulator, the test results in such a test environment are justified, as far as tested with various enough UE orientations and UE Rx beam patterns in the channel-model simulations.

Observation 11: FFS on a verdict of measurement results and corresponding inferences.

Proposal 2: As for the AI/ML beam management test, we evaluate the performance of the feature with the following test setup: BTS simulator + Fading simulator + 1AoA OTA chamber.

Proposal 3: Following aspects need further studies.

Specification of the report and the interface between the UE and the test system.

CDL models to be supported (LOS and/or NLOS)

Definitions of UE antenna beam steering patterns to be contained in the simulated channel model

UE orientations to be contained in the simulated channel model

2-layer test is the starting point. FFS for a need of 4-layer support.

Verdict of the measurement results and/or inferences.

Total test time.

Observation 12: Feedback to the issue 2-9 in the previous way forward [1] in a case we use the test system described at clause 2.3 is as follows.

CDL model: Same as existing CDL models. No need to simplify the legacy CDL models.

AoD range: Same as Set A.

Number of beams: Same as Set A.

Number of probes: One with cross polarized antenna (2 layers), FFS for 4-layer case.

# 4.	References

[1] R4-2417212, “WF on AI/ML”, Moderator (Qualcomm), RAN4 #112-bis, Hefei, China

[2] R4-2410570, “WF on AI/ML”, Moderator (Qualcomm), RAN4 #111, Fukuoka, Japan

[3] R4-2411587, “Discussion on test setup for AI/ML based beam management”, Rohde&Schwarz, RAN4 #112, Maastricht, Netherlands

[4] R4-2414930, “Discussion on testability issues of AI/ML RAN4 BM use case”, MediaTek Inc., RAN4 #112-bis, Hefei, China