3GPP RAN WG4 Meeting #113	R4-2417694

Orlando, US, November 18th – November 22nd, 2024

Title: 	Discussion on Performance requirements of SL positioning

Source: 	CATT

Agenda item:	5.22.2.2

Document for:	Discussion

Introduction

In RAN4#112bis meeting, a few issues about performance requirements and test cases for SL positioning were solved and the agreements were captured in the WF [1]. There is one issue about how to configure TX UEs in test cases and it needs to be further discussed. In this contribution, we will present our view on this issue.

Discussion

- Issue 1: Multiplexing of TX UEs in TCs

In last meeting, how to multiplex multiple TX UEs in SL test cases was discussed without any agreement.

UE capabilities [2] reproduced below play an important role in this issue.

- maxNumOfActiveSL-PRS-ResourcesInOneSlot

- maxNumOfSlotsWithActiveSL-PRS-Resources

- minTimeAfterEndofSlotCarryActiveSL-PRS-Resources

But since RAN4 has an agreement that the transmitted SL PRS resources from different anchor UEs do not overlap in time domain, the second and third IEs have more impacts in test cases. If the number of TX UEs configured in TCs is larger than the reported value of maxNumOfSlotsWithActiveSL-PRS-Resources, then RAN4 needs to configure enough time for UE to finish processing received SL PRS resources before any extra SL PRS resources, which exceeds UE capability, are transmitted. And a time period indicated by minTimeAfterEndofSlotCarryActiveSL-PRS-Resources can be considered for the above case.

Specifically, the applicable numbers of TX UEs in all SL TCs are 1, 2, and 3, and configuring 2 TX UEs is the most typical case, for which RAN4 can provide the following SL PRS configuration in clause A.3.21A.2.

Table A.3.21A.2.1-1: SL PRS.1 FR1: SL-PRS configuration

| SL PRS Parameters | Values |  |  |  |
| --- | --- | --- | --- | --- |
| Reference channel | SL PRS.1.1 FR1 | SL PRS.1.2 FR1 | SL PRS.1.3 FR1 | SL PRS.1.4 FR1 |
| SCS | 15 kHz, 30 kHz |  |  |  |
| SL-PRS comb size | 4 |  | 4 |  |
| Number of SL-PRS symbols | 2 |  | 4 |  |
| SL-PRS comb offset Note 1 | 1 |  | 1 |  |
| SL-PRS resource slot offset (slot) Note 1 | 0 | 1Note 2 | 0 | 1 Note 2 |
| PRB numbers containing SL PRS within channel Bandwidth Note 1 | 24 |  | 48 |  |
| SL-PRS starting PRB | 4 |  | 4 |  |
| NOTE 1: Unless otherwise specified in the test case NOTE 2: Unless otherwise specified in the test case. If reported value for maxNumOfSlotsWithActiveSL-PRS-Resources is ‘n1’, then slot offset is minTimeAfterEndofSlotCarryActiveSL-PRS-Resources for SCS 15kHz and 2*minTimeAfterEndofSlotCarryActiveSL-PRS-Resources for SCS 30kHz. |  |  |  |  |

Proposal 1: Configure slot offset as 1 slot in SL PRS configuration in clause A.3.21A.2 and add a note to handle cases that exceed UE capabilities:

Note: If reported value for maxNumOfSlotsWithActiveSL-PRS-Resources is ‘n1’, then slot offset is minTimeAfterEndofSlotCarryActiveSL-PRS-Resources for SCS 15kHz and 2*minTimeAfterEndofSlotCarryActiveSL-PRS-Resources for SCS 30kHz.

Similarly, for test cases configured with 3 TX UEs, the above analysis also applies. There are mainly three cases:

Case 1: maxNumOfSlotsWithActiveSL-PRS-Resources equals to ‘n1’

Then the time interval between any two of SL PRS resources should be equal to the value indicated in the note.

Case 2: maxNumOfSlotsWithActiveSL-PRS-Resources equals to ‘n2’

For this case, the time interval between SL PRS resources from TX UE1 and TX UE2 can be 1, and it should be the value indicated in the note for TX UE2 and TX UE3.

Case 3: maxNumOfSlotsWithActiveSL-PRS-Resources larger than ‘n2’

The time interval between any two of SL PRS resources can be equal to 1 slot.

In summary, we have the following proposal:

Proposal 2: For SL TCs configured with 3 TX UEs,

(1) If maxNumOfSlotsWithActiveSL-PRS-Resources equals to ‘n1’:

-	 The time offset between SL PRS resources from TX UE1 and TX UE2 is:

minTimeAfterEndofSlotCarryActiveSL-PRS-Resources for SCS 15kHz and 2*minTimeAfterEndofSlotCarryActiveSL-PRS-Resources for SCS 30kHz;

-	 The time offset between SL PRS resources from TX UE2 and TX UE3 is:

minTimeAfterEndofSlotCarryActiveSL-PRS-Resources for SCS 15kHz and 2*minTimeAfterEndofSlotCarryActiveSL-PRS-Resources for SCS 30kHz.

2) if maxNumOfSlotsWithActiveSL-PRS-Resources equals to ‘n2’:

-	 The time offset between SL PRS resources from TX UE1 and TX UE2 is:

1 slot;

-	 The time offset between SL PRS resources from TX UE2 and TX UE3 is:

minTimeAfterEndofSlotCarryActiveSL-PRS-Resources for SCS 15kHz and 2*minTimeAfterEndofSlotCarryActiveSL-PRS-Resources for SCS 30kHz.

3) if maxNumOfSlotsWithActiveSL-PRS-Resources is larger than ‘n2’:

-	 The time offset between SL PRS resources from TX UE1 and TX UE2 is:

1 slot;

-	 The time offset between SL PRS resources from TX UE2 and TX UE3 is:

1 slot.

A CR is submitted to capture the above changes [3].

Conclusions

This paper discussed the performance requirements for SL positioning and provided the following proposals:

Proposal 1: Configure slot offset as 1 slot in SL PRS configuration in clause A.3.21A.2 and add a note to handle cases that exceed UE capabilities:

Note: If reported value for maxNumOfSlotsWithActiveSL-PRS-Resources is ‘n1’, then slot offset is minTimeAfterEndofSlotCarryActiveSL-PRS-Resources for SCS 15kHz and 2*minTimeAfterEndofSlotCarryActiveSL-PRS-Resources for SCS 30kHz.

Proposal 2: For SL TCs configured with 3 TX UEs,

(1) If maxNumOfSlotsWithActiveSL-PRS-Resources equals to ‘n1’:

-	 The time offset between SL PRS resources from TX UE1 and TX UE2 is:

minTimeAfterEndofSlotCarryActiveSL-PRS-Resources for SCS 15kHz and 2*minTimeAfterEndofSlotCarryActiveSL-PRS-Resources for SCS 30kHz;

-	 The time offset between SL PRS resources from TX UE2 and TX UE3 is:

minTimeAfterEndofSlotCarryActiveSL-PRS-Resources for SCS 15kHz and 2*minTimeAfterEndofSlotCarryActiveSL-PRS-Resources for SCS 30kHz.

2) if maxNumOfSlotsWithActiveSL-PRS-Resources equals to ‘n2’:

-	 The time offset between SL PRS resources from TX UE1 and TX UE2 is:

1 slot;

-	 The time offset between SL PRS resources from TX UE2 and TX UE3 is:

minTimeAfterEndofSlotCarryActiveSL-PRS-Resources for SCS 15kHz and 2*minTimeAfterEndofSlotCarryActiveSL-PRS-Resources for SCS 30kHz.

3) if maxNumOfSlotsWithActiveSL-PRS-Resources is larger than ‘n2’:

-	 The time offset between SL PRS resources from TX UE1 and TX UE2 is:

1 slot;

-	 The time offset between SL PRS resources from TX UE2 and TX UE3 is:

1 slot.

References

[1] R4-2416870, WF on RRM requirements for NR_pos_enh2_Part2, CATT, RAN4#112bis.

[2] TS 38.355, V18.3.0.

[3] R4-2417691, Corrections to test cases for SL positioning, CATT, RAN4#113.