3GPP RAN WG4 Meeting #113	R4-2417702

Orlando, USA, November 18th – November 22nd, 2024

Title: 	Discussion on testability and interoperability issues for CSI prediction

Source: 	CATT

Agenda item:	7.17.3.1

Document for:	Discussion

Introduction

In RAN4#112bis meeting, RAN4 discussed Rel-19 AI/ML for NR air interface and agreements were captured in [1]. The relative throughput was chosen to be the metric for CSI prediction, but the baseline was not finally determined. In this contribution, we will continue to discuss the baseline.

Discussion

CSI prediction accuracy performance

Regarding AI/ML performance, there were some agreements achieved in last meeting:

Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

For CSI prediction, based on the agreements in RAN1#118 meeting, performance improvement is observed. Hence, the AI/ML based performance should be better than the legacy performance.

Agreement From RAN1 perspective, study of CSI prediction has been completed and performance improvement is observed with increased complexity.

Regarding the selection of legacy performance, there are two main options:

- Measurement based CSI

- R18 CSI prediction

In our previous contribution submitted to RAN4#112bis, the discussion was only based on measurement-based CSI method and a main concern about R18 CSI prediction is that this feature is optional. But after some further thinking and considering the motivation of AI/ML based CSI prediction is quite similar to the R18 CSI prediction, it is more suitable to use the performance requirements of R18 CSI prediction as the baseline for comparison with AI/ML based CSI prediction.

Proposal 1: Performance of AI/ML based CSI prediction should be better than legacy performance requirement.

- The legacy performance requirement is the requirement for R18 CSI prediction.

CSI prediction tests

Some proposals discussed how to conduct the CSI prediction test in detail:

- Proposals - Option 1: Use TDL model for testing and static condition (similar to legacy test) - Option 2: Define test requirements relative to performance with random PMI Type codebook - Option 3: RAN4 to consider Spatially Correlated Channel models (SCM) for the requirements/test on AIML CSI prediction. - Option 4: Define test requirements relative to performance with eTypeII Codebook - Option 5: others

Both R18 CSI prediction and AI/ML based CSI prediction are the internal implementations in user equipment. As we discussed the performance baseline in section 2.1, the requirements for R18 CSI prediction could be used as baseline for comparison, which means the test can be conducted in a similar environment to legacy test.

Proposal 2: RAN4 to use TDL model for testing and static condition (similar to legacy test).

Conclusions

This contribution discussed the issues related to AI/ML enabled CSI prediction and some proposals are provided:

Proposal 1: Performance of AI/ML based CSI prediction should be better than legacy performance requirement.

- The legacy performance requirement is the requirement for R18 CSI prediction.

Proposal 2: RAN4 to use TDL model for testing and static condition (similar to legacy test).

References

[1] R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated, RAN4#112bis.