3GPP TSG-RAN WG4 Meeting #113	R4-2417716

# Orlando , US, Nov 18 – 22, 2024

Title: 	Discussion on enhancements for Rel-19 ATG RRM requirements

Source: 	CATT

Agenda item:	7.10.4

Document for:	Discussion

- Introduction

A new RAN4 leading work item for Air-to-ground network for NR was approved in RAN#103 [1] and the revised WID was approved in RAN#104 and RAN#105 [2]. The core part objectives of the work item are as following:

The core part of the work item includes:

- Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]:

  - FR1 intra-band contiguous CA

    - Example band combination: CA_n79C

  - FR1+FR1 inter-band CA

    - Example band combination: CA_n3A-n39A

- Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4]

- Specify signaling and UE capabilities if necessary [RAN2].

In the Justification of WID, the following clarifications are presented:

Considering the particularity of ATG network deployment, the following aspects should be addressed in Rel-19 ATG work item.

- Enhance throughput to meet ATG capacity demand

- Fully utilize operators’ spectrum especially for the fragment spectrum

This document will discuss the general RRM issues about ATG and give our understanding and proposals.

- Discussion

  - 2.1 Timing

- 2.1.1 MRTD for ATG inter-band CA

In the last meeting, RAN4 discussed whether to define the MRTD requirements for ATG FR1 inter-band CA, and there was still controversy about the value of the MRTD.

For inter-band CA, 33us was defined for MRTD requirements in FR1 CA [4], and it included TAE (3us) and the propagation delay (30us) in FR1. For ATG FR1 inter-band CA, since only co-location scenario is considered, so the propagation delay can be 0, so technically speaking 3us may be enough.

However, since the normal UE can handle 33us MRTD in FR1 CA, considering that ATG UE has stronger capabilities, it is reasonable for ATG UE to handle larger MRTD compared to 3us. Therefore, we suggest considering relaxation on 3us, and whether it can be extended to 33us is related to specific UE types and UE capability. If the type of ATG UE with an antenna array on the bands of PCC and SCC and one common Rx beam assumed is considered in RRM requirements, extension to 33us may be challenging.

In our view, one possible option is to define a common value of MRTD for all types of ATG UEs, and the value of MRTD will be smaller than 33us. Another option is to define different values for different types of ATG UE, and 33us will be one of candidate values.

Observation 1: ATG UE has stronger capabilities compared to normal UEs.

Proposal 1: RAN4 to consider relaxation on 3us for MRTD requirements in FR1 inter-band CA.

Proposal 2: Whether MRTD can be extended to 33us is related to specific UE types and UE capability.

Proposal 3: If the type of ATG UE with an antenna array on the bands of PCC and SCC and one common Rx beam assumed is considered in RRM requirements,

- Option 1: Define a common value of MRTD for all types of ATG UEs, and the value of MRTD will be smaller than 33us.

- Option 2: Define different values for different types of ATG UE, and 33us will be one of candidate values, other candidate values are FFS.

Proposal 4: If the type of ATG UE with an antenna array on the bands of PCC and SCC and one common Rx beam assumed is not considered in RRM requirements, MRTD can be 33us for all types of ATG UEs.

  - 2.2 Signalling characteristics

- 2.2.1 Interruption requirements

Due to interruption requirements are only defined for the scenarios of CA and DC, they were not defined in R18 ATG. Since CA is considered in R19 ATG, at least the following interruption requirements will be defined for ATG:

- Interruptions at SCell addition/release

- Interruptions at SCell activation/deactivation

- Interruptions at direct SCell activation

- Interruptions at fast SCell activation

- Interruptions during measurements on deactivated SCC

- Interruptions due to SCell dormancy

- Interruptions due to Active BWP switching Requirement

- Interruptions due to UE-specific CBW change

- Interruptions when identifying CGI of an NR cell with autonomous gaps

The legacy requirements in TN can be reused for ATG CA as a baseline, RAN4 could further discuss the specific ATG related features.

Proposal 5: At least the following interruption requirements will be defined for R19 ATG:

- Interruptions at SCell addition/release

- Interruptions at SCell activation/deactivation

- Interruptions at direct SCell activation

- Interruptions at fast SCell activation

- Interruptions during measurements on deactivated SCC

- Interruptions due to SCell dormancy

- Interruptions due to Active BWP switching Requirement

- Interruptions due to UE-specific CBW change

- Interruptions when identifying CGI of an NR cell with autonomous gaps

Proposal 6: The existing interruption requirements in TN can be reused for R19 ATG CA as a baseline, RAN4 could further discuss the specific ATG related features.

- 2.2.2 SCell Addition/Activation mechanism

In the last meeting, the candidate proposal of SCell Addition/Activation mechanism is copied as below [3]:

- Proposal 1: Further consider distance triggered SCell addition and/or activation. (CMCC)

Technically speaking, we can understand the purpose of proposal 1. Considering that the specific scenario of R19 ATG such as co-located CA and LOS propagation conditions, the further optimization methods for SCell addition and activation can be considered, as the coverage areas of PCell and SCell are relatively fixed and SCell quality is related to the distance between UE and the center of cell. Omitting the measurements of SCell addition will achieve throughput gains.

However, we think that introducing distance triggered SCell addition and/or activation is complex and it will also involve RAN2’s work, from RAN4’s perspective, a better way is to define the corresponding enhanced requirements with some conditions, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted, and it depends on UE to apply there requirements.

Observation 2: The coverage areas of PCell and SCell are relatively fixed and SCell quality is related to the distance between UE and the center of cell, and omitting the measurements of SCell addition will achieve throughput gains.

Proposal 7: Considering that the specific scenario of R19 ATG such as co-located CA and LOS propagation conditions, the further optimization methods for SCell addition and activation can be considered

Observation 3: Introducing distance triggered SCell addition and/or activation is complex and it will also involve RAN2’s work.

Proposal 8: A better way is to define the corresponding enhanced requirements with some conditions, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted, and it depends on UE to apply these requirements.

- 2.2.3 Active BWP switch delay on multiple CCs

In the R18 ATG, the requirements of DCI and timer based BWP switch delay and RRC based BWP switch delay only apply to the case that the BWP switch is performed on the PCell with more than one BWP configurations configured. Since DL CA scenario has been considered for R19 ATG UE, so the DCI based, timer based and RRC based BWP switch delay on multiple CCs should be defined.

In legacy, both simultaneous and non-simultaneous DCI based/timer based/RRC based BWP switch delays on multiple CCs were defined in clause 8.6.2A/8.6.2B/8.6.3A in TS 38.133 [4]. However, since non-simultaneous DCI based and RRC based BWP switch delay on multiple CCs is defined for NR-DC and it is not in the scope of R19 ATG, so only simultaneous DCI based and RRC based BWP switch delay on multiple CCs need to be considered, and the legacy requirements can be reused as baseline. For timer based BWP switch delay on multiple CCs, both simultaneous and non-simultaneous related requirements should be defined.

Proposal 9: For active BWP switch delay on multiple CCs requirements for ATG UE, introduce the following requirements and the legacy can be reused as baseline:

- Simultaneous DCI based BWP switch delay on multiple CCs

- Simultaneous RRC based BWP switch delay on multiple CCs

- Simultaneous and non-simultaneous Timer based BWP switch delay on multiple CCs

- 2.2.4 Pre-configured measurement gap activation/deactivation delay upon SCell activation/deactivation

The candidate proposal on pre-configured measurement gap activation/deactivation delay upon SCell activation/deactivation in the last meeting is copied as follows [3]:

- Proposal 1: Introduce Pre-configured measurement gap activation/deactivation delay requirement upon SCell activation/deactivation, the legacy requirement can be reused.

The existing requirements of pre-configured measurement gap activation/deactivation delay only apply for an ATG UE configured with PCell in standalone NR. Considering that CA is introduced in R19 ATG, we support to introduce pre-configured measurement gap activation/deactivation delay requirement upon SCell activation/deactivation, and the legacy requirement can be reused.

Proposal 10: Introduce pre-configured measurement gap activation/deactivation delay requirement upon SCell activation/deactivation, the legacy requirement can be reused.

  - 2.3 Measurement procedure

- 2.3.1 Capabilities for Support of Event Triggering and Reporting Criteria

For capabilities for Support of Event Triggering and Reporting Criteria, considering that R19 ATG introduce CA, configured NR serving frequencies will include PCell and SCells carrier frequencies in the total number of NR reporting criteria for intra-frequency category when UE configured with SA operation mode.

Proposal 11: For capabilities for Support of Event Triggering and Reporting Criteria, configured NR serving frequencies will include PCell and SCells carrier frequencies in the total number of NR reporting criteria for intra-frequency category.

- 2.3.2 Pre-configured measurement gap requirement

Since CA has been introduced for R19 ATG, SCell related triggering conditions should be considered for network-controlled and autonomous activation/deactivation mechanism of pre-configured measurement gap, and the legacy requirement can be reused.

Proposal 12: SCell related triggering conditions should be considered for network-controlled and autonomous activation/deactivation mechanism of pre-configured measurement gap, and the legacy requirement can be reused.

- 2.3.3 SCell activation triggered measurement reporting requirement

In the last meeting, RAN4 discussed the requirement list of SCell Activation and Deactivation Delay for R19 ATG, and the agreements are copied as follows [3]:

Agreement: - SCell Activation and Deactivation Delay requirements should be introduced for ATG, reuse the current FR1 requirements as the starting point - SCell Activation Delay Requirement for Deactivated SCell - SCell Deactivation Delay Requirement for Activated SCell - Direct SCell Activation at SCell addition - Direct SCell Activation at Handover - Direct SCell Activation at RRC Resume - Fast SCell Activation Delay Requirement for Deactivated SCell - SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation - SSB-less SCell related requirement for intra-band CA and inter-band CA - Note: The appplicability of the requirement is up to UE capability supporting - Further discuss whether enhancement/update is needed considering band combinations, antenna types, and co-located deployment.

According to the above, since RAN4 agreed to define the requirements of SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation, so we think that the measurement reporting requirement for SCell activation Triggered Reporting shall also be introduced and the legacy requirements can be reused.

Proposal 13: The measurement reporting requirement for SCell activation Triggered Reporting shall be introduced and the legacy requirements can be reused.

- 2.3.4 Scheduling Availability/Restrictions

Considering that CA has been introduced for R19 ATG, the scheduling restriction extension to intra-band/inter-band CA should be identified, and it is highly related to UE antenna assumptions. The agreements of UE antenna type for CA in the last meeting are copied as follows [3]:

Agreement: - Refine the agreement from last meeting as: - For intra-band contiguous CA, same antenna type should be applied on a band. - For inter-band CA, either different or same antenna type can be applied on different bands. - For inter-band CA, consider the following cases: - 1. ATG UE with omnidirectional antennas on the bands of PCC and SCC. - 2. ATG UE with an omnidirectional antenna on the band of PCC and an antenna array on the band of SCC. - 3. ATG UE with an antenna array on the band of PCC and an omnidirectional antenna on the band of SCC. - [4. ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam assumed.] - 5. ATG UE with an antenna array on the bands of PCC and SCC, and two simultaneous separate Rx beams assumed.

When ATG UE with the antenna array and with omnidirectional antennas performing measurements on FR1 TDD band, for intra-band contiguous CA, the scheduling restrictions on transmitting PUCCH/PUSCH/SRS due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols. For inter-band CA, the scheduling restrictions on transmitting PUCCH/PUSCH/SRS due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols when simultaneousRxTxInterBandCA is not supported for the target measurement band and the serving cell’s band.

Proposal 14: The scheduling restriction extension to intra-band/inter-band CA should be identified for R19 ATG, and it is highly related to UE antenna assumptions.

Proposal 15: When ATG UE with the antenna array and omnidirectional antennas performing measurements on FR1 TDD band,

- For intra-band contiguous CA, the scheduling restrictions on transmitting PUCCH/PUSCH/SRS due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

- For inter-band CA, the scheduling restrictions on transmitting PUCCH/PUSCH/SRS due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols when simultaneousRxTxInterBandCA is not supported for the target measurement band and the serving cell’s band.

When ATG UE with the antenna array performing measurements on FR1, for intra-band contiguous CA, since RAN4 agreed that the same antenna type should be applied on a band, so the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols. For inter-band CA, for case 2 and case 3 where the UEs with independent antenna mode and case 5 where UE can form two separate Rx beams, the scheduling restrictions on receiving PDCCH/PDSCH/TRS/CSI-RS for CQI due to SCell/PCell will not apply to another serving cell in a different band. If case 4 is considered, ATG UE with the antenna array on both PCC and SCC will only form one Rx beam, the scheduling restrictions on receiving PDCCH/PDSCH/TRS/CSI-RS for CQI due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

Proposal 16: When ATG UE with the antenna array performing measurements on FR1,

- For intra-band contiguous CA, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

- For inter-band CA,

- For case 2, case 3 and case 5, the scheduling restrictions on receiving PDCCH/PDSCH/TRS/CSI-RS for CQI due to SCell/PCell will not apply to another serving cell in a different band.

- If case 4 is considered, the scheduling restrictions on receiving PDCCH/PDSCH/TRS/CSI-RS for CQI due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

- Conclusion

This document discussed preliminary issues for R19 ATG RRM requirements and presented following observations and proposals:

Observation 1: ATG UE has stronger capabilities compared to normal UEs.

Proposal 1: RAN4 to consider relaxation on 3us for MRTD requirements in FR1 inter-band CA.

Proposal 2: Whether MRTD can be extended to 33us is related to specific UE types and UE capability.

Proposal 3: If the type of ATG UE with an antenna array on the bands of PCC and SCC and one common Rx beam assumed is considered in RRM requirements,

- Option 1: Define a common value of MRTD for all types of ATG UEs, and the value of MRTD will be smaller than 33us.

- Option 2: Define different values for different types of ATG UE, and 33us will be one of candidate values, other candidate values are FFS.

Proposal 4: If the type of ATG UE with an antenna array on the bands of PCC and SCC and one common Rx beam assumed is not considered in RRM requirements, MRTD can be 33us for all types of ATG UEs.

Proposal 5: At least the following interruption requirements will be defined for R19 ATG:

- Interruptions at SCell addition/release

- Interruptions at SCell activation/deactivation

- Interruptions at direct SCell activation

- Interruptions at fast SCell activation

- Interruptions during measurements on deactivated SCC

- Interruptions due to SCell dormancy

- Interruptions due to Active BWP switching Requirement

- Interruptions due to UE-specific CBW change

- Interruptions when identifying CGI of an NR cell with autonomous gaps

Proposal 6: The existing interruption requirements in TN can be reused for R19 ATG CA as a baseline, RAN4 could further discuss the specific ATG related features.

Observation 2: The coverage areas of PCell and SCell are relatively fixed and SCell quality is related to the distance between UE and the center of cell, and omitting the measurements of SCell addition will achieve throughput gains.

Proposal 7: Considering that the specific scenario of R19 ATG such as co-located CA and LOS propagation conditions, the further optimization methods for SCell addition and activation can be considered

Observation 3: Introducing distance triggered SCell addition and/or activation is complex and it will also involve RAN2’s work.

Proposal 8: A better way is to define the corresponding enhanced requirements with some conditions, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted, and it depends on UE to apply these requirements.

Proposal 9: For active BWP switch delay on multiple CCs requirements for ATG UE, introduce the following requirements and the legacy can be reused as baseline:

- Simultaneous DCI based BWP switch delay on multiple CCs

- Simultaneous RRC based BWP switch delay on multiple CCs

- Simultaneous and non-simultaneous Timer based BWP switch delay on multiple CCs

Proposal 10: Introduce pre-configured measurement gap activation/deactivation delay requirement upon SCell activation/deactivation, the legacy requirement can be reused.

Proposal 11: For capabilities for Support of Event Triggering and Reporting Criteria, configured NR serving frequencies will include PCell and SCells carrier frequencies in the total number of NR reporting criteria for intra-frequency category.

Proposal 12: SCell related triggering conditions should be considered for network-controlled and autonomous activation/deactivation mechanism of pre-configured measurement gap, and the legacy requirement can be reused.

Proposal 13: The measurement reporting requirement for SCell activation Triggered Reporting shall be introduced and the legacy requirements can be reused.

Proposal 14: The scheduling restriction extension to intra-band/inter-band CA should be identified for R19 ATG, and it is highly related to UE antenna assumptions.

Proposal 15: When ATG UE with the antenna array and omnidirectional antennas performing measurements on FR1 TDD band,

- For intra-band contiguous CA, the scheduling restrictions on transmitting PUCCH/PUSCH/SRS due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

- For inter-band CA, the scheduling restrictions on transmitting PUCCH/PUSCH/SRS due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols when simultaneousRxTxInterBandCA is not supported for the target measurement band and the serving cell’s band.

Proposal 16: When ATG UE with the antenna array performing measurements on FR1,

- For intra-band contiguous CA, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

- For inter-band CA,

- For case 2, case 3 and case 5, the scheduling restrictions on receiving PDCCH/PDSCH/TRS/CSI-RS for CQI due to SCell/PCell will not apply to another serving cell in a different band.

- If case 4 is considered, the scheduling restrictions on receiving PDCCH/PDSCH/TRS/CSI-RS for CQI due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

- Reference

[1]	RP-240839, New WID on Enhancements for Air-to-ground network for NR, Apple

[2]	RP-242381, Revised WID on Enhancements for Air-to-ground network for NR, CMCC

[3]      R4-2416852, WF on RRM requirements for [112bis][208] NR_ATG_enh, CMCC

[4]      3GPP TS 38.133, NR Requirements for support of radio resource management, V18.7.0