3GPP TSG-RAN WG4 Meeting #113	R4-2417721

# Orlando , US, Nov 18 – 22, 2024

Title: 	Discussion on other RRM requirements for Rel-19 NTN phase3

Source: 	CATT

Agenda item:	7.26.4.2

Document for:	Discussion

- Introduction

A new RAN4 leading work item for Non-Terrestrial Networks (NTN) for NR Phase 3 was approved in RAN#102, and the WID have been revised in RAN#105 [1][2].

In the last meeting, the WF on RRM requirements for NR_NTN_Ph3_Part2 was approved [3]. This document will discuss other RRM requirements about NR NTN phase 3 and give our understanding and proposals.

- Discussion

2.1 Downlink coverage enhancements

- SSB periodicity enhancement

In the last meeting, RAN4 discussed the RRM impact for SSB periodicity enhancement and the agreements are copied as follows [3]:

Agreement: - RAN4 to wait for RAN1’s definitive decision on the maximum periodicity of SSB and the range of SMTC before discussing whether and how to modify the relevant RRM requirements. FFS on the detailed list of RRM requirements to be affected, e.g., - uplink timing requirements, RRM requirements during transmission between different SSB periodicities, default assumption on the SSB and SMTC periodicities in handover/satellite switch with re-sync/RRC re-establishment/RRC connection release with redirection, cell identification, measurements, etc.

This issue is related to RAN1’s conclusions. In RAN1 #118-bis meeting, the agreements on SSB periodicity for NR-NTN downlink coverage enhancement are copied as follows:

Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

According to the above, RAN1 has agreed the maximum of additional default value of SSB periodicity during initial access is at least 160 ms, and whether 320ms can be supported as the maximum of the additional default value was FFS.

From RRM requirements perspective, if the maximum of  SSB periodicity is 160ms, RRM requirements may not be affected. If the maximum of SSB periodicity can be 320ms, RAN4 need to further discuss the detailed RRM impact, e.g., uplink timing requirements and the aspects of DRX cycle, SMTC, and MGRP.

Observation 1: RAN1 agreed the maximum of additional default value of SSB periodicity during initial access is at least 160 ms, and whether 320ms can be supported as the maximum of the additional default value was FFS.

Proposal 1: RAN4 to wait for RAN1’s conclusion on the maximum periodicity of SSB and the range of SMTC before discussing whether and how to modify the relevant RRM requirements.

- If the maximum of SSB periodicity is 160ms, RRM requirements may not be affected.

- If the maximum of SSB periodicity can be 320ms, RAN4 need to further discuss the detailed RRM impact, e.g., uplink timing requirements and the aspects of DRX cycle, SMTC, and MGRP.

2.2 Regenerative payload

In the last meeting, RAN4 discussed some issues for regenerative payload and the agreements are copied as follows [3]:

### Issue 2-1: Uplink timing requirements Agreement: - No consensus on whether there is an issue due to non-negative value for common TA or whether negative values should be introduced for ta-common. - Note: No more discussion of this issue is expected in RAN4. Agreement: - No UE uplink timing requirement needs to be updated for regenerative payload NTN systems. If any necessary clarification is identified, it can be added accordingly. ### Issue 2-2: Mobility requirements Agreement: - For those requirements not applicable for regenerative payload NTN systems, if any, RAN4 to add an explicit statement to TS38.133 to clarify their applicability to regenerative payload NTN systems. FFS on the exact text and clause(s) for such clarifications. ### Issue 2-3: Other RRM impacts Agreement: - The conclusion on Issue 2-2 can be applied to the RRM requirements for network-verified UE positioning. No separate discussion is necessary.

This issue is related to the progress from other groups. In RAN2 #127-bis meeting, RAN2 confirmed that they will not further consider possible optimization. Therefore, at least for now, RAN4 does not need to discuss any RRM requirements for regenerative payload and all of existing NTN requirements can apply to regenerative payload. If some RRM impacts are needed in the future, RAN4 can revisit existing RRM requirements.

However, we believe that the applicability clarification for NTN requirements is needed in section 3.6.12 of TS 38.133 [4], such as all of the requirements defined for Satellite Access apply to both transparent and regenerative payload scenarios.

Observation 2: RAN2 confirmed that they will not further consider possible optimization in RAN2 #127-bis meeting.

Proposal 2: At least for now, RAN4 does not need to discuss any RRM requirements for regenerative payload and all of existing NTN requirements can apply to regenerative payload.

- If some RRM impacts are needed in the future, RAN4 can revisit existing RRM requirements.

Proposal 3: The applicability clarification for NTN requirements is needed in section 3.6.12, and the wording can be:

- The requirements defined for Satellite Access apply to both transparent and regenerative payload scenarios.

- Conclusion

This document discussed the issues about other RRM requirements for NR NTN Phase 3 and the following observations and proposals are presented:

Observation 1: RAN1 agreed the maximum of additional default value of SSB periodicity during initial access is at least 160 ms, and whether 320ms can be supported as the maximum of the additional default value was FFS.

Proposal 1: RAN4 to wait for RAN1’s conclusion on the maximum periodicity of SSB and the range of SMTC before discussing whether and how to modify the relevant RRM requirements.

- If the maximum of SSB periodicity is 160ms, RRM requirements may not be affected.

- If the maximum of SSB periodicity can be 320ms, RAN4 need to further discuss the detailed RRM impact, e.g., uplink timing requirements and the aspects of DRX cycle, SMTC, and MGRP.

Observation 2: RAN2 confirmed that they will not further consider possible optimization in RAN2 #127-bis meeting.

Proposal 2: At least for now, RAN4 does not need to discuss any RRM requirements for regenerative payload and all of existing NTN requirements can apply to regenerative payload.

- If some RRM impacts are needed in the future, RAN4 can revisit existing RRM requirements.

Proposal 3: The applicability clarification for NTN requirements is needed in section 3.6.12, and the wording can be:

- The requirements defined for Satellite Access apply to both transparent and regenerative payload scenarios.

- Reference

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei

- RP-241789 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- R4-2416866, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)

- 3GPP TS 38.133, NR Requirements for support of radio resource management