3GPP RAN WG4 Meeting #113		R4-2417741

Orlando, USA, 18th – 22nd November 2024

Agenda item:	7.2.4.2

Source:	Apple

Title:	On remaining UE IMT parameters for the 14800-15350MHz frequency range

WI/SI:	FS_NR_IMT_4400_7125_14800MHz

Release:	Rel-19

Document for:	Discussion

1	Introduction

The WRC-23 conference identified three frequency ranges as potential IMT candidates and tasked ITU-R WP5D to conduct the corresponding sharing and compatibility studies. In turn, the ITU-R WP5D sent an LS to several bodies, including 3GPP, asking to provide technical parameters for the candidate frequencies [1]. In response to that LS from WP5D and based on the technical input from RAN WG4 [2], the 3GPP RAN#103 meeting approved a new SI [3], purpose of which will be to study and provide the corresponding IMT technical parameters.

According to the WRC-23 resolution and the corresponding LS from ITU-R WP5D, there are three frequency ranges, for which RAN WG4 is tasked to provide the IMT-2030 parameters: 4400-4800MHz, 7125-8400MHz and 14800-15350MHz.  During the RAN4#111 meeting an initial discussion took place for the 14800-15300MHz range and RAN WG4 made several general assumptions on which options can be considered further. During the previous RAN WG4#112bis meeting several agreements were made for the 15GHz UE IMT parameters, but there are remaining open issues, such as channel bandwidth, emissions, etc. Thus, in this discussion paper we present our further views on the technical parameters for the 14800-15300MHz range, and especially for the UE RF antenna parameters, accounting for the previous work that 3GPP has done for e.g. 10-10.5GHz frequency range [4].

2	UE IMT parameters for 15GHz

## 2.1	General considerations

The 14800-15350MHz frequency range is a completely new band for 3GPP residing between the FR1 and FR2 ranges. 3GPP conducted in the past the study on IMT technical parameters for the 10-10.5GHz range, results of which can be found in TR 38.921. Based on the conclusions from the previous RAN WG4 meeting there are several open issues for the 15GHz UE IMT parameters, further considerations for which are presented below.

-	Channel bandwidth. The FR1 maximum single carrier channel bandwidth is 100MHz. In the previous 3GPP studies for the 10-10.5GHz range RAN WG4 already indicated that 100MHz can be assumed as a typical channel bandwidth for the co-existence analysis. From that perspective we believe that 100MHz can be also considered as a typical channel bandwidth for 14.8-15.3GHz. Even though we consider a new chunk of spectrum at a higher frequency range, its total size is 500MHz. Accounting for the fact that there are usually three/four operators in a country, we cannot see compelling reasons to assume that e.g. 200MHz can be regarded as a "typical" channel bandwidth. This of course does not preclude larger channel bandwidths that can be further discussed when 3GPP starts devising the corresponding system parameters for this range.

-	Signal bandwidth. The signal bandwidth can be calculated according to the following formula, NRB × SCS × 12, where the corresponding number of RBs (NRB) can be taken from TS 38.101-1.

-	Spurious emissions. According to the international regulations captured in ITU-R SM.329-13, there are several categories for the spurious emissions, whereupon Category A represents more relaxed requirements available in certain countries (e.g. US with -13dBm/MHz for 23dBm Tx power) and Category B corresponds to more stringent requirements (-30dBm/MHz) required for instance in EU/CEPT region. Since the considered frequence range is a potential IMT candidate in all ITU regions, our view is that we must adopt the ITU-R Category B spurious emission requirement of -30dBm/MHz.

-	Spectral mask. The generic 3GPP FR1 spectral mask with the general requirements for the out-of-band emissions can be also applicable to 14.8-15.3GHz range. Its floor value is -25dBm/MHz which logically matches the -30dBm/MHz requirement for the spurious emissions.

-	Noise figure. Referring to TR 38.921, "The noise figure of 9 dB is the baseline assumption and 13 dB is the optional assumption for co-existence study for 6425 – 7125 MHz and 10-10.5GHz". Furthermore, a noise figure of 13dB was finally agreed in the LS response to ITU-R WP5D for the 7-8GHz range. From that perspective the 14.8-15.35GHz range should have the noise figure of at least 13dB and might have even a higher noise figure of 14dB.

-	Sensitivity. The exact value for the UE sensitivity, or REFSENS, can be determined only when 3GPP assumes on how many Rx chains are used. However, this parameter is not strictly needed for the WP5D co-existence studies. During the previous studies this parameter was marked "to be specified" (see Annex A). It was also noted in the LS that "These are parameters pending detailed specification work that would not impact sharing and compatibility studies".

-	Blocking response. The 3GPP in-band and out-of-band blocking requirements from high-frequency bands, such as n79 and n104, can be applied also to this range.

-	ACS. Referring to sub-clause 7.2.4 in TR 38.921, the ACS value of 31dBc is assumed for 10-10.5 GHz. Since the considered range of 14.8-15.35GHz has even higher frequency, it can be studied further whether to proceed with 31dBc or whether even a smaller ACS value can be assumed. According to the co-existence simulation results 27-30dB ACS seems to be sufficient [6].

-	ACLR. According to the previous 3GPP studies in TR 38.921, sub-clause 7.1.3, it was agreed to consider 24dB ACLR for the 10-10.5GHz range. And according to the co-existence simulation results even a more relaxed ACLR of 22-24dB can be considered for this range [6]s.

As a summary of our considerations:

Proposal 1:	The typical channel bandwidth is 100MHz not precluding larger channel bandwidths.

Proposal 2:	UE spurious emissions follow the ITU-R Category B requirement of -30dBm/MHz (i.e. same requirement as FR1).

Proposal 3:	UE spectral mask is the same as FR1 mask.

Proposal 4:	UE noise figure is 13-14dB

Proposal 5:	UE ACLR is 22-24dB.

Proposal 6:	UE ACS is 27-30dB.

## 2.2	Summary of the UE IMT parameters

Table 2-1: IMT parameters for 14800-15350MHz (UE only)

| No. | Parameter | Base station (non-AAS) | Base station (AAS) | Mobile station |
| --- | --- | --- | --- | --- |
| 1 | Duplex Method |  |  | TDD |
| 2 | Channel bandwidth (MHz) |  |  | 100MHz (larger channel bandwidths are not precluded) |
| 3 | Signal bandwidth (MHz) |  |  | Derived from Channel Bandwidth: Signal bandwidth = NRB × SCS × 12 |
| 4 | Transmitter characteristics |  |  |  |
| 4.1 | Power dynamic range (dB) |  |  | 53dB |
| 4.2 | Spectral mask (dB) |  |  | See TS 38.101-1, sub-clause 6.5.2.2, Table 6.5.2.2-1. |
| 4.3 | ACLR (dB) |  |  | 22-24dB |
| 4.4 | Spurious emissions /out of band emissions |  |  | See TS 38.101-1, sub-clause 6.5.3.1 |
| 4.5 | Maximum output power (dBm) |  |  | 23dBm |
| 5 | Receiver characteristics |  |  |  |
| 5.1 | Noise figure (dB) |  |  | 13-14dB |
| 5.2 | Sensitivity (dBm) |  |  | To be specified |
| 5.3 | Blocking response |  |  |  |
| 5.4 | ACS |  |  | 27-30dB |
| 5.5 | SINR operating range (dB) |  |  | DL: -10..30 dB UL: -10..22 dB |

# 3	Conclusions

In this discussion paper we have presented our further considerations on the UE IMT technical parameters for the 14800-15350MHz range as tasked by ITU-R WP5D. Accounting for the existing studies for the 10-10.5GHz range and further simulations conducted specifically for the 14.8-15.35GHz range, we suggest the following UE IMT parameters

Proposal 1:	The typical channel bandwidth is 100MHz not precluding larger channel bandwidths.

Proposal 2:	UE spurious emissions follow the ITU-R Category B requirement of -30dBm/MHz (i.e. same requirement as FR1).

Proposal 3:	UE spectral mask is the same as FR1 mask.

Proposal 4:	UE noise figure is 13-14dB

Proposal 5:	UE ACLR is 22-24dB.

Proposal 6:	UE ACS is 27-30dB.

# 4	References

RP-240024, "LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4400-4800 MHz, 7125-8400 MHz and 14.8-15.35 GHz", ITU-R WP5D

R4-2102840, "LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (6.425 to 10.5 GHz)", RAN WG4.

RP-240765, "Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz"

3GPP TR 38.921, "Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz", v17.1.0

R4-2411143, "TP for 38.922 on UE antenna parameters for 14800-15350MHz", Apple

R4-2417740, "Co-existence simulation results for the 14800-15350MHz frequency range", Apple

Annex A: IMT-2020 technology-related and deployment-related parameters for bands between in 6425 to 7125 MHz and 10 to 10.5 GHz

TABLE 3A

IMT-2020 technology related parameters in 6425 – 7125 MHz and 10 – 10.5 GHz

|  |  | IMT |  |
| --- | --- | --- | --- |
| No. | Parameter | Base station (AAS) | Mobile station |
| 1 | Duplex Method | TDD | TDD |
| 2 | Channel bandwidth (MHz) | 100 MHz (typical) | 100 MHz (typical) |
| 3 | Signal bandwidth (MHz) | To be specified. Will be derived from Channel Bandwidth, see [1], § 5.3.2. | To be specified. Will be derived from Channel Bandwidth, see [2], § 5.3.2. |
| 4 | Transmitter characteristics |  |  |
| 4.1 | Power dynamic range (dB) | 0 dB | 56 dB |
| 4.2 | Spectral mask (dB) | Category A: (Note 1) See table 3B (Wide Area BS) (ΔfOBUE = 100 MHz) Category B: (Note 1) See table 3C (Wide Area BS) (ΔfOBUE = 100 MHz) | See Table 3D |
| 4.3 | ACLR (dB) | @6425 – 7125 MHz: 38 dB @10-10.5 GHz: 37 dB | @6425 – 7125 MHz: 26 dB @10-10.5 GHz: 24 dB |
| 4.4 | Spurious emissions | Category A: (Note 1) See [1], § 6.6.5, Table 6.6.5.2.1-1. Category B: (Note 1) @6425 – 7125 MHz See [1], § 6.6.5, Table 6.6.5.2.1-2. @10 – 10.5 GHz: See Table 3E | See [2], § 6.5.3. |
| 4.5 | Maximum/typical output power (dBm) | Defined by the conducted power per antenna element, see entry 1.9 in Table 4 for typical values. | 23 dBm |
| 5 | Receiver characteristics |  |  |
| 5.1 | Noise figure (dB) | @6425 – 7125 MHz: 6 dB (Wide Area BS) 11 dB (Medium Range BS) 14 dB (Local Area BS) @10-10.5 GHz 7 dB (Wide area BS) 12 dB (Medium Range BS) 15 dB (Local Area BS) For BS class definitions, see [1], § 4.4 | [9-13] dB |
| 5.2 | Sensitivity (dBm) | To be specified | To be specified |
| 5.3 | Blocking response | In-band blocking level: -43 dBm (Wide Area BS) -38 dBm (Medium Range BS) -35 dBm (Local Area BS) Interferer type: 20 MHz DFT-S-OFDM NR signal, 15 kHz SCS, 100 RB. Out-of-band blocking level: -15 dBm, Interferer type: CW ΔfOOB = 100 MHz (Note 2) | See [2], §7.6, Tables 7.6.2-4 and 7.6.3-4 |
| 5.4 | ACS | @6425 – 7125 MHz: 42 dB @10-10.5 GHz: 40 dB | @6425 – 7125 MHz: 32 dB @10-10.5 GHz: 31 dB |
| 5.5 | SINR operating range (dB) | See “SINR operating range and mapping function” in Annex 1. |  |