3GPP RAN WG4 Meeting #113s																	R4-2417753

Orlando, USA, 18th – 22nd November 2024

Agenda item:	6.17.2

Source:	Apple

Title:	On UE power back-off while operating in 2000-2020 MHz UL range of the NTN band n252

WI/SI:	NR_NTN_Sband-Core

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already at that time it was noted that there exist other satellite deployments. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI. For instance, after the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band (band n254) where the UL is on the L-band and the DL is on the S-band [2]. So-called extended L-band was also completed in Rel-18.

And after the RAN#104 meeting another spectrum WI was agreed for a satellite band operating in the S-band frequency range [3]. The UL block of a new NTN band covers the frequency range of 2000-2020MHz, which is close to the DL of another terrestrial band n25. And as discussed during the previous RAN4#112 meeting, there are some concerns on the potential impact from UL transmission on a new NTN band n252 to the DL receivers operating in the terrestrial band n25. Consequently, there were also concerns on the potential power back-off that band n252 UE may apply to meet existing requirements to protect band n25 receivers. RAN4#112bis concluded [5] to consider an option to "Define a new more relaxed spurious emission requirement for TN-NTN UE to UE coexistence compared to -50dBm/MHz ". To facilitate further technical discussions on potential trade-offs between the relaxed -50dBm/MHz requirements and required power back-off, we conduct a set of additional simulations accompanied by our preliminary considerations on anticipated power back-off values for different channel bandwidths and UL allocationss.

# 2	UE power back-off values

## 2.1	Power back-off simulations

Here we present power back-off results for the NTN band n252 with the following common assumptions:

-	Tx power: PC3 (+23dBm)

-	LO placement: always in the centre of the carrier

-	Additional requirements: -50dBm/MHz protection criterion for band n25

Figure 2.1-1 below presents the simulation results for the 5MHz channel, which is placed in the following locations within the band: left edge of the band (Fc=2002.5MHz) and two other centre frequencies shifted further away from the left edge (Fc=2007.5MHz and Fc=2012.5MHz).  For the sake of brevity, we present only DFT-s-OFDM results because as indicated in our previous studies CP-OFDM has even higher power back-off values. So, referring to Figure 2.1-1 the following general observations could be made for the 5MHz channel:

-	If the 5MHz channel is right at the left edge of the band, then large UL allocations may need large power back-off of up to 11dB. On the other hand, if we consider small UL allocations, which are likely to be the case for the NTN channels, then required power back-off is rather marginal. There is however a region constrained by RBstart=5 and LCRB=7 where even small UL allocations might have a noticeable power back-off.

-	If the 5MHz channel is shifted farther away from the band lower edge, then required power back-off reduces dramatically. Only large UL allocations may need power back-off, whereas small UL allocations either do not need any power back-off or the back-off stays within the limits of 0.5dB.

Figure 2.1-1: Power back-off values for the 5MHz channel with DFT-s-OFDM

Figure 2.1-2 presents power back-off values for the 10MHz channel considering three locations of the channel: right at the left edge of the band (Fc=2005), shifted to the right so that the offset from the band edge is 5MHz (Fc=2010), and with the offset of 10MHz (Fc=2015). The following general observations can be made:

-	For the first and the second case, i.e. when the 10MHz channel resides at the left edge of the band and is offset by 5MHz, large power back-off is needed even for small UL allocations.

-	If the 10MHz channel is offset by the 10MHz gap from the left edge, i.e. it resides at the right edge of the band, then most small UL allocations will experience either no or only marginal backoff (except several allocations at the left edge of the channel).

Figure 2.1-2: Power back-off values for the 10MHz channel with DFT-s-OFDM

Next, Figure 2.1-3 presents power back-off values for the 20MHz channel, which is the largest channel that can be configured in this band. So, there is only one scenario with the 20MHz channel placed within a new NTN band n252. Referring to the presented results, like for other NTN bands that have stringent out-of-band emission requirements, large allocations and/or small(-er) allocations at the edge of the band will require quite noticeable power back-off values.

Figure 2.1-3: Power back-off values for the 20MHz channel with DFT-s-OFDM

## 2.2	Further considerations

As discussed during the last RAN WG4, one way to avoid too large power back-off values is to consider trade-off(s) between relaxed -50dBm/MHz requirements in conjunction with smaller UL channel bandwidths and/or UL allocations. Below we present our initial considerations on potential options that can explored further by companies.

NOTE:	The considerations below take a simple assumption of 1:1 scaling of the relaxed protection requirement and required power back-off. However, this relationship is not always linear and thus additional power back-off simulations will be needed to cross-check actual power back-off values for a new protection limit.

Depending on target band n25 protection limit, the following observations can be made:

-	-50dBm/MHz. All 5MHz channels do not need extra power back-off for small UL allocations, except the left-most 5MHz channel. Thus, one potential configuration for this protection limit is to consider only 5MHz channels in UL with limited UL allocations. Another option is to consider e.g. 5+5+10MHz UL configuration because the 10MHz channel at the right edge of the band does not require large back-off for small allocations.

-	-45dBm/MHz. For this protection limit the same considerations as above will apply, with the only big noticeable difference is that for the 5+5+10MHz configuration none of the small UL allocations should require any extra power back-off.

-	-40dBm/MHz. With this protection limit even large UL allocations will not need extra power back-off at least for the UL 5+5+10MHz configuration. It is in principle possible to consider also the 10+10MHz UL configuration, but as can be seen from the existing results the left-most 10MHz channel still might require extra power back-off for some small UL allocations.

-	-35dBm/MHz. At this protection limit we anticipate no power back-off for small allocations at least for the 10+10MHz UL configuration. In fact, even large allocations should be feasible for the right-most 10MHz channel.

# 3	Conclusions

In this discussion paper we presented power back-off simulation results for a UE operating in the UL range of 2000-2020 MHz, i.e. NTN band n252 UL, in presence of the existing requirements to protect band n25 DL. These results can be used for further technical discussions on finding a suitable trade-off between the relaxed protection requirement and smaller UL channel bandwidths/allocations.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

RP-241661, "New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)", Dish Network, EchoStar, TerreStar, OmniSpace, Telus

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"

- R4-2417206, "WF on NTN and IoT-NTN bands", Moderator (Inmarsat)