3GPP RAN TSG-RAN4 Meeting # 113		R4-2417755

Orlando, USA, November 18 – 22, 2024

Agenda item:	7.1.1.3.4

Source:	Apple

Title:	Views on SRS AS IL Imbalance Solution for 6Rx

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction and Scope

In the previous RAN4 meeting (Meeting #112-Bis, Hefei, China, October 14 –18, 2024), discussions were held on the requirements for 6Rx for handheld and FWA, which are parts of the new  Release 19 WID on RF enhancements for NR FR1/FR2 and EN-DC [1]. As captured in the meeting WF [2], one of the 6Rx aspects that  were greatly debated is the SRS insertion loss imbalance solution. A summary of the decision made on this aspect is highlighted in the box below where potential candidate solutions to address the SRS IL imbalance issue are proposed.

## Sub-topic 4-2: SRS IL imbalance issue solutions Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: Companies are encouraged to provide comments and indication of support for the solution proposals listed below by RAN4#113. Proponent companies (in parentheses) for the candidate solution and solution proposals below are encouraged to provide expected benefit/performance gains and draftCRs so that the impacts to RAN1, RAN2, and RAN4 specifications (for RAN1 and RAN2, a description of the necessary changes is sufficient if draftCRs are not possible) can be further discussed. Other proposals including expected performance gains and draftCRs are not precluded. - Candidate solutions - Option 1: - UE performs self-compensation of SRS IL up to its maximum Tx power capabilities - UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g. - Per SRS resource power headroom reporting - Configured maximum output power per SRS resource reporting - SRS insertion loss value reporting (per SRS resource or per UE) - Other - Details of SRS IL self-compensation and assistance framework are FFS - Other options are not precluded

In this discussion paper, we will share our views on the proposed options from the last meeting for the SRS AS IL imbalance candidate solution.

# Discussion

  - SRS Insertion Loss Imbalance Solution

The SRS insertion loss imbalance issue has been highly debated in RAN4 and RAN1 meetings since the introduction of the 8Rx WID in Rel-18. Based on simulation results from several companies, it was recognized that, if this issue is not addressed, there could be a potential performance degradation in user throughput [4-9]. In RAN4#106 meeting (Athens, Greece, February 27 – March 3, 2023), an LS [3] from RAN4 was sent to RAN1 describing the SRS IL imbalance issue for 8Rx and its potential impact on inaccurate channel estimation at the gNB receiver, which could lead to an incorrect DL CSI estimation. In response to the RAN4 LS, a lot of companies provided simulation results to highlight the potential impacts of IL imbalance on the degradation of overall system performance. Some of those simulation results taken from topic summary [4] are shown in the figures below. It can be observed from the figures that SRS IL imbalance, without any reporting or compensation, can result in a non-negligible performance loss for both SU-MIMO and MU-MIMO.

Observation #1: During 8Rx discussions, based on simulation results from several companies, it was recognized that, if the SRS IL imbalance issue is not addressed, there could be a potential performance degradation in user throughput.

Figure 2.1-1:	Mean and cell-edge MU-MIMO user throughput with/without SRS IL imbalance [4]

Figure 2.1-2:	Mean and cell-edge MU-MIMO user throughput with/without SRS IL imbalance [4]

Although the majority of the companies saw the need for a solution to address the SRS AS IL imbalance issue, there was no agreement on a candidate solution during the 8Rx discussions. One of the objectives of the Rel-19 6Rx WID was then to continue the discussions on this issue and specify, if possible, a candidate solution to alleviate its negative impacts on the UE system performance. Over the past RAN4 meetings, several discussions have been held on the issue with no clear solution identified yet. Most of the companies have proposed so far solutions centered around UE power self-compensation and IL imbalance reporting to the NW.

For UE self-compensation, RAN4 identified in Meeting # 110-Bis three cases that determine the effectiveness of applying the compensation [9]:

- Case 1(non near max Tx power): In this scenario, the UE is not power-limited and should be able to compensate for the IL loss imbalance across the SRS ports if the PA headroom is higher than the maximum IL imbalance.

- Case 2 (near max Tx power): For this case, the UE is close to be power-limited so performing a reliable compensation for this case could be a challenge because there could be not enough PA headroom remaining. However, this would depend on UE implementation.

- Case 3 (max Tx power): The UE is power-limited in these conditions and would probably not be do power compensation.  the UE is close to be power-limited so performing a reliable compensation for this case could be a challenge because there could be not enough PA headroom remaining.

SRS AS IL imbalance reporting has also been considered as a solution. In this case, the UE provides some assistance to the network ( either statically or dynamically) on SRS IL imbalance and the network performs compensation. UE power headroom, UE maximum configured power, and SRS insertion loss imbalances have been proposed by several companies as the metrics to report to the network.

In RAN4 Meeting #112 (Maastricht, Netherlands, August 19-23, 2024), we proposed a hybrid solution based on both UE self-compensation and reporting where the UE always compensates when it is capable to do so in a non-power limited situations and does some reporting to the network when the power at an SRS port is above certain threshold [11].

Option 1 highlighted in the WF as shown above is in line with our previous proposal.

Proposal #1: For a candidate solution to address the SRS IL imbalance issue, our preference is Option 1 highlighted in the previous meeting WF. However, even if a solution is ultimately found, we recommend that this feature remains optional and should depend on UE implementation, given its potential implementation challenges.

# 3	Conclusions

Observation #1: During 8Rx discussions, based on simulation results from several companies, it was recognized that, if the SRS IL imbalance issue is not addressed, there could be a potential performance degradation in user throughput.

Proposal #1: For a candidate solution to address the SRS IL imbalance issue, our preference is Option 1 highlighted in the previous meeting WF. However, even if a solution is ultimately found, we recommend that this feature remains optional and should depend on UE implementation, given its potential implementation challenges.

# 4	References

RP-240828, “New WID: UE RF enhancements for FR1/FR2 and EN-DC , Phase 4”, by RAN4 chair (Huawei),  3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

R4-2417209, “WF on requirements for 6Rx”, by AT&T, 3GPP TSG-RAN WG4 Meeting #112Bis, Hefei, China, October 14-18, 2024

R4-2303519, “LS on the UE SRS IL imbalance issue”, 3GPP TSG-RAN WG4 Meeting #106, Athens, Greece, February 27 – March 3, 2023

R1-2306200, “FL summary on UE SRS IL imbalance issue”, 3GPP TSG RAN WG1 #112bis-e, Incheon, Korea, May 22nd – May 26th, 2023

R1-2304643, “Discussion on the UE SRS IL imbalance issue”, by Huawei, HiSilicon, 3GPP TSG-RAN WG1 Meeting #113,  Incheon, Korea, 22 – 26 May 2023

R1-2304433, “On SRS Imbalance”, by InterDigital, Inc., 3GPP TSG-RAN WG1 Meeting #113,  Incheon, Korea, 22 – 26 May 2023

R1-2305646, “Evaluation of SRS antenna switching with insertion loss”, by MediaTek, 3GPP TSG-RAN WG1 Meeting #113,  Incheon, Korea, 22 – 26 May 2023

R4-2300696, “8RX UE RF requirements”, by Qualcomm Inc., 3GPP TSG-RAN WG4#106 Athens, Greece, February 27th ‒ March 3rd, 2023

R1-2305838, “Discussion the UE SRS IL imbalance issue”, by vivo, 3GPP TSG-RAN WG1 Meeting #113,  Incheon, Korea, 22 – 26 May 2023

R4-2406585, “WF on requirements for 6Rx”, by AT&T, 3GPP TSG-RAN WG4 Meeting #110-bis,  Changsha, China, 15th Apr 2024 - 19th Apr 2024

R4-2411151, “On SRS IL imbalance issue for 6Rx”, by Apple, 3GPP RAN TSG-RAN4 Meeting # 112, Maastricht, NL, August 19 – 23, 2024

R4-2411883, “Further discussion on 6Rx Reference sensitivity requirements”, by ZTE Corporation, Sanechips,  3GPP TSG-RAN WG4 Meeting #110-bis,  Changsha, China, 15th Apr 2024 - 19th Apr 2024