3GPP RAN WG4 Meeting #113																	R4-2417759

Orlando, USA, 18th – 22nd November 2024

Agenda item:	7.23.2.2

Source:	Apple

Title:	On LP-WUR NF and REFSENS

WI/SI:	NR_LPWUS-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

LP-WUS/WUR shall be introduced to NR in Rel-19. The objectives are covered in the WID [1]. This contribution focuses on the RAN4 core requirements and the way forward [2] from last meeting. The WID lists RAN4 goals as:

# 2  Discussion

##### 2.1 On noise figure for WUR types

Noise figure and SNR are critical parameters influencing the RF hardware design and network performance. To achieve low noise figure the LNA is typically one of the first components in the receiver chain. Designing a LNA with low power consumption and low noise is immensely challenging. To achieve high linearity and high gain the LNA requires more stages translating into more power consumption. To effectively reduce the power consumption the gain and noise performance needs to be relaxed. RAN4 has held initial discussions on noise figure and captured the following in the WF [2]:

Issue 2-2-3: REFSENS, NF, IM, SNR aspects for FR1 OOK-based and OFDM-based LR Agreements: - Collect input on REFSENS with values for each element（NF, IM, SNR） in the table next meeting. The value for each element can be discussed. - Encourage companies to also input the justification for each element.

While typical NR FR1 receiver feature a noise figure of approximately 9 dB this value is expected to increase for LP-WUR. An OFDM type receiver is expected to re-use existing RF architecture from the main receiver. The battery lifetime of the device is enhanced by tweaking the LNA biasing for reduced current consumption. This decreases the noise figure performance. Therefore, a delta to the typical 9dB of the main receiver is required. We conducted measurements using an OFDM type WUR receiver and found that a delta of 7dB would be required to satisfy the expected current consumption improvements. Regarding implementation margin we propose to set the value no lower than 2.5dB.

Proposal: To achieve high linearity and high gain the LNA requires more stages translating in more power consumption. To effectively reduce the power consumption gain and noise performance needs to be reduced. While typical NR FR1 receiver feature a noise figure of approximately 9 dB this value is expected to increase considerably for LP-WUR. Measurements indicate that approximately 7dB delta to main receiver is required to satisfy the expected current consumption improvements. Regarding implementation margin we propose to set the value no lower than 2.5dB.

# 4	Conclusions

This contribution provides a discussion based on last meetings way forward on LP-WUR. The following observations and proposals are made:

Proposal: To achieve high linearity and high gain the LNA requires more stages translating in more power consumption. To effectively reduce the power consumption gain and noise performance needs to be reduced. While typical NR FR1 receiver feature a noise figure of approximately 9 dB this value is expected to increase considerably for LP-WUR. Measurements indicate that approximately 7dB delta to main receiver is required to satisfy the expected current consumption improvements. Regarding implementation margin we propose to set the value no lower than 2.5dB.

# 4	References

- RP-240801, “Low-power Wake-up Signal and Receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony, 3GPP TSG RAN Meeting #103

- R4-2417112, “WF on UE RF requirements for LP-WUS”, vivo, 3GPP TSG-RAN WG4 Meeting #112bis