3GPP RAN WG4 Meeting #113		R4-2417761

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	7.1.1.1.1

Source:	Apple

Title:	On Rel-19 HPUE CA/DC SAR compliance consideration

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The Rel-19 RAN4 non-spectrum WID/SID package has been approved in RAN #103 meeting. In consideration that the uplink coverage is crucial for 5G and 5G-adavanced roll-out and the continued strong interests from operators in higher UE maximum output power such as PC1.5 for their band combinations, one of the key objectives under the new Rel-19 “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” work item [1] is to specify the generic requirements of HPUE for NR uplink CA in FR1 and EN-DC with NR FR1 bands where the features include,

    - PC1.5 for intra-band contiguous UL CA with and without UL MIMO

    - PC1.5 Intra-band non-contiguous UL CA without UL MIMO

    - PC1.5 2-band inter-band UL CA with 2Tx or 3Tx for handheld UE and FWA

    - PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx for handheld UE and FWA UE

    - Increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band UL CA and EN-DC HPUE

In last RAN4 meeting, companies had provided their further views on the respective features above where the additional agreements were captured in the approved “WF on HPUE RF requirements” [2]. On the SAR compliance aspect, it was agreed that duty cycle solution for SAR will be discussed for PC1.5 intra-band and inter-band UL CA, and for PC1.5 and PC2 EN-DC, duty cycle solution for SAR will be discussed for the scenario without new IE introduction.

In this contribution, we share our views on the SAR compliance aspect for HPUE CA/DC combinations and propose not to further consider UL duty-cycle with ΔPPowerClass for SAR mitigation from Rel-19 and going forward.

# Discussion

SAR (Specific Absorption Rate) is a measure of the rate of RF energy absorption by the body from the source being measured, specifically a handheld UE from 3GPP perspective. SAR limit is a regulatory requirement which is to ensure the human exposure to RF energy is below the safety level while using the device. The term “SAR” however has never been officially defined in 3GPP specifications, instead, it is simply referred as the “applicable electromagnetic energy absorption requirements” but without explicit value specified.

Observation 1: The term “SAR” has never been officially defined in 3GPP specifications, instead, it is simply referred as the “applicable electromagnetic energy absorption requirements” but without explicit value specified.

In 3GPP, the SAR issue has been made more aware since the HPUE was first introduced in LTE for handheld devices. In nearly every HPUE related work item, SAR mitigation has been included as an objective which had seemingly led to a misconception that SAR solution is only needed for HPUE, but not for default power class (PC3) or even lower power UE. In fact, even before the introduction of HPUE, SAR mitigation has already been considered in 3GPP specifications, which is known as the P-MPR solution. In our view, the SAR issue being made more aware for HPUE is due to that a new solution, also known as “duty-cycle” approach was specifically introduced for HPUE. As 3GPP has been focusing so much on the duty-cycle solution for almost every HPUE UL configuration, the most fundamental solution of using P-MPR was often overlooked or taken for granted.

Observation 2: In 3GPP, the SAR issue has been made more aware since the HPUE was first introduced in LTE for handheld devices.

Observation 3: In fact, even before the introduction of HPUE, SAR mitigation has already been considered in 3GPP specifications, which is known as the P-MPR solution.

In our view, the original intention for UL “duty-cycle” solution was to allow UE to indicate its maximum UL duty cycle capability under HPUE operation via IEs such as maxUplinkDutyCycle-PC2-FR1, maxUplinkDutyCycle-PC1dot5-MPE-FR1, and maxUplinkDutyCycle-interBandCA-PC2 to network so that network may assist UE to fulfill its SAR requirement by scheduling UL duty cycle to less than UE’s capability. However, the UE maximum UL duty cycle capability is only applicable for either PC2 or PC1.5 under PCMAX. There is no provision to network on UE’s UL duty cycle capability at reduced UL power. On the other hand, if network has to accommodate different UEs with different maximum UL duty cycle capabilities, it may heavily constrain network’s scheduling flexibility. Therefore, despite UE maximum UL duty cycle capability has been introduced for NR since Rel-15, to our understanding, such capability has never been considered in network for UL scheduling. As a result, the so-called UL duty-cycle solution for SAR mitigation is only UE based, but not network based, and that also means UE maximum UL duty cycle capability is self-contained.

Observation 4: If network has to accommodate different UEs with different maximum UL duty cycle capabilities, it may heavily constrain network’s scheduling flexibility.

Observation 5: UE maximum UL duty cycle capability has never been considered in network for UL scheduling to assist UE on mitigating SAR issue.

Observation 6: The so-called UL duty-cycle solution for SAR mitigation is only UE based, but not network based.

The UE based UL duty-cycle solution as explicitly defined in current RAN4 specifications is associated with a power reduction term, called ΔPPowerClass, which can only be 3dB or 6dB. For PC2 UE, when UL duty cycle exceeds the maximum limit, let’s say 50%, the UE configured maximum output power would be reduced by 3dB. Similarly, for PC1.5 UE, when UL duty cycle exceeds the maximum limit, let’s say also 50%, the UE configured maximum output power would be reduced by 6dB. In our view, this solution is only a subset of P-MPR solution, as is shown in Table 2-1 where the power reduction level versus UL duty cycle is compared between the two solutions for a PC2 UE.

| UL Duty Cycle (%) | ΔPPowerClass (dB) | P-MPR (dB) |
| --- | --- | --- |
| 100 | 3 | 3.0 |
| 90 | 3 | 2.5 |
| 80 | 3 | 2.0 |
| 70 | 3 | 1.5 |
| 60 | 3 | 0.8 |
| 50 | 0 | 0 |
| 40 | 0 | 0 |
| 30 | 0 | 0 |
| 20 | 0 | 0 |
| 10 | 0 | 0 |

Table 2-1 Power reduction versus UL duty cycle between ΔPPowerClass and P-MPR for a PC2 UE

It can be seen that the UL duty-cycle solution with ΔPPowerClass has worse UL efficiency as compared to P-MPR solution for UL duty cycle between 50% and 100% as ΔPPowerClass is always 3 dB in this duty cycle range while P-MPR allows incremental power reduction between 0 and 3 dB.

Observation 7: UL duty-cycle solution with ΔPPowerClass is only a subset of P-MPR solution.

Observation 8: For PC2 UE, UL duty-cycle solution with ΔPPowerClass has worse UL efficiency as compared to P-MPR solution for UL duty cycle between 50% and 100%.

On the other hand, the UL duty-cycle solution with ΔPPowerClass has been causing a misconception of the so-called “power class fallback” in the current specifications where the UE configured maximum output power reduction would be mistakenly double counted, as brought up in [3].

Observation 9: The UL duty-cycle solution with ΔPPowerClass has been causing a misconception of the so-called “power class fallback” in the current specifications where the UE configured maximum output power reduction would be mistakenly double counted.

Last but not least, it is to our understanding that the UL duty-cycle solution with ΔPPowerClass has never been practically used by any UE in the field to mitigate SAR issue. Furthermore, there is also no conformance test case to verify the requirements. As a result, in our view it would not be worth RAN4’s time and efforts to continue introducing UL duty-cycle based solution with ΔPPowerClass for any new HPUE CA/DC UL configuration in Rel-19 and going forward.

Observation 10: The UL duty-cycle solution with ΔPPowerClass has never been practically used by any UE in the field to mitigate SAR issue.

Observation 11: There is no conformance test case to verify the UL duty-cycle with ΔPPowerClass requirements.

Observation 12: It would not be worth RAN4’s time and efforts to continue introducing UL duty-cycle with ΔPPowerClass based SAR solution for any new HPUE CA/DC UL configuration in Rel-19 and going forward.

Proposal: RAN4 do not consider UL duty-cycle with ΔPPowerClass based SAR solution for any new HPUE CA/DC UL configuration in Rel-19 and going forward.

# 3	Conclusion

In this contribution, we share our views on the SAR compliance aspect for HPUE CA/DC combinations and propose not to further consider UL duty-cycle with ΔPPowerClass for SAR mitigation from Rel-19 and going forward.

Observation 1: The term “SAR” has never been officially defined in 3GPP specifications, instead, it is simply referred as the “applicable electromagnetic energy absorption requirements” but without explicit value specified.

Observation 2: In 3GPP, the SAR issue has been made more aware since the HPUE was first introduced in LTE for handheld devices.

Observation 3: In fact, even before the introduction of HPUE, SAR mitigation has already been considered in 3GPP specifications, which is known as the P-MPR solution.

Observation 4: If network has to accommodate different UEs with different maximum UL duty cycle capabilities, it may heavily constrain network’s scheduling flexibility.

Observation 5: UE maximum UL duty cycle capability has never been considered in network for UL scheduling to assist UE on mitigating SAR issue.

Observation 6: The so-called UL duty-cycle solution for SAR mitigation is only UE based, but not network based.

Observation 7: UL duty-cycle solution with ΔPPowerClass is only a subset of P-MPR solution.

Observation 8: For PC2 UE, UL duty-cycle solution with ΔPPowerClass has worse UL efficiency as compared to P-MPR solution for UL duty cycle between 50% and 100%.

Observation 9: The UL duty-cycle solution with ΔPPowerClass has been causing a misconception of the so-called “power class fallback” in the current specifications where the UE configured maximum output power reduction would be mistakenly double counted.

Observation 10: The UL duty-cycle solution with ΔPPowerClass has never been practically used by any UE in the field to mitigate SAR issue.

Observation 11: There is no conformance test case to verify the UL duty-cycle with ΔPPowerClass requirements.

Observation 12: It would not be worth RAN4’s time and efforts to continue introducing UL duty-cycle with ΔPPowerClass based SAR solution for any new HPUE CA/DC UL configuration in Rel-19 and going forward.

Proposal: RAN4 do not consider UL duty-cycle with ΔPPowerClass based SAR solution for any new HPUE CA/DC UL configuration in Rel-19 and going forward.

# 4	References

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 Chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18th – 21st, 2024

R4-2417098 “WF on HPUE RF requirements”, Samsung, 3GPP TSG RAN WG4 Meeting #112bis, Hefei, China, October 14th – 18th, 2024

R4-2400180 “On misconception of power class fallback”, Apple, 3GPP TSG RAN WG4 Meeting #110, Athens, Greece, February 26th – March 1st, 2024