3GPP RAN WG4 Meeting #113		R4-2417763

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	7.1.1.1.2

Source:	Apple

Title:	On PC1.5 intra-band non-contiguous UL CA MPR

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The Rel-19 RAN4 non-spectrum WID/SID package has been approved in RAN #103 meeting. In consideration that the uplink coverage is crucial for 5G and 5G-adavanced roll-out and the continued strong interests from operators in higher UE maximum output power such as PC1.5 for their band combinations, one of the key objectives under the new Rel-19 “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” work item [1] is to specify the generic requirements of HPUE for NR uplink CA in FR1 and EN-DC with NR FR1 bands where the features include,

    - PC1.5 for intra-band contiguous UL CA with and without UL MIMO

    - PC1.5 Intra-band non-contiguous UL CA without UL MIMO

    - PC1.5 2-band inter-band UL CA with 2Tx or 3Tx for handheld UE and FWA

    - PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx for handheld UE and FWA UE

    - Increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band UL CA and EN-DC HPUE

In last RAN4 meeting, companies had provided their further views on the respective features above where the additional agreements were captured in the approved “WF on HPUE RF requirements” [2]. For PC1.5 intra-band UL CA, RAN4 has agreed that the MPR for contiguous UL CA will be evaluated and specified based on equal PSD between the two carriers, while for non-contiguous UL CA, the PSD imbalance assumption between the two carriers for MPR evaluation was still FFS.

In this contribution, we share our concern that there may not be any performance gain for PC1.5 intra-band non-contiguous UL CA as compared to its PC2 counterpart due to that the PA configuration may have been assumed to be (PC2 + PC2) for PC2 MPR requirements which is also the same PA configuration for PC1.5. Considering that supporting PC1.5 intra-band contiguous UL CA without UL MIMO could be a diminishing return, we propose to introduce UL Tx switching with UL MIMO as an enhancement feature for intra-band non-contiguous UL CA [3] to mitigate the coverage loss issue due to the considerable MPR under simultaneous non-contiguous carriers transmission.

# Discussion

Our concern and reasoning that there may not be any performance gain for PC1.5 intra-band non-contiguous UL CA as compared to its PC2 counterpart was based on the observation of comparable MPR values between the PC2 and PC3 requirements as summarized in Table 2-1.

For PC3 intra-band non-contiguous UL CA with dualPA-Architecture, there should be no doubt that the MPR requirements had been evaluated based on (PC3 + PC3) PA configuration where each PA would back off by 3 dB under PC3 maximum output power. On the other hand, for PC2 intra-band non-contiguous UL CA with dualPA-Architecture, the PA configuration could potentially be (PC3 + PC3), (PC3 + PC2), or (PC2 + PC2). Since (PC3 + PC3) is the PA configuration for PC3 MPR requirements, if the same PA configuration is applied for PC2, we would expect 3dB higher MPR than PC3. However, this is not the case as observed in Table 2-1 where the MPR values are quite comparable between PC2 and PC3.

| PC3 |  | PC2 |  |
| --- | --- | --- | --- |
| B (MHz) | MPR (dB) | B (MHz) | MPR (dB) |
| 0 ≤ B < 1.08 | 15.0 | 0 ≤ B < 1.44 | 15.5 |
| 1.08 ≤ B < 2.16 | 14.5 | 1.44 ≤ B < 2.88 | 15.0 |
| 2.16 ≤ B < 3.24 | 13.5 | 2.88 ≤ B < 5.76 | 14.0 |
| 3.24 ≤ B < 5.04 | 12.5 | 5.76 ≤ B < 10.8 | 12.0 |
| 5.04≤ B < 10.08 | 11.5 | 10.8 ≤ B < 23.04 | 10.5 |
| 10.08 ≤ B < 16.38 | 10.5 | 23.04 ≤ B | 9.0 |
| 16.38 ≤ B < 21.78 | 10.0 |  |  |
| 21.78 ≤ B | 9.0 |  |  |
| NOTE: B is the effective aggregated bandwidth in MHz |  |  |  |

Table 2-1 MPR requirements for intra-band NC UL CA with dualPA for -30dBm/MHz emission

Observation 1: For PC3 intra-band non-contiguous UL CA with dualPA-Architecture, MPR has been specified based on (PC3 + PC3) PA configuration.

Observation 2: For PC2 intra-band non-contiguous UL CA with dualPA-Architecture, the PA configuration could potentially be (PC3 + PC3), (PC3 + PC2), or (PC2 + PC2).

For (PC3 + PC2) PA configuration, though the PC2 PA is more linear, the MPR would be dominated by the emission from the PC3 PA. Using a conceptual 2-tone IMD3 analysis as illustrated in Figure 2-1, it can be seen that the IMD3 power level at (2fCC1 – fCC2) for (PC2 + PC3) PA configuration would be lower than that of (PC3 + PC3) PA configuration under the same total output power, owing to the better linearity of PC2 PA. However, the IMD3 power at (2fCC2 – fCC1) would be the same between (PC2 + PC3) and (PC3 + PC3) PA configurations under the same total output power.

Figure 2-1 A conceptual 2-tone IMD3 analysis between (PC3+PC3) and (PC2+PC3) PA configurations

Since both sides of the IMD3 need to fulfill the same emission requirement, the MPR is also expected to be the same between (PC3 + PC3) and (PC2 + PC3) PA configurations when both are operating at PC3. However, if (PC2 + PC3) PA configuration is operating at PC2, the MPR would be 3dB higher than PC3. As a result, to have comparable MPR between PC3 and PC2 as observed in Table 2-1, the only possible PA configuration for PC2 would be (PC2 + PC2). And that also implies MPR for PC1.5 with (PC2 + PC2) PA configuration would be 3dB higher than PC2 which means there is virtually no performance gain for PC1.5 intra-band NC UL CA as compared to its PC2 counterpart.

Observation 3: For PC2 intra-band non-contiguous UL CA with dualPA-Architecture, MPR based on both (PC3 + PC3) and (PC2 + PC3) PA configurations would be 3dB higher than that of PC3.

Observation 4: The comparable MPR values between PC3 and PC2 as defined in current RAN4 specifications implies that the only possible PA configuration for PC2 would be (PC2 + PC2).

Observation 5: As PC1.5 intra-band non-contiguous UL CA with dualPA-Architecture would be based on the same (PC2 + PC2) PA configuration for PC2, the PC1.5 MPR is expected to be 3dB higher than that of PC2.

Observation 6: There is virtually no performance gain for PC1.5 intra-band NC UL CA as compared to its PC2 counterpart.

Considering that supporting PC1.5 intra-band contiguous UL CA without UL MIMO could be a diminishing return, we propose to introduce UL Tx switching with UL MIMO as an enhancement feature for intra-band non-contiguous UL CA [3] to mitigate the coverage loss issue due to the considerable MPR under simultaneous non-contiguous carriers transmission.

Proposal: Introduce UL Tx switching for intra-band non-contiguous UL CA with UL MIMO as an enhancement feature to intra-band non-contiguous UL CA without UL MIMO.

# 3	Conclusion

In this contribution, we share our concern that there may not be any performance gain for PC1.5 intra-band non-contiguous UL CA as compared to its PC2 counterpart due to that the PA configuration may have been assumed to be (PC2 + PC2) for PC2 MPR requirements which is also the same PA configuration for PC1.5. Considering that supporting PC1.5 intra-band contiguous UL CA without UL MIMO could be a diminishing return, we propose to introduce UL Tx switching with UL MIMO as an enhancement feature for intra-band non-contiguous UL CA to mitigate the coverage loss issue due to the considerable MPR under simultaneous non-contiguous carriers transmission.

Observation 1: For PC3 intra-band non-contiguous UL CA with dualPA-Architecture, MPR has been specified based on (PC3 + PC3) PA configuration.

Observation 2: For PC2 intra-band non-contiguous UL CA with dualPA-Architecture, the PA configuration could potentially be (PC3 + PC3), (PC3 + PC2), or (PC2 + PC2).

Observation 3: For PC2 intra-band non-contiguous UL CA with dualPA-Architecture, MPR based on both (PC3 + PC3) and (PC2 + PC3) PA configurations would be 3dB higher than that of PC3.

Observation 4: The comparable MPR values between PC3 and PC2 as defined in current RAN4 specifications implies that the only possible PA configuration for PC2 would be (PC2 + PC2).

Observation 5: As PC1.5 intra-band non-contiguous UL CA with dualPA-Architecture would be based on the same (PC2 + PC2) PA configuration for PC2, the PC1.5 MPR is expected to be 3dB higher than that of PC2.

Observation 6: There is virtually no performance gain for PC1.5 intra-band NC UL CA as compared to its PC2 counterpart.

Proposal: Introduce UL Tx switching for intra-band non-contiguous UL CA with UL MIMO as an enhancement feature to intra-band non-contiguous UL CA without UL MIMO.

# 4	References

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 Chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18th – 21st, 2024

R4-2417098 “WF on HPUE RF requirements”, Samsung, 3GPP TSG RAN WG4 Meeting #112bis, Hefei, China, October 14th – 18th, 2024

R4-24xxxxx “Motivation and Rel-19 proposal on UL Tx switching for intra-band NC UL CA”, Orlando, USA, November 18th – 22nd, 2024