3GPP TSG-RAN4 Meeting #113					          	       R4-2417782

Orlando, US, Nov 18th – 22nd, 2024

Source: 	Tejas Networks

Title: 	Discussion on BS demodulation performance requirement using MMSE-IRC

Agenda Item:	7.16.3

Document for: 	Discussion

# Introduction

In the RAN4 #112 bis meeting RAN4 discussed test scenarios, interference modelling, and simulation parameter assumptions for the MMSE-IRC receiver, and the WF document [1] was agreed. According to WF [1], agreements were made on the following sub-topics.

Interference modelling criteria - On interference modelling criteria: - Use INR-based modelling and transfer the DIP value into INR value - Agreement on DIP -> INR transformation : - For 1 interferer only scenario, INR1=DIP11-DIP1 - For 2 interferers scenario, INR1=DIP11-DIP1-DIP2, INR2=DIP21-DIP1-DIP2. Test scope - Proposals: - Option 1: - 15kHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U - 30 kHz SCS TDD with pattern 7D1S2U, S=6D:4G:4U - Option 2: - 15kHz SCS FDD - 30 kHz SCS TDD with pattern 7D1S2U, S=6D:4G:4U - Option 3: - 15kHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U - 30 kHz SCS FDD and TDD with pattern 7D1S2U, S=6D:4G:4U - The same requirements are applicable to both FDD and TDD with different UL-DL patterns. - Agreement: - Option 3 with adding note in the spec that ‘same requirements are applicable to both FDD and TDD with different UL-DL pattern’. Channel bandwidth - Agreement: - For CP-OFDM: - For performance evaluation purposes, Minimum channel bandwidth for each SCS, i.e., 5MHz for 15kHz SCS and 10MHz for 30kHz. - Further discuss the CHBW for requirement definition Antenna configuration - Agreement - Cover 2/4/8 Rx with 1Tx for simulation evaluation purpose Waveform - Agreement - Cover CP-OFDM and FFS DFT-s-OFDM PUSCH mapping type - Agreement - Cover both PUSCH mapping Type A and Type B - PUSCH time domain allocation S=0, L=14 Test metric - Agreement: - Use 70% of maximum throughput as metric for NR BS MMSE-IRC demodulation

We present our views on remaining open issues based on WID [2] and WF document [1] of the RAN4 #112bis meeting. Additionally, we have also included simulation results for initial analysis.

# Discussion

## General

Test Scenarios

- On HomNet and HetNet scenarios:

  - Cover HomNet and HetNet scenarios.

  - FFS whether to introduce additional applicability rule.

- On scenario with unaligned TDD pattern:

  - For NR_demod_Ph5 WI, only consider interference model with aligned TDD pattern between cells.

According to the WF document [1], following options are considered for test scenarios:

In the WID [2] document it is agreed to specify the requirements of both HomNet and HetNet scenarios, as these are typical deployment scenarios for NR. We support covering both scenarios for defining performance requirements. Additionally, multiple factors, such as interference power and coverage area, significantly influence the deployment and performance of HomNet and HetNet environments. Hence RAN4 can discuss deployment-based applicability rules.

Proposal 1: We support covering both HomNet and HetNet scenarios. Additionally, RAN4 can discuss deployment-based applicability rules.

An unaligned TDD pattern results in a gNB-gNB CLI issue, which requires additional processing and modifications to the agreed baseline MMSE-IRC receiver algorithm. In addition, the WID objective considers interference only with aligned TDD patterns. Therefore, it is suggested to consider aligned TDD pattern between cells.

Proposal 2: We support considering an interference model with aligned TDD pattern between cells.

## Interference Modelling

Interference profile

- On interference modelling:

  - Reuse the same interference modelling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbour PUSCH

- On how to account for HPUE in existing LTE profiles:

  - Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE.

  - It is not precluded for companies to bring system level simulation on other profiles.

  - Proposals on how to account for HPUE in existing LTE profiles.

- Proposal 1: Perform new system-level simulations with the inclusion of HPUEs

- Proposal 2: Consider artificially increasing DIP/INR with additional 3 or 7 dB

In the WF document [1], following options are proposed for the interference profile:

For practical scenarios, inter-cell interference can have any modulation for NR. However, as proposed in the WF document [1], we support following the interference model same as legacy LTE and re-use same interference modelling as captured in B.6.2 in TS36.104, to transmit a random 16QAM symbol for the neighbour PUSCH.

Proposal 3: For the interference profile, we support to re-use the same interference modelling as captured in B.6.2 in TS36.104, to transmit a random 16QAM symbol for the neighbour PUSCH.

TR 36.884 [3] specifies methodologies and interference power values for LTE. However, the analysis made by TR 36.884 [3] should not be directly applied to the NR scenario as there exists an enormous difference in deployment scenario between NR and LTE.

Furthermore, increasing DIP values with 3 or 7 dB artificially can accommodate HPUEs in LTE-based power values. However, it still does not include NR-specific factors such as cell density (lower ISD), carrier frequency, channel type, delay, doppler, etc. Hence it is suggested to perform NR-specific new system-level simulations with the inclusion of HPUEs.

Proposal 4: To account for HPUEs in existing LTE profiles, we support proposal 1 to perform new system-level simulations with the inclusion of HPUEs.

Number Of Interferers

  - For 4Rx and 8Rx, consider 2 interferers.

  - Check the performance gain of 1 or 2 interferers for 2Rx and make decision based on simulation result.

In the WF document [1], following options are proposed for the number of interferers:

For the legacy LTE profile, initial simulation results for the 1-layer, 2-Rx configuration under a single interference scenario are provided below.

| SCS | BW | MCS for target cell | Modulation for interferer cell | Propagation condition for target cell | Propagation condition for interference cell | Scenario (DIP1, DIP2) | Passing SNR @ 70% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | MMSE | MMSE-IRC | SNR Gain |
| 15 KHz | 5 MHz | 2 | 16 QAM | TDL-B 100 400 | TDL-C 1000 100 | HetNet (-0.43, NA) | 6.83 | 4.95 | 1.88 |
| 15 KHz | 5 MHz | 12 | 16 QAM | TDL-C 300 100 | TDL-C 1000 100 | HetNet (-0.43, NA) | 14.88 | 11.89 | 2.99 |
| 15 KHz | 5 MHz | 20 | 16 QAM | TDL-A 30 10 | TDL-C 1000 100 | HetNet (-0.43, NA) | 20.34 | 17.15 | 3.19 |
| 15 KHz | 5 MHz | 2 | 16 QAM | TDL-B 100 400 | TDL-C 1000 100 | HomNet (-1.11, NA) | 2.49 | 1.46 | 1.03 |
| 15 KHz | 5 MHz | 12 | 16 QAM | TDL-C 300 100 | TDL-C 1000 100 | HomNet (-1.11, NA) | 10.97 | 9.41 | 1.56 |
| 15 KHz | 5 MHz | 20 | 16 QAM | TDL-A 30 10 | TDL-C 1000 100 | HomNet (-1.11, NA) | 16.69 | 15.2 | 1.49 |
| 30 KHz | 10 MHz | 2 | 16 QAM | TDL-B 100 400 | TDL-C 1000 100 | HetNet (-0.43, NA) | 5.95 | 3.16 | 2.79 |
| 30 KHz | 10 MHz | 12 | 16 QAM | TDL-C 300 100 | TDL-C 1000 100 | HetNet (-0.43, NA) | 14.88 | 12.17 | 2.71 |
| 30 KHz | 10 MHz | 20 | 16 QAM | TDL-A 30 10 | TDL-C 1000 100 | HetNet (-0.43, NA) | 20.50 | 17.77 | 2.73 |
| 30 KHz | 10 MHz | 2 | 16 QAM | TDL-B 100 400 | TDL-C 1000 100 | HomNet (-1.11, NA) | 2.01 | 0.61 | 1.40 |
| 30 KHz | 10 MHz | 12 | 16 QAM | TDL-C 300 100 | TDL-C 1000 100 | HomNet (-1.11, NA) | 11.00 | 9.38 | 1.62 |
| 30 KHz | 10 MHz | 20 | 16 QAM | TDL-A 30 10 | TDL-C 1000 100 | HomNet (-1.11, NA) | 16.83 | 15.82 | 1.01 |

Simulation results for two interference scenarios with legacy LTE assumptions are presented as follows.

| SCS | BW | MCS for target cell | Modulation for interferer cell | Propagation condition for target cell | Propagation condition for interference cell | Scenario (DIP1, DIP2) | Passing SNR @ 70% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | MMSE | MMSE-IRC | SNR Gain |
| 15 KHz | 5 MHz | 2 | 16 QAM | TDL-B 100 400 | TDL-C 1000 100 | HetNet (-0.43, -13.78) | 10.99 | 8.35 | 2.64 |
| 15 KHz | 5 MHz | 12 | 16 QAM | TDL-C 300 100 | TDL-C 1000 100 | HetNet (-0.43, -13.78) | 19.00 | 14.95 | 4.05 |
| 15 KHz | 5 MHz | 20 | 16 QAM | TDL-A 30 10 | TDL-C 1000 100 | HetNet (-0.43, -13.78) | 24.32 | 19.75 | 4.57 |
| 15 KHz | 5 MHz | 2 | 16 QAM | TDL-B 100 400 | TDL-C 1000 100 | HomNet (-1.11, -10.91) | 6.65 | 4.92 | 1.73 |
| 15 KHz | 5 MHz | 12 | 16 QAM | TDL-C 300 100 | TDL-C 1000 100 | HomNet (-1.11, -10.91) | 14.78 | 11.88 | 2.90 |
| 15 KHz | 5 MHz | 20 | 16 QAM | TDL-A 30 10 | TDL-C 1000 100 | HomNet (-1.11, -10.91) | 20.22 | 17.06 | 3.16 |
| 30 KHz | 10 MHz | 2 | 16 QAM | TDL-B 100 400 | TDL-C 1000 100 | HetNet (-0.43, -13.78) | 10.07 | 6.40 | 3.67 |
| 30 KHz | 10 MHz | 12 | 16 QAM | TDL-C 300 100 | TDL-C 1000 100 | HetNet (-0.43, -13.78) | 18.84 | 15.39 | 3.45 |
| 30 KHz | 10 MHz | 20 | 16 QAM | TDL-A 30 10 | TDL-C 1000 100 | HetNet (-0.43, -13.78) | 24.35 | 20.58 | 3.77 |
| 30 KHz | 10 MHz | 2 | 16 QAM | TDL-B 100 400 | TDL-C 1000 100 | HomNet (-1.11, -10.91) | 5.84 | 3.12 | 2.72 |
| 30 KHz | 10 MHz | 12 | 16 QAM | TDL-C 300 100 | TDL-C 1000 100 | HomNet (-1.11, -10.91) | 14.78 | 12.13 | 2.65 |
| 30 KHz | 10 MHz | 20 | 16 QAM | TDL-A 30 10 | TDL-C 1000 100 | HomNet (-1.11, -10.91) | 20.32 | 17.72 | 2.60 |

Based on the above simulation results, it is seen that for one interference case, SNR gains of almost 1dB to 3dB are observed, while for two interference scenarios, SNR gains of 2dB to 4dB are detected. As the SNR gain with an MMSE-IRC receiver does not vary much with the number of interferences, hence for 2Rx we propose to follow the legacy LTE interference scenario and consider only one interference for 2Rx.

Proposal 5: We propose the use of 1 interference for 2Rx cases, while 2 interferences for 4Rx and 8Rx antenna cases.

## Test parameters and simulation assumptions

Propagation Condition

In the WF document [1], following candidate options are proposed for the propagation condition of serving cell and interference cells:

- Candidate options:

  - For serving cell:

- Option 1: TDLA30-10 and TDLC300-100 for HetNet and HomNet respectively

- Option 2: TDLA30-10 and TDLB100-100 for HetNet and HomNet respectively

- Option 3: Cover TDLA30-10, TDLB100-400, TDLC300-100

- Option 4: Use TDLB100-400

  - For interference cell:

- Option 1: Introduce new propagation channel for testing with desired delay spread 1000ns for the interference PUSCH, i.e., TDLC1000

- Option 2: Same propagation condition for serving and interference PUSCH

- Option 3: Use TDLC300-100

TS 38.104 specifies the usage of TDL-A 30 10, TDL-B 100 400, and TDL-C 300 100 channels to cover NR-specific channel conditions and a variety of delay and Doppler values. As Doppler impacts covariance matrix estimation, the range of Doppler values must be exercised. So, we support the use of TDL-A 30 10, TDL-B 100 400, and TDL-C 300 100 channels for defining performance requirements.

Proposal 6: From the candidate options presented for propagation condition, we support candidate option 3 to cover TDLA30-10, TDLB100-400, and TDLC300-100 for serving cell.

For interference signals, the channel experiences a higher delay spread when compared to the desired signal. Hence, we support option 1 to introduce the channel with the desired delay spread of 1000ns for interference PUSCH.

Proposal 7: From the candidate options presented for propagation condition, we support candidate option 1 to introduce new channel for testing with a desired delay spread of 1000ns for interference PUSCH.

MCS

  - Option 1: Cover QPSK/16QAM/64QAM

- Option 1A: Use MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS 24(64QAM, R=772/1024)

- Option 1B: MCS index 2, 12, 20 in MCS table 1

- Option 1C: Cover MCS 2, 16, 20

  - Option 2: Choose only one MCS configuration

  - Option 3: Consider MCS 5(QPSK, R=379/1024) and MCS 13(16QAM, R=490/1024) and FFS 64QAM (ZTE)

In the WF document [1], following candidate options are proposed for MCS selection:

For NR, interference from HPUEs is a scenario that results in a lower code-rate requirement. Furthermore, the current discussion focuses on cell-edge users, where achieving a higher code rate for cell-edge UEs is not practical. Hence, we support option 1B to use MCS index 2, 12, 20 from MCS table 1.

Proposal 8: From the candidate options proposed for MCS selection, we support option 1B to choose MCS index 2, 12, 20 in MCS table 1.

DMRS Configuration

- Candidate options on DMRS type:

  - Option 1: Cover DMRS configuration type 1 and type 2

  - Option 2: DMRS configuration type 1

- Candidate options on DMRS sequence generation:

  - Option 1:

- NID0=0, nSCID =0 for CP-OFDM

- NID0=0, group hopping and sequence hopping are disabled for DFT-s-OFDM

  - Option 2:

- nSCID =0 for all cells

- Different NID0=0 for serving and interference cells

As per the WF document [1], following candidate options for the DMRS configuration are specified.

The number of DMRS sub-carriers per PRB, determined by the DMRS configuration type, significantly influences the accuracy of covariance matrix estimation, particularly under high Doppler channel conditions. An accurate covariance matrix estimation is essential for the MMSE-IRC for more effective inter-cell interference mitigation. Given the possibility of high Doppler circumstances in NR, it is recommended to include both DMRS configuration types 1 and 2.

Proposal 9: From the available candidate options for DMRS type, we support option 1 to cover DMRS configuration type 1 and type 2.

Since nSCID and NID0 play a crucial role in DMRS sequence generation, assigning identical values for both the serving and interfering cells introduces ambiguity at the receiver, resulting in corrupted channel estimates. Therefore, it is essential to ensure that the desired UE and interfering UE do not hold the same DMRS sequence, so we support option 2 of keeping nSCID=0 for all cells and keeping different NID0 = 0 for serving and interference cells.

Proposal 10: From the available candidate options for DMRS sequence generation, we support option 2 of keeping nSCID=0 for all cells and keeping different NID0 = 0 for serving and interference cells.

# Simulation Results

| Scenario | (DIP1, DIP2) dB | Transferred (INR1, INR2) dB |
| --- | --- | --- |
| HetNet with 1 interferer | (-0,43, N/A) | (9.82, N/A) |
| HetNet with 2 interferers | (-0,43, -13,78) | (12.38, -0.97) |
| HomNet with 1 interferer | (-1,11, N/A) | (5.35, N/A) |
| HomNet with 2 interferers | (-1,11, -10,91) | (7.28, -2.52) |

In this section, initial simulation results are provided to analyse the performance gain of MMSE-IRC receiver over MMSE. In the WF document [1], following simulation parameters are suggested to obtain initial results:

Table 1: PUSCH performance gain for HetNet scenario – SCS: 15KHz, BW: 5MHz

| Case No. | Tx | Rx | MCS for target cell | Propagation condition for target cell | Modulation for interference cell | Propagation condition for interference cell | No. of interference | (DIP1, DIP2) | Passing SNR @ 70% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | MMSE | MMSE-IRC | Gain |
| 1 | 1 | 2 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 6.83 | 4.95 | 1.88 |
| 2 | 1 | 2 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 14.88 | 11.89 | 2.99 |
| 3 | 1 | 2 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 20.34 | 17.15 | 3.19 |
| 4 | 1 | 4 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 6.83 | 1.78 | 5.05 |
| 5 | 1 | 4 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 11.77 | 7.16 | 4.61 |
| 6 | 1 | 4 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 16.68 | 13.5 | 3.18 |
| 7 | 1 | 8 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 6.00 | -1.63 | 7.63 |
| 8 | 1 | 8 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 9.33 | 5.63 | 3.70 |
| 9 | 1 | 8 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 14.51 | 9 | 5.51 |
| 10 | 1 | 2 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 10.99 | 8.35 | 2.64 |
| 11 | 1 | 2 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 19.00 | 14.95 | 4.05 |
| 12 | 1 | 2 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 24.32 | 19.75 | 4.57 |
| 13 | 1 | 4 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 10.88 | 4.87 | 6.01 |
| 14 | 1 | 4 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 15.81 | 10.78 | 5.03 |
| 15 | 1 | 4 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 20.62 | 15.24 | 5.38 |
| 16 | 1 | 8 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 10.35 | 1.67 | 8.68 |
| 17 | 1 | 8 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 13.41 | 9.22 | 4.19 |
| 18 | 1 | 8 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 18.56 | 12.66 | 5.90 |

Table 2: PUSCH performance gain for HomNet scenario – SCS: 15KHz, BW: 5MHz

| Case No. | Tx | Rx | MCS for target cell | Propagation condition for target cell | Modulation for interference cell | Propagation condition for interference cell | No. of interference | (DIP1, DIP2) | Passing SNR @ 70% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | MMSE | MMSE-IRC | Gain |
| 1 | 1 | 2 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 2.49 | 1.46 | 1.03 |
| 2 | 1 | 2 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 10.97 | 9.41 | 1.56 |
| 3 | 1 | 2 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 16.69 | 15.2 | 1.49 |
| 4 | 1 | 4 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 2.65 | -0.52 | 3.17 |
| 5 | 1 | 4 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 7.73 | 4.81 | 2.92 |
| 6 | 1 | 4 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 13.06 | 10.75 | 2.31 |
| 7 | 1 | 8 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 1.60 | -3.37 | 4.97 |
| 8 | 1 | 8 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 5.42 | 2.28 | 3.14 |
| 9 | 1 | 8 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 10.55 | 7.9 | 2.65 |
| 10 | 1 | 2 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 6.65 | 4.92 | 1.73 |
| 11 | 1 | 2 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 14.78 | 11.88 | 2.90 |
| 12 | 1 | 2 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 20.22 | 17.06 | 3.16 |
| 13 | 1 | 4 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 6.71 | 1.83 | 4.88 |
| 14 | 1 | 4 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 11.65 | 7.11 | 4.54 |
| 15 | 1 | 4 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 16.58 | 13.55 | 3.03 |
| 16 | 1 | 8 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 6.00 | -1.83 | 7.83 |
| 17 | 1 | 8 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 9.28 | 5.52 | 3.76 |
| 18 | 1 | 8 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 14.43 | 8.89 | 5.54 |

Table 3: PUSCH performance gain for HetNet scenario – SCS: 30KHz, BW: 10MHz

| Case No. | Tx | Rx | MCS for target cell | Propagation condition for target cell | Modulation for interference cell | Propagation condition for interference cell | No. of interference | (DIP1, DIP2) | Passing SNR @ 70% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | MMSE | MMSE-IRC | Gain |
| 1 | 1 | 2 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 5.95 | 3.16 | 2.79 |
| 2 | 1 | 2 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 14.88 | 12.17 | 2.71 |
| 3 | 1 | 2 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 20.50 | 17.77 | 2.73 |
| 4 | 1 | 4 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 4.51 | -1.31 | 5.82 |
| 5 | 1 | 4 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 11.89 | 6.92 | 4.97 |
| 6 | 1 | 4 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 16.61 | 11.7 | 4.91 |
| 7 | 1 | 8 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 3.16 | -4.37 | 7.53 |
| 8 | 1 | 8 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 9.86 | 4.08 | 5.78 |
| 9 | 1 | 8 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 14.79 | 9 | 5.79 |
| 10 | 1 | 2 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 10.07 | 6.4 | 3.67 |
| 11 | 1 | 2 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 18.84 | 15.39 | 3.45 |
| 12 | 1 | 2 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 24.35 | 20.58 | 3.77 |
| 13 | 1 | 4 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 8.60 | 1.17 | 7.43 |
| 14 | 1 | 4 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 16.09 | 9.87 | 6.22 |
| 15 | 1 | 4 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 20.67 | 14.35 | 6.32 |
| 16 | 1 | 8 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 7.42 | -2.41 | 9.83 |
| 17 | 1 | 8 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 13.91 | 5.6 | 8.31 |
| 18 | 1 | 8 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 18.87 | 11.17 | 7.70 |

Table 4: PUSCH performance gain for HomNet scenario – SCS: 30KHz, BW: 10MHz

| Case No. | Tx | Rx | MCS for target cell | Propagation condition for target cell | Modulation for interference cell | Propagation condition for interference cell | No. of interference | (DIP1, DIP2) | Passing SNR @ 70% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | MMSE | MMSE-IRC | Gain |
| 1 | 1 | 2 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 2.01 | 0.61 | 1.40 |
| 2 | 1 | 2 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 11.00 | 9.38 | 1.62 |
| 3 | 1 | 2 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 16.83 | 15.82 | 1.01 |
| 4 | 1 | 4 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 0.27 | -2.89 | 3.16 |
| 5 | 1 | 4 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 7.81 | 5 | 2.81 |
| 6 | 1 | 4 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 12.73 | 9.97 | 2.76 |
| 7 | 1 | 8 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | -1.15 | -5.91 | 4.76 |
| 8 | 1 | 8 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 5.79 | 1.91 | 3.88 |
| 9 | 1 | 8 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 10.89 | 8.08 | 2.81 |
| 10 | 1 | 2 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 5.84 | 3.12 | 2.72 |
| 11 | 1 | 2 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 14.78 | 12.13 | 2.65 |
| 12 | 1 | 2 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 20.32 | 17.72 | 2.60 |
| 13 | 1 | 4 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 4.39 | -1.34 | 5.73 |
| 14 | 1 | 4 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 11.79 | 6.9 | 4.89 |
| 15 | 1 | 4 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 16.51 | 11.67 | 4.84 |
| 16 | 1 | 8 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 3.10 | -4.36 | 7.46 |
| 17 | 1 | 8 | 12 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 9.71 | 4.01 | 5.70 |
| 18 | 1 | 8 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 14.73 | 8.94 | 5.79 |

Based on the simulation results presented above, the following observations are made:

Observation 1: Compared to the HomNet scenario, the HetNet scenario has a higher SNR gain. This higher SNR gain indicates the effectiveness of the MMSE-IRC algorithm in high-interference environment as HetNet has a greater INR than HomNet.

Observation 2: The SNR gain increases with the number of antennas, as more receiver antennas enhance interference rejection capability and improve diversity.

Observation 3: SNR gain of MMSE-IRC with two interference sources is always higher than single interference scenario, irrespective of the number of receiver antennas.

Observation 4: Channels with higher delay spread lead to lower SNR gain as interference mitigation becomes challenging due to poor channel quality.

Observation 5: The SNR gain with MMSE-IRC varies between almost 2 dB and 6 dB.  In the interference scenario, this amount of SNR gain is adequate to justify the use of MMSE-IRC for interference mitigation at BS.

Above mentioned simulation results and observations specifies the effectiveness and interference mitigation capability of the MMSE-IRC receiver over MMSE.

# Conclusion

In this contribution, we have given our views and proposals on the remaining issues of MMSE-IRC performance requirement for BS demodulation along with initial simulation results.

Proposal 1: We support covering both HomNet and HetNet scenarios. Additionally, RAN4 can discuss deployment-based applicability rules.

Proposal 2: We support considering an interference model with aligned TDD pattern between cells.

Proposal 3: For the interference profile, we support to re-use the same interference modelling as captured in B.6.2 in TS36.104, to transmit a random 16QAM symbol for the neighbour PUSCH.

Proposal 4: To account for HPUEs in existing LTE profiles, we support proposal 1 to perform new system-level simulations with the inclusion of HPUEs.

Proposal 5: We propose the use of 1 interference for 2Rx cases, while 2 interferences for 4Rx and 8Rx antenna cases.

Proposal 6: From the candidate options presented for propagation condition, we support candidate option 3 to cover TDLA30-10, TDLB100-400, and TDLC300-100 for serving cell.

Proposal 7: From the candidate options presented for propagation condition, we support candidate option 1 to introduce new channel for testing with a desired delay spread of 1000ns for interference PUSCH.

Proposal 8: From the candidate options proposed for MCS selection, we support option 1B to choose MCS index 2, 12, 20 in MCS table 1.

Proposal 9: From the available candidate options for DMRS type, we support option 1 to cover DMRS configuration type 1 and type 2.

Proposal 10: From the available candidate options for DMRS sequence generation, we support option 2 of keeping nSCID=0 for all cells and keeping different NID0 = 0 for serving and interference cells.

Observation 1: Compared to the HomNet scenario, the HetNet scenario has a higher SNR gain. This higher SNR gain indicates the effectiveness of the MMSE-IRC algorithm in high-interference environment as HetNet has a greater INR than HomNet.

Observation 2: The SNR gain increases with the number of antennas, as more receiver antennas enhance interference rejection capability and improve diversity.

Observation 3: SNR gain of MMSE-IRC with two interference sources is always higher than single interference scenario, irrespective of the number of receiver antennas.

Observation 4: Channels with higher delay spread lead to lower SNR gain as interference mitigation becomes challenging due to poor channel quality.

Observation 5: The SNR gain with MMSE-IRC varies between almost 2 dB and 6 dB.  In the interference scenario, this amount of SNR gain is adequate to justify the use of MMSE-IRC for interference mitigation at BS.

# Reference

- R4-2416548, “WF on [112bis][319] NR_demod_Ph5_Part1_General_BS”, China Telcom, RAN#112bis, Hefei, China.

- RP-240831, “New WID: WI resulting from discussion on Demodulation performance requirements enhancements”, Vivo, RAN#103, Maastricht, Netherlands.

- TR 36.884 “Performance requirements of MMSE-IRC receiver for LTE BS.”