# 3GPP TSG-RAN WG4 Meeting #113	                                R4-2417787

Orlando, US, 18th – 22nd November, 2024

Source:                Tejas Networks Pvt. Ltd.

Title:                     Discussion on Potentially new requirements for SBFD operation

Agenda Item:       7.20.2.1

Document for:     Discussion

# Introduction

This contribution presents our views on the potential new requirements for SBFD operation. In the RAN4 #112bis meeting, RAN4 agreed to further investigate the duration and characteristics of the transient period between SBFD and non-SBFD operation, as outlined in the following agreement [1].

Way Forward: - FFS location of transient period between SBFD and non-SBFD: - Option 1: No need to discuss and decide whether the transmitter transient period shall be located within the SBFD symbol or not. - Option 2: Place the transient period outside of both SBFD and non-SBFD slots. - Option 3: The location of the transient period between SBFD and non-SBFD shall be within the SBFD symbol).

# Discussion

## Location of transient period between SBFD and non-SBFD

Figure 1: SBFD configuration

Figure 1 shows a SBFD configuration. The possible switching cases may be [DLàSBFD], [SBFDàDL], [NON SBFD UL à SBFD UL] and [SBFD UL à NON SBFD UL].

Observation 1: In general, the transient time should be the same for both full TDD switching and switching related to SBFD, as the total transmit (TX) switching time budget must be consistent for both switching between the complete uplink/downlink (UL/DL) and the SBFD resource blocks (RBs). For the transitions between DL-SBFD UL and SBFD-DL, the existing TDD switching time can be used, since the power amplifier (PA) will always remain on during this transition. Additionally, because the user equipment (UE) operates in half-duplex mode, the transition from non-SBFD DL to SBFD UL should account only for the PA switching time. As a result, no new requirements are anticipated from the base station (BS) side.

Proposal 1: The location of the transient period between SBFD and non-SBFD shall be within the SBFD symbol.

# Conclusion

In this contribution, we summarised the potentially new requirements for SBFD operation with the following proposals and observations.

Observation 1: In general, the transient time should be the same for both full TDD switching and switching related to SBFD, as the total transmit (TX) switching time budget must be consistent for both switching between the complete uplink/downlink (UL/DL) and the SBFD resource blocks (RBs). For the transitions between DL-SBFD UL and SBFD-DL, the existing TDD switching time can be used, since the power amplifier (PA) will always remain on during this transition. Additionally, because the user equipment (UE) operates in half-duplex mode, the transition from non-SBFD DL to SBFD UL should account only for the PA switching time. As a result, no new requirements are anticipated from the base station (BS) side.

Proposal 1: The location of the transient period between SBFD and non-SBFD shall be within the SBFD symbol.

# Reference

- R4-2416584: Way forward for [112bis][306] NR_duplex_evo_General.