3GPP TSG-RAN WG4 Meeting#113							         		R4-2417794

Orlando, Florida, USA, 18 – 22 November 2024

Title: 	Simplification of HPUE 2UL IMD MSDs

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.1.1.1.1 General aspects (incl. duty cycle solutions, MSD rules) [NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In RAN#112bis, different options on how to simplify the HPUE MSD requirement process or whether to remove these requirements were captured in way forward [1]. In this contribution, we expose preliminary proposals on the 2UL HPUE MSD cases, which require further elaboration in order to provide delta MSD LUT based on calculations and observations in [2]. While another paper [3] make similar proposals for calculations based LUTs for 1UL HPUE MSDs.

# Discussion

Based on the observations collected for this meeting in [2], we formulate a number of propositions for the 2UL HPUE IMD MSDs for the Option 2 described in way forward [1] from R4#112bis where the HPUE MSD is derived from calculated curves of delta MSD captured in LUTs. This complements the similar proposals for 1UL HPUE MSDs in [3].

## 2.1 HPUE cases with issues.

HPUE cases with incomplete or incorrect requirements

Observations in [2]:

- CA_n7-n20 is an FDD-FDD combination which was not in scope for R18 HPUE that is not in the HPUE power class table and has a missing PC3 MSD and a PC2 MSD. It seems that this is a PC3 combination MSD that was captured in the PC2 table instead of PC3.

- CA_n20-n78/n78(2A) have missing PC2 or PC3 MSD while the other MSD is specified and is not in the HPUE power class table. It seems that this is a PC3 combination MSD that was captured in the PC2 table instead of PC3 for the CA_n20-n78(2A) case.

- CA_n28-n78/n78(2A) have missing PC2 or PC3 MSD while the other MSD is specified and is valid in the HPUE power class table. Maybe the CA_n28-n78(2A) is only for PC3, but then CA_n28-n78 is missing PC2 MSD.

- CA_n25-n77 has PC1.5 3Tx power class configuration but not PC2 3Tx power class configuration which should be there as a fallback.

- CA_n70-n77 has PC2 3Tx power class configuration, but not PC2 2Tx power class configuration, which seems to be an omission compared to other cases where combinations supporting 3Tx PC2 also support 2Tx PC2.

- CA_n18-n77 has two sets of MSDs for both IMD4 and IMD5, with one having N/A for the PC3 case. The valid cases should be sorted.

- Some cases have a large PC2 MSD, but no PC3 MSD: CA_n25-n41C, CA_n8-n77, CA_n3-n28-n78 and CA_n28-n78(2A). This needs to be understood.

- Some cases have a PC3 MSD, but no PC2 MSD: CA_n28-n78, CA_n5-n77, CA_n5-n7-n78 and CA_n7-n28-n78. These cases are incomplete.

Proposal: Bands combinations and UL configurations that have incorrectly captured MSDs cannot be considered complete in Release 18:

- MSD captured in the wrong power class table.

- MSD captured for UL configurations that are not specified in the 2Tx or 3Tx CA power class tables.

- Case where there is an PC3 MSD but no PC2 MSD.

- Cases where there is a large PC2 MSD and no PC3 MSD should be justified.

Incorrect HPUE power class

Observations in [2]:

- There are HPUE cases that are specified in the power class table but are not valid in the BCS table: CA_n28A-n41C, CA_n28A-n79C, CA_n40A-n79C, CA_n41A-n79C and CA_n41C-n79A.

- There are HPUE cases that have an HPUE MSD specified but are not in the power class table, notably the cases where one band supports intra-band ULCA: CA_n1-n77(2A), CA_n1-n78(2A), CA_n3-n77(2A), CA_n3-n78(2A), CA_n5-n77(2A), CA_n5-n78(2A), CA_7-n77(2A), CA_n7-n78(2A), CA_n25-n41C, CA_n25-n77(2A), CA_n25-n78(2A), CA_n28-n77(2A), CA_n28-n78(2A), CA_n41C-n66, CA_n41C-n77, CA_n41-n77(2A), CA_n66-n77(2A), CA_n66-n78(2A) and CA_n71-n77(2A).

Proposal: HPUE bands combinations and UL configurations that are not consistently captured in the BCS tables and the 2Tx and 3Tx power classes cannot be considered complete in release 18.

HPUE IMD cases with incorrect MSD.

Observations in [2]:

- Except for CA_n41C-n66, CA_n25-n41C and CA_n41C-n77 which are correct, all the band combinations have a significant delta MSD between PC3 and PC2 that seems proportional to the IMD order like for the two band IMD case.

- In fact, the delta should be zero as the IMD levels for the 1UL band intra-band CA is limited by the SEM mask (the UE can apply MPR for this) which is constant for all power classes and thus the IMD levels are constant:

  - IMD3 is limited to -13dBm/MHz and IMD5 to -30dBm/MHz for contiguous intra-band ULCA and higher order IMDs are thus also limited to the same level independently of the band power class.

  - IMD3 is limited to -30dBm/MHz for non-contiguous intra-band ULCA and higher order IMDs are thus also limited to the same level independently of the band power class.

- Consequently, the HPUE MSD related to 1UL band with intra-band CA should be equal to the PC3 value and the Delta MSD is 0dB.

Proposal: The HPUE MSD related to a one TDD band UL with intra-band ULCA should be equal to the PC3 value. The Delta MSD is 0dB as the IMD4/5/7/9 levels are clamped by the SEM mask. Many cases need correction for Release 18.

## 2.2 Proposal for Release 18 PC2 cases

Figure 1 and Figure 2 are derived from the annex Tables A1 and A2 in [2] covering PC2 two DL and three DL bands respectively, where the cases with issues described above are removed.

The two figures illustrate the delta MSD between PC2 and PC3 versus PC3 MSD with the following aspects:

- Calculated curves based on equations provided on the annex of [2] which assume that:

  - PC2 interference level = PC3 interference level + 3 * the IMD order, with:

  - Blue curve for IMD2, green curve for IMD3, orange curve for IMD4 and red curve for IMD5, plain curves.

- The Delta MSD based on the specified PC3 and PC2 MSDs with:

  - Blue triangle for IMD2, green diamond for IMD3, orange dots for IMD4 and red squares for IMD5.

- A proposed simplified curve that can be captured in a LUT or a simple equation.

  - Blue curve for IMD2, green curve for IMD3, orange curve for IMD4 and red curve for IMD5, dashed curves.

Figure 1: Two DL band PC2 delta MSD versus PC3 MSD. Specification points and calculated curves vs IMD order.

Figure 2: Three DL band PC2 delta MSD versus PC3 MSD. Specification points and calculated curves vs IMD order.

Observations in [2]:

- The calculated curves based on 3xIMD order increase in interference power is valid for all R18 PC2 cases which, although the TDD band may have different power capability (PC3 or PC2) or architecture (1Tx or 2Tx), is consistent with the fact that the worst case is for the FDD PC3 PA that is unchanged.

- These curves can be approximated by a linear curve in the 0.5 to 10dB PC3 MSD range which is clamped above 10dB PC3 MSD.

Proposal for release 18 PC2 Delta MSD: The 2UL IMD delta MSD are derived from following equations:

- IMD2 PC2 delta MSD = Min(6 , 0.45*PC3MSD+1.5) [dB]

- IMD3 PC2 delta MSD = Min(9 , 0.6*PC3MSD+3) [dB]

- IMD4 PC2 delta MSD = Min(12 , 0.7*PC3MSD+3) [dB]

- IMD5 PC2 delta MSD = Min(15 , 0.8*PC3MSD+7) [dB]

- FFS if captured in an LUT or use a 0.5dB step ceiling function.

## 2.3 Proposal for Release 18 PC1.5 cases

Figure 3 is derived from the annex Tables A1 covering PC1.5 two DL bands, where the cases with issues described above are removed. The figure illustrates the delta MSD between PC1.5 and PC3 versus PC3 MSD with the following aspects:

- Calculated curves based on equations provided on the annex of [2] but modified to account for the fact that the TDD PA is increased to 27.7dBm but in fact by 24.7dBm per PA with one coupling more tightly to the FDD PA:

  - PC1.5 interference level 	= PC3 interference level + 3 * the IMD order + 1.7*(IMDorder-1)

= PC3 interference level + 4.7 * the IMDorder – 1.7dB, with:

  - Blue curve for IMD2, green curve for IMD3, orange curve for IMD4 and red curve for IMD5, plain curves.

- The Delta MSD based on the specified PC3 and PC1.5 MSDs with:

  - Blue triangle for IMD2, green diamond for IMD3, orange dots for IMD4 and red squares for IMD5.

- A proposed simplified curve that can be captured in a LUT or a simple equation.

  - Blue curve for IMD2, green curve for IMD3, orange curve for IMD4 and red curve for IMD5, dashed curves.

Figure 3: Two DL band PC1.5 delta MSD versus PC3 MSD. Specification points and calculated curves vs IMD order.

Observations in [2]:

- There is only one PC1.5 MSD value in Release 19 as the only valid implementation is PC3 1Tx FDD + PC1.5 2TX TDD. However, in release 19 more cases are anticipated.

- All the specified values seem to account for the sum of the TDD PAs power increase (may be assuming fully separate antennas for the two TDD PAs from the PC3 FDD PA?). Further check on PC1.5 assumptions is needed, but a modified calculation based on an interference power increasing with the order of the IMD will work.

Proposal for release 18 PC1.5 Delta MSD: The 2UL IMD delta MSD using a simplified equation of the form:

- IMDX PC1.5 delta MSD = Min(Z , A*PC3MSD+B) [dB]

- FFS if captured in an LUT or use a 0.5dB step ceiling function

- If 23dBm+27.7dBm power is considered, Z, A and B can be derived from assuming an interference increase of at least:

  - PC3 interference level + 4.7 * the IMDorder – 1.7dB

  - Other assumptions resulting in a higher interference level increase is not precluded.

For information: assuming an interference increase of PC3 interference level + 4.7 * the IMDorder – 1.7dB, the 2UL IMD delta MSD can be derived from following equations:

- IMD2 PC2 delta MSD = Min(7.7 , 0.6*PC3MSD+1.7) [dB]

- IMD3 PC2 delta MSD = Min(12.4 , 0.8*PC3MSD+4.4) [dB]

- IMD4 PC2 delta MSD = Min(17,1 , 0.9*PC3MSD+8.1) [dB]

- IMD5 PC2 delta MSD = Min(21.8 , 1*PC3MSD+11.8) [dB]

- FFS if captured in an LUT or use a 0.5dB step ceiling function

## 2.2 Proposal for Release 19 cases

In Release 19, there are more PC1.5 cases and PC2 FDD can be considered. This significantly increases the number of cases to be studied and the complexity of the increased level of IMD interference versus PC3.

Table 1 shows the different PC2 and PC1.5 inter-band power class implementations, the associated release and related 1UL and 2UL MSDs with:

- Inter-band ULCA power class and its maximum power level

- Per band implementation scenario and power class

- Per band 1UL power levels for MSD

- Per band 2UL power levels for MSD

- Per band back off condition at 2UL power for MSD

- Maximum total power

- Possibility for increased power and its value

- Anticipated interference power increase versus PC3

- Incremental interference power increase versus other cases.

Table 1: PC3, PC2 and PC1.5 inter-band power class cases up to R19

| case |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ULCA power class | Type | PC3 2Tx | PC2 2Tx | PC2 2Tx | PC2 3Tx | PC2 2Tx | PC2 3Tx | PC1.5 2Tx | PC1.5 3Tx | PC1.5 3Tx | PC1.5 3Tx |
|  | PCmax | 23 | 26 | 26 | 26 | 26 | 26 | 29 | 29 | 29 | 29 |
| per band power class | Bd1 | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC2 1TX | PC2 1TX | PC2 1TX | PC2 1TX | PC3 1Tx | PC2 1TX |
|  | Bd2 | PC3 1Tx | PC3 1Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC1.5 2Tx | PC1.5 2Tx |
| Valid Release |  | R15 | R17 | R17 | R18 | R19 | R19 | R19 | R19 | R18 | R19 |
| 1UL power | Bd1 | 23 | 23 | 23 | 23 | 26 | 26 | 26 | 26 | 23 | 26 |
|  | Bd2 | 23 | 23 | 26 | 2x23 | 26 | 2x23 | 26 | 2x23 | 2x26 | 2x26 |
| 2UL power | Bd1 | 20 | 23 | 23 | 23 | 23 | 23 | 26 | 26 | 23 | 26 |
|  | Bd2 | 20 | 23 | 23 | 2x20 | 23 | 2x20 | 26 | 2x23 | 2x24.7 | 2x23 |
| 2UL BO condition | Bd1 | -3 | 0 | 0 | 0 | -3 | -3 | 0 | 0 | 0 | 0 |
|  | Bd2 | -3 | 0 | -3 | -3 | -3 | -3 | 0 | 0 | -1.3 | -3 |
| Max power |  | 26 | 26 | 27.8 | 27.8 | 29 | 29 | 29 | 29 | 30 | 30.8 |
| Increased power |  | N (+3dB) | N | Y (+1.8dB) | Y (+1.8dB) | N (+3dB) | N (+3dB) | N | N | Y? (+1dB) | Y? (+1.8dB) |
| 2UL interference level |  | Ref | +3xorder | +3xorder for FDD | +3xorder for FDD | +3dB | +3dB -MRC | +3dB +3xorder | +3dB +3xorder -MRC | ≤+4.7*order -1.7dB for FDD side | ≤+3dB +3xorder -MRC |
| case relation |  | Ref | =(1) +3xorder | bd1=(2) ,Bd2<(2) | bd1=(2) ,Bd2= (3-MRC) | =(1) +3dB | =(5) -MRC | =(2) +3dB | =(7) -MRC | bd1=(4) +1.7*(IMDorder-1) | =(4)+3dB |

Observations:

- There are new PC2 cases in Release 19 but the interference increase is simple (+3dB) thus it can be modeled with a unique curve for all IMDs clamped at 3dB similar to the proposed curves for PC2 R18 cases.

- There are also new PC1.5 cases that can be easily modeled as they are just a 3dB shift of the proposed curves for PC2 R18 cases.

- This assumes that MRC imbalance changes are ignored like for the PC2 R18 cases.

Proposal: New Release 19 PC2 and PC1.5 cases are specified in a similar way than the R18 cases proposed in this contribution by adding a 3dB shift and/or adjusting the rate of interference increase with the IMD order for post MRC MSD calculations that can be ported to an equation or LUT.

# Conclusion

In this contribution, based on the analysis of all the available HPUE 2UL MSDs in the latest available R18 specification in [2] we make the following proposals.

Proposal: Bands combinations and UL configurations that have incorrectly captured MSDs cannot be considered complete in Release 18:

- MSD captured in the wrong power class table.

- MSD captured for UL configurations that are not specified in the 2Tx or 3Tx CA power class tables.

- Case where there is an PC3 MSD but no PC2 MSD.

- Cases where there is a large PC2 MSD and no PC3 MSD should be justified.

Proposal: HPUE bands combinations and UL configurations that are not consistently captured in the BCS tables and the 2Tx and 3Tx power classes cannot be considered complete in release 18.

Proposal: The HPUE MSD related to a one TDD band UL with intra-band ULCA should be equal to the PC3 value. The Delta MSD is 0dB as the IMD4/5/7/9 levels are clamped by the SEM mask. Many cases need correction for Release 18.

Proposal for release 18 PC2 Delta MSD: The 2UL IMD delta MSD are derived from following equations:

- IMD2 PC2 delta MSD = Min(6 , 0.45*PC3MSD+1.5) [dB]

- IMD3 PC2 delta MSD = Min(9 , 0.6*PC3MSD+3) [dB]

- IMD4 PC2 delta MSD = Min(12 , 0.7*PC3MSD+3) [dB]

- IMD5 PC2 delta MSD = Min(15 , 0.8*PC3MSD+7) [dB]

- FFS if captured in an LUT or use a 0.5dB step ceiling function.

Proposal for release 18 PC1.5 Delta MSD: The 2UL IMD delta MSD using a simplified equation of the form:

- IMDX PC1.5 delta MSD = Min(Z , A*PC3MSD+B) [dB]

- FFS if captured in an LUT or use a 0.5dB step ceiling function.

- If 23dBm+27.7dBm power is considered, Z, A and B can be derived from assuming an interference increase of at least:

  - PC3 interference level + 4.7 * the IMDorder – 1.7dB

  - Other assumptions resulting in a higher interference level increase is not precluded.

Proposal: New Release 19 PC2 and PC1.5 cases are specified in a similar way than the R18 cases proposed in this contribution by adding a 3dB shift and/or adjusting the rate of interference increase with the IMD order for post MRC MSD calculations that can be ported to an equation or LUT.

# References

[1] R4-2417098 WF on HPUE RF requirements, Samsung, Skyworks, Nokia, RAN4#112bis

[2] R4-2417793 Observations on HPUE 2UL IMD MSDs, Skyworks Solutions Inc., RAN4#113

[3] R4-2419699 1UL HPUE NRCA MSD Simplifications for Rel-19, Skyworks Solutions Inc., RAN4#113