# 3GPP TSG-RAN WG4 Meeting #113		R4-2417796

Orlando, US, 18th – 22nd November, 2024

Source:	China Telecom

Title:	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

Agenda Item:	7.16.2

Document for:	discussion

Introduction

In RAN#104 meeting, the Rel-19 new WID on NR demodulation performance: Phase 5 is approved.

The following objectives are approved for UE with 8Rx:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices:

  - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

    - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline

  - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver

    - Reuse the existing interference model for 2/4Rx UE as a baseline

The WF was approved in [1]. In this contribution, our views on the open issues are given.

Discussion

In the following table, we have summarized the key parameters used for MMSE-IRC requirements defined for 2/4Rx UE in Rel-17:

Table 1. Key parameters used for Rel-17 MMSE-IRC requirements for 2/4Rx UE

- PDCSH requirements with inter cell interference - Receiver type: - MMSE-IRC - Network type: - Synchronized network only - Cover both HomNet and Hetnet scenarios with the following applicability rule [2] - ‘If UE supporting both TDD and FDD with same Rx number, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode’ - Interference profile: INR based inter cell interference modeling with the following interference level: - INR1 = 7.77dB and INR2 = 2.29dB for HomNet - INR1 = 7.58dB for HetNet - Random rank with 70% and 30% probability for rank 1 and rank 2 for the interference cell(s) - Test and PDSCH configuration - TDLC300-100, MCS 13, 2x2 ULA Low, rank1 for HomNet - TDLA-30-10, MCS 13, 2x2 ULA Low, rank1 for HetNet - PDCSH requirements with intra cell inter user interference - Receiver type: - MMSE-IRC and R-ML - 1 co-scheduled UE with random 16QAM symbols for MMSE-IRC test - Rank configuration: - Both rank 1+1 and rank 2+2 are covered - Precoding method: - Target UE: Random precoding - Co-scheduled UE: - Random precoding for MMSE-IRC tests with rank 1+1; - Orthogonal precoding with the target UE for MMSE-IRC tests with rank 2+2 and for R-ML tests - Test and PDSCH configuration for the target UE - TDLC300-100, MCS 13, 2x2 and 2x4 ULA Low for MMSE-IRC tests with rank 1+1 - TDLA30-10, MCS 13, 4x4 ULA Low for MMSE-IRC tests with rank 2+2 - DMRS port allocation between target and co-scheduled UEs: - Same DMRS group for rank 1+1 test, i.e., {1000} for target and {1001} for co-UE - Different DMRS groups for rank 2+2 test, i.e., {1000, 1001} for target and {1002, 1003} for co-UE - Wideband CQI reporting requirements with inter-cell interference - Receiver type: - MMSE-IRC - Test criteria: - the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ ; - when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to 0.02. - Configuration for the target cell: - TDLA30-5, 2x2 and 2x4 ULA Low - Configuration for the target cell: - AWGN 1x2 and 1x4, INR = 10.04dB - Tested SINR = -2dB

In general, we think most of the test settings and parameters can be reused for 8Rx UEs. At the same time, we would also like to share our views on the following parameters.

Test applicability rule and release independent

Test applicability rule and release independent

Status in the WF [1]: Test applicability - Option 1: The new requirements for 8Rx MMSE-IRC receiver shall be mandatory requirements for all 8Rx UEs without additional test applicability rule - Option 2: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. - Option 3: Introduce applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, if same test scenarios/configurations as 2RX/4RX are defined. - Option 4: - The new requirements for 8Rx MMSE-IRC receiver to be mandatory requirements for all 8Rx UEs - If agreed to define two set of requirements (baseline and simplified receiver), introduce new applicability rules based on Table 5.1.1.3-1 of TS38.101-4. - Option 5: Introduce applicability rule to skip 2/4Rx requirements if 8Rx tests are passed regardless the concrete test setup. - Other options not precluded. Release independency - Option 1: The new requirements for 8Rx should be release independent from Rel-18 - Option 2: The requirement should be released independent from Rel-18 if no new signalling or UE capability introduced. - Option 3: Postpone the discussion on release independency until agreements are made on baseline assumptions, test parameters and test metrics. - Option 4: Discuss release independence requirements at a later stage in the WI. - Other options not precluded.

MMSE-IRC is a mandatory without capability signaling feature for 2/4Rx UEs from Rel-17. As a new feature for Rel-18, we believe all 8Rx UEs should at least implement MMSE-IRC receiver. Therefore, the new requirements for 8Rx MMSE-IRC receiver should also be mandatory requirements for all 8Rx UEs without additional test applicability rule.

For UEs supporting both 8Rx and 2/4Rx, in Rel-18, we have already introduced test applicability rules to skip the 2/4Rx requirements if the corresponding 8Rx requirements are passed as shown below. I believe the same approach can be reused for the Rel-19 new requirement to be defined.

Table 5.1.1.2-1: Requirements applicability Supported RX antenna ports; Test type; Test list. UE supports 8Rx, 4Rx and 2Rx; PDSCH; All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.4 (Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6. or; PDCCH; All tests in Clause 5.3.3 (Note 2,3). UE supports both 8Rx and 4Rx; PBCH; All tests in Clause 5.4.3 (Note 1). UE supports both 8Rx and 2Rx; PDSCH; All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.4 (Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4. PDCCH; All tests in Clause 5.3.2 (Note 2,4). PBCH; All tests in Clause 5.4.2.

For release independent, the 8Rx UE feature is introduced in Rel-18 and we have already discussed the possibility to cover MMSE-IRC requirements during the performance part discussion under WI ‘NR_ENDC_ RF_FR1_enh2’. Such requirements are not introduced in Rel-18 mainly because of the limited time and huge workload. Therefore, we think the new requirements for 8Rx should be release independent from Rel-18.

Proposal 1: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule

Proposal 2: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Proposal 3: The new requirements for 8Rx should be release independent from Rel-18.

PDCSH requirements with inter cell interference

Interference modelling and power levels

Agreement: - Reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx. - TBD: Further discuss applicability rules to potentially reduce UE effort to only comply to either 1 or 2 interference cells.

In the last meeting, the test scenarios as well as the interference power level are agreed. It is now still FFS on the test applicability rule to reduce the test burden.

In the 2/4Rx requirement definition phase, companies have agreed the following applicability rule for the UE to only select one scenario for TDD and FDD respectively [2]:

‘If UE supporting both TDD and FDD with same Rx number, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode’

In the final test requirement definition, the test applicability rule is as below:

- 5.1.1.10 Applicability of requirements for PDSCH with inter cell interference Tests; Applicability notes. All tests in Clause 5.2.2.1.15, 5.2.3.1.15, 5.2.2.2.16 and 5.2.3.2.16; If UE supporting both duplex mode TDD and FDD with 2RX, only test 1-1 in clause 5.2.2.1.15 and test 1-2 in clause 5.2.2.2.16 will be applied. If UE supporting both duplex mode TDD and FDD with 4RX, only test 1-1 in clause 5.2.3.1.15 and test 1-2 in clause 5.2.3.2.16 will be applied..

We propose to reuse the same test applicability rule as 2/4Rx MMSE-IRC requirements.

Proposal 4: Reuse the same test applicability rule as 2/4Rx MMSE-IRC requirements, i.e., if UE supporting both TDD and FDD with 8Rx, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode.

Other parameters

Agreement - Use 2Tx for interreference cell. - Frequency shift to the target cell: 300Hz for interference cell 1, -100Hz for interference cell 2 - SSB configuration as Rel-17 - Transmission rank: Random rank with 70% and 30% probability for rank 1 and rank 2 for all interference cells Remaining proposals - Option 1: - For PDSCH configuration, DMRS configuration, TRS configuration, HARQ, precoding, Tx EVM, Test metric and other parameters, reuse the configuration from 8Rx requirement as the baseline. - For PCI and SSB configuration of serving cell and interference cell(s), Rank/MCS, precoding, timing and frequency offset, and other interference cell modeling parameters, reuse the configuration from R17 2/4Rx inter-cell interference suppressing requirement as the baseline. - Option 2: For not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC as a starting point for initial evaluation.

For the other remaining parameters, we propose to reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC as a starting point.

Proposal 5: For the other remaining parameters, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC as a starting point.

PDCSH requirements with intra cell inter user interference

Precoding method for the co-scheduled UE

Status in the WF [1]: Agreement: For initial simulation alignment use: - Rank 1+1: random and orthogonal. - Rank 2+2: orthogonal. - Rank 4+4: orthogonal. For orthogonal precoding, select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH

For 2/4Rx MMSE-IRC tests, random precoding was used with rank 1+1 and orthogonal precoding was used for rank 2+2 tests. Same time, orthogonal precoding was used for all R-ML tests.

In our understanding, RAN4 has chosen to use only orthogonal precoding for R-ML tests because the test configurations for R-ML is stricter in terms of MCS and antenna correlation. Higher modulation order or higher antenna correlation makes it hard for the network to use random precoding for co-UE anymore.

Therefore, if we consider using the same test setting and PDSCH configuration as MMSE-IRC, we propose to reuse the same precoding method as MMSE-IRC, i.e., random precoding was used with rank 1+1 and orthogonal precoding was used for rank 2+2 tests.

Proposal 6: Reuse the same precoding method as MMSE-IRC, i.e., random precoding was used with rank 1+1 and orthogonal precoding was used for rank 2+2 tests, if we consider using the same test setting and PDSCH configuration as MMSE-IRC.

DMRS ports

Status in the WF [1]: Agreement: - Rank 1+1 and 2+2, reuse same configuration as Rel-17 - Rank 4+4, use different CDM group for 4+4 case. Serving UE with ports {1000, 1001,1004, 1005} and co-scheduled UE with ports {1002, 1003, 1006, 1007} as a starting point.

For rank 4+4, we are fine to use different CDM group for DMRS ports from different UEs, which is aligned with the test setup for rank 2+2 in Rel-17.

Proposal 7: For rank 4+4, use different CDM group for DMRS ports from different UEs, i.e., Serving UE with ports {1000, 1001,1004, 1005} and co-scheduled UE with ports {1002, 1003, 1006, 1007}.

Definition of MMSE-IRC receiver capability for 8Rx UE

In the last meeting, there were companies pointed it out that the Rel-18 8Rx UE receiver capability definition cannot be applicable for MU-MIMO scenarios:

SU-MIMO 8Rx receiver - Baseline SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector in FR1 - Simplified SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors in FR1.

In Rel-17, instead of introducing new mandatory UE capability for 2/4Rx MMSE-IRC, we only defined applicability rules to clearly state that the MU-MIMO requirements are applicable for all UEs with 2/4Rx. With the common understanding that the 8Rx UE receiver implementation is the same as Rel-18, the following solutions are proposed for RAN4 discussion:

  - Option 1: Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE supporting SU-MIMO 8Rx receiver.

  - Option 2: Update the existing Rel-18 UE capability definition as a maintenance part of the R18 FR1 RF enhancement WI.

  - Option 3: Introduce new Rel-19 UE capabilities for MU-MIMO 8Rx receiver. And clearly state that the UE supporting ‘MU-MIMO 8Rx receiver’ are also required to support ‘SU-MIMO 8Rx receiver’ capability.

Among the above options, option 1 has the lowest impact to the existing specs and is more preferable from us. We also think this issue can be further discussed when we decide the UE applicability rule as well as the release independent.

Proposal 8: Regarding the Rel-19 8Rx UE capability definition for MU-MIMO scenario, the following solutions are proposed for further discussion:

  - Option 1 (more preferable for us): Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE supporting SU-MIMO 8Rx receiver.

  - Option 2: Update the existing Rel-18 UE capability definition as a maintenance part of the R18 FR1 RF enhancement WI.

  - Option 3: Introduce new Rel-19 UE capabilities for MU-MIMO 8Rx receiver. And clearly state that the UE supporting ‘MU-MIMO 8Rx receiver’ are also required to support ‘SU-MIMO 8Rx receiver’ capability.

Wideband CQI reporting requirements with inter-cell interference

CQI calculation algorithm

Status in the WF [1]: - Option 1: Define CQI reporting requirements as below: - Baseline SU-MIMO 8Rx receiver with Baseline CQI calculation - Simplified SU-MIMO 8Rx receiver with Simplified CQI calculation. - Option 2: RAN4 to define CQI requirements for two receiver types: - Baseline receiver: UE performs demodulation and CSI calculation with 8Rx joint processing. - Simplified receiver: UE performs demodulation and CSI calculation with two separate 4Rx processing. - Option 3: Only define one CQI calculation assumption to match both receiver types. - Option 4: Introduce one CQI calculation assumption independent of receiver type. Reconsider if later proven that two CQI calculation assumptions matching the two receiver types are needed. - Option 5: RAN4 to evaluate the throughput ratio for both receiver assumptions based on the agreed parameters/settings. If the throughput ratio is close, then we can go for one requirement. Otherwise, RAN4 can define two sets of requirements for different type of receiver. - Other options not precluded.

We have already defined 2 types of 8Rx receivers in TS38.306. We believe the UE reported CQI should be matched with the PDSCH receiving algorithm. And we support to cover both types of 8Rx receivers thus we propose to define CQI reporting requirements as below:

  - Baseline SU-MIMO 8Rx receiver with Baseline CQI calculation

  - Simplified SU-MIMO 8Rx receiver with Simplified CQI calculation.

Proposal 9: Define CQI reporting requirements as below:

  - Baseline SU-MIMO 8Rx receiver with Baseline CQI calculation

  - Simplified SU-MIMO 8Rx receiver with Simplified CQI calculation.

Test metric

Status in the WF [1]: - Candidate options: - Option 1: Reuse the same test metric for Rel-17 2/4Rx a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥  b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.. - Other options are not precluded

We support to reuse the same test metric for Rel-17 2/4Rx.

Proposal 10: Reuse the same test metric for Rel-17 2/4Rx:

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ 

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

Propagation condition

Status in the WF [1]: - Option 1: Reuse the same configuration for Rel-17 2/4Rx: - TDLA30-5 for the serving cell - AWGN for the interference cell. - Option 2: Further study whether to also include fading channel conditions. - Option 3: CQI requirements under fading channel (If time allows) - Option 4: Consider the assumption assumed for Rel-17 CQI reporting fading tests with inter-cell interference as baseline - Other options not precluded

According to the existing test metric, both AWGN and fading channel are involved in the test. Unless we are trying to define a new test metric for CQI reporting with MMSE-IRC, we propose to reuse the same configuration for Rel-17 2/4Rx, i.e., TDLA30-5 for the serving cell and AWGN for the interference cell.

Proposal 11: Reuse the same configuration for Rel-17 2/4Rx, i.e., TDLA30-5 for the serving cell and AWGN for the interference cell.

Interference power level

Status in the WF [1]: - Option 1: Reuse the same configuration for Rel-17 2/4Rx, i.e., INR = 10.04dB with rank 1 - Other options are not precluded

We support to reuse the same configuration for Rel-17 2/4Rx, i.e., INR = 10.04dB.

Proposal 12: Reuse the same configuration for Rel-17 2/4Rx, i.e., INR = 10.04dB with rank 1

Antenna configuration, rank and MIMO correlation for the serving cell

We are supportive to consider higher rank compared with Rel-17 2/4Rx, we propose to cover rank 1,2 and 4 for initial study and further make decision based on simulation results.

Proposal 13: Cover rank 1,2 and 4 for initial study and further make decision based on simulation results.

Antenna configuration the interference cell

Status in the WF [1]: - Option 1: 1T8R - Other options not precluded

For the interference cell, we support to reuse the same antenna configuration as Rel-17 2/4Rx, i.e., 1T8R.

Proposal 14: For the interference cell, reuse the same antenna configuration as Rel-17 2/4Rx, i.e., 1T8R.

Tested SINR value

In Rel-17, the tested SINR value is chosen based on the simulation results with reasonable performance gain. Therefore, we propose to decide the detailed SINR value based on simulation results.

Proposal 15: The tested SINR value should be decided based on simulation results with reasonable performance gain.

Besides the above proposal, for the other parameters for 8Rx UE new requirements, we propose to reuse the same configuration for Rel-17 2/4Rx as summarized in Table 1.

Proposal 16: For the other parameters for 8Rx UE new requirements, we propose to reuse the same configuration for Rel-17 2/4Rx as summarized in Table 1.

Conclusion

Proposal 1: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule

Proposal 2: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Proposal 3: The new requirements for 8Rx should be release independent from Rel-18.

Proposal 4: Reuse the same test applicability rule as 2/4Rx MMSE-IRC requirements, i.e., if UE supporting both TDD and FDD with 8Rx, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode.

Proposal 5: For the other remaining parameters, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC as a starting point.

Proposal 6: Reuse the same precoding method as MMSE-IRC, i.e., random precoding was used with rank 1+1 and orthogonal precoding was used for rank 2+2 tests, if we consider using the same test setting and PDSCH configuration as MMSE-IRC.

Proposal 7: For rank 4+4, use different CDM group for DMRS ports from different UEs, i.e., Serving UE with ports {1000, 1001,1004, 1005} and co-scheduled UE with ports {1002, 1003, 1006, 1007}.

Proposal 8: Regarding the Rel-19 8Rx UE capability definition for MU-MIMO scenario, the following solutions are proposed for further discussion:

  - Option 1 (more preferable for us): Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE supporting SU-MIMO 8Rx receiver.

  - Option 2: Update the existing Rel-18 UE capability definition as a maintenance part of the R18 FR1 RF enhancement WI.

  - Option 3: Introduce new Rel-19 UE capabilities for MU-MIMO 8Rx receiver. And clearly state that the UE supporting ‘MU-MIMO 8Rx receiver’ are also required to support ‘SU-MIMO 8Rx receiver’ capability.

Proposal 9: Define CQI reporting requirements as below:

  - Baseline SU-MIMO 8Rx receiver with Baseline CQI calculation

  - Simplified SU-MIMO 8Rx receiver with Simplified CQI calculation.

Proposal 10: Reuse the same test metric for Rel-17 2/4Rx:

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ 

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

Proposal 11: Reuse the same configuration for Rel-17 2/4Rx, i.e., TDLA30-5 for the serving cell and AWGN for the interference cell.

Proposal 12: Reuse the same configuration for Rel-17 2/4Rx, i.e., INR = 10.04dB with rank 1

Proposal 13: Cover rank 1,2 and 4 for initial study and further make decision based on simulation results.

Proposal 14: For the interference cell, reuse the same antenna configuration as Rel-17 2/4Rx, i.e., 1T8R.

Proposal 15: The tested SINR value should be decided based on simulation results with reasonable performance gain.

Proposal 16: For the other parameters for 8Rx UE new requirements, we propose to reuse the same configuration for Rel-17 2/4Rx as summarized in Table 1

Reference

R4-2416555 WF for [112bis][320] NR_demod_Ph5_Part2_UE, Nokia, TSG-RAN WG4 Meeting #112bis

R4-2120707, WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC, Intel, 3GPP TSG-RAN WG4 Meeting #101-e, Nov 2021