3GPP TSG-RAN WG4 # 113	R4-2417801

Orlando, USA, 18th Nov 2024 – 22nd Nov 2024

Agenda item:	7.14.2

Source:	MediaTek Inc.

Title:	Discussion on Spatial Channel Modelling

Document for:	Discussion

# Introduction

The Rel-19 study item [1] aims to investigate practical spatial channel modelling methodologies for both SU-MIMO and MU-MIMO demodulation and CSI reporting requirements. It has been identified that the current TDL-based models and spatial correlation matrices poorly match with realistic propagation conditions that have been used as the design assumption for the NR downlink precoder (PMI) codebooks. We also believe that the test scenarios and channel models should match with realistic transmit strategies and channel conditions in order to correctly support UE algorithm design. When RAN4 test cases are unrealistic, the UE algorithm designers may need to compromise between real-world behavior/performance and RAN4 tests.

In our view, the problem with the existing RAN4 spatial correlation models is that they amplify the signal only in the broadside direction (AOD/AOA) of the TX and RX antenna arrays. In order to support a higher number of layers, multiple separate spatial signal directions should exist. This can be achieved by introducing spatial clusters, where each cluster has its own separable mean AOD/AOA.

The current RAN4 MIMO tests employ TDL (Tapped Delay Line) channel models with specific spatial correlation matrices. For spatial channel model development, both CDL (Clustered Delay Line) and TDL models can be considered as baseline. In our earlier contribution, we have described how to achieve any desired spatial channel properties by employing simple “multi-cluster TX-RX beam steering” on top of TDL channels [2].

# Discussion

# 2.1 PDSCH demodulation testing

Cases for SU-MIMO

Agreement:

- Single-User PDSCH:

- 8Rx: 8 Layer, MCS 13 on both codewords (Table 1) (type I codebook) – PMI Choice (FFS Fixed, Random) Companies encouraged to bring Fixed PMI choice to next meeting

- (FFS Whether it is already covered in PMI) 4Rx: 4 Layer, MCS 13 (Table 1)  (type I codebook) – PMI Choice (FFS Fixed, Random) Companies encouraged to bring Fixed PMI choice to next meeting

- 4 CSI-RS Ports (2,1) for 4 Layer

- 8 CSI-RS Ports (4,1) for 8 Layer

For the purpose of aligning simulation results between companies, we have simulated MCS13 PDSCH MIMO demodulation with random precoding and LMMSE receiver for both 4TX-4RX-4L and 8TX-8RX-8L cases with multiple AAV options in CDL-C UMa (827) channel. For the full set of simulation results and details see [3].

As can be seen from Figure 1, for 8TX-8RX-8L, some of the AAV options require very high SNR (>30dB) to reach maximum tput. Without AAV, maximum tput is not reached at any SNR. We believe that using a fixed precoder that matches well with the MIMO channel, a more feasible test setup could be obtained.

Observation #1: 8-layer demodulation with random precoding and LMMSE MIMO demodulation is challenging and not feasible for all evaluated CDL-C UMa AAV cases.

Proposal #1: Instead of random precoding, we propose using a carefully chosen fixed precoder for 8-layer PDSCH demodulation testing in CDL channels.

Additionally, we have observed that for 4L and 8L MIMO, R-ML demodulation can provide significant performance gains compared to LMMSE. We therefore urge all companies to employ strictly LMMSE demodulation to enable alignment of the results.

Observation #2: For 4L and 8L MIMO, R-ML demodulation can provide significant performance gains compared to LMMSE demodulation.

Proposal #2: We propose that companies employ strictly LMMSE MIMO demodulation to enable alignment of the results.

In addition to CDL channels, we have simulated our multi-cluster TX-RX beam steering extension on top of TDL, as described in [2]. Here, the channel comprises a small number of clusters, each cluster employing a TDL channel model with XP-High spatial correlation. By setting a handful of cluster direction and power parameters, almost arbitrary spatial channel properties can be achieved, without the need of AAV modelling. For the simulated parameter details see [3]. The results are plotted for comparison in Figure 1.

Proposal #3: We propose RAN4 to consider the option of multi-cluster TX-RX beam steering with TDL model for spatial channel modelling as a much simpler and controllable choice than CDL.

Figure 1: 4TX-4RX-4L and 8TX-8RX-8L PDSCH demodulation tput.

Cases for MU-MIMO

Agreement:

- Interested companies to bring to RAN4#113 views on the following cases:

- PDSCH

  - 2+2 layers with IRC (type I orthogonal and random precoding, to distinguish target and co-scheduled UE)

  - Interested companies are encouraged to assess 2+2 layers with R-ML / E-IRC (type I orthogonal and random precoding, to distinguish target and co-scheduled UE)

- Proponents to highlight the difference between a SCM and TDL for MU-MIMO.

We see SU-MIMO as the priority for alignment and channel model observations. However, when SU-MIMO simulation results and analysis are widely available we are open to study also MU-MIMO scenarios.

Proposal #4: We propose to focus on SU-MIMO first but discuss potential MU-MIMO scenarios for study.

# 2.2 Follow-PMI testing

Cases for SU-MIMO

Agreement:

- Single-User PMI

- 4Rx: 4 Layer (type I) – Full Throughput Curves (PMI Follow, PMI Random)

- 4Rx: 4 Layer (eType II) – Full Throughput Curves (PMI Follow)

- 4Rx: 2 Layer (type I) – Full Throughput Curves (PMI Follow, PMI Random)

- 4Rx: 2 Layer (eType II) – Full Throughput Curves (PMI Follow)

- Interested companies can use initially 8 CSI-RS Ports, more ports can be used to identify and show relevant impacts.

- Note: Several companies would prefer to focus on PDSCH until the modelling is stable

We simulated 8TX-4RX follow PMI performance with LMMSE receiver for both 2L and 4L cases with multiple AAV options in CDL-C UMa (827) channel. For the full set of simulation results and parameter details see [3]. These simulations help in identifying feasible AAV options for follow-PMI testing.

As can be seen from Figure 2, for rank 2 with AAV via fixed TX-BF as specified in TR38.827, the SNR gain from Type1 precoding vs random precoding is negligible (up to 1dB at 90% tput), while the gain from eType2 remains modest (up to 3dB). Furthermore, simple fixed port selection precoding mostly outperforms both Type1 and eType2 follow PMI. This suggests that further precoding on top of the fixed beamforming does not make much sense.

Observation #3: CDL-C UMa with AAV via fixed TX-BF as specified in TR38.827, provides only modest gains for Type1/eType2 follow-PMI when compared to random precoding.

Proposal #5: We propose that AAV via fixed TX-BF as specified in TR38.827 is not used for follow-PMI testing.

As can be seen from Figure 2, for rank 2 without AAV, the SNR gain from both Type1 and eType2 precoding vs random precoding is very high (more than 7 dB at 90% tput). Thus, it can be concluded that using no AAV provides a good setup for testing PMI feedback.

Observation #4: CDL-C UMa without AAV, provides significant gains for both Type1 and eType2 follow-PMI when compared to random precoding.

Proposal #6: We propose to consider no AAV for follow-PMI testing in CDL channels.

Figure 2: Examples of 8TX-4RX follow-PMI tput in CDL-C.

One of the issues raised by some companies is that we should find use cases where the potential tput gain of eType2 over Type1 precoding is demonstrated. According to our simulations, eType2 provides no gain when the UE speed or Doppler is high (30km/h / 100Hz). Furthermore, for rank 4, eType2 does not perform as well as for rank 2. Therefore, we believe eType2 PMI testing is best feasible with low Doppler and low rank.

Observation #5: eType2 precoding provides tput gain over Type1 precoding only when the UE speed (Doppler) is low, and only when the rank is low (2).

Proposal #7: We propose that eType2 follow PMI testing should concentrate to low UE speed or Doppler, and rank 2.

In addition to CDL channels, we have simulated an example of our multi-cluster TX-RX beam steering extension on top of TDL, as described in [2]. For the simulated parameter details see [3]. The PMI-follow results are plotted for comparison in Figure 3. As can be seen, solid precoding gain against random precoding is achieved with both Type1 and eType2 precoding. Furthermore, at 10Hz Doppler, eType2 provides SNR gain of 0.6dB against Type1.

Observation #6: Simple multi-cluster TX-RX beam steering with TDL-C channel is well feasible for both Type1 and eType2 follow-PMI testing.

Figure 3: 8TX-4RX follow-PMI tput with fixed multi-cluster TX-RX beam steering in TDL-C.

# 2.3 Discussion of CDL-C UMa FR1 channel model

In this section, we share some observations about implementing and simulating the CDL-C UMa FR1 channel model. We have noticed that implementing the CDL model is not trivial as the many details of the model are scattered between multiple TRs and their many sections. Therefore, there is a need for a more focused and self-contained description of the link-level model.

Proposal #8: The upcoming TR should provide self-contained and focused descriptions of the agreed channel models.

One of our observations is that due to different AAV beamforming options, the average channel power response does not necessarily converge to one (while for legacy RAN4 channel models it does). Therefore, the definition of SNR must be agreed on to align the results. We have defined SNR in our results by measuring the average power of the 4x4, 8x8, and 8x4 channels over the simulation, and by adding this result to SNR in dB. However, it would be better to have a power normalization factor available prior to simulation. Note that without AAV, additional power normalization is not needed.

Observation #7: With different AAV TX beamforming options, the average power response of the CDL-C UMa FR1 is not equal to one.

Proposal #9: Depending on each AAV and spatial CDL channel combination, a numerical transmit beamformer power normalization factor should be specified to ensure unit power response.

Subcluster Correlation

Proposals:

- Option 1: RAN4 should discuss and clarify whether the potential correlation between subclusters is an issue, and whether there should be mechanisms to avoid it.

Another observation is that the CDL-C model includes subclusters for which the propagation delays and powers differ, but the TX-RX (departure/arrival) angles are identical as shown by the TR-snippet in Figure 4. Due to identical ray-specific incident angles, the ray-specific Doppler shifts for the subclusters appear to become identical as well. Thus, there is a risk that the fading processes between the sub-clusters becomes correlated.

However, TR 38.827 and TR 38.901 have specified a complicated special treatment for the sub-clusters as highlighted by the TR-snippet in Figure 5. The intention is that instead of allocating 20 rays for every cluster, the subclusters each get only a subset of the 20 rays. Specifically, the 1st sub-cluster has 10 rays, the 2nd subcluster 6 rays, and the 3rd subcluster 4 rays.

Observation #8: There is a risk that the fading processes between the sub-clusters become correlated unless ray-splitting is performed.

Proposal #10: To avoid inter-cluster correlation, RAN4 companies should make sure they are performing ray-splitting for the sub-clusters as specified by TR 38.827 and TR 38.901.

Figure 4: Channel model parameters for UMa CDL-C at 3.5 GHz.

Figure 5: Handling of sub-clusters via ray-splitting according to TR 38.827.

Radiation Pattern

Proposals:

- Option 1: For the purposes of CDL alignment, disregard radiation pattern parameters included in TR 38.827 Section 7.2 and consider Omnidirectional antenna pattern

- Option 2: The antenna pattern of a single BS antenna element could use the radiation power pattern defined in 38.901 Table 7.3-1

We have used antenna radiation pattern for BS as proposed in Option 2 in our simulations and we see it as feasible options.

Proposal #11: We propose to use BS radiation pattern as defined in TR38.901 Table 7.3-1.

Proposal #12: We propose to use Omnidirectional antennas for UE.

Frequency Bands

Proposals:

- Option 1: RAN4 to study if the same CDL channel model can be defined for all carrier frequencies/ bands in given frequency range

We see it valuable feasibility check to verify channel model in more bands and carrier frequencies. However, we see that we should focus first to 3.5GHz carrier frequency and check other frequencies in the end of feasibility study.

Proposal #13: We propose to focus on 3.5GHz carrier frequency first but check other frequencies in the end of feasibility study.

Time Varying Beam Direction

Proposals:

- Option 1: Further evaluate the method for time variant beam directions in CDL model.

Existing TDL channel models implement time varying beam direction for more extensive PMI testing. We would like to consider similar possibility available for CDL channels too.

Proposal #14: We propose to further evaluate the method for time variant beam directions in CDL model.

UE Speed

Agreement

Interested companies to use 30 km/h and/or 3 km/h for UE speed/velocity.

TDL doppler values to be used for simulation:

- 100 Hz for 30 km/h

- 10 Hz for 3 km/h

Assumption: 3.5 GHz carrier to be used for simulations (aligned with FR1)

In demodulation simulations for this meeting, we have used UE speed of 30 km/h, whereas for PMI simulations we have used both UE speeds 3 and 30 km/h. We see it important to evaluate proposed channel models in lower and higher doppler conditions.

Proposal #15: We propose to evaluate proposed channel models in lower and higher doppler conditions.

Antenna Coordinates, Angles, and other parameters for gNB (for UMa-CDL-C)

Proposals:

- Option 1: For the gNB, use the following parameters for alignment with UMa CDL-C model:

  - GCS Location Coordinates for the gNB:

  - Height = 25m (based on UMa);

  - Azimuth = 0; (placement on the x axis);

  - GCS to LCS Conversion angles (α,β,γ = bearing, downtilt, slant) for the gNB;

  - α,β,γ=(0,0,0);

- Option 2:

  - Frequency Fc = 3.5 GHz, height BS = 25m, and D2D = 100m.

  - BS orientation: (alpha = 0 degrees, beta = 0 degrees, gamma = 0 degrees). Intention is that BS points towards horizon along x-axis.

- Other options not precluded

We have used BS (α, β, γ) = (0°, 0°, 0°) and BS polarization slant = (45°, -45o) in our simulations. More specifically we have directly followed TR38.827 Table 7.2.1-8: Channel model parameters for UMa CDL-C at 3.5 GHz as is that is calculated with assumptions of hUT = 1.5 m, and d2d = 100 m.

Proposal #16: We propose to use BS (α, β, γ) = (0°, 0°, 0°).

In premeeting email discussion there seem to be proposals to use 10 degree mechanical down tilt. We are also fine with that but in this case, we could update Table 7.2.1-8: Channel model parameters for UMa CDL-C at 3.5 GHz to match updated down tilt assumption.

Proposal #17: We are also fine with (α, β, γ) = (0°, 10°, 0°).

Proposal #18: We propose to use BS polarization slant = (45°, -45°).

Antenna Coordinates, Angles, and other parameters for UE (for UMa-CDL-C)

Proposals:

- Option 1: For the UE, use the following parameters for alignment with UMa CDL-C model:

  - Height = 25m (same as gNB)

  - Azimuth = 0; (placement on the x axis);

  - α,β,γ=(90,0,45);

- Option 2:

  - Frequency Fc = 3.5 GHz, height UE = 1.5m and D2D = 100m.

  - UE orientation: (alpha = -90 degrees, beta = -13.2 degrees, gamma = 45 degrees). Intention is that UE points towards BS and uses "[+ + ... +]" crosspol elements.

- Other options not precluded.

We have used UE (α, β, γ) = (180°, 0°, 0°) and UE polarization slant = (0°, 90o) in our simulations.

Proposal #19: We propose to use UE (α, β, γ) = (180°, 0°, 0°).

Proposal #20: We propose to use UE polarization slant = (0°, 90°).

BS Antenna Virtualiser choice

Proposals:

- Option 1: Down Tilt Virtualiser : -10 degrees

- Option 2: Broadside

- Option 3: Max Receive Power

- Option 4: DFT Virtualiser according to TR 36.897 with application of panning and tilting angles (as per Table 7.8-1 of 38.901)

We have used broadside BS antenna virtualizer in our simulations. From premeeting email discussions, we have understood that most of the companies prefer mechanical down tilt over digital down tilt. Therefore, we see that we can keep broadside digital antenna virtualizer and discuss if we need or want 10-degree mechanical down tilt or not.

Proposal #21: We propose to use broadside antenna virtualizer.

# Conclusion

In this paper we provided the view on the spatial channel modelling. The following observations and proposals are made:

Observation #1: 8-layer demodulation with random precoding and LMMSE MIMO demodulation is challenging and not feasible for all evaluated CDL-C UMa AAV cases.

Proposal #1: Instead of random precoding, we propose using a carefully chosen fixed precoder for 8-layer PDSCH demodulation testing in CDL channels.

Observation #2: For 4L and 8L MIMO, R-ML demodulation can provide significant performance gains compared to LMMSE demodulation.

Proposal #2: We propose that companies employ strictly LMMSE MIMO demodulation to enable alignment of the results.

Proposal #3: We propose RAN4 to consider the option of multi-cluster TX-RX beam steering with TDL model for spatial channel modelling as a much simpler and controllable choice than CDL.

Proposal #4: We propose to focus on SU-MIMO first but discuss potential MU-MIMO scenarios for study.

Observation #3: CDL-C UMa with AAV via fixed TX-BF as specified in TR38.827, provides only modest gains for Type1/eType2 follow-PMI when compared to random precoding.

Proposal #5: We propose that AAV via fixed TX-BF as specified in TR38.827 is not used for follow-PMI testing.

Observation #4: CDL-C UMa without AAV, provides significant gains for both Type1 and eType2 follow-PMI when compared to random precoding.

Proposal #6: We propose to consider no AAV for follow-PMI testing in CDL channels.

Observation #5: eType2 precoding provides tput gain over Type1 precoding only when the UE speed (Doppler) is low, and only when the rank is low (2).

Proposal #7: We propose that eType2 follow PMI testing should concentrate to low UE speed or Doppler, and rank 2.

Observation #6: Simple multi-cluster TX-RX beam steering with TDL-C channel is well feasible for both Type1 and eType2 follow-PMI testing.

Proposal #8: The upcoming TR should provide self-contained and focused descriptions of the agreed channel models.

Observation #7: With different AAV TX beamforming options, the average power response of the CDL-C UMa FR1 is not equal to one.

Proposal #9: Depending on each AAV and spatial CDL channel combination, a numerical transmit beamformer power normalization factor should be specified to ensure unit power response.

Observation #8: There is a risk that the fading processes between the sub-clusters become correlated unless ray-splitting is performed.

Proposal #10: To avoid inter-cluster correlation, RAN4 companies should make sure they are performing ray-splitting for the sub-clusters as specified by TR 38.827 and TR 38.901.

Proposal #11: We propose to use BS radiation pattern as defined in TR38.901 Table 7.3-1.

Proposal #12: We propose to use Omnidirectional antennas for UE.

Proposal #13: We propose to focus on 3.5GHz carrier frequency first but check other frequencies in the end of feasibility study.

Proposal #14: We propose to further evaluate the method for time variant beam directions in CDL model.

Proposal #15: We propose to evaluate proposed channel models in lower and higher doppler conditions.

Proposal #16: We propose to use BS (α, β, γ) = (0°, 0°, 0°).

Proposal #17: We are also fine with (α, β, γ) = (0°, 10°, 0°).

Proposal #18: We propose to use BS polarization slant = (45°, -45°).

Proposal #19: We propose to use UE (α, β, γ) = (180°, 0°, 0°).

Proposal #20: We propose to use UE polarization slant = (0°, 90°).

Proposal #21: We propose to use broadside antenna virtualizer.

# Reference

- RP-241610, “3GPP Work Item Description: Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

- R4-2415382, “Discussion on Spatial Channel Modelling”, MediaTek Inc

- R4-2417802, “Simulation results of spatial channel modelling”, MediaTek Inc