3GPP TSG-RAN WG4 Meeting #113	R4-2417804

Orlando, USA, 18th Nov 2024 – 22nd Nov 2024

Agenda item:	7.17.2.2

Source:	MediaTek Inc.

Title:	Discussion on CSI compression AIML two-sided model logistics

Document for:	Discussion

# Introduction

In RAN4#112-bis, RAN4 agreed interested companies to share decoder and encoder ML models and dataset in next meeting (RAN4#113) [1][2]. In this contribution, we provide information on our data sharing.

# Discussion

In this Chapter, we provide information on our data sharing.

Issue 4-1: Simulation results discussion for Option 3

Agreement:

Next step:

Interested companies to share decoder and encoder ML models and dataset in next meeting (RAN4#113)

- Model and dataset used to generate the data submitted to RAN4#112-bis

We share decoder, encoder, and dataset in this meeting.

Agreement for AI/ML formats:

- Use at least ONNX format for sharing.

- Companies can also share PyTorch model

- The models in different formats from one company shall be the same

- Use version v1.16 or later

- ONNX version and opset version number to be included in the file

We share the AI/ML models in ONNX format version 1.16.1.

Agreement for dataset format:

- Use NumPy for dataset sharing

- Do not use pickled data(compression mechanism)

- Use npy – single array in each file

We share the dataset in NumPy format (.npy) in single array that is not pickled.

Agreement for training data file format:

Dataset/model input file format:

    - N (samples) X 2 (IQ) X nSB (number of subbands) X nPorts (number of CSI-RS ports) X nLayers (number of layers)

        - In other words, dataset should content following info:

        - 1st dimension: Number of samples

        - 2nd dimension: Real and imaginary

        - 3rd dimension: Number of sub-bands

        - 4th dimension: Number of antenna ports

        - 5th dimension: Number of layers – present only if >1 layer

        - Note: Each element of the dataset will be a float32 real number

The dataset includes eigenvectors calculated based on average covariance channel matrix for the given MIMO layer for each sub-band based on pre-processing assumptions in WF R4-2414447.

Single file for fixed scalar quantization (only dataset), TBD whether additional file is needed for variable/trainable quantization.

Dataset files can be split into multiple files to enable easier upload.

- Use the 2 digits for split files starting from 00, increment for each additional file.

Split files and then archive each file.

We share the dataset in requested 5-dimensional array with dimensions (600000, 2, 13, 32, 2).

Agreement:

- Folders for AI/ML data sharing and current WI/use case to be created under “RAN4 folder”.

- Subfolder created for each meeting.

File naming scheme (ML model file and dataset file):

- a unique identifier for the company (4 characters – list to be maintained by RAN4 secretary)

| Source | Identifier | Secretary Remarks |
| --- | --- | --- |
| MediaTek | MTK0 | This is for AI/ML data sharing |

- meeting number

- differentiate model and dataset by identifier (2 characters ml and ds)

- additional identifier with 2 characters (TBD)

  - ec for encoder

  - dc for decoder

  - ei for encoder input

  - do for decoder output

  - lt for latent (decoder input or encoder output)

  - others to be added as needed

- dataset can be split in multiple files – 2 digits

- files to be compressed to zips and uploaded

We share models and dataset with the following filenames compressed to zips.

- Encoder model with filename: MTK0R4_113mlec.onnx

- Decoder model with filename: MTK0R4_113mldc.onnx

- Dataset with filename: MTK0R4_113dsei.npy

Note, each zip file also contains copyright notice letter for your reference.

# Conclusion

In this contribution we provide information on our data sharing.

# Reference

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm inc.

- R4-2414323, WF on AI/ML, Qualcomm inc.