3GPP TSG-RAN WG4 Meeting #112                                                                                       R4-2417829

Orlando, US, 18th –22nd November, 2024

Source: 	CATT

Title: 	Further discussion on system parameters for Ku-band for NR NTN

Agenda item:	7.9.3

Document for:	Approval

- Introduction

In RAN4#112bis meeting, a WF[1] on NTN Ku band system parameter was agreed and remaining issues on Ku-band system parameter in WF require further discussion.

In this contribution, we provide our views on system parameters for Ku-band for NR NTN.

- Discussion

2.1 Frequency range designation for Ku-band

Max Doppler shift after pre/post compensation and SCS

The Max Doppler shift after pre/post compensation for S-band and Ka-band in Table 6.1.1.1-8 of TR 38.821 is copied as below:

| Max Doppler shift if pre/post compensation mechanism is assumed at satellite payload side | Scenario C2/D2: ● Satellite alt. = 600 km - beam diameter = 50 km (Set 1 - S-band): 1.05 ppm - beam diameter = 20 km (Set 1 - Ka-band): 0.42 ppm - beam diameter = 90 km (Set 2 - S-band): 1.88 ppm - beam diameter = 50 km (Set 2 - Ka-band): 1.05 ppm - beam diameter = 1000 km (max beam foot print size): 15.82 ppm |
| --- | --- |

Referring to above table, the max Doppler shift after pre/post compensation for S-band and Ka-band is 1.88ppm and 1.05ppm, respectively. so max Doppler shift after pre/post compensation for Ku-band can be calculated as (1.88+1.05)/2 =1.465ppm, which is corresponding to 20.51kHz for 14GHz, Therefore, from perspective of Doppler shift, a SCS of at least twice 20.51 kHz is required to overcome the Doppler effect. So, 60 kHz and wider SCSs are more applicable for the Ku-band.

Observation 1:  From perspective of max Doppler shift, the 60kHz SCS and wider is more applicable for Ku-band.

For SCS of data signal, 60 kHz and120 kHz SCS are specified for FR2-NTN, and 15 kHz, 30 kHz, and 60kHz SCS are specified for FR1-NTN. From perspective of Doppler shift, for FR2-NTN, 60 kHz and120 kHz SCS of data signal are applicable for Ku-band, and for FR1-NTN, only 60kHz SCS of data signal is applicable for Ku-band.

Observation 2: From perspective of max Doppler shift, for FR2-NTN, 60 kHz and120 kHz SCS of data signal are applicable for Ku-band, and for FR1-NTN, only 60kHz SCS of data signal is applicable for Ku-band.

For SCS of SSB signal, the 120 kHz and 240 kHz SCS are specified for FR2-NTN SSB, and 15 kHz, 30 kHz SCS are specified for FR1-NTN SSB. So, From perspective of Doppler shift, 120 kHz and 240 kHz SCS specified for FR2-NTN SSB are applicable for Ku-band, and 15 kHz, 30 kHz SCS specified for FR1-NTN SSB are not applicable for Ku-band.

Observation 3: From perspective of max Doppler shift, 120 kHz and 240 kHz SCS specified for FR2-NTN SSB are applicable for Ku-band, and 15 kHz, 30 kHz SCS specified for FR1-NTN SSB are not applicable for Ku-band.

UL timing accuracy and SCS

The UL timing error limit Te_NTN in Table 7.1C.2-1 for FR1-NTN, Table 7.1C.2-2 for fixed VSAT, and Table 7.1C.2-3 for mobile VSAT in TS 38.133 are copied as below:

Table 7.1C.2-1: Te_NTN Timing Error Limit Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te_NTN. FR1-NTN; 15; 15; 29*64*Tc. 30; 24*64*Tc. 60; N/A. 30; 15; 24*64*Tc. 30; 22*64*Tc. 60; N/A. Note 1: Tc is the basic timing unit defined in TS 38.211 [6]. Table 7.1C.2-2: Te_NTN Timing Error Limit for fixed VSAT is served by GSO and fixed VSAT is served by NGSO Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te_NTN. FR2-NTN; 120; 60; 13*64*Tc. 120; 7.5*64*Tc. 240; 60; 13*64*Tc. 120; 7.5*64*Tc. Note 1: Tc is the basic timing unit defined in TS 38.211 [6]. Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te_NTN. FR2-NTN; 120; 60; 13*64*Tc. 120; [10]*64*Tc. 240; 60; 13*64*Tc. 120; [10]*64*Tc. Note 1: Tc is the basic timing unit defined in TS 38.211 [6].

The agreement on Te_NTN for 60kHz and 120kHz in WF [2] in RAN4#108bis meeting are copied as below:

From Infra vendor’s perspective, the network margin Tm in above is more important factor for gNB receiver, using Ts = 64*Tc (defined in TS38.211) as unit, the UL timing error limit Te_NTN = Td + Tp + Tr + Ta + Tf + Tm, so the Tg = 0.5Tcp - Te_NTN, where Tcp is in Ts unit. The Tg can be considered as additional margin, which can be used for Tm, So the additional margin for FR1-NTN, FR2-NTN for fixed VSAT, and FR2-NTN for mobile VSAT are shown in following Table 7.1C.2-1, Table 7.1C.2-2, and Table 7.1C.2-3, respectively.

Table 7.1C.2-1: Te_NTN Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN | 0.5Tcp | Additional margin (Tg) |
| --- | --- | --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc | 72*64*Tc | 43*64*Tc |
|  |  | 30 | 24*64*Tc | 36*64*Tc | 12*64*Tc |
|  |  | 60 | N/A | 18*64*Tc | N/A |
|  | 30 | 15 | 24*64*Tc | 72*64*Tc | 48*64*Tc |
|  |  | 30 | 22*64*Tc | 36*64*Tc | 14*64*Tc |
|  |  | 60 | N/A | 18*64*Tc | N/A |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |  |

Table 7.1C.2-2: Te_NTN Timing Error Limit for fixed VSAT is served by GSO and fixed VSAT is served by NGSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN | 0.5Tcp | Additional margin (Tg) |
| --- | --- | --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc | 18*64*Tc | 5*64*Tc |
|  |  | 120 | 7.5*64*Tc | 9*64*Tc | 1.5*64*Tc |
|  | 240 | 60 | 13*64*Tc | 18*64*Tc | 5*64*Tc |
|  |  | 120 | 7.5*64*Tc | 9*64*Tc | 1.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |  |

Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN | 0.5Tcp | Additional margin (Tg) |
| --- | --- | --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc | 18*64*Tc | 5*64*Tc |
|  |  | 120 | [10]*64*Tc | 9*64*Tc | -1*64*Tc |
|  | 240 | 60 | 13*64*Tc | 18*64*Tc | 5*64*Tc |
|  |  | 120 | [10]*64*Tc | 9*64*Tc | -1*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |  |

Based on above discussion on Max Doppler shift, here we only analyse Te_NTN Timing Error Limit for 60kHz and 120kHz SCS of uplink signals. The above Table 7.1C.2-1 shows that Te_NTN for 60kHz SCS of uplink signals for FR1-NTN is not applicable, so there is no additional margin (Tg) can be used for Tm needed at gNB receiver. The above Table 7.1C.2-2 shows Te_NTN defined 60kHz and 120kHz SCS of uplink signals for FR2-NTN for fixed VSAT have at least 1.5*64*Tc additional margin can be used for Tm needed at gNB receiver. And the Table 7.1C.2-3 shows that Te_NTN defined 60kHzSCS of uplink signals for FR2-NTN for mobile VSAT have at least 5*64*Tc additional margin can be used for Tm needed at gNB receiver, but Te_NTN defined 60kHzSCS of uplink signals for FR2-NTN for mobile VSAT have not additional margin can be used for Tm needed at gNB receiver. For 60kHz and 120kHz SCS of uplink signals, Te_NTN for 60kHz SCS of uplink signals for FR1-NTN is not defined and not applicable for Ku-band, Te_NTN defined 60kHz and 120kHz SCS of uplink signals for FR2-NTN for fixed VSAT is applicable for Ku-band, and Te_NTN defined 60kHzSCS of uplink signals for FR2-NTN for mobile VSAT is applicable for Ku-band.

Observation 4: From perspective of UL timing error limit, Te_NTN for 60kHz SCS of uplink signals for FR1-NTN is not defined and not applicable for Ku-band, Te_NTN defined 60kHz and 120kHz SCS of uplink signals for FR2-NTN for fixed VSAT is applicable for Ku-band, and Te_NTN defined 60kHzSCS of uplink signals for FR2-NTN for mobile VSAT is applicable for Ku-band.

Phase noise and SCS

Phase noise causes inter-carrier interference, and larger SCS implies greater phase noise robustness. The 60kHz and 120kHz SCS are specified for Ka-band, which have good phase noise robustness, and 64QAM EVM is defined for FR2-NTN in TS 38.108. So we think that for 14GHz Ku-band NTN, the 60kHz and 120kHz SCS also have good phase noise robustness for Ku-band.

Beam hopping granularity and SCS

According to agreements in coexistence, it was assumed that the antenna gain and beamwidth fo Ku-band SAN are same as those for Ka-band SAN, we believe that beam hopping granularity of Ku-band should be same as that of Ka-band, and thus the SCS of SSB for Ku-band can be same as that for Ka-band.

CBW and SCS

The transmission bandwidth configuration for FR1-NTN and FR2-NTN in TS 38.108 are copied as below:

### 5.3.2 Transmission bandwidth configuration The transmission bandwidth configuration NRB for each SAN channel bandwidth and subcarrier spacing is specified in table 5.3.2.-1 for FR1-NTN and table 5.3.2-2 for FR2-NTN. Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1-NTN SCS (kHz); 5 MHz; 10 MHz; 15 MHz; 20 MHz; 30 MHz. NRB; NRB; NRB; NRB; NRB. 15; 25; 52; 79; 106; 160. 30; 11; 24; 38; 51; 78. 60; N/A; 11; 18; 24; 38. Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-NTN SCS (kHz); 50 MHz; 100 MHz; 200 MHz; 400 MHz. NRB; NRB; NRB; NRB. 60; 66; 132; 264; N/A. 120; 32; 66; 132; 264. NOTE: All Tx and Rx requirements are defined based on transmission bandwidth configuration specified in table 5.3.2-1 for FR1-NTN and table 5.3.2-2 for FR2-NTN.

Larger SCS implies larger bandwidth, according to above transmission bandwidth, for CBW <50MHz, the SCS for FR1-NTN is more reasonable, but for CBW>50MHz, the SCS for FR2-NTN is more reasonable.

Based on above analysis, we think that the 120kHz SCS of SSB and 60kHz or 120kHz SCS of data signal for FR2-NTN are more applicable for Ku-band.

Observation 5: The 120kHz SCS of SSB and 60kHz or 120kHz SCS of data signal for FR2-NTN are more applicable for Ku-band.

Based on above observations, we prefer to extend FR2-NTN downwards for Ku-band as following table 2-1.

Table2-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 7125 |
| FR2-NTN (NOTE 2) | 10700 – 30000 |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2 band when references from other specifications. |  |

Proposal 1: Extend FR2-NTN downwards for Ku-band as following table 2-1:

Table2-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 7125 |
| FR2-NTN (NOTE 2) | 10700 – 30000 |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2 band when references from other specifications. |  |

- Conclusion

This contribution provides analysis on system parameters for Ku-band for NR NTN, and the following proposals and observations are concluded as follows:

Observation 1:  From perspective of max Doppler shift, the 60kHz SCS and wider is more applicable for Ku-band.

Observation 2: From perspective of max Doppler shift, for FR2-NTN, 60 kHz and120 kHz SCS of data signal are applicable for Ku-band, and for FR1-NTN, only 60kHz SCS of data signal is applicable for Ku-band.

Observation 3: From perspective of max Doppler shift, 120 kHz and 240 kHz SCS specified for FR2-NTN SSB are applicable for Ku-band, and 15 kHz, 30 kHz SCS specified for FR1-NTN SSB are not applicable for Ku-band.

Observation 4: From perspective of UL timing error limit, Te_NTN for 60kHz SCS of uplink signals for FR1-NTN is not defined and not applicable for Ku-band, Te_NTN defined 60kHz and 120kHz SCS of uplink signals for FR2-NTN for fixed VSAT is applicable for Ku-band, and Te_NTN defined 60kHzSCS of uplink signals for FR2-NTN for mobile VSAT is applicable for Ku-band.

Observation 5: The 120kHz SCS of SSB and 60kHz or 120kHz SCS of data signal for FR2-NTN are more applicable for Ku-band.

Proposal 1: Extend FR2-NTN downwards for Ku-band as following table 2-1:

Table2-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 7125 |
| FR2-NTN (NOTE 2) | 10700 – 30000 |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2 band when references from other specifications. |  |

- References

- R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#112bis

- R4-2317374,RRM WF and CR work split for NR_NTN_enh,Moderator (Qualcomm Incorporated),RAN4#108bis

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#112

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104