3GPP TSG-RAN WG4 Meeting #113                                                                                                      R4-2417832

Orlando, US, 18th –22nd November, 2024

Source: 	CATT

Title: 	Further discussion on UE spurious emissions for UE in mmWave in NR

Agenda item:	6.21.2

Document for:	Approval

- Introduction

In RAN4#112bis meeting, A WF[1] on UE RF requirements for EESS protection was agreed and remaining issues on UE RF requirements for EESS protection need to further discussion.

In this contribution, we provide our views on UE RF requirements for EESS protection.

- Discussion

General requirements for n257

As per the WID [1], the objectives concerning general requirement for n257 is shown as below:

FFS to define the EESS protection requirements as general requirements for n257.

For spurious emission requirement, the introduction of additional spurious emission requirements of -5dBm/200Mz for 23.6-24 GHz frequency range for n257 and n258 will impact the following parts in TS 38.101-2:

- 6.5.3.1	Spurious emission band UE co-existence

- 6.5.3.2	Additional spurious emissions

- 6.5A.3.1 Spurious emission band UE co-existence for UL CA

- 6.5A.3.2	 Additional spurious emissions

For spurious emission band UE co-existence in sub-clause 6.5.3.1 of TS 38.101-2, the spurious emission in Table 6.5.3.1-1 is copied as below:

Table 6.5.3.1-1: Requirements

| NR Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band/frequency range | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n257 | NR Band n260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
|  | Frequency range | 23600 | - | 24000 | 1 | 200 | 3 |
| n258 | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n259 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 36000 | - | 37000 | 7 | 1000 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n260 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n261 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n262 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n263 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 258 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 259 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
| NOTE 1: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 NOTE 2: Void NOTE 3: The protection of frequency range 23600-24000 MHz is meant for protection of satellite passive services. |  |  |  |  |  |  |  |

According to CR[2], the spurious emission band UE co-existence for n257 in sub-clause 6.5.3.1 of TS 38.101-2 and spurious emission band UE co-existence for UL CA for n257 in sub-clause 6.5A.3.1 of TS 38.101-2 is general requirement.

Observation 1: The spurious emission band UE co-existence for n257 in sub-clause 6.5.3.1 of TS 38.101-2 and spurious emission band UE co-existence for UL CA for n257 in sub-clause 6.5A.3.1 of TS 38.101-2 is general requirement.

According to WID[1], to support the commercial deployment, the more stringent UE spurious requirement of −35 dBW/200 MHz or -5dBm/200MHz is needed according the studies on mmWave usage.

- The -5dBm/200MHz on 23.6GHz-24GHz UE spurious emission for commercial deployment needs to be satisfied in China.

- EU Decision 2020-590 has decided to implement −35 dBW/200 MHz (i.e. equivalent to -5dBm/200MHz) on 23.6GHz-24GHz from Jan. 1, 2024.

From our understanding, the updated -5dBm/200MHz is for additional requirement instead of general requirement, so the existing EESS protection of 1dBm/200MHz for n257 as general spurious emission requirement is sufficient.

Observation 2: The existing EESS protection of 1dBm/200MHz for n257 as general spurious emission requirement is sufficient.

Based on above observations, we propose,

Proposal 1: Not to update the general spurious emission requirement for EESS protection for n257.

- Conclusion

This contribution provides analysis on impacts of EESS protection for n257 and n258 on UE specification, and the following observations are concluded as follows:

Observation 1: The spurious emission band UE co-existence for n257 in sub-clause 6.5.3.1 of TS 38.101-2 and spurious emission band UE co-existence for UL CA for n257 in sub-clause 6.5A.3.1 of TS 38.101-2 is general requirement.

Observation 2: The existing EESS protection of 1dBm/200MHz for n257 as general spurious emission requirement is sufficient.

Proposal 1: Not to update the general spurious emission requirement for EESS protection for n257.

- References

- R4-2417083, WF on UE RF requirements for EESS protection, Huawei, China Unicom, RAN4#112bis

- R4-2010614, CR for introduction of EESS protection for n257 into general spurious emission, RAN4#96-e

- RP-242371, New WID on mmWave UE spurious emssion, China Unicom, Deutsche Telekom, Vodafone, CATT, Huawei, HiSilicon, OPPO, Qualcomm, Xiaomi, ZTE, RAN#105