3GPP TSG-RAN WG4 Meeting #113	                                                                                            R4-2417845

Orlando, US, 18th – 22nd November, 2024

Source: 	CATT

Title: 	Discussion on SBFD system parameters

Agenda item:	7.20.1

Document for:	Discussion

- Introduction

This contribution provides further discussion on the open issues for SBFD parameters according to the WF [1] in last RAN4 meeting. Channel BW, subband configuration and guard band size are discussed.

- Discussion

  - Channel bandwidth

In last RAN4 meeting, the following agreement is made,

Issue 1-1-1: Minimum channel bandwidth for SBFD (X MHz)

Agreement:

- Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle:

  - The supported channel bandwidth(s) is declared by gNB vendors

  - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104

  - The supported channel bandwidth shall be no less than X MHz

    - X = 40 as the default value

      - For band n39, X = 20MHz is used instead.

For the agreements on the 40 MHz CBW, it seems it applies to both FR1 and FR2. In the current specification, the minimum CBW for FR2 is 50 MHz, so all of the FR2 CBW can use SBFD operation if the above agreement is followed. However, one of the RAN1 working assumptions in RAN1#118b should be noticed that SSB configuration in SBFD subbands is mentioned. The CBW restriction may need to be revisited or additional side condition may be needed.

In RAN1#118bis, the following working assumption is made,

Working Assumption

For SSB symbols configured with SBFD subbands,

- Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

  - Note: The SSB block is assumed to be within DL subband

With the above working agreement, DL subband bandwidth should be larger than (SSB BW + sync raster step size).

The justification can be found in TR 38.817-2, the equation is copied as following,

ΔFSC,Raster ≤ BWConfig – BWPBCH + ΔFCH,Raster	(4.3.1.1-1)

The explanation figure is also copied in Figure 1.

Figure 1: Figure copied from TR 38.817-2

The SSB BW for different SCS can be calculated in Table 1.

Table 1: SSB BW for different SCS

|  | FR1 |  | FR2-1 |  |
| --- | --- | --- | --- | --- |
| SSB SCS (Hz) | 15k | 30k | 120k | 240k |
| SSB BW (MHz) | 3.6 | 7.2 | 28.8 | 57.2 |

For FR1, if 3MHz CBW is not considered, the sync raster step size is 50kHz for <3GHz, 1.44 MHz for >3GHz, respectively. For FR2, sync raster step size is 17.28MHz. So for FR1, DL SB CBW should be ≥ 5MHz for < 3GHz with 15kHz SSB SCS, ≥ 10 MHz for > 3GHz with 15kHz SSB SCS and for 30kHz SSB SCS of all FR1 bands. For FR2, DL SB CBW should be ≥ 50 MHz for 120 kHz SSB SCS and ≥ 80MHz for 240 kHz SSB SCS.

Considering the DL SB CBW restriction, there may be no issues for the whole CBW for FR1 because there are sufficient single carrier CBW choices if UL SB reuses the current CBW. For FR2, only 4 CBWs are supported, i.e. 50, 100, 200 and 400MH. Considering DL SB CBW should be ≥50MHz, FR2 minimum CBW to support SBFD is 100MHz.

Proposal 1: For SBFD, at least one DL SB BW should be larger than (SSB BW + sync raster step size).

Proposal 2: FR2 minimum CBW to support SBFD is 100MHz.

  - Subband configuration

After last RAN4 meeting discussion, the sub-band configuration can be further discussed according to option 2b with the following remaining issues [1],

  - For DUD case, the UL subband should be placed in the middle of channel bandwidth

  - FFS the number of DL subband sizes to be declared and tested

    - Alt-1: For FR1 WA BS, one DL subband size is declared to support SBFD and tested, operation with other DL subband sizes than the tested one is not allowed.

      - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS.

    - Alt-2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested

      - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS.

  - Restriction on UL subband size based on the specific channel bandwidth

    - FFS to specify a range of ratio for UL subband size over channel bandwidth.

    - FFS the restriction is different for DU and DUD

    - FFS the restriction is applied for FR2 case.

For the UL SB size open issues, the ratio of UL SB size may not be very low considering the real deployment. From system performance perspective, if the ratio is too low, it’s not necessary to use SBFD operation when the implementation cost is taken into account. But it’s not clear how to decide the exact ratio from technical aspect. In the SI study, the lowest ratio for UL SB is 20% among all of the cases. If a range of ratio for UL subband size over channel bandwidth is set, minimum 20% can be considered. However, as now we agreed that it’s based on declaration, we don’t think it’s necessary to define a UL SB size ratio.

Proposal 3: No restriction on the UL SB size ratio in the core requirements.

There’s one issue which may be overlooked that UL transmission bandwidth reuses current non-SBFD values in option 2b, but no clarification that if the center frequency for the transmission bandwidth is fixed or can be flexible. If it’s fixed, the straightforward way is to reuse the center frequency of the CBW, i.e. the center of the UL SB is the center of the UL transmission bandwidth. In this case, DU configuration guard band size at the CBW edge of U side and CBW is the same with the value of single carrier in current spec. For DUD configuration, the guard band flexibility still exist because U is not at the edge of the whole CBW. Some thinking is needed that if the guard band size flexibility brings benefit. RAN1 design supports this flexibility. According to the following RAN1 working assumption in RAN1#118,

RAN1 Working Assumption in RAN1#118

For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

- One or two SBFD DL subbands can be configured

RAN1 design can support all of the flexibility including zero GB, i.e. all of the GB between DL SB and UL SB can be used as DL if UL transmission BW is fixed first. Considering the implementation capability may be different for different vendors, keeping the guard band size flexibility can bring benefit to operators that spectrum can be used more efficiently. So we propose the UL SB center frequency can be flexible and up to the implementation in RAN4 specification.

Proposal 4: Flexibility of the UL SB transmission center frequency should be allowed that it may not be the center frequency of UL SB.

How to do the conformance test may need more thinking, one choice can be that it is based on the declaration.

  - Guard band size

From the analysis in 2.2, the guard band size can be flexible, i.e. option 3a in [2]

    - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guandband:

Proposal 5: No restriction on maximum/minimum number of RBs for guard band in RAN4 specification.

- Conclusion

This contribution provides our further analysis for SBFD general issues, we have the following observation and proposals.

Proposal 1: For SBFD, at least one DL SB BW should be larger than (SSB BW + sync raster step size).

Proposal 2: FR2 minimum CBW to support SBFD is 100MHz.

Proposal 3: No restriction on the UL SB size ratio in the core requirements.

Proposal 4: Flexibility of the UL SB transmission center frequency should be allowed that it may not be the center frequency of UL SB.

Proposal 5: No restriction on maximum/minimum number of RBs for guard band in RAN4 specification.

- References

[1] R4-2416584, “Way Forward for [112bis][306] NR_duplex_evo_General”, Samsung, RAN4#112b

[2] R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General”, Samsung, RAN4#112