3GPP TSG-RAN WG4 Meeting # 113                                                            R4-2417853

Orlando, US, 18th – 22nd November, 2024

Title: 	Discussion on modification of existing Rx requirements for SBFD operation

Source: 	CATT

Agenda item:	7.20.2.3

Document for:	Discussion

- Background

WF [1] of RAN4#112bis concluded that further studies are needed for some Rx RF requirements. This contribution provides our further analysis of the requirements.

- Impact on existing BS Rx RF requirement

  - OTA sensitivity

About OTA sensitivity degradation value caused by SI, according to the SI feasibility analysis results of each company in [1], 1.0dB is a more appropriate value.

Proposal 1: 1.0 dB degradation is agreed for OTA sensitivity requirement.

  - Dynamic range requirement

In [3], dynamic range simulation considered both uplink signal and gNB-to-gNB CLI signal.

| Scenario | Frequency | Aggressor -> Victim | Accuracy | IoT (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 100% grid-shift |  |  | 10% grid-shift |  |  |
|  |  |  |  | 95% CDF | 99% CDF | 100% CDF | 95% CDF | 99% CDF | 100% CDF |
| 1 | FR1 | Urban Macro -> Urban Macro | 0dB | 7.9 | 15.5 | 21.0 | 10.6 | 16.8 | 24.0 |
|  |  |  | ±9dB | 9.3 | 15.9 | 22.8 | 11.9 | 17.4 | 23.5 |
| 3 | FR1 | Indoor -> Indoor | 0dB | 13.3 | 15.3 | 16.7 | —— | —— | —— |
|  |  |  | ±9dB | 15.0 | 18.2 | 21.0 | —— | —— | —— |
| 5 | FR1 | Micro -> Micro | 0dB | 3.5 | 10.0 | 15.5 | 6.5 | 13.0 | 18.5 |
|  |  |  | ±9dB | 5.1 | 12.0 | 21.5 | 8.1 | 15.1 | 24.6 |
| 6 | FR2-1 | Urban Macro -> Urban Macro | 0dB | 4.0 | 8.9 | 8.9 | 6.3 | 11.5 | 12.2 |
|  |  |  | ±9dB | 5.2 | 17.6 | 19.6 | 7.5 | 21.5 | 21.8 |
| 9 | FR2-1 | Indoor -> Indoor | 0dB | -1.4 | 0.5 | 1.6 | —— | —— | —— |
|  |  |  | ±9dB | 0.2 | 2.6 | 7.6 | —— | —— | —— |

It is necessary to decide CFD ratio and grid shift first, 95% CDF was used for LTE system, and we can use data corresponding to 95% CDF and 100% grid shift.

For both 100% grid shift and 10% grid shift, the IoTs at 99% CDF of scenario1, scenario 3 and scenario 5 are all less than 20dB, and the IoTs of scenario 6 are much higher than existing requirements, IoT of scenario 9 is 2.6dB, and which is close to existing requirement. So scenario 1, scenario 3, scenario 5 and scenario 9 can reuse existing requirements.

For both 100% grid shift and 10% grid shift, The IoTs at 99% CDF are close to 20dB, and 20dB IoTs are achievable; The IoTs at 95% CDF of scenario 6 are close to existing requirement, and needn’t to be defined.

Observation 1: For both 100% grid shift and 10% grid shift of FR2-1 macro scenario:

- Option 1: The IoTs at 95% CDF and 100% grid shift is 5.2dB, and at 95% CDF and 10% grid shift is 7.5dB.

- Option 2: The IoTs at 99% CDF are close to 20dB, and 20dB IoT is achievable.

Proposal 2: We propose to specify dynamic range requirements based on data at 95% CDF and 100% grid shift.

Proposal 3: The dynamic range requirements of FR1 can reuse existing requirement.

Proposal 4: The dynamic range requirements of FR2-1 can reuse existing requirement.

  - In-band blocking

In WF [2], in-band blocking requirements need to be further studied.

Agreement:

- For FR1 local area SBFD BS, re-use existing FR1 local area BS in-band blocking power requirements.

WF:

- FFS on the followings:

  - For FR1 WA BS, RAN4 need to discuss whether a unified in-band blocking requirement is defined?

    - Option 1: a unified blocking requirement to consider both scenarios: coexistence with legacy TDD system in adjacent channel, and coexistence with new SBFD system in adjacent channel.

    - Option 2: separate requirements are defined,

      - One set of requirements for coexisting with legacy TDD system in adjacent channel

      - One set of requirements for coexisting with new SBFD system in adjacent channel

  - For FR1 WA BS co-existence study, FFS grid-shift values should be considered.

    - Option 1: 10%

    - Option 2: 20%

    - Option 3: 50%

    - Option 4: 100%

  - In-band blocking requirement for FR1 MR SBFD BS

  - In-band blocking requirement for FR2-1 WA and MR SBFD BS

  - For FR2 local area SBFD BS, re-use existing FR2 local area BS in-band blocking power requirements.

For FR1 WA BS，whether the SBFD is configured as {DU} or {DUD}, the in-band interference signal level of the victim SBFD coexisting with the legacy TDD system of the adjacent channel can cover the interference signal level coexisting with the new SBFD system of the adjacent channel, so we propose to define a unified in-band blocking requirement.

Proposal 5: For FR1 WA BS, unified in-band blocking requirement can be considered for two scenarios if no implementation issue identified.

According to [3], the simulation results of in-band blocking for FR1 and FR2-1 are shown in table 1 and table 2, and the interference are from TDD DL to SBFD UL.

Table 1: SBFD conducted in-band blocking simulation results (FR1, TDD DL to SBFD UL subband)

| Scenario | Frequency | Aggressor -> Victim | Conducted in-band blocking simulation results (dBm) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 99% CDF (100% grid-shift) | 99% CDF (10% grid-shift) |
| 1 | FR1 | Urban Macro -> Urban Macro | -25.4 | 0.3 |
| 5 | FR1 | Micro -> Micro | -32.1 | -17.2 |

Table 2: SBFD OTA in-band blocking simulation results (FR2-1, TDD DL to SBFD UL subband)

| Scenario | Frequency | Aggressor -> Victim | in-band blocking simulation results (dBm) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 99% CDF (100% grid-shift) | 99% CDF (10% grid-shift) |
| 6 | FR2-1 | Urban Macro -> Urban Macro | -51.7 | -39.8 |
| 9 | FR2-1 | Indoor -> Indoor | -56.5 | —— |

For FR1 urban macro scenario, the in-band blocking signals power levels are much higher than the existing IBB requirement.

For FR1 micro scenario, the in-band blocking signals power level of 100% grid shift is close to the existing IBB requirement, but the power level of 10% grid shift is much higher than the existing IBB requirement.

For FR2-1 urban macro scenario, the in-band blocking signals power level is close to the existing IBB requirement, and can reuse the existing requirements.

For FR2-1 indoor scenario, the in-band blocking signals power level is lower than the existing IBB requirement, and can reuse the existing requirements.

Observation 2: The simulation result of IBB for FR1 macro scenario is much higher than existing represent.

Observation 3: In the FR1 micro-scenario, the simulation result of IBB is higher than the existing requirements when the grid shift is 10%, and the simulation result of IBB is close to the existing requirements when the grid shift is 100%.

Observation 4: The simulation results of IBB for FR2-1 urban macro and indoor scenarios are lower than the existing requirements.

Proposal 6: We propose to specify in-band blocking requirements based on data at 100% grid shift.

Proposal 7: The IBB requirements of FR1 micro scenario can reuse existing requirements.

Proposal 8: The IBB requirements of FR2-1 can reuse existing requirements.

  - Rx intermodulation

In WF [2], the necessity of new RX intermodulation requirement with one interfering signal needs to be further studied.

WF:

- FFS on the followings:

  - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement.

  - Option 2: RAN4 to study further RX intermodulation scenario with 1 interfering signal.

For the issue of if single interfering signal scenario is covered by IBB requirement, it depends on the frequency position of the single interfering signal. For some SBFD DL and UL spectrum configurations, the single interference signal scenario is implicitly captured by the SBFD RX blocking requirement, even if the frequency range of the single interference signal is from out-of-band to in-band rather than only in-band.

Observation 5: The single interference signal scenario is implicitly captured by the SBFD RX blocking requirement.

Proposal 9： The single interference signal scenario isn’t needed.

WF:

- FFS on the following:

  - Option 1:

    - For the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal.

    - For receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver.

  - Option 2: New IMD requirement with 2 interfering signal scenario is not needed.

  - Option 3: RAN4 to study further

For the power level of the receiver intermodulation interference signal, whether BS2BS CLI needs to be considered depends on the power level values of the interference, and the power level can be simulated. The in-band blocking of the SBFD slot has been simulated by RAN4, and the interference signals are from BS of frequency adjacent system, these interference signals can be considered by legacy Rx intermodulation, and new RX intermodulation requirement with two interfering signals is not necessary.

Observation 6: Interference signals from BS of frequency adjacent system can be considered by legacy Rx intermodulation requirement.

Proposal 10 New RX intermodulation requirement with two interfering signals is not necessary.

- Summary

In this contribution, we have the following proposals and observations.

Proposal 1: 1.0 dB degradation is agreed for OTA sensitivity requirement.

Observation 1: For both 100% grid shift and 10% grid shift of FR2-1 macro scenario:

-  Option 1: The IoTs at 95% CDF and 100% grid shift is 5.2dB, and at 95% CDF and 10% grid shift is 7.5dB.

-	  Option 2: The IoTs at 99% CDF are close to 20dB, and 20dB IoT is achievable.

Proposal 2: We propose to specify dynamic range requirements based on data at 95% CDF and 100% grid shift.

Proposal 3: The dynamic range requirements of FR1 can reuse existing requirement.

Proposal 4: The dynamic range requirements of FR2-1 can reuse existing requirement.

Proposal 5: For FR1 WA BS, unified in-band blocking requirement can be considered for two scenarios if no implementation issue identified.

Observation 2: The simulation result of IBB for FR1 macro scenario is much higher than existing represent.

Observation 3: In the FR1 micro-scenario, the simulation result of IBB is higher than the existing requirements when the grid shift is 10%, and the simulation result of IBB is close to the existing requirements when the grid shift is 100%.

Observation 4: The simulation results of IBB for FR2-1 urban macro and indoor scenarios are lower than the existing requirements.

Proposal 6: We propose to specify in-band blocking requirements based on data at 100% grid shift.

Proposal 7: The IBB requirements of FR1 micro scenario can reuse existing requirements.

Proposal 8: The IBB requirements of FR2-1 can reuse existing requirements.

Observation 5: The single interference signal scenario is implicitly captured by the SBFD RX blocking requirement.

Proposal 9： The single interference signal scenario isn’t needed.

Observation 6: Interference signals from BS of frequency adjacent system can be considered by legacy Rx intermodulation requirement.

Proposal 10 New RX intermodulation requirement with two interfering signals is not necessary.

- Reference

[1] TR 38.858

[2] R4-2416585, “Way Forward for [112bis][307] NR_duplex_evo_BSRF”, Huawei, HiSilicon, RAN4#112bis

[3] R4-2415106, “SBFD in-band blocking and dynamic range simulation results”, CATT, RAN4#112bis